# Design and Development of a Solar-powered Water Purifier Prototype

ABSTRACT

Access to safe drinking water remains a significant public health concern, particularly in educational institutions where students rely on shared water sources daily. This study aimed to design and develop a solar-powered water purifier prototype and evaluate its effectiveness in improving selected water quality indicators at Mindanao State University – Maigo College of Education, Science, and Technology (MSU–MCEST). An experimental-developmental research design was employed. Water samples were collected from selected campus locations, specifically the Senior High School (SHS) Building and the Administration Building (ADMIN). The collected samples were tested before and after treatment using the developed prototype. Water quality assessment focused on selected heavy metal indicators and microbiological quality through the use of heavy metal strips and bacteria testing kits.

The findings revealed that cadmium (Cd), which was detected in several pre-treatment water samples, was no longer present after treatment, indicating that the prototype contributed to reducing selected harmful contaminants. However, zinc (Zn) remained detectable in one SHS post-treatment sample, suggesting that the purification process had limited effectiveness in reducing certain heavy metal indicators. In terms of microbiological quality, all pre-treatment water samples tested positive for bacterial contamination, while all post-treatment samples tested negative after passing through the solar-powered water purifier prototype. These results indicate that the prototype was effective in reducing bacterial contamination in the tested water samples.

Overall, the study concluded that the solar-powered water purifier prototype was effective in improving microbiological quality and partially effective in improving selected chemical quality indicators. The prototype demonstrates potential as a practical, sustainable, and environmentally friendly solution for improving water safety within educational institutions.

Keywords: solar-powered water purifier, water purification, bacterial contamination, solar energy, water safety

INTRODUCTION

A ccess to safe drinking water is essential for health, learning, and daily functioning. However, water quality in shared campus water sources is not always guaranteed. According to the World Health Organization (WHO, 2023), contaminated drinking water remains one of the major sources of preventable disease, especially where monitoring and maintenance are inconsistent. The United Nations (2025) further emphasized that access to safe water and sanitation remains a global priority under Sustainable Development Goal 6. Prüss-Ustün et al. (2019) also reported that unsafe water, sanitation, and hygiene remain major contributors to preventable diseases worldwide.

In the Philippines, studies have detected microbial contaminants such as coliform bacteria and Escherichia coli in community water sources and refilling stations, indicating that water safety cannot always be assumed (Cambarihan et al., 2022; Cuenca et al., 2021). Healing Waters International (2024) explained that waterborne diseases can negatively affect students’ learning because illness may lead to poor attendance and reduced academic focus. Tetteh and Tettey (2025) also highlighted that unsafe drinking water negatively affects public health and productivity. UNICEF (2019) emphasized the importance of clean drinking water and sanitation in schools to protect student health and learning outcomes.

Solar-powered water purification technologies have emerged as sustainable and environmentally friendly approaches to improving water quality. Methods such as Solar Water Disinfection (SODIS), ultraviolet sterilization, and solar distillation have been shown to effectively reduce microbial contamination while utilizing renewable energy (García-Gil et al., 2021; Ahmad et al., 2024). Bharathi et al. (2021) explained that solar-powered purification systems equipped with ultraviolet sterilization technologies effectively improve water safety while remaining energy efficient and environmentally sustainable. Dey et al. (2024) emphasized that integrating solar energy into water purification systems minimizes electricity dependence while improving accessibility to clean water.

Research by Hendrickson et al. (2020) demonstrated that decentralized solar-powered purification systems significantly reduced bacterial contamination in rural communities. Similarly, Ahmed et al. (2024) highlighted that integrating Internet-of-Things monitoring systems with solar-powered purification technologies improved operational efficiency and water quality monitoring. Gürsu (2024) also reported that solar purification systems are useful in underserved regions because they support potable water production in areas with limited conventional resources. Kim et al. (2022) explained that ultraviolet sterilization technologies effectively reduce bacterial contamination in drinking water purification systems.

In the Philippine setting, solar-powered water treatment systems have demonstrated effectiveness and community acceptance in improving water quality, particularly in areas with limited access to electricity (Yu Jeco et al., 2019). The Department of Science and Technology (DOST, 2021) also promoted research and development programs on sustainable water resources as a major priority. The International Renewable Energy Agency (IRENA, 2023) further noted that renewable energy technologies contribute to long-term environmental sustainability and reduced energy dependence.

Theoretical foundations related to sustainability and public health further support the development of renewable-energy-based water systems. Sustainable Development Theory emphasizes balancing environmental protection, social well-being, and future resource needs (Brundtland Commission, 1987). The United Nations Sustainable Development Goals, particularly SDG 3, SDG 6, and SDG 7, further support efforts toward clean water, improved health, and renewable energy integration (United Nations, 2015). Rosenstock (1974) further explained that health-related practices are influenced by individuals’ awareness and perception of health risks.

Despite the growing interest in solar-powered purification systems, limited studies have focused on their application within academic institutions where students rely heavily on communal water sources. These findings highlight the need for practical and sustainable solutions that can improve water quality within educational institutions.

METHODOLOGY

This study employed an experimental-developmental research design. The developmental aspect focused on designing and constructing a functional solar-powered water purifier prototype, while the experimental component evaluated the effectiveness of the prototype in improving selected water quality indicators.

The primary instrument used in the study was the solar-powered water purifier prototype developed by the researchers. The prototype consisted of a solar panel, charge controller, solar battery, power inverter, and a water purifier equipped with ultraviolet sterilization.

Water samples were collected from selected campus locations within MSU–MCEST using sterile containers. Samples were obtained before treatment to establish baseline water quality conditions. The collected samples were tested using bacteria testing kits and heavy metal test strips to determine initial water quality conditions. Afterwards, the water samples were processed through the solar-powered water purifier prototype.

Post-treatment water samples were then collected and tested using the same procedures to determine whether improvements in water quality occurred after purification. Descriptive statistical methods, including means and standard deviations, were utilized to summarize water quality results before and after treatment.

RESULTS AND DISCUSSION

The solar-powered water purifier prototype integrated solar energy technology with a water purification system. Solar energy served as the primary power source of the system. The prototype consisted of a solar panel, charge controller, solar battery, power inverter, and a water purification unit equipped with ultraviolet sterilization technology.

The purification system was designed to improve water quality by reducing bacterial contamination and selected heavy metal indicators. The use of solar energy allowed the system to operate sustainably while minimizing electricity consumption.

Table 1. Materials and Estimated Expenses for the Solar-Powered Water Purifier Prototype

| Materials/Components | Function |
| --- | --- |
| Water Purifier with UV Sterilizer | Purifies and disinfects water using ultraviolet technology |
| Solar Battery (12V) | Stores electrical energy produced by the solar panel |
| Solar Panel with Charge Controller and Power Inverter | Converts solar energy into electrical energy and regulates power supply |
| Bacteria Testing Kit | Detects bacterial contamination in water samples |
| Heavy Metal Test Strips | Detects selected heavy metal indicators in water |

The materials used in the prototype were selected based on affordability, availability, and suitability for a small-scale solar-powered purification system.

Table 2. Pre-Treatment and Post-Treatment Heavy Metal Indicator Results

| Sample Location | Cadmium Before Treatment | Cadmium After Treatment | Zinc Before Treatment | Zinc After Treatment |
| --- | --- | --- | --- | --- |
| SHS Building | Detected | Not Detected | Detected | Detected |
| Administration Building | Detected | Not Detected | Detected | Not Detected |

Table 3. Pre-Treatment and Post-Treatment Bacterial Results

| Sample Location | Before Treatment | After Treatment |
| --- | --- | --- |
| SHS Building | Positive | Negative |
| Administration Building | Positive | Negative |

The findings revealed significant improvements in microbiological water quality after treatment using the solar-powered water purifier prototype. All pre-treatment water samples tested positive for bacterial contamination, while all post-treatment samples tested negative after purification. Cadmium detected in several pre-treatment samples was no longer present after treatment. However, zinc remained detectable in one post-treatment sample, indicating that the purification process had limited effectiveness in reducing certain heavy metal indicators.

These findings support previous studies emphasizing the effectiveness of ultraviolet sterilization and solar-powered purification systems in improving microbiological water quality (Kim et al., 2022; Bharathi et al., 2021). García-Gil et al. (2021) also explained that solar-powered disinfection systems effectively eliminate harmful microorganisms while utilizing renewable energy. The persistence of zinc in one sample suggests that further enhancement of the filtration system may still be necessary to improve overall purification performance.

Overall, the study demonstrates that combining solar energy with water purification technologies can provide a practical and environmentally sustainable solution for improving water safety in educational institutions.

CONCLUSION

Based on the findings of the study, the solar-powered water purifier prototype was effective in improving microbiological water quality and partially effective in improving selected chemical water quality indicators. The prototype successfully reduced bacterial contamination in all tested water samples and contributed to the reduction of selected harmful contaminants such as cadmium.

However, certain heavy metal indicators such as zinc remained detectable in some post-treatment samples, indicating that further enhancement of the filtration system is still necessary. Overall, the study demonstrated that solar-powered purification systems have strong potential as sustainable and environmentally friendly technologies for improving water safety within educational institutions.

ACKNOWLEDGEMENT

The researchers would like to express their deepest gratitude to everyone who contributed to the successful completion of this study entitled “Design and Development of a Solar-Powered Water Purifier Prototype.” Above all, the researchers thank the Almighty God for His wisdom, guidance, and strength throughout the conduct of the research.

The researchers sincerely extend their appreciation to their research adviser, Sir Oscar Romero Jr., for his invaluable guidance, encouragement, and professional support in improving the quality of the study.

REFERENCES

- Ahmad, A., Khan, Z. M., & Farid, H. U. (2024). A potential solution for clean water supply: Multi-effect solar still and water quality analysis. Water Supply, 24(2), 329–340. https://doi.org/10.2166/ws.2024.003

- Ahmed, R., Ahamed, E., & Islam, F. (2024). IoT-based solar-powered water purification systems. American International University-Bangladesh. https://dspace.aiub.edu/jspui/bitstream/123456789/2130/1/Capstone%20Project%20Project%20Book%202023.2.28.pdf

- Bharathi, M. L., Rhani Fathima, K. B., Kanagaraj, V., Santhiya, M., & Saranya, R. (2021). IoT enabled solar powered water purification system for rural areas. https://doi.org/10.1109/ICMNWC52512.2021.9688336

- Brundtland Commission. (1987). Our Common Future. https://www.brundtland.co.za/wp-content/uploads/2022/08/Brundtland-Report-1987-Our-Common-Future.pdf

- Cambarihan, S. D., Patricio, E. R. P., & Lumogdang, L. P. (2022). Detection and enumeration of coliforms in drinking water sources in the selected barangay in Santa Maria, Davao Occidental, Philippines. https://doi.org/10.9734/ajob/2022/v15i130227

- Cuenca, M., Dizon, A., Manalo, J., Reyes, K., & Tolentino, D. (2021). Assessment of water refilling stations. https://thelawbrigade.com/wp-content/uploads/2021/03/AJMRR_Marish-Cuenca-4-others.pdf

- Department of Science and Technology. (2021). DOST makes R&D programs on sustainable water resources a major priority. https://www.dost.gov.ph/knowledge-resources/news/72-2021-news/2373-dost-makes-r-d-programs-on-sustainable-water-resources-a-major-priority.html

- Dey, T., Bhattacharya, S., Paul, A. K., & Ghosh, S. (2024). Solar-powered water purification: A sustainable solution for remote communities. https://www.researchgate.net/publication/382563128_Solar-Powered_Water_Purification_A_Sustainable_Solution_for_Remote_Communities

- García-Gil, Á., García-Muñoz, R. A., McGuigan, K. G., & Marugán, J. (2021). Solar water disinfection to produce safe drinking water: A review of parameters, enhancements, and modelling approaches to make SODIS faster and safer. Molecules, 26(11), 3431. https://www.mdpi.com/1420-3049/26/11/3431

- Gürsu, H. (2024). Assessment of theoretical and test performance considerations of concentrated solar water purification system “Parabosol” in underserved regions. https://doi.org/10.3390/su16156611

- Hendrickson, C., et al. (2020). Decentralized solar-powered drinking water ozonation in Western Kenya: An evaluation of disinfection efficacy. Gates Open Research, 4, 56. https://gatesopenresearch.org/articles/4-56

- International Renewable Energy Agency. (2023). Renewable energy – powering a safer and prosperous future. https://www.irena.org/-/media/Files/IRENA/Agency/Publication/2023/Jul/IRENA_Renewable_energy_statistics_2023.pdf

- Kim, H. J., Yoon, H. W., Lee, M. A., Kim, Y. H., & Lee, C. J. (2022). Impact of UV-C irradiation on bacterial disinfection in a drinking water purification system. https://doi.org/10.4014/jmb.2211.11027

- Tetteh, A. A., & Tettey, E. (2025). Water pollution and public health. In R. C. Brears (Ed.), The Palgrave Handbook of Ecosystems and Wellbeing. Palgrave Macmillan. https://doi.org/10.1007/978-3-031-24952-5_21-1

- Prüss-Ustün, A., et al. (2019). Burden of disease from inadequate water, sanitation and hygiene for selected adverse health outcomes. https://doi.org/10.1016/j.ijheh.2019.05.004

- Rosenstock, I. M. (1974). The Health Belief Model and preventive health behavior. https://doi.org/10.1177/109019817400200405

- Healing Waters International. (2024, March 12). Common water-borne diseases & their effect on learning. https://healingwaters.org/common-water-borne-diseases-their-effect-on-learning/

- UNICEF. (2019). Water, sanitation, and hygiene in schools. https://www.unicef.org/eap/sites/unicef.org.eap/files/2020-05/EAPRO%20WASH%20Results%20Report%202019_FINAL.pdf

- United Nations. (2015). The 17 goals. https://sdgs.un.org/goals

- United Nations. (2025). Goal 6: Ensure access to water and sanitation for all. https://www.un.org/sustainabledevelopment/water-and-sanitation/

- World Health Organization. (2023, September 13). Drinking-water. https://www.who.int/news-room/fact-sheets/detail/drinking-water

- Yu Jeco, B. M. F., Larroder, A. C., & Oguma, K. (2019). Technosocial feasibility analysis of solar-powered UV-LED water treatment system in a remote island of Guimaras, Philippines. https://doi.org/10.1117/1.JPE.9.043105