Improve of Sugar content and purity by adding  Beet juice to Cane juice

Prof.Yasir A. Mohamed1 ,Dr.Elrafie A. A.Allah1,Omyma Altayib Haj Almakki1

1.Department of Chemical Engineering, Faculty of Engineering, University

of ElImam ElMahdi, Kosti, Sudan

Yasir13000@yahoo.com,rafieah@gmail.com,ssa@mahdi.edu.sd

Abstract

The study aimed to improve the quality of Cane juice by adding sugar Beet juice and determining the best addition ratio of sugar Beet juice to sugar Cane juice. The experiment is conducted  by  mixing beet juice with the cane juice by different ratio, begin experiences by ratio %5 of beet sugar juice and 95% of sugar cane juice until we reach the ratio equal of Beet juice and cane juice and to know the change that happens to Cane juice after mixing. where the calculation were carried out by the formula for calculating the concentration of mixtures.

This study and calculation improves quality parameters of the mixture of  Beet juices and cane juices Positively, there was an increase in pol from 10.5 to 21.5, Brix increased from 13.4 to 25.1, the purity of the blended juice increased significantly where before the addition of sugar Beet juice was 78.3 and after addition of sugar Beet juice recorded purity 88.65, the pH value increased from 5.2-6.65 and reducing sugar value decreased from 1 to 0.1 improving the quality standard is positively reflected in improving the production processes of purification, evaporation, crystallization and in the amount of sugar produced. Each increased percentage added Beet juice note the improvement in the properties of mixed juice, especially in the degree of purity, which is also reflected in the amount of sugar produced. Every time the percentage of sugar beet juice added to sugar cane juice has increased, notice an improvement in the mixed juice, especially in the degree of purity, which reflects too on increasing the amount of sugar produce.Therefore recommendation introducing sugar Beet juice to sugar factories in Sudan to improve the quality of Cane juice and conduct more studies on the effect of adding sugar Beet juice to sugar Beet juice.

Key words: Improve ,Beet juice ,Cane juice ,Sugar content ,Purity

1.Introduction

Sugar production was considered as an important chemical industry the raw   material for this processing can be obtained from cane and beet.Beet sugar plant the Mediterranean basin and bred and developed as an alternative to cane in sugar production due to lack of success of the sugar cane and cold regions and thus the difficulty of obtaining sugar and in recent decades moved sugar beet south to the sub-tropics where produced in Pakistan, Egypt and morocco large amounts but in can be said that the production of sugar from beet in Egypt increased its production of sugar cane in recent years.And more recently has been producing some varieties of sugar beet suited to tropical conditions and named tropical varieties and conducted research in Sudan for a period of more than ten years the results indicated the possibility of sugar beet cultivation in Sudan with great success in the winter semester in Sudan.Although the process that the sugar industry of the cane is similar in its large operation production of the beet ,variation in one of the stages and some of the properties of the final product to make the industrial  disappear from the cane to the beet , this difference led to the separation of industry every type of the modern ,the appeared the idea was to mix the two types any operation mixing the beets with the cane and is the idea aimed at the development of the industry and take advantage the properties to increase the quality, In previous years it has been conducting this experiment in the Sudan in Aljenad factory by mixed ratio 15%.The objective of this experiment  mixing beet juice with the cane juice by different ratio, look experiences by ratio 5% of beet sugar juice and 95% of sugar cane juice until we reach the ratio equal of beet juice and cane juice and to know the change that happens to cane juice after mixing.

This study and calculation improves quality parameters of the mixture of beet juices and cane juices positively, there was an increase in pol and purity of the mixture, which is reflected in the improvement of the production processes of purification, evaporation, crystallization and in the amount of sugar produced.

2.Material and Methods

Analysis of samples before and after the completion of the experiment include analysis of the elements below:

1. Cane juice

2.Beet juice

3.  Mixed juice

Take reading for  Polarization-pol and Purity-Pty

Conduct analyses of the juice of beet and cane juice before mixed

Calculation equation of mixer concentration

Emix=Ec*Xc + EB*XB                                                                                 (1)

Ec     parameter value of cane,

XC   ratio addition of cane juice

EB    parameter value of Beet  ,

XB   ratio addition of Beet juice

Experiments take place by mixing beet juice with the juice of the sugar cane by different ratio, look experiences by ratio 5% of beet sugar juice and 95% of sugar cane juice until we reach the ratio equal of beet juice and cane juice and to know the change that happens to juice cane after mixing.In the following table shows the added ratio of beet juice to cane juice.

Table (1): Added ratio of Beet juice to cane juice

| Ratio beet juice % | Ratio cane juice % |
| --- | --- |
| 5 | 95 |
| 10 | 90 |
| 15 | 85 |
| 20 | 80 |
| 25 | 75 |
| 30 | 70 |
| 35 | 65 |
| 40 | 60 |
| 45 | 55 |
| 50 | 50 |

3.Results and Discussions

# Samples were taken over several months during the production season October, November, January, February and April.The months were chosen based on the beginning of the season, end of the season and the middle of the season.

Table (2): determination of sugar content in mixed juice after adding (5-50) % of beet juices to cane juices during the five months production

| Month | adding ratio of Beet juice |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0% | 5% | 10% | 15% | 20% | 25% | 30% | 35% | 40% | 45% | 50% |
| October | 12.8 | 13.2 | 13.6 | 14 | 14.5 | 14.9 | 15.3 | 15.8 | 16.2 | 16.6 | 17.1 |
| November | 13.7 | 14.1 | 14.5 | 15 | 15.4 | 15.9 | 16.3 | 16.8 | 17.2 | 17.7 | 18.1 |
| January | 17.2 | 17.6 | 18 | 18.4 | 18.9 | 19.3 | 19.7 | 20.1 | 20.6 | 21 | 21.4 |
| February | 16.2 | 16.7 | 17.2 | 17.7 | 18.2 | 18.7 | 19.2 | 19.7 | 20.2 | 20.7 | 21.2 |
| April | 10.5 | 10.8 | 11.1 | 11.5 | 11.8 | 12.2 | 12.5 | 12.9 | 13.2 | 13.5 | 13.9 |

Table 2 show a noticeable increase in the percentage of sugars content during the five months mentioned above Also noticed that, at the beginning of the production season in October, the value of sugars content before addition of beet juice was 12.8, and after the addition of beet juice the value of sugars content increased to 17.1. In January which is considered as the one of the best production months, the value of sugars content before the adding of Beet juice was 17.2, this meaning the value of sugars content rise to the point which  it became equivalent to the best value of sugars in the best month.

Fig (1): Added ratio of Beet juice with pol mix graph

Table (3): Study the effect the adding a ratio (5-50) % of beet juices to cane juices on purity in a

five month production period

| Month | Adding ratio of Beet juice |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0% | 5% | 10% | 15% | 20% | 25% | 30% | 35% | 40% | 45% | 50% |
| October | 79 | 79.4 | 79.8 | 80.2 | 80.6 | 81 | 81.4 | 81.8 | 82.2 | 82.6 | 83 |
| November | 82.5 | 82.6 | 82.7 | 82.8 | 82.9 | 83 | 83.1 | 83.2 | 83.3 | 83.4 | 83.5 |
| January | 81.5 | 81.8 | 82.1 | 82.5 | 82.8 | 83.2 | 83.5 | 83.8 | 84.2 | 84.5 | 84.9 |
| February | 88.5 | 88.51 | 88.53 | 88.54 | 88.56 | 88.57 | 88.59 | 88.6 | 88.62 | 88.63 | 88.65 |
| March | 78.3 | 78.4 | 78.6 | 78.7 | 78.9 | 79 | 79.2 | 79.3 | 79.5 | 79.6 | 79.8 |

Table 3 shows the improvement in the purity of the mixed juices, especially in October which is considered the beginning of the production season and also the same effect on April which is considered the end of the season.

The increase in the quality of the juice and the more the degree of purity of the juice increased, the more the accompanying processes of purification, evaporation and crystallization process were going better, increase the value of the crystal 0.1 is equivalent to thousands of tons of the amount of sugar produced an increase in quality means an increase in the amount of sugar produced.

Fig (2): Added ratio of Beet juice with Purity mix graph

4.Conclusions:

The effects of different values of adding beet juice to cane juice on pol and purity properties have been studied, in this research, these effects were identified and explained in tables and chart.At the end of this research, it became clear the importance of mixing sugar cane juice with sugar beet juice, the importance of mixing became clear from the results of the experiment, which were to improve the quality of the mixed juice, as the sugar content in the mixed juice increased significantly from 10.5 to 21.5 and the purity of the mixed juice increased significant, where before the addition of sugar Beet juice was 78.3 and after the addition of sugar Beet juice recorded purity 88.65, improving the quality standard is positively reflected in improving the production processes of purification, evaporation, crystallization and in the amount of sugar produced.   Each increased percentage added Beet juice note the improvement in the properties of mixed juice, especially in the degree of purity, which is also reflected in the amount of sugar produced.The study aims to develop the sugar industry in Sudan, in addition to the sugar industry from cane and the development of agricultural.

References

[1] Asadi, M., 2011, Sugarbeet processing in beetsugar Handbook, John and Sons Inc, New Jersey, USA, Pp 100-465

[2] Yaser, A., Salaheldin, A., Ahmed, O., and  Mohamed, k. 2011, Attempts To Mix And Crush Sugar Cane And Sugar Beet At Guneid Sugar Factory, Sudan, 621.798:641

[3] Paturar, j., Byproduct of the cane sugar industry, New York, USA

Arteaga, R., Larrahano ., J.E., and Lamprea, E. 2012, TSB Add Mix Test Columbia

[4] Mathur,R.B.L.(1990).Composition of Cane and Juice Clarification Bleaching in:”Hand Book of Cane Sugar Technology”2nd

[5] Dambalkar VS, Rudrawar BD, Poojari VR. Study of physico-chemical properties and sensory attributes of beetroot-orange RTS drink. International Journal of Science and Research. 2015; 4(10):589-594.

[6] LalMathur R.B.,1990,HandBook of Sugar Technology

Kale, RG., Sawate, AR., Kshirsagar, RB., Patil, BM., and Mane RP,2018, Studies on evaluation of physical and chemical composition of beetroot (Beta vulgaris L.)

[7] Maria, l., 2017, Milling Test Using a Mixture of Sugarbeet and sugar cane, personal communication.

[8] Bentley, G. A., Miller, K. F., Palmer, B.C. (1999) Proc. Austr. Soc. Sugar Cane Technol.193-202.