# Design & Development of Object Identification using ESP 32 CAM Modules

Mohini D. More1 ,		Prof. Swami S.G.2 mohinimored@gmail.com	swamishivputra@gmail.com Department of Computer Science and Engineering, M.S.Bidve Engineering College, Latur.

## Abstract

The project “Design and Development of Object Identifying Algorithm using ESP32- CAM Module” focuses on creating a low-cost, efficient, and real-time object recognition system[6] using edge-based artificial intelligence. The ESP32-CAM module, equipped with an onboard camera and Wi-Fi capabilities, serves as the core hardware for capturing and processing images. The system employs the Edge Impulse platform for developing and training a custom machine learning model capable of identifying multiple objects—such as vegetables (tomato, potato, and onion)—directly on the microcontroller without requiring cloud computation.

The process involves collecting and preprocessing image datasets, training a Convolutional Neural Network (CNN) on the Edge Impulse platform, and deploying the trained model onto the ESP32-CAM. The captured images are analyzed locally, and the recognition results are displayed on an OLED screen in real time. This approach demonstrates the potential of edge computing for intelligent embedded systems by reducing latency, power consumption, and dependency on external servers.

The project showcases a practical application of AI on low-power hardware, suitable for IoT-based automation, smart surveillance, and agricultural product classification. It highlights how AI and embedded systems can be effectively integrated to achieve cost-effective, portable, and scalable smart vision solutions.

Keywords: edge computing; tiny machine learning; ESP32; object detection.

- Introduction:

Recent advances in embedded hardware and lightweight machine learning[1] have made it possible to perform visual recognition directly on low-power devices. This project implements an object-identification system using the ESP32-CAM[2], a compact microcontroller board that includes a camera and wireless connectivity. Using Edge Impulse for model development, the project trains a compact image classifier that can be deployed to the ESP32-CAM so that inference runs entirely on-device. Images captured by the camera are preprocessed and fed into the onboard model; detection outcomes are then presented locally (for example, on an attached OLED or via serial output). The solution demonstrates how constrained devices can execute practical computer-vision tasks without relying on cloud resources, enabling fast responses and simpler deployment.

Need of Project Many conventional vision applications depend on remote servers for processing, causing latency, dependence on network availability, and potential privacy issues. For scenarios such as field-level agricultural sorting, remote monitoring, or portable automation, these limitations reduce practicality. A compact, locally running recognition system solves these problems by offering low-latency decisions, reduced bandwidth use, and improved data privacy. Additionally, building an efficient object-identification pipeline for the ESP32-CAM provides a low-cost, accessible platform for students and practitioners to prototype edge-AI [3] applications. This project therefore addresses the demand for affordable, autonomous, and privacy-conscious vision systems suitable for real-world, resource-limited environments.

## Literature Survey:

  - Existing Work:

Carried out Object detection and image recognition have been widely explored using deep learning and embedded vision technologies. Most existing systems rely on high-performance platforms such as Raspberry Pi, Jetson Nano, or cloud-based services for real-time image processing. These devices can run convolution neural networks (CNNs)[4] like YOLO, MobileNet, or TensorFlow Lite, providing good accuracy but at higher cost and power consumption. Several studies have demonstrated the use of Edge Impulse[5] with microcontrollers for on-device inferencing. Researchers have successfully used the ESP32-CAM module to classify simple objects like fruits, vegetables, or traffic signs by training lightweight models compatible with its limited computational power. These projects typically follow a workflow of image dataset collection, preprocessing, model training, and deployment using Aurduino IDE or Micro Python. Moreover, edge-based implementations have shown the potential for low-latency and offline operation, making them suitable for IoT, agricultural, and industrial applications. However, the majority of available implementations focus on limited object categories and lack optimized accuracy for complex real world conditions such as poor lighting or background noise.

  - Research Gap: While many previous works have applied image recognition on embedded systems, there remains a gap in balancing model accuracy and hardware constraints. Most existing ESP32-CAM–based models struggle with performance when deployed for multi-class detection or in dynamic environments. In addition, earlier implementations often rely on small datasets, which limits the generalization capability of the trained models. Another significant gap lies in the integration of hardware and AI platforms—many projects do not fully utilize tools like Edge Impulse for optimized model conversion and on-device deployment. There is also limited research on enhancing user interaction, such as providing real-time visual feedback through OLED displays or integrating local data logging. This project addresses these challenges by developing a custom-trained object identification algorithm optimized for ESP32-CAM, capable of performing reliable real-time detection on low-power hardware. It aims to improve model efficiency, reduce inference latency, and deliver better usability through real-time on-screen output.

## Objectives:

The primary aim of this project is to design and develop an intelligent object identification system capable of detecting and classifying different objects in real time using the ESP32-CAM module. The project emphasizes the use of Edge AI — that is,

performing machine learning inference directly on the embedded hardware without external cloud support. This

approach enhances processing speed, ensures data privacy, and minimizes network dependency. By integrating the ESP32-CAM with a trained lightweight deep learning model, the system can identify and display the recognized object on a connected OLED screen, demonstrating the potential of low-cost AI based embedded systems for practical applications such as smart monitoring, automation, and agricultural systems.

## Objective 1:

To Develop a Real-Time Object Identification System[6] The first and most essential objective of this project is to create a real-time visual recognition system that can efficiently capture, process, and classify images using the ESP32-CAM’s built-in camera. This involves designing a pipeline that includes:

- Continuous image capturing through the camera interface.

- Preprocessing of captured images for feature extraction and input normalization.

- Running the trained inference model directly on the ESP32-CAM for object identification. The system is expected to function with minimal delay, offering near real-time detection performance. This requires optimizing both hardware and software components to operate within the limited memory and computational power of the ESP32 microcontroller. Additionally, special attention is given to factors such as lighting variation, camera angle, and background noise to ensure the model performs accurately under practical conditions. Through this objective, the project demonstrates how embedded vision and AI can be combined to create efficient, standalone detection systems for various real-world applications. Object Detection using ESP32 Cam Module.

## Objective 2:

To Implement Edge-Based Machine Learning using Edge Impulse[5] The second objective focuses on implementing Edge-based Machine Learning (ML)[1] using the Edge Impulse platform. This stage involves the complete workflow of building a custom-trained AI model suitable for deployment on resource-constrained hardware. The steps include:

- Dataset collection using the ESP32-CAM or other sources to capture multiple images of selected objects (e.g., vegetables like tomato, onion, and potato).

- Data preprocessing and labeling, where images are resized, filtered, and annotated to improve model accuracy.

- Model training and optimization using Edge Impulse’s graphical interface to build a Convolutional Neural Network (CNN)[4] capable of running efficiently on embedded devices.

- Deployment of the trained model as an Arduino-compatible library that integrates seamlessly with the ESP32-CAM. By executing inference directly on the hardware, the project avoids cloud-based delays, ensures faster results, and strengthens data privacy. This objective highlights the practicality of AI on the Edge, where devices become self-reliant and responsive without internet dependency.

- To improve user experience by showing the detection result and its confidence level clearly on screen. This feature also makes the system suitable for IoT-based smart devices [3] where quick and simple status reporting is essential. For example, in smart agriculture setups, the device could automatically recognize the type of crop or vegetable and provide instant on-screen feedback.

## Methods and Materials:

  - Experimental Setup / Layout:

The experimental setup consists of the ESP32-CAM module, a USB-to-Serial (FTDI) programmer, and an OLED display connected on a breadboard for easy prototyping. The figure below shows the connection layout of the system.

Figure 4.1: Experimental setup of ESP32-CAM with FTDI programmer and OLED display.

Figure 4.2: Actual setup of ESP32-CAM with FTDI programmer and OLED display.

Object Detection using ESP32 Cam Module Description of Setup the ESP32-CAM module serves as the core processing unit that captures images through its built-in OV2640 camera. It performs real-time image acquisition and inference using a trained AI model deployed from the Edge Impulse platform. The FTDI programmer is used to upload code and provide serial communication between the ESP32-CAM and a computer. An OLED display (0.96 inch, I2C interface) is connected to the ESP32-CAM to display the detection results — such as the object name (e.g., “Tomato”) and confidence level (e.g., “92%”). Power is supplied via USB through the FTDI board or an external 5V source.

## Pin Configuration Summary:

| Component | ESP32-CAM Pin | Connection Description |
| --- | --- | --- |
| FTDI TX | U0R (GPIO 3) | Serial data reception |
| FTDI RX | U0T (GPIO 1) | Serial data transmission |
| FTDI 5V | 5V | Power supply |
| FTDI GND | GND | Ground connection |
| OLED SDA | GPIO 15 | I2C Data line |
| OLED SCL | GPIO 14 | I2C Clock line |
| GND (OLED) | GND | Common ground |
| VCC (OLED) | 3.3V | Power supply for display |

Note: GPIO 0 should be connected to GND while uploading code and disconnected afterward for normal operation.

## Flow Diagram / Process Diagram:

The working of the system can be represented through the following process flow:

## Step-by-step Workflow:

Figure 4.2: Flow diagram of ESP32-CAM object identification system.

- Start / Power ON: The ESP32-CAM module initializes camera and OLED hardware.

- Image Capture: The onboard camera captures an image of the object placed in front of it.

- Preprocessing: The captured image is resized and converted into grayscale or RGB format suitable for the trained model input.

- Inference / Object Detection: The preprocessed image is passed to the Edge Impulse trained model running on the ESP32-CAM. The model predicts the object category and confidence score.

- Display Results: The identified object name and confidence percentage is displayed on the OLED screen.

- Repeat Process: The camera continuously captures new frames and repeats detection in real time. This closed-loop process ensures continuous and autonomous operation, allowing the system to identify objects instantly after each frame capture. Object Detection using ESP32 Cam Module

## Materials Required and Procedure:

- Materials Required:

| Sr. No | Component | Specification / Description | Quantity |
| --- | --- | --- | --- |
| 1 | ESP32-CAM Module | AI Thinker version with OV2640 camera | 1 |
| 2 | FTDI USB to Serial Converter | 5V output, compatible with ESP32-CAM | 1 |
| 3 | OLED Display | 0.96 inch, I2C interface (SSD1306 driver) | 1 |
| 4 | Breadboard | For prototyping connections | 1 |
| 5 | Jumper Wires | Male-to-male and male-to-female wires | As required |
| 6 | Power Supply | 5V DC via USB or external adapter | 1 |
| 7 | Computer with Arduino IDE | For programming and debugging | 1 |

## Procedure:

  - Hardware Setup:

    - Connect the ESP32-CAM to the FTDI module as per the wiring diagram.

    - Ensure that GPIO 0 is connected to GND during code upload.

    - Connect the OLED display to GPIO 14 (SCL) and GPIO 15 (SDA).

## Software Configuration:

    - Open Aurduino IDE and install the required ESP32 board package.

    - Install the EloquentEsp32cam and Adafruit SSD1306 libraries. o Import the trained model library from Edge Impulse. Object Detection using ESP32 Cam Module

## Model Training (on Edge Impulse):

    - Capture or upload datasets (e.g., images of tomato, onion, and potato).

    - Label and preprocess images. o Train and optimize a CNN model suitable for ESP32 hardware. o Download the model as an Arduino library and integrate it with your project.

## Code Upload:

    - Compile the code in Aurduino IDE.

    - Upload the sketch via FTDI while GPIO 0 is grounded.

    - After upload, disconnect GPIO 0 from GND and press the reset button.

## Testing:

    - Place different objects (e.g., vegetables) in front of the camera. o Observe the detection output on the OLED display or serial monitor.

    - Evaluate the accuracy and response time of the system.

## Optimization (if needed):

    - Adjust image size or lighting conditions.

    - Retrain the model with more data if accuracy is low. Object Detection using ESP32 Cam Module

## Result and Discussion

  - Result:

After completing the implementation and deployment of the trained model onto the ESP32-CAM module, the system successfully performed real-time object identification for the selected objects — tomato, onion, and potato. The camera captured live images, processed them locally, and displayed the detected object name and confidence percentage on the OLED screen. The system demonstrated stable operation with smooth inference at a low latency. On average, the model achieved a classification accuracy of 90–93%, depending on the lighting conditions and object distance. The detection output refreshed continuously, confirming that the ESP32-CAM could efficiently handle lightweight machine learning models optimized through the Edge Impulse platform. The results also validated the project’s main objective — performing AI-based image recognition directly on edge hardware without the need for internet connectivity or cloud-based computation. Observed Results Summary Parameter Observation Hardware Used ESP32-CAM (AI Thinker) Model Type CNN (trained using Edge Impulse) Average Detection Accuracy 90–93% Power Requirement 5V DC Display Output OLED (128x64) showing object name and confidence Response Time ~1.5 seconds per frame Detected Objects Tomato, Onion, Potato The system thus met the functional and performance goals set in the initial project objectives. Object Detection using ESP32 Cam Module.

## Result Discussion and Conclusion:

The obtained results show that ESP32-CAM, despite its low cost and limited hardware resources, can perform real-time image classification effectively when supported by optimized machine learning models. The Edge Impulse platform simplified model training and deployment, enabling the device to recognize multiple object categories with satisfactory accuracy. The OLED output ensured user-friendly visual feedback, while the edge-based inference reduced processing delay and improved system independence. In conclusion, the developed prototype successfully demonstrates the practicality of Edge AI implementation on microcontrollers, opening new opportunities for low-cost smart vision applications such as agricultural classification, automated sorting, and IoT-enabled monitoring.

## Conclusion and Future Scope:

  - Conclusion:

The project “Design and Development of Object Identifying Algorithm using ESP32-CAM Module” successfully achieved its intended goals by integrating machine learning with embedded edge computing. The system proved capable of recognizing and classifying objects in real time without cloud connectivity, using the ESP32-CAM’s internal resources and a lightweight CNN model trained via Edge Impulse. The performance results validated that edge AI can be both cost-effective and efficient when optimized properly. The hardware configuration was simple, and the use of an OLED display provided a clear interface for users to view results. This demonstrates how AI can be embedded into small-scale IoT devices to enable autonomous and intelligent functionality. Overall, the project highlights the potential of combining embedded systems, AI, and IoT technologies to build innovative and energy-efficient solutions for real-world applications. Object Detection using ESP32 Cam Module

## Future Scope:

There are several ways this project can be expanded and enhanced in the future:

- Model Expansion: The dataset can be extended to include a larger variety of objects, allowing for multi-class classification with higher accuracy.

- Improved Hardware: Using modules like ESP32-S3 or ESP-EYE with higher RAM can enable more complex AI models to run efficiently.

- Mobile or Web Integration: The ESP32-CAM can be connected to a smartphone app or a cloud dashboard for remote monitoring and data logging.

- Edge Optimization: Model compression and quantization can further reduce inference time and power consumption.

- Industrial and Agricultural Use: The system can be adapted for sorting fruits, identifying defective products, or monitoring crops in smart farming applications.

- Autonomous Systems: Integrating the module with actuators or robotic systems can enable self operating intelligent machines.

## References:

- A. Sharma, “Machine Learning Approach for Crop Prediction,” IEEE Transactions on AI Systems, vol. 4, no. 2, pp. 112–118, 2024.

- R. Krishna, “Object Detection using ESP32-CAM and Edge Impulse,” CircuitDigest, Jul. 2024.

- P. Gupta and M. Kumar, “Edge AI Implementation on Low-Power IoT Devices,” IEEE Internet of Things Journal, vol. 9, no. 6, pp. 4842–4853, 2023.

- S. R. Patil, “Optimized Convolutional Neural Networks for Embedded Vision Systems,” IEEE Access, vol. 11, pp. 17456–

17467, 2023.

- Edge Impulse Inc., Edge Impulse Documentation: AI on Edge Devices, 2024. [Online]. Available: https://edgeimpulse.com/docs

- A. S. Pandey and D. Verma, “Real-Time Object Classification using Edge Computing,” International Journal of Embedded Systems Research, vol. 5, no. 3, pp. 210–219, 2024.

- Espressif Systems, ESP32-CAM Technical Reference Manual, 2023. [Online]. Available: https://www.espressif.com