# A Comparative Performance Analysis of Quantum-dot Cellular Automata (QCA) Digital Code Converters

Abstract— As conventional CMOS technology approaches its fundamental physical scaling limits, researchers are exploring alternative computing paradigms to sustain the progress predicted by Moore's Law. Quantum-dot Cellular Automata (QCA) has emerged as a promising, transistor less alternative, offering significant advantages such as ultra-low power dissipation, higher device density, and faster switching speeds. This paper presents a critical evaluation and comparative analysis of various digital code converters implemented using QCA technology. The study synthesizes existing research on fundamental converter types, including Binary-to-Gray, Gray-to-Binary, and BCD-based converters. By systematically comparing different architectural designs based on key performance metrics namely cell count, occupied area, and latency this analysis reveals a clear hierarchy of efficiency. The key finding is that circuit efficiency improves through two approaches: component-level optimizations, such as improved XOR gate designs, and paradigm-level advancements, such as the adoption of scalable tile-based methods instead of traditional gate-based layouts.

### Keywords: Quantum-dot Cellular Automata (QCA), Digital Code Converters, Binary-to-Gray, Gray-to-Binary, BCD Converters, XOR Gate Optimization, Tile-based Layout

### 1. Introduction

#### 1.1. The Post-CMOS Era and the Rise of Nanotechnology

For decades, the relentless downscaling of Complementary Metal-Oxide-Semiconductor (CMOS) technology has been the primary engine of the digital revolution, a trend famously encapsulated by Moore's Law. However, this progress is now encountering fundamental physical barriers. As feature sizes shrink to the nanometer scale, challenges such as prohibitive power consumption, quantum tunneling effects, and high leakage currents threaten to halt further advancement. This has spurred a global search for a "post-CMOS" technology capable of carrying computation into a new era.

Quantum-dot Cellular Automata (QCA) represents a revolutionary, transistor less computing paradigm proposed as a powerful alternative to overcome these limitations. Unlike conventional circuits that encode information using voltage levels, QCA utilizes the spatial arrangement of electrons within quantum dot cells to represent binary states. This fundamental difference enables a host of advantages over CMOS, including:

- Higher Packaging Density

- Lower Power Dissipation

- Higher Switching Speeds

- Nanometer Scalability

1.2. Objective and Scope

The central objective of this research paper is to synthesize and critically analyze the existing body of research on QCA-based digital code converter circuits. Code converters are fundamental components in digital systems, enabling communication between subsystems that use different binary encoding schemes. A thorough understanding of their performance in a QCA framework is crucial for evaluating the technology's practical potential.

This analysis focuses on a comparative assessment of various design implementations for essential circuits, including:

- Binary-to-Gray Code Converters

- Gray-to-Binary Code Converters

- Binary-Coded Decimal (BCD) to Excess-3 Converters

- BCD-to-Gray Code Converters

The comparison is grounded in key quantitative performance metrics that define circuit efficiency in the QCA paradigm: cell count, occupied area, and latency (measured in clock cycles).

#### 1.3. Paper Structure

This paper is structured to guide the reader from fundamental concepts to a comprehensive comparative analysis. Section 2 provides an overview of the fundamentals of QCA, including the basic cell structure, logic implementation, and clocking. Section 3 analyzes the design of the XOR gate, a critical building block for many converters. Section 4 presents a focused comparison of Binary-to-Gray code converters, followed by a broader evaluation of other converter architectures in Section 5. Section 6 discusses the overarching trends and the impact of different design methodologies on circuit performance. Finally, Section 7 concludes the paper with a summary of findings and identifies promising directions for future research. This structured approach begins with an essential review of QCA's foundational principles.

### 2. Fundamentals of Quantum-dot Cellular Automata (QCA)

To fully appreciate the design and performance trade-offs of the circuits analyzed in this paper, it is essential to first understand the fundamental principles of QCA. This technology operates on mechanisms entirely different from conventional electronics. This section covers the basic QCA cell, the implementation of core logic functions, and the clocking mechanism that orchestrates the directional flow of information.

#### 2.1. The QCA Cell and Binary Encoding

The fundamental unit of a QCA circuit is the QCA cell. A standard cell consists of four quantum dots arranged in a square array, containing two mobile electrons. These electrons are capable of tunneling between the dots within the cell but cannot leave it. Due to electrostatic Coulombic repulsion, the two electrons will always seek to maximize their separation, forcing them into one of two stable, energetically equivalent states where they occupy antipodal corners of the square.

These two distinct configurations encode binary information:

- Polarization P = +1: Corresponds to binary logic '1'.

- Polarization P = -1: Corresponds to binary logic '0'.

Crucially, in QCA, information is represented by the position of electrons, not by the voltage or current levels that define logic states in CMOS. This is the core principle that enables its ultra-low-power operation.

#### 2.2. Fundamental Logic Primitives

All complex combinational logic in QCA can be constructed from two fundamental logic primitives: the Majority Gate (MG) and the Inverter.

The three-input Majority Gate is the primary logic element. Its output is determined by the polarization of the majority of its three inputs. The corresponding Boolean function is:

M(A, B, C) = AB + BC + AC

This versatile gate can be programmed to perform simpler, more familiar logic functions. By fixing one of the inputs (the "programming" input) to a constant logic level, the Majority Gate can be configured as:

- A two-input AND gate by setting one input to logic '0' (M (A, B, 0) = AB).

- A two-input OR gate by setting one input to logic '1' (M (A, B, 1) = A + B).

The Inverter, which performs the Boolean NOT operation, complements the Majority Gate. Together, they form a logically complete set, enabling the construction of any arbitrary Boolean function.

#### 2.3. Clocking, Interconnection, and Data Flow

Unlike conventional circuits where a global clock triggers simultaneous state changes, QCA relies on a Zone Clocking Scheme to control the direction of information flow and enable pipelined computation. The circuit layout is partitioned into distinct clocking zones, and each zone is driven by a periodic, four-phase clock signal. The four phases are:

- Switch

- Hold

- Release

- Relax.

This four-phase system ensures that data propagates in a controlled, pipelined manner, with each clock zone acting like a D-latch. Furthermore, QCA cells perform the dual function of logic structures and interconnections (wires). Data is transmitted along a line of cells through local Coulombic interactions. Wire crossovers, a critical aspect of complex circuit design, can be implemented using either coplanar or multilayer layouts. These fundamental concepts are combined to build essential logic blocks, such as the XOR gate, which serves as a cornerstone for many digital circuits.

### 3. Analysis of QCA-based XOR Gate Designs

The Exclusive-OR (XOR) gate is a critical building block for a wide range of digital circuits, including adders, parity generators, and, most relevantly for this paper, code converters. The efficiency of the XOR gate design—in terms of its physical size, component count, and processing speed—has a direct and significant impact on the overall performance of any larger circuit that employs it. Therefore, an analysis of XOR gate implementations provides a foundational understanding of the performance variations seen in more complex QCA architectures.

#### 3.1. Architectural Comparison

Early QCA designs for the XOR gate were often direct translations of CMOS logic. For example, the design presented by Guleria (2017) is composed of OR, AND, and inverter gates, which are themselves implemented using multiple QCA majority gates. While functional, this approach often leads to suboptimal designs with high cell counts and latency.

More recent research, such as the work by Khakpour et al. (2020), has focused on creating more compact and efficient XOR gates by optimizing the logic at the QCA cell level. Their design uses a more direct implementation that minimizes the number of majority gates and interconnecting wires, resulting in a significantly improved architecture. The performance differences are summarized in the table below.

| Metric | Guleria (2017) | Khakpour et al. (2020) |
| --- | --- | --- |
| Cell Count | 51 | 20 (First proposed) |
| Occupied Area (µm²) | 0.08 | Not Specified |
| Latency (Cycles) | 1.0 | 0.75 |

#### 3.2. Performance Implications

The data clearly demonstrates the impact of architectural optimization. The design from Khakpour et al. represents a substantial improvement over the earlier implementation, achieving a reduction in cell count of over 60% and a 25% decrease in latency. This highlights a critical trend in QCA research: moving beyond simple CMOS-to-QCA translations toward designs that are natively optimized for the QCA paradigm. These highly efficient building blocks are then used to construct more complex systems, such as the Binary-to-gray code converters discussed in the next section.

### 4. Comparative Analysis of Binary-to-Gray Code Converters

Binary-to-gray code converters are essential components in digital systems, particularly in applications like rotary encoders and state machines, where they help prevent transient errors by ensuring that only one-bit changes between consecutive values. As a common benchmark circuit, the QCA implementation of this converter provides an excellent case study for assessing the impact of design efficiency at both the gate and system levels.

#### 4.1. Design Architectures and Performance Metrics

Different research efforts have produced QCA-based Binary-to-Gray converters with vastly different performance characteristics, largely dependent on the underlying logic gates used.

The 4-bit converter design proposed by Guleria (2017) is constructed from three of their previously detailed XOR gate designs, resulting in a total circuit composition of 9 majority gates and 3 inverters, and it utilizes multilayer crossovers for signal routing. In contrast, the design from Khakpour et al. (2020) is built using their more compact and faster XOR gate. This architectural choice at the component level translates directly into system-level performance gains, as detailed in the comparison table below.

| Metric | Guleria (2017) | Khakpour et al. (2020) |
| --- | --- | --- |
| Cell Count | 225 | 99 |
| Occupied Area(µm²) | 0.43 | Not Specified |
| Latency (Cycles) | 1.0 | 0.75 |

#### 4.2. Evaluation of Efficiency

The comparative data reveals substantial performance improvements in the more recent design by Khakpour et al. Their architecture achieves a cell count reduction of approximately 56% and a latency reduction of 25%. These impressive gains are not the result of a complete architectural rethinking of the converter itself, but are directly attributable to the optimization of the XOR gate used as its fundamental building block. This underscores the principle that component-level efficiency is a primary driver of overall system performance in QCA. This analysis can now be expanded from this specific converter type to a broader examination of other code converter architectures.

### 5. Evaluation of Other QCA Code Converter Architectures

The principles of QCA design optimization, particularly the importance of efficient building blocks and layout strategies, extend far beyond the Binary-to-gray converter. To provide a more comprehensive view of the technology's capabilities and design trade-offs, this section surveys a wider range of code converter circuits, comparing their performance metrics and highlighting the impact of different design methodologies.

#### 5.1. Survey of Converter Designs

Researchers have implemented numerous code converters in QCA, including Gray-to-Binary, BCD-to-Gray, and BCD-to-Excess-3. Early designs often followed a traditional gate-based approach, translating standard logic diagrams into QCA layouts. However, more recent work has explored alternative design paradigms and addressed practical operational constraints.

Concurrently, the introduction of analyses for practical operational parameters, such as temperature stability and energy dissipation, signals the maturation of the QCA field. For instance, recent multicode converters have been rigorously tested for energy efficiency, reporting low dissipation levels such as 171 meV. The growing focus on such real-world viability factors, in parallel with architectural advancements like the tile-based and block-based methods, illustrates a critical evolution from theoretical simulation toward practical, robust, and scalable circuit design.

One notable architectural innovation is the "tile-based" method, which organizes circuits into localized, repeatable tiles. This approach can offer superior scalability and eliminate the need for separate inverter gates, thereby reducing latency by creating logic functions within the tile structure itself. Building upon this concept of integrated logic, recent advancements have introduced multi-functional block designs, such as the decimal-to-multicode converter. This specific architecture utilizes a novel 4-input block that can be programmed to perform AND, OR, NAND, or NOR operations depending on the polarization of a "select" cell. By using these high-density blocks, researchers can now generate multiple code outputs—such as BCD, Excess-3, and gray codes—simultaneously within a single, optimized circuit.

#### 5.2. Comprehensive Performance Comparison

The following table compiles performance metrics from various research papers, offering a holistic comparison of different converter types and design methodologies. This unified view allows for the identification of broader trends and trade-offs in QCA circuit design.

| Circuit | Type | Cell Count | Area (µm²) | Latency (cycles) | Scalability | Inverter Gate | Circuit Layer type |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Binary to Gray (4-bit) | Guleria (2017) | 225 | 0.43 | 1.0 | No | 19 | Multi-layer Crossover |
| Binary to Gray (4-bit) | Khakpour et al. (2020) | 99 | N/A | 0.75 | No | 4 | Optimized XOR Gate |
| Binary to Gray (4-bit) | Khakpour et al. (2020) | 76 | N/A | 2.25 | No | 3 | Optimized XOR Gate |
| BCD to Gray | Khakpour et al. (2020) | 78 | N/A | 1.0 | No | 4 | Optimized XOR Gate |
| BCD to Gray | Fouladinia et al. (2024) | 127 | 0.18 | 0.75 | No | 3 | Tile-Based |
| BCD to XS3 | Fouladinia et al. (2024) | 190 | 0.25 | 1.75 | No | 7 | Tile-Based |
| BCD to XS3 | Referenced by Fouladinia et al. | 191 | 0.29 | 3.00 | No | 6 | Single-Layer |
| BCD to XS3 | Referenced by Fouladinia et al. | 254 | 0.31 | 1.00 | No | 5 | Multi-layer |
| Thermometer to Gray | Das et al. (2021) | 134 | 0.20 | 1.75 | Yes | 1 | Single-layer |
| Binary to XS3 | Bhowmik et al. (2022) | 176 | 0.20 | 1.00 | No | 7 | Single-layer |
| Decimal to-3 Codes | F. Fouladinia, Gholami (2023) | 280 | 0.29 | 1.75 | Yes | 3 | Novel-4-input block, Multilayer, |

Table 1: Primary Performances metrics

| Circuit layer Type | Design Methodology | Fault Tolerance | Inverter Gates | Temperature Stability | Crossover Technique |
| --- | --- | --- | --- | --- | --- |
| Multi-layer | Gate based | No | 19 | Not analysed | Multi-layer |
| Single layer | Optimized XOR based | No | 4 | Not analysed | Coplanar |
| Single layer | Optimized XOR based | No | 3 | Not analysed | Coplanar |
| Single layer | Gate based | No | 4 | Not analysed | Coplanar |
| Single layer | Tile based | No | 3 | Analysed | None |
| Single layer | Tile based | No | 7 | Analysed | None |
| Single layer | Gate based | No | 6 | Not analysed | Coplanar |
| Multi-layer | Gate based | No | 5 | Not analysed | Multi-layer |
| Single layer | Gate based | No | 1 | Analysed | Coplanar |
| Single layer | Gate based | Yes | 7 | Analysed | Coplanar |
| Multi-layer | Block based (4-input) | No | 3 | Analysed | Multi-layer |

Table 2: Design and Layout Characteristics

This comprehensive data provides the basis for a deeper discussion of performance drivers and design choices in QCA circuits.

6.Discussion

The comparative data synthesized in the preceding sections provides a clear vantage point from which to interpret the state of QCA code converter design. This section analyzes the overarching trends revealed by the data, evaluates the impact of distinct design methodologies, and discusses the broader implications for the future development of practical and efficient QCA circuits.

#### 6.1. Analysis of Performance Trends

A detailed analysis of the compiled data reveals two primary drivers of performance in QCA circuits: lower-level component optimization and higher-level design methodology. The dramatic improvement in the Binary-to-Gray converter when moving from Guleria's (2017) design to that of Khakpour et al. (2020) is a direct consequence of optimizing the underlying XOR gate. This directly demonstrates that component-level efficiency gains propagate upward to create significantly more efficient systems.

The data also highlights critical performance trade-offs dictated by circuit topology and algorithmic depth. For instance, the optimized Gray-to-Binary converter from Khakpour et al. has a remarkably low cell count (76 cells) but a significantly higher latency (2.25 clock cycles) compared to its Binary-to-Gray counterpart (99 cells, 0.75 cycles). This disparity arises from the inherent logical structure of the conversion algorithms. Binary-to-Gray conversion is a highly parallel task, where each output bit can be computed independently and simultaneously. In contrast, Gray-to-Binary conversion is fundamentally sequential; the calculation of each output bit depends on the result of the previous bit's calculation, creating a cascade of operations that dictates the circuit's overall latency. This illustrates that while component optimization is vital, the intrinsic structure of the computational task imposes hard limits on performance.

#### 6.2. Impact of Design Methodology

The comparison between traditional gate-based designs and the more recent tile-based and block-based approaches reveals the profound impact of design philosophy3. The performance data reveals a critical design trade-off: while the gate-based BCD-to-Gray converter from Khakpour et al. (2020) achieves a lower cell count (78 cells)4, the tile-based design from Fouladinia et al. (2024) demonstrates superior latency (0.75 vs. 1.0 cycles) and a significantly smaller area (0.18 $µm^2$ vs. N/A), despite requiring more cells (127).

This evolution is further exemplified by the Decimal to Multicode Converter6666. By utilizing a novel 4-input gate, this design creates a multi-functional block capable of generating 4-input AND, OR, NAND, and NOR outputs. This methodology-driven approach allows a single circuit to accept 10 decimal inputs and produce 12 simultaneous outputs (Excess-3, BCD, and Gray codes) while maintaining a competitive footprint of 0.29 $µm^2. This highlights a shift from optimizing for component count alone towards holistic optimization for functionality, latency, and physical footprint.

Integrated functional blocks achieve lower latency by creating complex logic within a single structure, effectively reducing the need for separate, delay-inducing gates. For instance, the multicode converter requires only one inverter gate, compared to the 3 to 19 inverters used in more traditional architectures. This shift represents a maturing of QCA design principles from assembling discrete gates to designing integrated systems.

Furthermore, the introduction of analyses for practical viability factors, such as temperature stability and power dissipation (estimated at 171 meV for the multicode converter), signals an important evolution in the field. While early research focused predominantly on logical correctness and minimizing abstract metrics like cell count, these newer considerations—including scalability—are essential for moving QCA design from a purely theoretical exercise toward real-world application. A circuit that is compact and fast but fails to operate reliably or scale efficiently is of limited use. Therefore, a holistic design approach that balances logical efficiency with physical robustness and multi-functional scalability is paramount for the future of QCA.

7. Conclusion and Future Scope

#### 7.1. Summary of Findings

This paper has provided a structured comparative analysis of digital code converter circuits implemented in Quantum-dot Cellular Automata (QCA) technology. By synthesizing data from multiple studies, this work demonstrates a clear performance hierarchy among different design approaches and highlights the key factors driving circuit efficiency. The analysis confirms that QCA offers a viable platform for implementing complex digital logic with potential benefits in density and power consumption over conventional CMOS.

A critical conclusion of this review is that significant improvements in circuit performance are directly linked to the optimization of fundamental building blocks (such as the XOR gate) and the adoption of advanced, scalable design paradigms like the tile-based method and multi-functional block architectures. Specifically, the introduction of novel 4-input blocks has enabled the transition from single-purpose converters to multi code systems capable of simultaneously producing BCD, Excess-3, and gray codes from a single decimal input. These findings underscore a clear path forward for QCA research, emphasizing a shift from direct translation of CMOS architectures to the development of methodologies natively suited to QCA’s unique operational principles.

#### 7.2. Future Research Directions

Based on the analysis and the future scope identified in the source literature, several promising avenues for future research emerge. Advancing the field will require concerted efforts in the following areas:

- Scalability: Implementation and verification of higher-bit converters (e.g., 8-bit, 16-bit, and beyond) to assess the practical scalability of current design methodologies and identify potential bottlenecks in larger systems.

- Complexity: Design of more complex sequential and arithmetic circuits, such as memories, multipliers, and full Arithmetic Logic Units (ALUs), to demonstrate QCA's capability for general-purpose computing.

- Optimization: Further research into automated optimization of interconnects, routing, and minimizing wire crossovers, which remain significant challenges in designing dense, multilayer QCA circuits.

- Practicality: Expanded analysis of critical operational parameters, such as power dissipation (verified at 171 meV for multicode converters) and fault tolerance, to bridge the gap between simulation and physical implementation.

Pursuing these research directions will be essential for maturing QCA from a promising nanotechnology into a practical and powerful computing platform.

### 8. References

- C. S. Lent, P. D. Tougaw, W. Porod, and G. H. Bernstein, "Quantum cellular automata," Nanotechnology, vol. 4, no. 1, p. 49, 1993.

- N. Guleria, "Binary to Gray code converter implementation using QCA," in 2017 International Conference on I-SMAC (IoT in Social, Mobile, Analytics and Cloud) (I-SMAC), 2017,

pp. 694-699.

- M. Khakpour, M. Gholami, and S. Naghizadeh, "Parity generator and digital code converter in QCA nanotechnology," International Nano Letters, vol. 10, pp. 49–59, 2020.

- F. Fouladinia, M. Gholami, and P. Karimi, "Three novel digital code converters in QCA nanotechnology," Heliyon, vol. 10, e40446, 2024.

- N. Nagaraju, A. Rani, S. Teena Sri, B. B. Subrahmanyam, and P. S. V. V. Srisha, "QCA based cost-efficient code converters with temperature stability," RB Journal of Lib & Information Science, vol. 13, no. 3, 2023.

- A. Kaity and S. Singh, "Optimized area efficient quantum dot cellular automata based reversible code converter circuits: design and energy performance estimation," Journal of Supercomputing, vol. 77, pp. 11160–11186, 2021.

- H. Rashidi and A. Rezai, “High-performance full adder architecture circuit quantum-dot cellular automata,” J.Eng., vol.1, no.1, pp.1–9, Jun.2021.

[8] M. Balalia. Rezai, H. Balali, F.Rabiei, and S.Emadi, “Towards coplanar quantum-dot cellular automata adders based on efficient three-input XOR gate,” Results Phys., vol.7, pp.1- 9, 2020..

- M. G. Waje and P. K. Dak hole, “Design and implementation of 4-bit arithmetic logic unit using Quantum Dot Cellular Automata,”2022, pp.1022–1029.

- Khan, A., Safoev, N. & Arya, R. Modeling of excess-3 to BCD code converter for nano system using quantum-dot cellular automata technology. Int. J. Numer. Model. Electron. Newt. Devices Fields 36(2), e3055 (2023).

- Gholami, M., Movahedi, M. & Aminzadeh, Z. Latch and fip-fop design in QCA technology with a minimum number of cells. Computer. Electr. Eng. 102, 108186 (2022).

- Raj, M., Ahmed, S. & Lakshminarayanan, G. Subtractor circuits using different wire crossing techniques in quantum-dot cellular automata. J. Nano photon. 14(2), 026007 (2020).

- F. Fouladinia, M. Gholami, Decimal to excess-3, BCD, and gray code converters with a novel 4-inputs block in QCA, Opt. Quant. Electron. 55 (10) (2023 Oct) 862.

- D. Bhowmik, J. Pal, M. Chandra, A. Kumar Saha, and N. Kumar, “QCA based design of cost-efficient code converter with temperature stability and energy efficiency analysis,” Materials Today: Proceedings, vol. 49, pp. 3585–3594, 2022.

- F. Fouladinia and M. Gholami, "Design of a Novel Decimal to Multicode Converter in QCA Technology," Journal of Electrical and Computer Engineering, vol. 2023, Article ID 6615790, 13 pages, 2023.

[16] J. C. Das and D. De, “Design of thermometer code-to- gray code converter circuit in quantum-dot cellular automata for nano-computing network,” Photonic Network Communications, vol. 41, no. 3, pp. 259–273, 2021

[17] F. Fouladinia, M. Gholami, and P. Karimi, “New tile-based circuits converting BCD code to Gray, excess-3, and Aiken codes in quantum-dot cellular automata (QCA) nanotechnology,”
Heliyon, vol. 10, 2024, DOI: 10.1016/j.heliyon. 2024.e40446.

[18] Design of energy, cost-efficient binary to Gray code converter with temperature and stability analysis, Results in Optics, vol. 16, 2024, DOI: 10.1016/j.rio.2024.100712.

[19] C. Xie, et al., “Design and analysis of a fault tolerance nano-scale code converter based on quantum-dots, “Nano Communication Networks, vol. 42, 2024, DOI: 10.1016/j.nancom.2024.100530.

[20] A. Khan, N. Safoev & R. Arya, “Modelling of Excess-3 to BCD code converter for nano systems using quantum-dot cellular automata technology,” Int. J. Numer. Model. Electron. Newt. Devices Fields, 2022.

[21] Decimal to Excess-3 and Excess-3 to Decimal Code Converters in QCA Nanotechnology, (Research work reported as QCA code converter designs with significant improvements — search on Google Scholar for full citation/DOI)

[22] Energy-efficient implementation of BCD to Excess-3 code converter for nano-communication using QCA, with DOI 10.23919/JCC.ea.2021-0883.202401

[23] Decimal to excess-3, BCD, and gray code converters with a novel 4-inputs block in QCA, reported 2023(Google scholar DOI)