STUDY ON THE ASSESSMENT OF PHYSICO-CHEMICAL PARAMETERS OF NARASAPURA LAKE, KARNATAKA

Abstract

The present study was conducted to evaluate the seasonal variation of selected physico-chemical parameters in Narasapura Lake, Kolar, Karnataka, from January to December 2024. Key parameters analysed included pH, total alkalinity, total dissolved solids (TDS), magnesium, calcium, total hardness, iron, chloride, biochemical oxygen demand (BOD), and chemical oxygen demand (COD). The results revealed noticeable seasonal fluctuations in all parameters, primarily influenced by monsoon dynamics. The pH remained within the acceptable range (6.5–8.5) throughout the study period. Alkalinity and TDS showed higher values during pre-monsoon months and declined significantly during monsoon. Magnesium levels remained relatively stable. Calcium, total hardness, Iron and chloride exhibited higher concentrations during dry months and lower values during the monsoon season. BOD and COD values were higher during the initial months, indicating organic and chemical load, but showed a declining trend towards the end of the year, suggesting improvement in water quality. The variations in water quality were mainly attributed to natural factors such as rainfall, evaporation, and water inflow. However, results showed most parameters were within acceptable limits, with no significant pollution observed.

Keywords: pH, Alkalinity, TDS, Magnesium, Calcium, Hardness, Iron, Chloride, BOD, COD.

Introduction

Lakes are non-marine, standing water bodies that are functionally linked to their surrounding landscapes (watersheds) through hydrological and atmospheric inputs. As relatively well-defined and self-contained ecosystems, they provide a clear framework for understanding the interactions among physical, biogeochemical, and biological processes. The distinct boundaries between water and land, as well as between water and air, facilitate strong coupling among ecosystem components, making lakes highly suitable for ecological study and informed management. Despite covering only 3% of the Earth’s land surface, lakes store approximately 87% of the world’s liquid freshwater (Yao et al., 2023), providing invaluable ecosystem services to more than 131 million people (Jenny et al., 2020).  The ever-increasing population, rapid urbanization, and modernization have intensified problems related to sewage disposal and the contamination of surface water bodies such as lakes. In addition to anthropogenic factors, natural processes like weathering of rocks, soil leaching, and mining activities also contribute to water pollution. These factors collectively lead to nutrient enrichment in lakes, a process known as eutrophication. Furthermore, changes in land use patterns and longer growing seasons promote increased use of fertilizers, which often leach into nearby watercourses, rivers, and lakes. This nutrient influx accelerates eutrophication, ultimately resulting in the degradation of water quality and loss of aquatic biodiversity. It is essential to test water quality before it is used for drinking, domestic, agricultural, or industrial purposes. Water may contain various types of impurities, including floating, dissolved, suspended, as well as microbiological and bacteriological contaminants. Therefore, comprehensive analysis using different physico-chemical parameters is necessary. Physical tests are conducted to assess characteristics such as temperature, pH, turbidity, and total dissolved solids (TDS). In addition, chemical tests are performed to determine parameters like biochemical oxygen demand (BOD), chemical oxygen demand (COD), dissolved oxygen (DO), alkalinity, hardness, and other relevant properties (Kumar, et al., 2023). These assessments help ensure the safety and suitability of water for its intended use (Sanath and Ramakrishna 2025). Conventional water quality assessments mainly emphasize physico-chemical parameters such as turbidity, pH, and conductivity. While these indicators are important, they are insufficient to fully represent the complexity of water quality. Various environmental factors also play a crucial role, including topographical features like slope and aspect, and geological characteristics such as lithology and soil type. These factors influence water quality both directly and indirectly by affecting processes like runoff, erosion, and the movement of nutrients and pollutants, ultimately shaping the overall condition of aquatic systems (Choudhury, et al., 2022; Gupta, et al.,2022). Several research works have been carried out on the water quality of different lakes in India (Mishra Amarnath, 2020; Prachi Vasistha, 2020; Chaudhary, 2021; Singh, 2022). Studies on the physico-chemical characteristics of ponds and lakes provide valuable insights into water quality status and emphasize the need for effective management strategies to maintain ecological balance. Freshwater has become a scarce resource due to overexploitation, pollution, population growth, rapid urbanization, industrialization, and agricultural runoff (Gupta and Shukla, 2006). In karnataka lakes, several researchers have attempted to assess the water quality periodically. Panikkar et al., (2022) have studied hydrogeochemistry and water quality index (WQI), in different small and medium reservoirs of Karnataka, Purandara et al., (2012) in surface water quality evaluation and modelling of Ghataprabha River, Ravikumar et al., (2013) in Sankey tank and Mallathahalli Lake, Bangalore urban district, and Kumar and Prabhu, (2019) in Kushawathi watershed area Chikkaballapur District. However, no comprehensive water quality survey has been conducted in the study area to date. Hence, the present study has been undertaken to assess the water quality of Narasapura Lake, Karnataka, with particular reference to its physico-chemical parameters.

Materials and methods

# Study area and fish collection

Narasapura Lake, situated in the Kolar district of Karnataka, is an important freshwater body covering an area of approximately 327.35 acres (131 hectares). Geographically, the lake is located at 13°07′59.7″N latitude and 77°59′59.0″E longitude, about 15 km from Kolar city and nearly 55 km from Bengaluru, making it a significant water resource in the region. The lake exhibits an irregular, roughly circular morphology, with considerable variation in depth across different parts, influenced by seasonal changes, sediment deposition, and water inflow patterns. Hydrologically, Narasapura Lake is connected to the Koramangala–Challaghatta (KC) Valley Project, through which treated wastewater is supplied. This inflow plays a crucial role in maintaining the water level of the lake, especially during dry periods, but may also influence its physico-chemical characteristics. The lake supports a rich aquatic ecosystem and is particularly known for its high fish productivity. Its relatively shallow nature, combined with nutrient availability, creates favorable conditions for fish growth and breeding, making it a prominent site for local fishing activities. In addition to its ecological importance, the lake also contributes to the livelihoods of nearby communities and plays a role in groundwater recharge and microclimatic regulation. However, increasing anthropogenic pressures and changes in water quality necessitate continuous monitoring and effective management to sustain its ecological health and productivity.

Surface water sampling

The surface water samples were collected from three different locations namely; Inlet of the Lake, Lake bund area and lake backwater area  of Narasapura Lake From January 2024 - December 2024.The water samples were collected on a monthly basis in sterilized polythene bottles for a period of 12 months, starting from January 2024 and ending in December 2024. The collected samples were subjected to comprehensive physicochemical analysis. General parameters such as water temperature, pH, electrical conductivity, and total dissolved solids were measured at the sampling site immediately after obtaining the samples. For the remaining parameters, the water samples were transferred to the laboratory and stored in a refrigerator at 4 °C for further analysis. All the physicochemical analyses were performed in accordance with the American Public Health Association (APHA, 2017; Gopal, et al., 1984).

RESULTS AND DISCUSSIONS

The water analysis was carried out to understand the level of some chemical parameters like pH, Alkalinity, TDS, Magnesium, Calcium, Hardness, Iron, Chloride, BOD and COD in  Narasapura lake water from January 2024 to December 2024 are shown in the figures 2 - 11.

Fig.2. pH Content in the water                              Fig.3. Total Alkalinity Content in the water

Fig.4. TDS Level in the water                              Fig.5. Magnesium Content in the water

Fig.6. Calcium Level in the water                              Fig.7. Hardness of water

Fig.8. Iron Content in the water                              Fig.9. Chloride Content in the water

Fig.10. BOD Level in the water                              Fig.11. COD Level in the water

The results of the present study showed (Figures 2 to 11) monthly variation of selected physico-chemical parameters—pH, total alkalinity, total dissolved solids (TDS), and magnesium—in Narasapura Lake at three sampling locations: inlet water, lake bund middle, and backwater region over the study period.

The pH values exhibit slight but consistent seasonal variation, remaining within the acceptable range of 6.5 to 8.5 throughout the study period. At the beginning of the year (January to April), pH values are relatively stable, ranging between approximately 7.5 and 8.0. A noticeable decline is observed during the pre-monsoon and early monsoon months (May to July). This reduction may be attributed to dilution effects and increased organic matter decomposition. Total alkalinity shows moderate temporal and spatial variation across the sampling locations. Higher values are observed during the initial months (January to April), indicating greater buffering capacity of the water. A significant decline occurs during the monsoon period (June to August), with the lowest values recorded in July, especially in the backwater region. This decrease is likely due to dilution from rainwater and reduced concentration of bicarbonates.

TDS levels are relatively high and stable across all sampling sites. Further, a sharp decline is observed during May to July, with an extreme drop in June and July, especially in the backwater region where values approach minimal levels. This trend clearly indicates dilution effects caused by monsoon inflow and increased water volume.

Magnesium concentration exhibits relatively minor fluctuations throughout the study period. By December, values tend to stabilize again. Overall, magnesium levels remain within acceptable limits across all sampling points. Calcium level ranged between ~30–60 mg/L. however, the higher values are observed during January–March and October–November, especially at the inlet, due to mineral inflow and evaporation and lowest values occur during June–July, caused by rainwater dilution during monsoon. The total hardness ranges from ~150–300 mg/L showed higher level in January–April and October, especially at the inlet and bund middle and lower during June–August because of monsoon dilution. Whereas, Back water has slightly lower hardness compared to other sites. The Iron concentration ranges between 0.08 to 0.20 mg/L.. Slight increase is seen during February–April and October and further lowered during June–August due to rainfall dilution. Back water sometimes shows slightly higher iron than other sites. Chloride values range from 100 to 200 mg/L. Higher values are observed in summer months (April–June) due to evaporation and concentration. Lower values occur during monsoon (July–September) due to dilution. COD ranges from ~70 to 220 mg/L. Higher values are observed in early months (Jan–Apr), especially at the bund middle. Further the values declined May onwards, reaching lower values by September–December. The BOD values are high at the beginning (50–80 mg/L) and decrease gradually. A sharp decline occurs after July, reaching very low values (5 mg/L) by November–December. Later low values indicate improved water quality.

The experiment results showed there was no abnormal values observed in any of the parameters or in any season, most of the parameter’s values are within the acceptable limit and few are within the permissible limit. However, the fluctuation in the all the parameters showed due to inflow of water, prolong stagnation and none domestic interference.  Previous studies have similarly reported lakes (Chandra et al.,2012; Musa et al., 2020; Bikila, et al., 2020; Akinbile & Yusuff, 2021; Pratibha et al., 2021; Simeon et al., 2022).

References

Akinbile, C. O., & Yusuff, R. O. (2021). Assessment of water quality in urban and rural areas in southwestern Nigeria. Environmental Monitoring and Assessment, 193(9), 1–12.

Bikila, E. T., Akinyemi, M. B., & Alabi, B. O. (2020). Assessment of the physicochemical and microbiological quality of drinking water sources in rural areas of Ethiopia. Water Quality Research Journal of Canada, 55(1), 1–9.

Chandra, S., Kumar, A., & Tomar, P. K. (2012). Assessment of water quality values in Porur Lake (Chennai), Hussain Sagar (Hyderabad), and Vihar Lake (Mumbai), India. Chemical Science Transactions, 1(3), 508–515.

Choudhury, S., Verma, S., & Kumar, R. (2022). Seasonal variations in the physicochemical properties of lake water: A comprehensive study. Journal of Environmental Management, 300, 113642.

Gupta, R., Sinha, A., & Sharma, N. (2022). Impacts of seasonal changes on water quality: A review. Water Research, 209, 117970.

Gupta, S., & Shukla, D. N. (2006). Physico-chemical analysis of sewage water and its effect on seed germination and seedling growth of Sesamum indicum. Journal of Natural Resources Development, 1, 5–19.

Jennings, W. C., Chern, E. C., O'Donohue, D., Kellogg, M. G., & Boehm, A. B. (2018). Frequent detection of a human fecal indicator in the urban ocean: Environmental drivers and covariation with enterococci. Environmental Science: Processes & Impacts, 20(3), 480–492.

Kumar, S., Singh, R., & Gupta, P. (2023). Impact of monsoon on turbidity and sedimentation in freshwater bodies. Environmental Science and Pollution Research, 30(4), 4621–4635.

Kumar, T. M. M., & Prabhu, N. (2019). Geochemical modelling of Kushawathi watershed area, Chikkaballapur District, Karnataka. International Journal of Scientific and Technology Research, 8(12), 1405–1409.

Mahendra Pratap Chaudhary, S. S. (2021). Assessment of water quality parameters of Man Sagar Lake, Jaipur, Rajasthan, India. Pollution Research, S90–S96.

Mishra, A., & B. (2020). Study of water quality in Udaipur region, India. Asian Journal of Water, Environment and Pollution, 17(1), 51–57.

Musa, R., Bello, S. A., & Adamu, S. M. (2020). Water quality and pollution status of selected water sources in Nigeria: Implications for sustainable development. Environmental Sustainability. https://doi.org/10.1007/s42152-020-00043-6

Panikkar, P., Saha, A., Prusty, A. K., Sarkar, U. K., & Das, B. K. (2022). Assessing hydrogeochemistry, water quality index (WQI), and seasonal pattern of plankton community in reservoirs of Karnataka, India. Arabian Journal of Geosciences, 15, 82.

Pratibha, R., Verma, S., & Rani, M. (2021). Toxicological impact of heavy metals on the environment and human health. Environmental Toxicology and Pharmacology, 79, 103413.

Purandara, B. K., Varadarajan, N., Venkatesh, B., & Choubey, V. K. (2012). Surface water quality evaluation and modeling of Ghataprabha River, Karnataka, India. Environmental Monitoring and Assessment, 184, 1371–1378.

Ravikumar, M. A., Mehmood, R. K., & Somashekar, R. K. (2013). Water quality index to determine the surface water quality of Sankey Tank and Mallathahalli Lake, Bangalore, India. Applied Water Science, 3, 247–261.

Simeon, T., Azazi, J. A., & Aliyu, M. A. (2022). Microbial contamination and health risks in drinking water sources: A study in rural Nigeria. Journal of Environmental Health Science & Engineering, 20(1), 207–215.

Singh, A. S. (2022). Heavy metal contamination of water and their toxic effects on living organisms. IntechOpen.

Vasistha, P., & G. (2020). Water quality assessment of natural lakes and its importance: An overview. Materials Today: Proceedings, 544–552.

Yao, F., Livneh, B., Rajagopalan, B., Wang, J., Crétaux, J.-F., Wada, Y., & Berge-Nguyen, M. (2023). Satellites reveal widespread decline in global lake water storage. Science, 380(6646), 743–749.