Dynamic Analysis of a Motor–Generator Feedback System with Voltage Amplification: A Detailed Analysis

Abstract

A self-coupled motor–generator feedback system is sometimes proposed as a method to sustain continuous power generation by feeding the electrical output of a generator back into the motor that drives it. In theory, if the output power equals or exceeds the input power, the system could operate continuously without external energy input. However, due to inefficiencies inherent in electrical machines and transformers, the total efficiency of such a system is always less than unity. This paper presents a detailed theoretical analysis of a motor–generator feedback loop and investigates whether the introduction of a step-up transformer can increase the feedback power sufficiently to sustain operation. Using energy conservation principles and efficiency models of the motor, generator, and transformer, it is demonstrated that the system cannot achieve self-sustained operation because total system losses always exceed any apparent voltage gain provided by a transformer.

Keywords—Motor–Generator Set, Energy Conservation, Thermodynamics, Efficiency, Perpetual Motion, Electromechanical Energy Conversion.

Introduction

Electric generators and electric motors are fundamental electromechanical devices. A generator converts mechanical energy into electrical energy, whereas a motor converts electrical energy into mechanical energy. When electrically and mechanically coupled, they form a closed energy conversion system commonly used in laboratories for performance testing, in industry for voltage/frequency conversion, and in renewable systems for energy transfer studies.

The coupling between these two machines can be:

1. Mechanical Coupling  – via shaft connection

2. Electrical Coupling  – via direct electrical connection

3. Electromechanical Coupling – combination of both

Motor–generator (MG) sets are widely used in electrical engineering applications such as frequency conversion, voltage stabilization, and electrical isolation. However, proposals occasionally emerge suggesting that a motor mechanically coupled to a generator could sustain itself by feeding back generated electrical power to the motor.  The concept of a self-sustaining motor–generator system has been proposed in various engineering discussions as a means of generating continuous electrical power without external energy input. In such a system, an electric motor drives a generator, and the electrical output of the generator is fed back to the motor to maintain rotation.

At first glance, it may appear that by manipulating electrical parameters—such as using a step-up transformer to increase voltage in the feedback path—the system could provide sufficient power to sustain itself. However, such systems are constrained by thermodynamic laws, particularly the First Law of Thermodynamics (energy conservation) and the Second Law of Thermodynamics (entropy increase).

The purpose of this paper is to analyze the power balance in a self-coupled motor–generator feedback system and to determine whether a step-up transformer can compensate for system losses and evaluates the feasibility of such a system using rigorous physical principles.

Discussion

Basic Principles of Operation

Electric Generator

A generator operates on Faraday’s Law of Electromagnetic Induction, which states:

e = -N (dф/dt)

where:

e   = induced emf

N  = number of turns

ф = magnetic flux

Mechanical rotation of the rotor in a magnetic field induces an electromotive force (EMF) in the stator windings.

Electric Motor

A motor works on the Lorentz Force Principle, where a current-carrying conductor placed in a magnetic field experiences a force:

F = B I L

This force produces torque:

T = k ф Ia

where:

T  = torque

ф = flux

Ia  = armature current

Types of Generator–Motor Coupling

Mechanical Shaft Coupling

In this method, the motor drives the generator through a rigid or flexible shaft coupling.

Applications:  Motor–Generator (MG) sets, Laboratory testing benches,  Frequency conversion systems

Advantages: Simple construction, Easy measurement of torque and speed

Electrical Coupling

The generator output feeds directly into the motor input.

Example: DC generator supplying DC motor,  Alternator supplying induction motor

Here, power balance equation applies:

P mechanical  = P electrical  + Losses

Combined Electromechanical Coupling

Used in closed-loop systems where the motor drives the generator, and generator output feeds back to the motor through control circuitry.

Used in:  Regenerative braking systems,  Renewable energy conversion

Power Transfer Analysis

For steady-state condition:

Pmotor input = V I cos ф

Pgenerator output = V I

Total system efficiency:

ηtotal​  = ηmotor ​× η generator

Losses include:

Copper losses ( I2R ), Iron losses (hysteresis + eddy currents),  Mechanical losses (friction and windage)

Dynamic Characteristics

Torque–Speed Matching

Proper coupling requires matching torque-speed characteristics: DC motor: Linear torque-speed curve, Induction motor: Non-linear curve, Mismatch may cause: Instability, Overheating,  Mechanical stress

Synchronization

In AC systems: Frequency of generator must match motor requirement, Phase sequence must be correct, Voltage regulation must be controlled

Control Strategies

1. Voltage Control – regulates motor speed

2. Field Control – adjusts generator output

3. Power Electronic Converters – improves efficiency and stability

4. Feedback Control Systems – PID-based regulation

Modern systems use:

PWM drives, Variable Frequency Drives (VFDs), Smart controllers

Applications

Motor–Generator (MG) Sets

Used for: Voltage stabilization, Frequency conversion,  Electrical isolation

Renewable Energy Systems: Wind turbine (generator) feeding motor-driven loads, Solar power systems

Electric Vehicles : Regenerative braking involves motor operating as generator.

Laboratory Testing; Used to determine:  Efficiency, Load characteristics, Performance curves

Experimental Setup (Typical Laboratory Arrangement)

Components:  DC/AC Motor, Generator, Coupling shaft, Tachometer, Voltmeter & Ammeter, Load bank

Procedure:

1. Start motor

2. Gradually apply mechanical load

3. Measure voltage, current, speed

4. Calculate efficiency

9. Advantages and Limitations

Advantages: Energy conversion flexibility,Voltage/frequency control, Educational importance

Limitations: Combined efficiency less than 100%, Mechanical wear, Alignment issues

System Description

The proposed system consists of: (i) an electric motor, (ii) a mechanical shaft coupling, (iii) an electric generator, and (iv) a feedback control circuit. Initial electrical power is supplied to start the motor. The generator output is then partially fed back to the motor while the remainder is intended for external load usage.

Energy Conversion Chain:

Electrical Energy → Mechanical Energy → Electrical Energy

Electromechanical Energy Conversion Theory

For a motor, electrical input power is given by:

Pin = V I

Mechanical output power of the motor:

Pmech = T ω

Motor efficiency is defined as:

ηm = Pmech / Pin

For the generator, electrical output power is:

Pout = Vg Ig

Generator efficiency is:

ηg = Pout / Pmech

Combined System Efficiency

The total system efficiency is the product of motor and generator efficiencies:

ηtotal = ηm × ηg

Since both ηm < 1 and ηg < 1, their product must satisfy:

ηtotal < 1

For self-sustained operation, Pout ≥ Pin is required. However:

Pout = ηtotal × Pin

Thus, because ηtotal < 1, it follows that Pout < Pin. Therefore, the system cannot sustain continuous operation.

Rigorous Thermodynamic Analysis

According to the First Law of Thermodynamics (Energy Conservation):

ΔU = Q − W

For steady-state operation in a cyclic system, ΔU = 0. Therefore:

Q = W

In real electromechanical systems, irreversible losses convert useful work into heat (Q_loss). Hence:

Woutput = Winput − Qloss

Since Q_loss > 0 due to entropy production, it follows that:

Woutput < Winput

From the Second Law of Thermodynamics, entropy generation (S_gen) must satisfy:

Sgen ≥ 0

This guarantees that no real cyclic energy conversion system can achieve 100% efficiency.

Numerical Example

Assume ηm = 0.92 and ηg = 0.91.

ηtotal = 0.92 × 0.91 = 0.8372

If Pin = 1000 W, then Pout = 837.2 W. A deficit of 162.8 W prevents self-sustained operation.

Discussion

The analysis confirms that the proposed feedback system falls under the category of a Perpetual Motion Machine of the First Kind. Such systems violate fundamental energy conservation laws. Practical motor–generator sets require continuous external energy input.

Basic Configuration of the Motor–Generator Feedback System

System Components: The system consists of:

- Electric Motor

- Mechanical Shaft Coupling

- Electric Generator

- Step-Up Transformer

- Feedback Circuit

Conceptual Operation

- External power initially starts the motor.

- The motor converts electrical power into mechanical power.

- The generator converts mechanical power into electrical power.

- Part of the generator output is fed back to the motor.

- The transformer increases voltage in the feedback loop.

Block Diagram of the System

Mathematical Model of the System

Let

Pin = Electrical input power to motor
Pm = Mechanical power output of motor
Pg = Electrical power output of generator

Motor efficiency:

ηₘ = Pm / Pin

Generator efficiency:

ηg = Pg / Pm

Therefore

Pg = Pin × ηₘ × ηg

Total system efficiency:

ηtotal = ηₘ × ηg

Power Balance in the Feedback Loop

For continuous operation:

Pg ≥ Pin

Substituting:

Pin × ηₘ × ηg ≥ Pin

Dividing by Pin:

ηₘ × ηg ≥ 1

However, since:

ηₘ < 1
ηg < 1

Therefore

ηₘ × ηg < 1

Hence

Pg < Pin

Thus the system cannot sustain continuous operation.

Effect of Introducing a Step-Up Transformer

A transformer changes voltage and current, but not power (ignoring small losses).

For an ideal transformer: Pprimary = Psecondary

If voltage increases by factor k: V₂ = kV₁

Current decreases: I₂ = I₁ / k

Thus

P₂ = V₂ × I₂
P₂ = (kV₁)(I₁/k)
P₂ = V₁I₁ = P₁

Therefore a transformer cannot increase power.

Transformer Efficiency

Real transformers also have losses: ηt ≈ 0.95 – 0.98. Thus Pfeedback = Pg × ηt

Total efficiency becomes:   ηtotal = ηₘ × ηg × ηt

Example:

Motor efficiency = 0.9
Generator efficiency = 0.9
Transformer efficiency = 0.97

ηtotal = 0.9 × 0.9 × 0.97

ηtotal = 0.7857

Thus only 78.6% of input power returns to the motor.

Therefore: Pfeedback < Pin

External power is always required.

Thermodynamic Considerations

Energy First Law of Thermodynamics

cannot be created or destroyed.

Pin = Pout + Losses

Losses include:

- Copper losses

- Iron losses

- Mechanical friction

- Windage losses

- Transformer losses

Second Law of Thermodynamics

Every real process increases entropy.

Energy degradation occurs through:

- Heat generation

- Electrical resistance

- Magnetic hysteresis

Therefore 100% efficiency is impossible.

Entropy Generation Analysis

Entropy generation due to losses:

Ṡgen = Qloss / T

Where

Qloss = Pin − Pfeedback

Since losses > 0

Ṡgen > 0

Thus the system moves toward thermodynamic equilibrium and cannot sustain itself.

Practical Engineering Example

Assume

Motor input power = 1000 W

Motor efficiency = 90%

Mechanical power

Pm = 1000 × 0.9
Pm = 900 W

Generator efficiency = 90%

Electrical output

Pg = 900 × 0.9
Pg = 810 W

Transformer efficiency = 97%

Feedback power

Pfeedback = 810 × 0.97
Pfeedback = 785.7 W

Thus   Loss = 1000 − 785.7  = 214.3 W

External power of 214 W must continuously be supplied.

Although a step-up transformer increases voltage, it does not increase energy. Because the product of voltage and current determines power, any increase in voltage corresponds to a proportional decrease in current.

Consequently: Voltage gain ≠ Power gain.

The motor-generator feedback loop always suffers cumulative losses from:

- electromagnetic conversion

- mechanical friction

- resistive heating

- magnetic hysteresis

- transformer losses

These losses guarantee that the total efficiency remains below unity.

Conclusion

The coupling between electric generator and electric motor represents a fundamental electromechanical energy conversion system. Proper mechanical alignment, electrical matching, and control strategies are essential to ensure efficient and stable operation. Advances in power electronics and smart control techniques have significantly improved performance, making generator–motor coupling systems highly relevant in renewable energy, electric vehicles, and industrial applications.

A rigorous electromechanical and thermodynamic analysis demonstrates that a self-coupled motor–generator feedback system cannot operate autonomously. The total system efficiency is inherently less than unity due to unavoidable loss mechanisms. Continuous energy generation without external input is therefore physically impossible.

This study analyzed a self-coupled motor–generator feedback system with a step-up transformer. Mathematical modeling and thermodynamic principles demonstrate that:

- The total system efficiency is the product of individual efficiencies.

- Since each efficiency is less than unity, the overall efficiency is also less than unity.

- A transformer cannot increase power, only voltage.

- Energy losses in the motor, generator, and transformer prevent continuous self-operation.

Therefore, a self-sustaining motor–generator system is physically impossible, even with the inclusion of a step-up transformer.

Scope for Future Research

Future research may focus on:

- High-efficiency motor–generator sets

- Energy storage assisted feedback systems

- Smart power management using power electronics

- Hybrid renewable-energy coupled generator systems

Such systems cannot create energy but may improve overall system efficiency.

References

- Fitzgerald, A. E., Kingsley, C., & Umans, S. D. Electric Machinery. McGraw-Hill.

- Chapman, S. J. Electric Machinery Fundamentals. McGraw-Hill.

- Callen, H. B. Thermodynamics and an Introduction to Thermostatistics. Wiley.

- Van Valkenburg, M. E. Network Analysis. Prentice Hall.

- IEEE Power Engineering Society, Energy Conversion Systems.

- H. Goldstein, C. Poole, and J. Safko, Classical Mechanics, 3rd ed. Pearson, 2002

- Y. A. Çengel and M. A. Boles, Thermodynamics: An Engineering Approach, 8th ed. McGraw-Hill, 2015

- S. J. Chapman, Electric Machinery Fundamentals, 5th ed. McGraw-Hill, 2011

- D. P. Kothari and I. J. Nagrath, Electric Machines, 4th ed. McGraw-Hill, 2010

- R. C. Dorf and J. A. Svoboda, Introduction to Electric Circuits, 9th ed. Wiley, 2014.