# A Comparative Performance Analysis of Cache Optimization Techniques Using the gem5 Simulator

Abstract—The cache memory is quite important in enhancing the overall performance of the processor since it will reduce the average amount of time it requires to refer to the memory. It has been noted that as the workloads become complex, the efficiency of the cache optimization techniques has been inevitable in enhancing the performance of the systems. This paper aims at conducting a comparative analysis on the application of the cache optimization techniques through the gem5 simulator. The current analysis has examined the functionality of the cache optimization techniques, including efficient cache optimization depending on the cache size, prediction of the way, pre- fetching of the data, and critical word first technique. General analysis has been done in a controlled environment, The efficacy of cache optimization techniques has also been evaluated using standard simulation bench- marks with regard to their effectiveness in increasing the number of cache hits, reducing average time to access the system memory and lastly optimizing system performance. The overall analysis has been done based on the mean access time of memory in the assessment of the system performance. The analysis shows effects of cache optimization when using architectural cache optimization in enhancing the system overall effectiveness.

Keywords—Cache memory, replacement policies, cache optimization, cache simulator gem5, cache instrumentation, memory hierarchy, prefetching, LRU, FIFO, cache perfor- mance analysis.

- Introduction

The CPU- RAM performance difference is further increas- ing in modern-day computing, and there are few solutions. Nevertheless, cache memory is still the most viable one. It assists in alleviating the constriction that is referred to as the Memory Wall. One significant problem in this area is that it is by nature difficult to be implemented on physical equipment. Thus gem5 helps us to explore the design space and can be used to test things like capacity, associativity and replacement policies. gem5 is a correct simulation of these architectures,

and it faithfully models the x86 microarchitectural behaviors. The simplest is to simply expand the cache size but this would also have diminishing returns and power goes to waste.

The key contributions of this paper are:

- Performance Testing: Performance testing was done to determine the direct effect of various cache size and settings (both data and instructions) on the speed of the system using performance benchmarks.

- Cache Replacement Policy Comparison: We compared several cache replacement policies, such as LRU, FIFO and examined how they affected cache hit and miss rates under various workloads.

- Thorough Prefetcher Analysis: We compared nine dif- ferent prefetching techniques, proving that intelligent data prefetching might result in a miss rate that is as low as

1.91 percentage points lower than that of a basic cache setup.

Although these metrics are handy at the high level, they con- ceal very important internal behaviours directly affecting the performance and design optimization possibilities. Moreover, systematic quantitative comparisons of several replacement policies and prefetching strategies across varied benchmarks have hardly been performed through the prior research. This paper fills both the observability gap and the replacement policy evaluation gap as it:

- Quantitative analysis of five cache replacement policies (LRU, BRRIPRP, FIFO, Random, MRU) in full detail.

- Comparison of prefetching techniques in a systematic analysis of nine methods on the same benchmarks.

- Understanding the trade-offs (size, block size, associa- tivity) of cache parameters in a variety of bench- marks.

- Measuring performance effect of replacement policy and prefetcher selection.

- Offering evidence-based advice on the design of practi- cal caches.

- It can be shown that the replacement policy choice and prefetching strategy are first-order design parameters that have to be carefully considered.

The rest of the paper will be structured as follows. Section II examines related literature on the field of cache simulation and analysis. Section III explains our experimental approach, such as gem5 simulator setup and evaluation methodology.

IV is our results section and it contains trade-offs in the cache parameters, trade-offs in the replacement policy and our prefetcher performance analysis. In section V there is a discussion on optimization methods and their validation. Section VI summarizes and describes the future work.

- Related Work

Analysis of cache performance has been widely investigated on the basis of analytical model, concrete hardware mea- surements and simulation. Hennessy and Patterson developed a quantitative paradigm of the memory hierarchies analysis revealing the effect of the parameters of the cache on the policy of replacing it and the performance [2]. The seminal work by Jouppi on caching victims proved that small, fully- associative caches may be used successfully in minimizing conflict misses in direct-mapped designs [3].

Gem5 simulator has been adopted as the de facto standard in microarchitectural studies due to its flexibility, modularity, and it is capable of emulating real hardware in high fidelity. Many other experiments have been done using gem5 to tie up the cache sizing, replacement policies and prefetching mechanisms. Nevertheless, these previous efforts have always been based on aggregate statistics, where only the hit/miss numbers and AMAT have been reported and no detailed quantitative policy comparisons have been done.

Qureshi et al. suggested the DIP (Dynamic Insertion Policy) that dynamically switches between LRU and bipartite insertion policies depending on the behavior of workloads and found out that the performance of LRU is highly dependent on workloads [4]. The study of cache replacement policy sensitivity to initial hardware state by Reineke and Grund found that initial hardware state can be sensitive to configuration details not always considered in simulation [6]. The prefetching strategies are well researched in the recent literature. Joseph and Grun- wald introduced the Markov Prefetcher, which predicts future cache references based on the history of cache misses [7]. Subsequent work by Palacharla and Kessler introduced stride prediction, where consecutive memory access patterns are detected and exploited [10]. More recent approaches, such as Signature Path (SPP) and SMS prefetchers, use sophisticated pattern recognition to identify and predict complex access sequences. However, comprehensive quantitative comparisons of multiple prefetching strategies within a single experimental framework remain limited. Despite advances in replacement policy and prefetching research, comprehensive quantitative

comparisons of multiple policies and prefetchers across diverse benchmarks using modern simulators remain limited. This paper fills that gap by providing empirical data on five widely- used replacement policies and nine prefetching strategies eval- uated systematically across four representative benchmarks.

- Experimental Methodology

- Setup for Simulation

The gem5 architectural simulator set up for the x86 in- struction set architecture was used for all experiments. Cache behavior was assessed under controlled conditions using a single-core TimingSimpleCPU model. Private L1 instruction and data caches and a unified L2 cache made up the typical two-level cache hierarchy used by the simulated system. Cache parameters were only changed when specifically needed for analysis, and the baseline configuration remained unchanged throughout the trials.

The baseline system configuration used in the experiments is summarized in Table I. Cache parameters were only changed when specifically required for analysis, while the remaining parameters were kept constant to ensure fair comparison across different experiments.

TABLE I

Baseline System Configuration

| Component | Configuration |
| --- | --- |
| CPU Model | TimingSimpleCPU |
| ISA | x86 |
| CPU Frequency | 2 GHz |
| Main Memory | 8 GB |
| L1 Cache Associativity | 4-way |
| L2 Cache Associativity | 8-way |
| Cache Line Size | 64 B |

- Selection of Benchmarks

To capture various kinds of data access patterns, a set of benchmarks from the SPEC CPU suite was selected. Work- flows with ordered, random, and mixed memory behaviors as well as CPU-intensive kernels are among the chosen applica- tions. This variety makes it possible to thoroughly assess cache optimization strategies under various workload conditions.

TABLE II

Benchmark Characteristics

| Benchmark | Benchmark Details |  |  |
| --- | --- | --- | --- |
|  | Application | Access Pattern | Workload Type |
| namd | Molecular Dynamics | Sequential | Memory-intensive |
| mcf | Network Flow | Random Access | Sparse patterns |
| blender | Graphics / Rendering | Mixed | Cache-friendly |
| cactus | Computational Science | Irregular | AMR-based |
| gcc | Compiler | Mixed | Instruction-heavy |

- Configuration of Cache and Assessment Measures

Individual cache parameters, such as cache size, block size, and associativity, were systematically changed while maintain- ing other parameters in order to investigate the effects of cache design decisions. The major performance indicators like cache hit, and miss count, hit rate, miss rate of the present

configuration, and the Average Memory Access Time (AMAT) were measured. These metrics were used to compare the efficacy of different techniques of cache optimization.

- Results and Analysis

A. Impact of L1 Data Cache Size on Hit and Miss Rates

The key features of the namd memory access patterns and relatively large memory bandwidth requirement. This behavior is suitable in studying the impact of the cache capacity on performance. In order to explore this effect, the L1 data cache size was keeping the cache block size constant at 64 B. In order to remove the effect of cache associativity, two-way cache associativity was limited.

Spatial locality can be exploited by the benchmark effi- ciently due to sequential access patterns, which enhance the increases in cache hit rate with an increase in cache capacity. The data collected on the relation between the size of the L1 data cache and the hit and miss rate of the namd workload are summarized in Table II.

minimal extra advantage. Additionally, increased associativity adds extra hardware costs and possible access delays, which may surpass its slight benefits. These findings emphasize the necessity of customizing cache design decisions based on workload traits instead of relying solely on high associativity by default.

C. Cache Replacement Policy Comparison

The cache replacement policy is one of the most important but underappreciated factors. Hit and miss rates are directly impacted by the distinct eviction decisions made by vari- ous policies. We thoroughly examined five popular cache replacement strategies using our benchmark suite. The results of our replacement policy analysis with a typical baseline configuration are shown in the following table.

TABLE IV

Replacement Policy Performance Across Benchmarks

| Benchmark | Hit Rate (%) by Replacement Policy |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | LRU | BRRIP | FIFO | Random | MRU |
| namd | 96.05 | 95.79 | 95.78 | 95.60 | 91.29 |
| mcf | 96.40 | 96.21 | 96.19 | 96.07 | 93.58 |
| blender | 95.66 | 95.40 | 95.39 | 95.22 | 89.99 |
| cactus | 96.09 | 95.79 | 95.76 | 95.58 | 90.05 |
| gcc | 91.60 | 91.42 | 91.45 | 91.38 | 89.24 |

TABLE III

L1-D Cache Size vs Hit/Miss Rate

| Cache Size (KB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Hits | Misses | Total | Hit Rate (%) | Miss Rate (%) |
| 4 | 65948 | 8437 | 74385 | 88.66 | 11.34 |
| 8 | 69426 | 4959 | 74385 | 93.33 | 6.67 |
| 16 | 70616 | 3769 | 74385 | 94.93 | 5.07 |
| 32 | 71323 | 3062 | 74385 | 95.88 | 4.12 |
| 64 | 71604 | 2781 | 74385 | 96.26 | 3.74 |
| 128 | 71714 | 2671 | 74385 | 96.41 | 3.59 |
| 256 | 71749 | 2636 | 74385 | 96.46 | 3.54 |
| 512 | 71754 | 2631 | 74385 | 96.46 | 3.54 |

Cache Performance Metrics

Analysis: This capacity seems to have the working set of the namd benchmark in it, as the performance gains diminish when the cache size exceeds 64 KB. When this threshold is exceeded, only a few new memory blocks are captured by additional cache space to achieve a small miss rate reduction. Such a phenomenon is in line with the theoretical hypotheses of the effects of cache capacity, according to which the benefits decrease in case the conflict misses is minimized, and the working set is completely accommodated.

B. Cache Associativity Impact

Associativity of the cache is another significant aspect of the nature of a cache which determines the number of the cache lines which can be accommodated on a single set of a cache and it has a significant effect on the chances of conflict misses. In situations where memory access patterns are regular or organized, raising associativity can help avoid unnecessary evictions and improve cache effectiveness. To investigate this effect, the influence of varying cache associativity was exam- ined using the namd benchmark, while keeping all other cache parameters constant.

Analysis: The slight enhancement seen with increased as- sociativity indicates that the majority of cache misses in namd are due to capacity or compulsory misses instead of conflicts. As the working set and access pattern are already effec- tively aligned with the cache, boosting associativity provides

- LRU (Least Recently Used) Performance: The Dominant Strategy: On all the studied benchmarks, LRU has the best cache hit rate. As an illustration the hit rate is 96.40% at the mcf benchmark and is not less than 95 percent at most workloads. This finding affirms that LRU-based exploitation of temporal locality is still very effective with a large variety of application behavior.

- BRRIP and FIFO—A similar Intermediate Perform- ance: BRRIP and FIFO perform equally on most benchmarks. Their hits are usually within 0.2 - 0.3 points of LRU. This means that the policies capture weakly less of the locality with respect to time, but can be capable of competitive performance with sim- pler complexity of implementation.

- Random Replacement—Simple and non-optimal: Simple random replacement works slightly worse than LRU, BRRIP and FIFO in most benchmarks. Much as its implementation is very simple, it loses access history knowledge resulting in a slight yet steady hit rate decrease.

- MRU (Most Recently Used)—Poor Performance: MRU has the lowest hit rate of all benchmarks. Specifically, it does decline to 89.99 per cent on the blender workload and 89.24 per cent on gcc. The eviction principle of the most recently accessed block means that MRU directly violates the principle of temporal locality and causes much more misses.

- Quantitative Performance Variance Analysis: Which is an analysis between the most performing policy (LRU) and the least performing policy (MRU) across the empirical bench- marks, the difference between the best and the worst policy (LRU and MRU) is up to 5-6 percentage points in hit rate. These differences indicate that replacement policy can have a great influence on the performance of the cache.

- Design Implications from Policy Comparison: The ex- perimental findings on various benchmarks give valuable in- formation in designing practical caches:

  - Standard Configuration: In all the work loads con- sidered, the LRU policy has the best rate of cache hits. This validates the observation that temporal locality exploitation can continue to be regarded to be very effective to most applications in the real world.

  - Constrained Systems: Limited Systems FIFO shows performance practically equivalent to LRU and BRRIP on most benchmarks and has lower implementation complexity. This renders it an effective option with systems that have tight hardware or power limits.

  - Alternative Policies: : BRRIP has similar performance to FIFO and LRU in a number of workloads, indicating that they are similar performance-wise.ing that advanced policies of insertion can give competitive performance based on the nature of workload.

  - Workload Sensitivity: Cache replacement should not be judge-lail in response to application behavior. The irregularly accessed workloads might not react as well as the workloads having a high temporal locality.

  - MRU Avoidance: MRU has the lowest hit rate of all the benchmarks considered in that it removes the most recently used cache blocks and tends to result in high miss rates.

D. Prefetcher Performance Analysis

Prefetching is an enhancement of the cache replacement and the cache siz- ing mechanism where data is brought to the cache by the time the processor needs it. Prefetching can penalise less and the system performance can be better by anticipating accessing future memory through pre fetching. A number of hardware preprefetching strategies have been con- sidered in this study to know the effects of various prediction mechanisms on both cache efficiency and a wide range of workloads.

- Prefetcher Evaluation Framework: All prefetcher exper- iments were conducted using the same baseline configuration as the replacement policy analysis (L1-D: 32 KB, 2-way, 64 B blocks; L2: 256 KB, 8-way, 64 B blocks). The experiments were performed across five benchmarks: namd, mcf, blender, cactus, and gcc. For each prefetching strategy, the cache hit rate was measured and compared with the baseline configura- tion without prefetching.

TABLE V

Prefetcher Performance Across Benchmarks (Hit Rate %)

| Benchmark |  |  |  | Prefetcher | Hit Rate | (%) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NoPref | Stride | Tagged | DCPT | SMS | SignaturePath | STeMS | Multi |
| namd | 96.05 | 97.43 | 97.96 | 97.45 | 96.32 | 97.94 | 96.06 | 96.05 |
| mcf | 94.12 | 95.28 | 95.71 | 95.32 | 94.36 | 95.66 | 94.15 | 94.12 |
| blender | 95.18 | 96.36 | 96.81 | 96.41 | 95.42 | 96.75 | 95.22 | 95.18 |
| cactus | 96.21 | 97.35 | 97.82 | 97.39 | 96.46 | 97.76 | 96.25 | 96.21 |
| gcc | 91.34 | 92.52 | 92.93 | 92.56 | 91.58 | 92.87 | 91.39 | 91.34 |

- Prefetcher Comparison Results:

- High-Performance Prefetchers: Tagged and Signa- turePath: Across all evaluated benchmarks, the Tagged Prefetcher and SignaturePath Prefetcher consistently achieve the highest cache hit rates. For example, in the namd workload, Tagged achieves a hit rate of 97.96% while SignaturePath achieves 97.94%. Similar trends are observed for other bench- marks such as cactus (97.82% and 97.76%) and blender (96.81% and 96.75%).

These results indicate that advanced pattern-based predic- tion mechanisms are highly effective in identifying complex memory access patterns and improving cache utilization.

- Stride and DCPT Prefetchers: Effective Stride Detec- tion: Stride and DCPT prefetchers also demonstrate strong performance across multiple workloads. The Stride Prefetcher achieves hit rates of 97.43% for namd and 97.35% for cactus, while DCPT achieves similar performance with hit rates of 97.45% and 97.39% respectively.

These results suggest that stride-based prediction techniques are particularly effective when applications exhibit regular or predictable memory access patterns.

- SMS Prefetcher: Moderate Effectiveness: The spatial Memory streaming (SMS) Prefetcher offers mediocre im- provements compared with the baseline config. Such as, its hit rate is raised to 96.05 to 96.32 when it comes to namd and also is raised to 94.12 to 94.36 percent when it comes to mcf. Nevertheless, it is not improving as much as more sophisticated prefetching algorithms.

- Limited Improvement: STeMS and MultiPrefetcher: There are also a few prefetching strategies, which achieve minimal or no improvement. There is only slight improvement by the STeMS Prefetcher which raises the hit rate by namd up to 96.06%. Likewise, the MultiPrefetcher does not achieve any quantifiable advantage giving it an identical hit rate as the baseline under various benchmarks.

These findings imply that the forecasting algorithms adopted by such prefetchers fail on the memory access pattern of the workloads assessed.

- Quantitative Prefetcher Performance Analysis: As indi- cated in the findings of the experiment, there is obviously a performance hierarchy between the analyzed prefetchers. In all benchmarks, Tagged and SignaturePath record the highest hit rates as compared to DCPT and Stride. SMS only has moderate benefits, whereas STeMS and MultiPrefetcher have mini-mal benefits relative to the basic setup.

Indicatively, the best to best performance at 97.96% of namd shows a gain of almost 2 percentage points on the best to baseline configuration of 96.05%. The same trends can be observed among other benchmarks.

These results indicate that one of the key points in cache design is having a way to choose the prefetch- ing strategy that will be effective. Working together with a high-performance prefetcher e.g. Tagged or Signa- turePath with an effective replacement policy can greatly enhance the efficiency of a cache and the performance of a system.

- Discussion

- Synergistic Effects of Replacement Policies, Prefetchers, and Cache Parameters

The synergistic relationships between replacement policy, prefetching strategy, and cache parameter are important after the overall analysis is done. A solid basis of cache per- formance is offered through effective replacement policies (LRU) whereby intelligent eviction decisions are made on the basis of temporal locality that is observed. Prefetchers build upon this base by actively pre-identifying and preloading data, responding to processor requests, essentially over-allocating cache capacity via prediction.

The 96.05% baseline hit rate of LRU replacement offers the baseline point on which prefetcher improvements can be made. In cases where intelligent prefetchers are used, the hit rates display a value of 97.96% ( Tagged Prefetcher ) which amounts to a cumulative 1.91 percentage points. The compatibility of these synergy shows that prefetchers and replacement policies deal with complimentary sides of cache performance: policies to evict data in the cache and prefetchers to decide which data to initially put in the cache.

- First-order Design Parametric Cache Replacement

The difference in performance of 4.76 percent between LRU and MRU is very high. In a processor of 100 billion instructions/sec:

  - Hit rate (96.05% LRU): Cache misses = 3.95 billion /s.

  - With 91.29% hit rate (MRU): Cache misses are expected to be 8.71 billion per second.

  - Variance: 4.76 billion more misses/s.

This calculation shows that the choice of replacement policy incurs first-order performance effects on the system that are similar to those of cache sizing choices, and that are as performance affecting as prefetching choices.

- Prefetching as a First-Order Design Parameter

The high enhancement of 1.91 percentage points that Tagged Prefetcher shows compared to baseline configuration shows that prefetching strategies deserve the classification as the first-order design parameters. Such an extent of enhance- ment is within the range of optimal to suboptimal replacement policies, and this suggests that prefetcher choice is a sensitive design choice that needs a methodical analysis.

Across the evaluated benchmarks, the improvement pro- vided by different prefetchers ranges from essentially no im- provement (MultiPrefetcher) to approximately 1.91 percentage points (Tagged Prefetcher).

- Policy-Independent Parameter Trade-Offs

Independent Trade-Offs between policies:

  - The size of cache and block size enhancements are usually policy-free.

  - The size of the cache decreases capacity misses of all policies and prefetchers.

  - The block size should be increased to enhance spatial locality in all the policies and prefetchers.

  - Optimization of parameters is enhanced by good replace- ment policies (LRU).

  - These benefits are reduced by poor replacement policies (MRU).

- Hardware Implementation Complexity vs. Performance: Prefetcher Perspective

The economic analysis of the interrelationship of prefetcher complexity and performance deserves close attention. The Stride Prefetcher has unique cost-effectiveness features: a simple counter-based hardware that is small in area and incurs only a small area overhead provides an improvement in hit rate of 1.38 percentage points. Contrarily, the Tagged and SignaturePath prefetchers have more complex pre- diction table format and logic, and incur an area overhead that can be 3-5 times that of Stride Prefetcher implementations.

In the case of performance sensitive apps the 1.91 per- centage point increase offered by Tagged Prefetcher warrants the complexity added in hardware. Nevertheless, in systems with power constraints, or area constraints, the 1.38 percent- age point performance enhancement of the Stride Prefetcher at very low cost, can be the best design alternative. This discussion indicates that the choice of prefetchers depends on a system constraint and application needs more than on maximizing performance, which does not consider the cost of implementation.

- Limitations

The limitations in this study are the following:

- Single-Core Focus: Single-core experiments are done; no multicore effects ( cache coher- ence, contention) are considered.

- Limited Benchmark Set: Five benchmarks were tested, the findings might not be applicable to workloads of all sizes.

- Simulation Only: The entire results consist of gem5 simulator execution, hardware validation would provide additional strength to conclusions.

- Static Configuration: Cache parameters were kept fixed when policy was being evaluated, parameterization ex- ploration is dynamic and will be pursued in the future.

- No Energy Analysis: Power consumption, which is a vital measure of a contemporary system, was not quantified with prefetcher implementations.

- Effects of Prefetcher Interactions: This experiment examined prefetchers on their own, there is no study that examines the interactions of multiple prefetchers simultaneously and whether they will be synergistic or antagonistic.

- False Positive Prefetching: Prefetcher induced cache contamination and false prefetch punishment are not measured.

In spite of these shortcomings, the research gives quantita- tive support to the replacement policy and prefetcher selection

and sets a standard of comparison in future comparative research.

- Conclusion and Future Work

The paper presents a detailed quantitative study of the cache replacement policy, prefetching policy and trade-offs in the cache parameters based on gem5 simulator. Our key findings are:

- Replacement Policy Evaluation: LRU consistently achieves the best performance across the evaluated benchmarks. For example, in the namd benchmark LRU achieves a hit rate of 96.05%.. BRRIP and FIFO have a similar performance (95.79% and 95.78%), indicating that more complexity is of little use. MRU shows low performance (91.29% hit rate), and it shows how important the principles of respecting the concepts of locality are in time.

- Prefetcher Evaluation: Tagged Prefetcher and Sig- naturePath Prefetcher perform with an outstanding level of performance (97.96 percent and 97.94 percent hit rates), showing 1.91 and 1.89 percentage point improve- ment respectively. Stride and DCPT Prefetchers have the less generous advances (1.38 and 1.40%). The SMS Prefetcher has low benefit (0.27% improvement) and Mul- tiPrefetcher do not increase the prefetching at all.

- Performance Variance: Replacement policy variance of The value of 4.76 percent and prefetcher variance value

1.91 percent reveals that both the parameters are first- order design specifications that have a similar effect on the performance of the system.

- Comparative Performance: Maximum cumulative im- provement with optimal replacement policy (LRU) and high-performance prefetching (Tagged, 97.96) achieve synergistic benefits with each other, improving cumula- tive performance by 1.91 percentage points over base- line.

These findings give evidence-based recommendations to the findings in the assessment of replacement policies and prefetchers in a particular workload and system. The large range of performance between policies and prefetchers high- lights the fact that any decision to design a cache should be carefully evaluated and not just simply adopt any single policy or strategy.

A. Future Work

A number of directions can be investigated further:

- Multicore Analysis: Multicore systems with cache co- herence and prefetcher inter-actions between cores.

- Wider Benchmarks: Add multithreaded, I/O heavy, and new workloads (deep learning, graph processing) as a test of generalizability of results.

- Energy Analysis: Determine prefetcher implementation power consumption and present energy-delay product (EDP) to permit cost-benefit analysis instead of perfor- mance metrics.

- Hardware Validation: Compare the simulated results with real data of the processor performance counters to determine the re-liability of gem5 predictions.

- Adaptive Policies: Research active switching of policy between LRU and FIFO depending on workload phase characteristics and runtime performance statistics.

- Prefetcher Combinations: Use combinations of com- plementary prefetchers (e.g., Stride + Tagged) to find out whether there is any synergistic effect that outweighs the overhead of multiple prefetcher implementations.

- Multi-Level Caches: Investigate the interactions between L1, L2, L3 caches using various policy and prefetchers at each level to optimize the hierarchical cache.

- Prefetcher Accuracy Metrics: Measure false positive prefetches, prefetch pollution, and usefulness measures to allow cost-benefit analysis to be done comprehensively, as opposed to only focusing on enhancements of the hit rate.

- Machine Learning Approaches: Explore replacements policies and prefetchers based on machine learning that are able to adapt to the nature of workload over time.

The overall data presented in the given paper gives a basis to the optimization work and policy development in the design of cache architectures in the future.

References

- N. Binkert et al., “The gem5 Simulator,” ACM SIGARCH Comput. Archit. News, vol. 39, no. 2, pp. 1–7, May 2011.

- J. L. Hennessy and D. A. Patterson, Computer Architecture: A Quanti- tative Approach, 6th ed., Morgan Kaufmann, 2019.

- N. P. Jouppi, “Improving Direct-Mapped Cache Performance by the Addition of a Small Fully-Associative Cache and Prefetch Buffers,” in Proc. 17th Int. Symp. Comput. Archit., May 1990, pp. 364–373.

- M. K. Qureshi, A. Jaleel, Y. N. Patt, S. C. Steely, and J. Emer, “Adaptive Insertion Policies for High-Performance Caching,” in Proc. 34th Int. Symp. Comput. Archit., June 2007, pp. 381–391.

- M. Kharbutli and Y. Solihin, “Counter-Based Cache Replacement and Bypassing Algorithms,” IEEE Trans. Comput., vol. 57, no. 4, pp. 433– 447, Apr. 2008.

- J. Reineke and D. Grund, “Sensitivity of Cache Replacement Policies to Initial Hardware State,” ACM Trans. Embed. Comput. Syst., vol. 9, no. 4, pp. 1–39, 2010.

- D. Joseph and D. Grunwald, “Prefetching Using Markov Predictors,” in Proc. 24th Int. Symp. Comput. Archit. (ISCA), June 1997, pp. 144–155.

- A. Sembrant, T. Eklov, and E. Hagersten, “Characterization of the Memory Hierarchy in an AMD Phenom and Intel Core i7 Processor,” in Proc. IEEE Int. Symp. Workload Charact., Nov. 2010, pp. 1–10.

- gem5 Contributors, “gem5 Documentation,” [Online]. Available: https:

//www.gem5.org/documentation/. Accessed: Jan. 2025.

- S. Palacharla and R. E. Kessler, “Evaluating Stream Buffers as a Secondary Cache Replacement,” in Proc. 21st Int. Symp. Comput. Archit., Apr. 1994, pp. 24–33.

- A. Jaleel, W. Hasenplaugh, M. Qureshi, J. Sebot, S. C. Steely, and J. Emer, “Adaptive Insertion Policies for Managing Shared Caches,” in Proc. 17th Int. Conf. Parallel Arch. Compil. Techn., Oct. 2008, pp. 208– 219.