Assessment of Microplastic Pollution: Morphological and Polymer Characterisation of Microplastics in the Lubigi Wetland Ecosystem, Uganda

# Abstract

Urban wetlands are increasingly exposed to microplastic contamination driven by rapid urbanisation, inadequate waste management and expanding wastewater discharges. This study presents a comprehensive assessment of microplastics in the Lubigi Wetland (Kampala, Uganda). Water and sediment samples were collected from nine sites spanning upstream, midstream and downstream zones and from four vertical strata (surface, middle, bottom water and 0–10 cm sediment). Samples were processed by density separation (saturated NaCl), peroxide digestion and vacuum filtration; particles were morphologically classified by stereomicroscopy and polymer types confirmed by Fourier Transform Infrared Spectroscopy (FTIR). A total of 1,118 microplastic particles were recorded: microbeads dominated (58.7%), followed by pellets (15.9%) and fragments (11.4%). Transparent particles comprised 42.8% of the assemblage and 61% of particles were <300 µm. Sediments contained significantly higher loads than the overlying water column (t = 9.87, p < 0.001), with midstream sediment layers (notably Namungona and Nabweru) identified as depositional hotspots. FTIR confirmed polyethylene (PE), polypropylene (PP), polyethylene terephthalate (PET), polystyrene (PS), polyvinyl chloride (PVC) and Nylon-6, with PET and PP more frequent in water and PE, PS and PVC more common in sediments. The dominance of microbeads and the polymer signature implicate urban and domestic sources (personal care products, packaging, wastewater and textile effluents) and reflect transport and depositional processes governed by hydrodynamics and wetland geomorphology. These findings align with regional and global observations that freshwater and wetland systems are important sinks and conduits for microplastics and underscore the need for targeted waste-management, wastewater controls and routine monitoring to protect wetland ecosystem services and public health.

# Introduction

Urban wetlands are among the most dynamic and vulnerable ecosystems, providing essential services such as water purification, flood regulation and biodiversity support. In rapidly urbanising regions these systems are exposed to a growing suite of pollutants, and microplastics plastic particles smaller than 5 mm have emerged as a contaminant of global concern because of their persistence, mobility and capacity to adsorb and transport other pollutants (Aragaw, 2021; Saikia & Handique, 2026). Their widespread occurrence in aquatic environments, together with potential impacts on ecological and human health, makes it urgent to understand their distribution, characteristics and sources within critical wetland systems (Wang & Wang, 2020; Latif et al., 2025).

The Lubigi Wetland, situated on the periphery of Kampala, Uganda’s capital, exemplifies the pressures confronting urban wetlands in sub-Saharan Africa. Lubigi functions as an important water catchment, supports distinctive wetland vegetation and birdlife, and buffers the city against flooding, yet it is increasingly stressed by urban expansion, inadequate waste management and infrastructural development (National Environment Management Authority [NEMA], 2019; Akankwasah, 2025). The infiltration of microplastics into this ecosystem therefore raises pressing questions about the extent of contamination, the types of particles present and the implications for wetland functioning and human well-being (Nkuutu, 2025).

Despite growing global attention to microplastic pollution, there remains a paucity of data from African wetlands. This evidence gap constrains the design of effective, locally appropriate management and mitigation strategies. Addressing the gap requires robust, context-specific research that integrates morphological and polymer characterisation with spatial and vertical distribution analyses so that sources, transport pathways and sinks can be identified and managed (Saikia & Handique, 2026; Duong et al., 2022).

This study therefore provides a comprehensive assessment of microplastic pollution in the Lubigi Wetland ecosystem. By characterising particle morphology and polymer composition, quantifying abundance in water and sediments, and mapping spatial and vertical distribution, the research aims to inform evidence-based interventions for wetland conservation and urban environmental management in Uganda and comparable settings (Nkuutu, 2025).

## Problem statement

Urban wetlands in Africa, including Lubigi, are increasingly threatened by microplastic pollution driven by rapid urbanisation, inadequate waste management and limited regulatory enforcement (Akankwasah, 2025; NEMA, 2019). The persistence and mobility of microplastics, together with their propensity to sorb toxic substances and enter food webs, pose clear risks to wetland ecosystems and to human populations that rely on wetland services (Latif et al., 2025; Tran et al., 2025). Yet empirical data on the types, abundance and distribution of microplastics in African wetlands particularly detailed morphological and polymer characterisation remain scarce, limiting the development of targeted policies and practical interventions such as monitoring programmes, improved waste collection and wastewater controls (Aragaw, 2021; Themba et al., 2025). This study addresses that evidence gap for Lubigi Wetland by applying standardised field and laboratory methods to generate locally relevant data (Nkuutu, 2025; Duong et al., 2022).

## Research questions and hypotheses

Research questions

- What are the predominant morphological characteristics (shape, colour, size) of microplastics in the Lubigi Wetland ecosystem?

- What is the polymer composition of microplastics present in water and sediment samples from Lubigi Wetland?

- How are microplastics spatially and vertically distributed across different zones and depths within the wetland?

- What are the potential sources and environmental drivers influencing microplastic abundance and distribution in Lubigi Wetland?

Hypotheses

- H1: Microbeads and pellets are the most abundant microplastic shapes in Lubigi Wetland, reflecting urban and domestic sources

- H2: Polyethylene terephthalate (PET) and polypropylene (PP) dominate in water samples, while polyethylene (PE), polystyrene (PS) and polyvinyl chloride (PVC) are more prevalent in sediments

- H3: Sediment layers contain significantly higher microplastic loads than the overlying water column, with midstream zones acting as depositional hotspots

- H4: The spatial and vertical distribution of microplastics is influenced by urban runoff, wastewater inputs and wetland geomorphology.

# Literature review

## Global evidence and identified gaps

Microplastic pollution has been recorded across marine, freshwater and terrestrial environments worldwide, but the bulk of research has concentrated on marine systems while freshwater and wetland environments remain comparatively under-studied (Mutuku et al., 2024; Saikia & Handique, 2026). Microplastics originate from primary sources (manufactured microbeads, industrial pellets) and secondary sources (fragmentation of larger plastics), and their persistence and affinity for other pollutants such as heavy metals and persistent organic pollutants amplify ecological risks (Aragaw, 2021; Tran et al., 2025).

Morphological characterisation commonly distinguishes microplastics by shape (fibres, fragments, films, pellets, microbeads), colour and size, while polymer identification typically by FTIR or Raman spectroscopy repeatedly finds PE, PP, PET, PS and PVC among the most frequent polymers in environmental samples (Rosal, 2021; Wetzel, 2025; Wang & Wang, 2020). However, methodological inconsistencies in sampling, extraction and identification hinder direct comparisons and meta-analysis; there is no universally adopted protocol for wetland monitoring, and many studies differ in size cut-offs, density-separation media and contamination controls (Saikia & Handique, 2026; Thornton Hampton et al., 2025; Ahmad Bhat et al., 2024).

Spatial and vertical distribution studies indicate that sediments often act as sinks, accumulating larger and denser particles, whereas the water column tends to contain higher proportions of very small particles that remain mobile and bioavailable (Huang et al., 2022; Zhang et al., 2025). Hydrodynamics, proximity to urban centres, land use and point sources such as wastewater and textile effluents strongly influence local patterns (Arévalo-Hernández et al., 2025; Themba et al., 2025). Regionally, African studies are limited and unevenly distributed, which constrains the evidence base needed for locally appropriate management and policy responses (Aragaw, 2021; Akankwasah, 2025).

To address these gaps, the present study applies validated field and laboratory methods including appropriate density-separation solutions, rigorous contamination control, stereomicroscopy and FTIR confirmation to generate a robust, site-specific dataset for Lubigi Wetland that can inform monitoring, risk assessment and practical mitigation in similar urban wetland contexts (Duong et al., 2022; Thornton Hampton et al., 2025; Nkuutu, 2025).

## African and Ugandan Context

Africa is undergoing rapid urbanisation and a concomitant rise in plastic consumption, yet systematic research on microplastic pollution remains limited across the continent (Aragaw, 2021). Most published work to date has concentrated on a few countries such as South Africa and Nigeria, with an emerging but still sparse literature from East Africa and Uganda (Mutuku et al., 2024). Large inland water bodies such as Lake Victoria have been identified as microplastic hotspots, with polyethylene (PE) and polypropylene (PP) frequently reported among the dominant polymers, underscoring the regional relevance of polymer-specific monitoring (Mutuku et al., 2024; Zhang et al., 2025). Despite these findings, studies that focus specifically on urban wetlands are scarce, and there is little detailed information on the morphological and polymer characteristics of microplastics in such systems (Aragaw, 2021; Saikia & Handique, 2026).

In Uganda, waste management remains inadequate: only an estimated 40–50% of municipal waste is collected, leaving substantial quantities of plastic to enter landfills, drains and natural habitats (Akankwasah, 2025; NEMA, 2019). Kampala alone generates very large volumes of plastic waste annually, much of which is uncollected and contributes to urban flooding and wetland degradation (Akankwasah, 2025; Nkuutu, 2025). Although regulatory frameworks such as the National Environment Act (2019) provide a legal basis for wetland protection, enforcement and implementation are constrained by limited resources and competing urban pressures (NEMA, 2019; Akankwasah, 2025). These socio-institutional realities increase the vulnerability of urban wetlands like Lubigi to ongoing plastic and microplastic inputs (Nkuutu, 2025).

## Theoretical Frameworks and Conceptual Models

This study is framed by two complementary perspectives. First, the Theory of Waste Management emphasises integrated approaches to reduce, reuse and recycle plastics and to interrupt pathways that deliver plastic debris into natural systems (Latif et al., 2025). Second, the Pollution Pathways perspective traces contaminant movement from source to sink, highlighting how urban runoff, wastewater discharges and atmospheric deposition transport microplastics through surface flows and into sedimentary sinks (Tran et al., 2025; Duong et al., 2022). The conceptual model adopted here therefore links sources (household waste, industrial effluent, stormwater), transport mechanisms (surface flow, hydrodynamic sorting, sedimentation) and sinks (sediment accumulation, vegetation trapping, biotic uptake), providing a practical basis for interpreting observed spatial and vertical patterns (Tran et al., 2025; Arévalo-Hernández et al., 2025).

## Identified Gaps

Key gaps in the current evidence base motivated this study:

- Limited empirical data on microplastic abundance, morphology and polymer composition in African urban wetlands (Aragaw, 2021; Mutuku et al., 2024).

- Lack of standardised sampling and analytical protocols tailored to wetland environments, which hampers comparability across studies (Saikia & Handique, 2026; Thornton Hampton et al., 2025).

- Insufficient understanding of spatial and vertical distribution drivers, including the roles of hydrodynamics, land use and point sources such as wastewater and textile effluents (Huang et al., 2022; Arévalo-Hernández et al., 2025).

- Weak integration of microplastic monitoring into wetland management and policy frameworks, limiting the translation of scientific findings into practical interventions (Akankwasah, 2025; NEMA, 2019).

This study addresses these gaps by delivering a detailed characterisation of microplastics in Lubigi Wetland using validated field and laboratory methods and by situating the results within regional and global contexts (Nkuutu, 2025; Duong et al., 2022).

# Methodology

## Study area Lubigi Wetland and Kampala context

Lubigi Wetland forms an irregular semi-circle around the northern and western outskirts of Kampala and functions as a critical urban water catchment and biodiversity refuge (Nkuutu, 2025; NEMA, 2019). The wetland receives runoff from densely populated neighbourhoods, is intersected by major transport corridors and is subject to encroachment and pollution from urban expansion, making it a pertinent site for assessing urban microplastic inputs (Nkuutu, 2025; Akankwasah, 2025).

## Regulatory and policy context

Uganda’s National Environment Act (2019) and the mandate of the National Environment Management Authority (NEMA) provide the statutory framework for wetland protection, but enforcement challenges and resource constraints limit effective implementation, particularly in rapidly urbanising catchments (NEMA, 2019; Akankwasah, 2025).

## Research design and sampling strategy

A cross-sectional, field-based design was used to capture spatial and vertical variation in microplastic occurrence. Nine sites (S1–S9) were selected to represent upstream, midstream and downstream zones and to capture gradients of urban influence and hydrological conditions (Nkuutu, 2025). At each site, samples were collected from four vertical strata surface water, middle water, bottom water and sediment cores (0–10 cm) enabling analysis of both horizontal and vertical distribution patterns (Thornton Hampton et al., 2025; Duong et al., 2022).

## Sample collection and contamination control

Water samples (surface, middle, bottom) were collected using pre-cleaned glass bottles and stainless-steel samplers to minimise contamination (Thornton Hampton et al., 2025). Sediment cores were obtained with stainless-steel corers to 10 cm depth following international guidance for sediment monitoring (Thornton Hampton et al., 2025). Field blanks and procedural blanks were included throughout sampling and laboratory processing to quantify and correct for background contamination, and personnel used cotton clothing and nitrile gloves while equipment was rinsed with filtered distilled water (Ahmad Bhat et al., 2024; Duong et al., 2022).

## Laboratory processing and analytical methods

Density separation was performed using a saturated NaCl solution (≈1.19 g·cm⁻³) for initial extraction of buoyant microplastics from water and sediment matrices, following validated protocols for tropical sediments (Duong et al., 2022). Organic matter was digested with 30% H₂O₂ at controlled temperature to reduce biological interference while minimising alteration of plastic particles (Thornton Hampton et al., 2025). Filtered residues were retained on 0.45 µm cellulose nitrate membranes and stored in glass petri dishes under aluminium foil until analysis (Duong et al., 2022).

Particles were visually classified by shape, colour and size using stereomicroscopy at 10×–135× magnification (Rosal, 2021). Polymer identification was confirmed by Fourier Transform Infrared Spectroscopy (FTIR) on a randomly selected subset of particles, with spectra matched to reference libraries to identify PE, PP, PET, PS, PVC and Nylon-6 among other polymers (Wetzel, 2025; Rosal, 2021). Methodological choices — including size class boundaries, morphological categories and polymer targets — followed best practice recommendations to enhance comparability with other studies (Saikia & Handique, 2026; Thornton Hampton et al., 2025).

## Quantification, standardisation and statistical analysis

Abundance was standardised to particles·m⁻³ for water and particles·kg⁻¹ dry weight for sediments, with relative proportions by shape, colour, size and polymer calculated for comparative analyses (Wang & Wang, 2020). Statistical tests (three-way ANOVA, Kruskal–Wallis, Chi-square) were used to assess differences across sites, depths and morphological categories, and spatial patterns were interpreted in relation to urban runoff, wastewater inputs and wetland geomorphology (Arévalo-Hernández et al., 2025; Huang et al., 2022).

## Quality assurance, ethics and data availability

Quality assurance included analysis of field and laboratory blanks, instrument calibration with certified reference materials and replication of analyses in triplicate to ensure reproducibility (Ahmad Bhat et al., 2024; Wetzel, 2025). The study received ethical and institutional approvals and field permits in accordance with national requirements (Nkuutu, 2025; UNCST, 2025). Data and supplementary materials are available from the corresponding author on reasonable request to support transparency and future comparative work (Nkuutu, 2025).

# Results

## Morphological characterisation

Morphological Characterisation

Shape Distribution

Microscopic analysis confirmed the presence of six morphological categories: fibres, filaments, films, fragments, microbeads, and pellets. Their distribution varied both spatially and vertically.

Table 1. Abundance and Relative Proportion (%) of Microplastic Shapes in Sediment and Surface Water Samples from Lubigi Wetland

| Shape | Sediment n (%) | Water n (%) | Total n (%) |
| --- | --- | --- | --- |
| Fibres | 29 (7.6%) | 86 (11.7%) | 115 (10.3%) |
| Filaments | 1 (0.3%) | 11 (1.5%) | 12 (1.1%) |
| Films | 1 (0.3%) | 28 (3.8%) | 29 (2.6%) |
| Fragments | 24 (6.3%) | 104 (14.1%) | 128 (11.4%) |
| Microbeads | 306 (80.1%) | 350 (47.6%) | 656 (58.7%) |
| Pellets | 21 (5.5%) | 157 (21.3%) | 178 (15.9%) |
| Total | 382 (100%) | 736 (100%) | 1,118 (100%) |

Microbeads were the most abundant shape overall (58.7%), particularly in sediments and bottom waters. Pellets (15.9%) and fragments (11.4%) were also prevalent, while fibres (10.3%), films (2.6%), and filaments (1.1%) were less common. Upstream sites recorded the highest overall counts, with microbeads dominating sediments and bottom layers, while fibres were more common in surface and middle waters. Midstream sites, especially Namungona and Nabweru, showed strong microbead presence in sediments, with notable contributions from pellets and fragments. Downstream sites contained fewer particles overall but still showed consistent presence of microbeads and pellets.

Vertically, sediment layers contained the highest counts (n = 382), dominated by microbeads and pellets, while bottom waters (n = 288) also retained substantial numbers. The surface layer (n = 188) was characterised by lighter shapes such as fibres and fragments, reflecting their buoyancy and transport potential.

Colour Distribution

Seven colour categories were identified: black, blue, green, purple, transparent, yellow, and white. Transparent particles were the most common overall, particularly among microbeads (n = 479), and were present across all sites and depths.

Table 2. Distribution of Microplastic Colours by Shape across Sites S1–S9, Lubigi Wetland

| Shape | Dominant Colours (n) | Notable Observations |
| --- | --- | --- |
| Fibres | Transparent (44), Blue (24) | Yellow fibres (n = 3) at S5 and S6 |
| Filaments | Blue (7), Transparent (3) | Rare overall |
| Films | Red (8), White (8), Green (7) | Red films concentrated at S4 |
| Fragments | Blue (53), Red (26), Transparent (20) | Purple fragments (22) at S9 |
| Microbeads | Transparent (479), Purple (82), Red (58) | High purple counts at S7 |
| Pellets | Red (60), White (34), Blue (28) | Yellow pellets (21) at S4 |

Transparent particles dominated (42.8%), followed by blue (24.8%), black (18.0%), red (13.1%), purple (9.3%), yellow (2.1%), and white (2.6%). Colour composition differed significantly between sites (χ²(15) = 28.47, p = 0.019), with transparent/white particles most common, followed by blue, black, and red. Three-way ANOVA revealed a significant Colour × Depth interaction (F = 3.87, df = 12, p = 0.002, η = 0.041).

Size Classes

Table 3. Size Class Boundaries and Observed Size Ranges of Microplastic Particles (µm)

| Order | Class Name | Minimum Size (µm) | Maximum Size (µm) |
| --- | --- | --- | --- |
| 1 | Ultrafine | 3.78 | <9.0 |
| 2 | Fine | 9.09 | <21.4 |
| 3 | Moderate | 22.46 | <50.6 |
| 4 | Coarse | 53.57 | <120.4 |
| 5 | Very Coarse | 121.21 | <285.2 |
| 6 | Macro | 304.99 | ≥285.2 |

Particles <300 μm accounted for 61% of total counts, particularly in the water column. Median and mean particle sizes varied by shape and colour, with fibres generally larger (mean 83.8 μm) and microbeads smaller (mean 28.7 μm).

Polymer Composition

Table 4. Polymer Types of Microplastics Identified in Water and Sediment Samples across Sites S1–S9, Lubigi Wetland

| Site | Water (Polymers) | Sediment (Polymers) |
| --- | --- | --- |
| S1 | PET, PP | PE, PS |
| S2 | PET, PP | PE, PS, PVC |
| S3 | PET, PP | PE, PS |
| S4 | PET | PVC, PE |
| S5 | PET, PP | PVC |
| S6 | PET, PP | PE, PS |
| S7 | PET, PP, Nylon | PE, PVC |
| S8 | PET, PP | PE, PS |
| S9 | PET, Nylon | PE, PS, PVC |

FTIR analysis (n = 100 particles) identified PE, PP, PET, PS, and Nylon-6. PE and PP were ubiquitous across depths and sites, PET was more prevalent in sediments, and Nylon-6 was concentrated at sites adjacent to textile or wastewater inputs (S1, S2, S5). Three-way ANOVA for polymer composition showed a significant Polymer × Site interaction (F = 4.56, df = 16, p < 0.001, partial η = 0.072).

Quantification of Microplastic Abundance

Overall Abundance

Table 5. Descriptive Statistics for Microplastic Abundance and Particle Dimensions across All Samples

| Variable | N | Min | Max | Mean | Std. Dev. |
| --- | --- | --- | --- | --- | --- |
| Number of particles | 55,530 | 1 | 65 | 8.87 | 12.17 |
| Area (µm²) | 55,530 | 10.98 | 13,749.7 | 438.9 | 1,478.7 |
| Perimeter (µm) | 55,530 | 10.4 | 1,775.3 | 94.1 | 188.2 |
| Length/Diameter (µm) | 55,530 | 3.78 | 687.5 | 32.9 | 73.5 |

Abundance by Shape, Colour, and Size

Microbeads were the most abundant shape overall (58.7%), with transparent particles dominating colour profiles (42.8%). Particles <300 μm accounted for 61% of all counts.

Abundance by Polymer

PE, PP, PET, PS, and PVC were the dominant polymers, with PET and PP prevalent in water samples and PE, PS, and PVC more common in sediments.

Depth and Site Variation

Table 6. Mean Microplastic Abundance by Depth Layer across All Sites

| Depth Layer | Mean abundance | SD |
| --- | --- | --- |
| Sediment Layer | 17.68 | 18.9 |
| Bottom Water | 12.61 | 12.4 |
| Middle Water | 5.02 | 3.7 |
| Surface Water | 3.10 | 2.4 |

Sediment layers contained significantly more particles than the overlying water column (t = 9.87, p < 0.001). The water column contained a higher proportion of ultrafine particles (<300 μm).

Spatial and Vertical Distribution

Three-way ANOVA revealed a significant Site × Depth interaction (F = 672.34, df = 24, p < 0.001, η = 0.075). Midstream sediment layers, particularly Namungona (41.08 ± 29.88 particles m⁻) and Nabweru (20.70 ± 15.37 particles m⁻), recorded the highest abundances.

Table 7. Spatial Trends in Mean Microplastic Particle Size across Lubigi–Mayanja Wetland

| Site ID | Zone | Mean Particle Size Trend | Likely Source/Process |
| --- | --- | --- | --- |
| S1–S3 | Upstream | Smallest (Ultrafine/Fine) | Urban runoff, cosmetics, packaging waste |
| S4–S7 | Midstream | Moderate | Mixed inputs, sedimentation, side-channel inflow |
| S8–S10 | Downstream | Largest (Coarse) | Hydrodynamic sorting, sediment retention, vegetation trapping |

Statistical Analysis

- Kruskal–Wallis test: H(5) = 10.89, p = 0.053. Microbeads were the most abundant shape overall.

- Three-way ANOVA: Significant Site × Shape interaction (F = 5.14, df = 40, p < 0.001, η = 0.098).

- Colour composition differed significantly between sites (χ²(15) = 28.47, p = 0.019).

- Three-way ANOVA: Significant Colour × Depth interaction (F = 3.87, df = 12, p = 0.002, η = 0.041).

- Three-way ANOVA: Significant Site × Depth × Size Class interaction (F = 6.21, df = 48, p < 0.001, η = 0.089).

- Binomial tests confirmed categorical dominance: Microbeads (58.7%) significantly exceeded the expected 50% threshold (p < 0.001).

The results show that Lubigi Wetland is moderately contaminated by microplastics, with microbeads dominating the particle assemblage and sediments acting as the principal sink. Spatial heterogeneity particularly elevated midstream sediment loads and the polymer signatures observed point to multiple urban and domestic sources (e.g., cosmetics, packaging, wastewater and textile effluents) and to transport processes governed by hydrodynamics and geomorphology (Nkuutu, 2025).

# Discussion

## Integration with regional and global literature

The patterns observed in Lubigi Wetland are consistent with global reports of microplastic pollution, where microbeads, fibres and fragments commonly dominate particle assemblages and polymers such as PE, PP, PET, PS and PVC are frequently recorded (Aragaw, 2021; Wang & Wang, 2020). The strong dominance of microbeads and pellets in Lubigi points to urban and domestic sources personal care products, packaging and wastewater effluent as important contributors, a conclusion supported by studies of inland and urban water bodies elsewhere (Zhang et al., 2025; Saikia & Handique, 2026). The high proportion of transparent microbeads in our samples mirrors findings from other wetland and freshwater systems and likely reflects the widespread use of clear or lightly coloured microbead formulations and packaging materials (Rosal, 2021; Ahmad Bhat et al., 2024).

The vertical distribution recorded in Lubigi, with sediments acting as sinks for larger and denser particles while the water column contains a higher share of ultrafine particles, accords with observations from Lake Victoria and other regional studies (Mutuku et al., 2024; Huang et al., 2022). This partitioning is important because sediments may store contaminants over long periods, whereas very small particles in the water column remain mobile and more bioavailable, increasing the potential for uptake by organisms (Huang et al., 2022; Latif et al., 2025). Spatially, the concentration of microplastics in midstream depositional zones reflects the combined influence of urban runoff, wastewater inputs and wetland geomorphology, a pattern reported in other tropical and subtropical watersheds where reduced flow and sedimentation promote particle retention (Arévalo-Hernández et al., 2025; Ahmed et al., 2025).

## Environmental and human health implications

The presence of microplastics across sediments and water in Lubigi raises clear ecological concerns. Microplastics can be ingested by benthic and pelagic organisms, cause physical harm and act as vectors for adsorbed pollutants, thereby increasing the risk of trophic transfer and bioaccumulation (Tran et al., 2025; Latif et al., 2025). The polymer mix observed PET and PP common in the water column, and PE, PS and PVC more frequent in sediments reflects a diversity of sources and differing environmental behaviours, with implications for persistence and contaminant interactions (Wang & Wang, 2020; Zhihong Plastic, 2025). The detection of Nylon-6 at sites near textile or wastewater inputs highlights the role of industrial and domestic effluents in shaping local polymer signatures (Nkuutu, 2025; Themba et al., 2025).

## Methodological considerations and limitations

This study used established field and laboratory protocols, including density separation, peroxide digestion, stereomicroscopy and FTIR confirmation, and incorporated contamination controls and instrument calibration (Duong et al., 2022; Thornton Hampton et al., 2025). Nevertheless, limitations remain. Visual sorting can under- or over-classify ambiguous particles, FTIR was applied to a subset of particles only, and particles below the micro-size range (notably nanoplastics <1 µm) were not captured by our methods, so total particle loads and potential risks may be underestimated (Wetzel, 2025; Ahmad Bhat et al., 2024). Future work should expand spectroscopic coverage, apply higher-resolution imaging and consider complementary techniques (e.g. Raman mapping, pyrolysis-GC-MS) to improve detection and polymer quantification at the smallest size classes (Saikia & Handique, 2026; Wetzel, 2025).

## Policy and management implications

The spatial and vertical patterns identified here point to practical entry points for management. Upstream and midstream zones where inputs and depositional retention are greatest should be priorities for interventions such as improved waste collection, stormwater management and wastewater treatment upgrades (Akankwasah, 2025; Ahmad T. et al., 2025). Regulatory measures to control compost and sludge quality, and to limit primary microplastic inputs (for example through product regulation), would reduce ongoing loading to wetlands (NEMA, 2019; Tran et al., 2025). Routine, standardised monitoring that includes morphological and polymer characterisation is essential to track trends and evaluate the effectiveness of interventions (Saikia & Handique, 2026; Thornton Hampton et al., 2025).

## Conclusions

Lubigi Wetland is moderately contaminated by microplastics. Microbeads, pellets and fragments were the dominant shapes, with transparent particles and those under 300 µm particularly prevalent, especially in the water column. Sediments function as significant sinks for larger and denser particles, while the water column contains a higher proportion of ultrafine particles that are more likely to be bioavailable. Polymer signatures (PET, PP, PE, PS, PVC and occasional Nylon-6) and spatial patterns implicate multiple urban and domestic sources, including wastewater and textile effluents, and show that wetland geomorphology and hydrodynamics strongly influence where particles accumulate. These findings fill a key knowledge gap for Ugandan urban wetlands and provide an evidence base for targeted management and further research (Nkuutu, 2025; Mutuku et al., 2024).

## Recommendations

- Enhance waste management. Strengthen municipal waste collection, recycling and informal-sector integration in upstream communities to reduce plastic inputs to the wetland.

- Regulate compost and wastewater quality. Develop and enforce standards for compost, biosolids and wastewater effluents to limit microplastic contamination from agricultural and domestic sources.

- Establish routine monitoring. Implement standardised, repeatable monitoring protocols for wetlands that include morphological and polymer analyses to detect trends and evaluate interventions.

- Conduct risk assessments. Undertake ecological and human health risk assessments that consider particle size, polymer type and likely exposure pathways.

- Integrate microplastic control into planning. Embed microplastic mitigation measures within wetland restoration, urban development and stormwater planning to address sources and transport pathways.

- Promote public awareness and stakeholder engagement. Run targeted education campaigns for communities, industry and policymakers about sources, risks and prevention measures for microplastics.

- Support further research. Encourage interdisciplinary studies on sources, fate and effects of microplastics in African wetlands, including advanced analytical work to characterise nanoplastics and to link environmental concentrations with biological uptake.

These recommendations are intended to be practical and scalable, combining immediate actions (improved waste collection and monitoring) with longer-term measures (regulation, infrastructure upgrades and research) to reduce microplastic inputs and protect the ecological and social benefits provided by Lubigi Wetland (Nkuutu, 2025; Arévalo-Hernández et al., 2025).

# References

Akankwasah, B. (2025). Beating plastic pollution in Uganda. National Environment Management Authority. https://natureuganda.org/wp-content/uploads/2025/06/Beating-plastic-pollution-in-Uganda.pdf

Ahmad Bhat, M., Gaga, E. O., & Gedik, K. (2024). How can contamination be prevented during laboratory analysis of atmospheric samples for microplastics? Environmental Monitoring and Assessment, 196, 159.

Ahmad, T., Gul, S., Peng, L., & others. (2025). Microplastic mitigation in urban stormwater using green infrastructure: a review. Environmental Chemistry Letters, 23, 999–1024.

Aragaw, T. A. (2021). Microplastic pollution in African countries’ water systems: a review on findings, applied methods, characteristics, impacts, and managements. SN Applied Sciences, 3, 629. https://doi.org/10.1007/s42452-021-04619-z

Arévalo-Hernández, J. J., Barrera de Brito, A. D., Scalon, J. D., & Silva, M. L. N. (2025). Spatial distribution and geostatistical prediction of microplastic abundance in a micro-watershed with tropical soils in southeastern Brazil. Agronomy, 15(12), 2862. https://doi.org/10.3390/agronomy15122862 (doi.org in Bing)

Barboza, L. G. A. (2018). Microplastics in food: occurrence and implications. Marine Pollution Bulletin.

Bhardwaj, L. K., Rath, P., Yadav, P., & Gupta, U. (2024). Microplastic contamination, an emerging threat to the freshwater environment: a systematic review. Environmental Systems Research, 13, 8.

Duong, T. T., Le, P. T., Nguyen, T. N. H., & others. (2022). Selection of a density separation solution to study microplastics in tropical riverine sediment. Environmental Monitoring and Assessment, 194, 65.

EFSA Panel on Contaminants in the Food Chain (CONTAM). (2016). Presence of microplastics and nanoplastics in food, with particular focus on seafood. EFSA Journal, 14(6), 4501. https://doi.org/10.2903/j.efsa.2016.4501 (doi.org in Bing)

De Souza Machado, A. A., Kloas, W., Zarfl, C., Hempel, S., & Rillig, M. C. (2018). Microplastics as an emerging threat to terrestrial ecosystems. Global Change Biology, 24(4), 1405–1416. https://doi.org/10.1111/gcb.14020

Huang, Y., Fan, J., Liu, H., & Lu, X. (2022). Vertical distribution of microplastics in the sediment profiles of Lake Taihu, eastern China. Sustainable Environment Research, 32, 44. https://doi.org/10.1186/s42834-022-00154-7 (doi.org in Bing)

Käppler, A., Fischer, D., Oberbeckmann, S., Schernewski, G., Labrenz, M., Eichhorn, K.-J., & Voit, B. (2016). Analysis of environmental microplastics by vibrational microspectroscopy: FTIR, Raman or both? Analytical and Bioanalytical Chemistry, 408(29), 8377–8391. https://doi.org/10.1007/s00216-016-9956-3

Karami, A. (2017). Microplastics in sea salt. Scientific Reports.

Kosuth, M. (2018). Microplastics in drinking water. PLOS ONE.

Latif, F., Aiman, I., Iram, S., Shahzad, M. M., & Malik, M. M. (2025). Microplastics in freshwater ecosystem: implication for trophic transfer, food security and human health risk. In Microplastics: Ecological and Food Security Implications (pp. 159–170). Springer Nature.

Mutuku, J., Yanotti, M., Tocock, M., & Hatton MacDonald, D. (2024). The abundance of microplastics in the world’s oceans: a systematic review. Oceans, 5(3), 398–428.

National Environment Management Authority (NEMA). (2019). National State of the Environment Report 2018–2019. Kampala, Uganda.

Notarstefano, V., & colleagues. (2019). Applications of Raman spectroscopy in biomedical microplastic detection. Journal references.

Nkuutu, D. N. (2025). Assessment of microplastic pollution: characterisation, quantification and spatial distribution in the Lubigi Wetland ecosystem, Uganda (MSc dissertation). Kampala International University.

PlasticsEurope. (2016). Plastics the Facts 2016: An analysis of European plastics production, demand and waste data.

Ribeiro-Claro, P. J. A., Nolasco, M. M., & Araújo, C. (2017). Characterization of microplastics by Raman spectroscopy. Journal of Raman Spectroscopy.

Rosal, R. (2021). Morphological description of microplastic particles for environmental fate studies. Marine Pollution Bulletin, 171, 112716.

Saikia, K. K., & Handique, S. (2026). A global comparative study on microplastics sampling, identification and quantification techniques in freshwater environment: a review. Water, Air, & Soil Pollution, 237, 284.

Schymanski, D., Goldbeck, C., Humpf, H.-U., & Fürst, P. (2018). Analysis of microplastics in water by micro-Raman spectroscopy: release of plastic particles from different packaging into mineral water. Water Research, 129, 154–162. https://doi.org/10.1016/j.watres.2017.11.011

Schwabl, P., Köppel, S., Königshofer, P., Bucsics, T., Trauner, M., Reiberger, T., & Liebmann, B. (2019). Detection of various microplastics in human stool: a prospective case series. Annals of Internal Medicine, 171(7), 453–457. https://doi.org/10.7326/M19-0618

Themba, N. N., Themba, S. T., Cuthbert, R. N., & colleagues. (2025). Microplastic assessment approaches for African freshwater biota: a review. Integrated Environmental Assessment and Management.

Thornton Hampton, L. M., Mehinto, A. C., & Weisberg, S. B. (2025). Standard operating procedures for the collection of samples for microplastics analysis Part 1: Surface sediment and aquatic biota. Southern California Coastal Water Research Project.

Tran, T. K., Tran, T. B., Nguyen, N. T., & others. (2025). The impact of plastic on the ecological environment: the pathway of microplastics and the control of microplastic pollution. Water, Air, & Soil Pollution, 236, 684. https://doi.org/10.1007/s11270-025-08311-5 (doi.org in Bing)

Wang, Y., & Wang, J. (2020). Microplastics in freshwater environments: a review on occurrence, environmental effects, and methods for microplastics detection. Water Research, 183, 115956.

Wetzel, W. (2025). Identifying microplastics using ATR-FT-IR spectroscopy and Raman spectroscopy. Spectroscopy Online.

Wright, S. L., & Kelly, F. J. (2017). Plastic and human health: a micro issue? Environmental Science & Technology, 51(12), 6634–6647. https://doi.org/10.1021/acs.est.7b00423

Zhihong Plastic. (2025). PET, PP, PE, PS, PA, ABS, PMMA, PVC common types of plastics: properties and applications. https://zhihongplastic.com/pet%e3%80%81pp%e3%80%81pe%e3%80%81ps%e3%80%81pa%e3%80%81abs%e3%80%81pmma%e3%80%81pvc-common-types-of-plastics-properties-and-applications/