# Life Cycle Emission Analysis and Case Specific Mitigation Strategies for Hostel Buildings in the Lower Himalayan Region.

# ABSTRACT

Buildings account for a significant share of global carbon emissions, yet comprehensive lifecycle frameworks for assessing their full environmental impact remain limited in the Indian context. The study evaluates the environmental impact of a residential building in the lower region of north India. This case specific framework integrates life cycle assessment (LCA) in terms of operational energy analysis, embodied energy accounting, and carbon emission estimation. A detailed material and energy inventory was compiled, with embodied energy coefficients and emission factors sourced from peer-reviewed literature. Results indicate that the operational stage dominates the building's carbon profile, contributing 73% of total lifecycle emissions driven primarily by occupant metabolic CO₂ and electricity consumption. The total carbon footprint is estimated at 12.77 million kg CO₂ equivalent, with brick and reinforced cement concrete identified as the most energy and emission intensive materials. Three mitigation strategies were evaluated: replacing grid electricity with photovoltaic panels reduces electricity related emissions by 96%; substituting LPG with kitchen waste biogas achieves a 15.6% reduction in cooking emissions; and plantation of 90 mature trees enables full carbon sequestration. Collectively, these measures reduce the building's lifetime emissions by approximately 38%. The proposed framework offers a replicable, case specific approach to carbon assessment and mitigation planning, applicable to other residential buildings seeking to align with sustainable construction goals.

Keywords emissions, embodied energy, hostel building, sustainability, operational stage.

# INTRODUCTION

Sustainable development has now been embossed in human consciousness (Brandon & Lombardi, 2011). It has led to changes in the priorities of global agendas such as the Sustainable Development Goals and the 2015 Paris Agreement. Scholars and environmentalists have long warned of the consequences of unchecked human activity on the planet, including global warming, habitat loss, resource depletion, and widespread pollution (Gore & Gore, 1999; Jackson, 2009; Lovelock, 2009; Rees, 2009). Despite widespread awareness, no single solution has yet proven sufficient to reverse these complex trends.

Peter and Brandon (Brandon & Lombardi, 2011) have highlighted a spectrum of views for sustainable development. At one extreme is the ecological viewpoint that emphasizes harmony with nature, urging radical lifestyle changes and curbs on urban expansion. At the other is the techno-centric perspective, which advocates technological innovation to mitigate environmental harm while maintaining development trajectories.

India, ranked as the third-largest emitter of greenhouse gases (GHGs) among the world's leading economies (World Bank: Washington, DC, USA, 2021), exemplifies the scale of this challenge. In 2022 alone, its CO₂ emissions rose by 6.52%, adding over 164 million metric tons to previous levels (India CO2 Emissions, 2024). Acknowledging this trajectory, India pledged at COP26 to achieve net-zero emissions by 2070 (Ministry of Environment, Forest and Climate Change, 2023).

The building construction industry is a crucial sector for achieving these climate targets. It is one of the world's largest energy consumers, accounting for 40% of total energy consumption (Mathumitha et al., 2024) and 37% of energy and process-related CO2 emissions globally (United Nations Environment Programme, 2021). Consequently, sustainable construction methods are increasingly prevalent to alleviate the environmental and energy crises. Although sustainable construction involving new materials may incur higher initial costs, they offer longer economic value in the long run. Sustainable buildings can save money by reducing carbon emissions, optimizing input usage, and lowering operation costs. Moreover, good design enhances the comfort and natural living experience of occupants. Materials used in sustainable construction tend to be more environment friendly and harmless, safeguarding occupants from harmful substances (Y. Wang, 2023). A triple-bottom-line approach is important for any design and construction practice where social, environmental, and economic principles are integrated to achieve more sustainable outcomes (Goh et al., 2020).

Assessing the sustainability involved in the life cycle of any construction helps in identifying the scope and direction of improvement for sustainable and cost-effective decision-making (Chen et al., 2025; Y. Dong et al., 2023; Fan et al., 2025; Figueiredo et al., 2024; Milić & Bleiziffer, 2024; Ullah et al., 2023). ISO 14040:2006 provides a detailed framework for Life Cycle Assessment of any construction (ISO, 2006). It is an intricate process involving various phases, viz., setting up the goal and scope, the life cycle inventory analysis (LCI) phase, the life cycle impact assessment (LCIA) phase, interpretation, and reporting phase. The sub-processes involved may differ in importance due to the specific characteristics of the built environment project (Moncaster & Song, 2012).

Studies have also suggested broadening the scope of LCA by considering factors like indoor environmental quality, building location, and social factors (Chau et al., 2015). LCA also offers a framework for evaluating buildings' resource use and enviromental impacts (W. Dong et al., 2024), and its early application can greatly improve sustainability (Hansen et al., 2023; Schneider-Marin & Lang, 2020).

The construction and management of built environment utilize enormous resources and energy, affecting the environment and human health in numerous ways (Ioppolo et al., 2019). These concerns must be understood from a life cycle perspective (Asdrubali et al., 2013; Chau et al., 2015). Several approaches have been proposed to appraise the environmental impacts of any product or process. These can be classified into three major streams viz. Life Cycle Assessment (LCA), Life Cycle Energy Assessment (LCEA), and Life Cycle Carbon Emissions Assessment (LCCO2A) (Chau et al., 2015). LCA evaluates the all-inclusive environmental impacts of products in terms of their energy consumption, material usage, and discharges into the atmosphere (Eq. 1). It encompasses entire cradle-to-grave phase of the product, viz., extraction and processing of raw materials, their transportation, application (re-application), maintenance, and disposal.

I = IExtraction + IManufacture + IOnsite + IOperation +IDemolition + IRecycling +IDisposal                                                                      (1)

Where,

I = life cycle environmental impact, and

Ij = environmental impacts of jth building phase.

LCEA assesses the total energy used by a building in its lifespan (Eq. 2). However, it fails to present a genuine picture of the negative impacts as it does not segregate the renewable and non-renewable energies.

E = EExtraction + EManufacture + EOnsite + EOperation + EDemolition + ERecycling +EDisposal                                                                                             (2)

Where, E = total energy consumed during the whole life cycle of a building,

Ej = energy consumed during the jth building phase.

LCCO2A determines the total CO2 emitted by the building in its lifespan.

LCA approach is adopted for comprehending the environmental impacts of built environment round the globe, viz. India (Varun et al., 2012), China (Hao et al., 2020; J. Wang et al., 2018; Yang et al., 2018), Taiwan (Chou & Yeh, 2015), Japan (Endo & Takamura, 2021).  Australia (Shahana Y. Janjua et al., 2019), United States (Ben-Alon et al., 2021; Hasik et al., 2019; Liang et al., 2020), Canada (Rezaei et al., 2019) Netherlands (Santos et al., 2020), Norway (Rabani et al., 2021) Sweden (Österbring et al., 2019; Petrovic et al., 2019), Argentina (Arena & de Rosa, 2023), Italy (Ardente et al., 2008; Blengini, 2009), Nigeria (Adeyeye et al., 2023), Thailand (Kofoworola & Gheewala, 2008).

# METHODOLOGY

A building consumes energy and emits harmful pollutants during all processes involved in its construction and operation. Several materials are required for its construction, which must be extracted or prepared and transported to the site. All these processes consume energy, contributing to their embodied energy and associated carbon emissions as shown in Figure 1. Post-construction and during building occupancy, various operations run for its functioning, each consuming energy and resources in some form or another. These operations, alongside repairs and refurbishments, run throughout the building's lifespan. The End-of-Life stage addresses demolition, waste transport, recycling, and disposal. Finally, Beyond the Lifecycle includes reuse and recovery opportunities to mitigate further emissions.

To reduce the carbon footprint of building structures, it is essential to evaluate embodied carbon during the early design stages, since the structural framework often reflects the initial conceptual form of the building (Torabi & Evins, 2024). The study calculates the emissions and embodied energy in both the phases of the building, starting from the preparation (extraction, transportation, and manufacturing) of various building materials such as cement, brick, and MS frames to operational activities such as energy (electricity) usage, water consumption, cooking, and human respiration. The life span of the building for this case study is taken as 50 years. Emissions due to construction processes, and maintenance is not considered in the study.

Fig. 1 Building Operations responsible for carbon emissions ans embodied energy

## Carbon Emission

The total environmental impact of building in terms of carbon emissions of any building is the total pollutant (CO₂, CH4, N2O, SO2, etc.)  released to the atmosphere during the construction and operation of the building. It is the sum of the emissions due to all the materials used and activities involved during its construction and operation (Eq.3).

For each material or activity, emissions were estimated using:

E=Q×EF                                                                                                                                                                         (3)

Where:

E = total emissions (kg CO₂- eq.)

Q = quantity used or consumed

EF = emission factor specific to the material or activity

Emission factor (EF) is used to calculate the GHG emissions of a process or activity. An EF of any material is a value that represents the amount of a pollutant released to the atmosphere during the activity or manufacturing of the material. The potential impact of all the GHG gases is converted into an equivalent effect of carbon dioxide (CO₂) and is cumulatively expressed in terms of CO₂ equivalent (CO₂- eq.) which is a unit of measurement used to express the impact of different GHGs on global warming.

Emission factors for individual materials and operational carbon (electricity, Liquified Petroleum Gas (LPG), and user's exhalation) were derived from a review of relevant literature with similar research  (Shams et al., 2011) and Inventory of Carbon & Energy (ICE) database. Emissions from furniture are not considered. Grid electricity emission factor remains constant over the study period.

## Embodied energy

Embodied Energy encompasses the total energy consumed throughout the lifecycle of a building material from extraction and processing to transportation and installation at the construction site. This measure accounts for all direct and indirect energy inputs, including those associated with manufacturing, logistics, on-site assembly, and eventual renovation or demolition. It is a key parameter in evaluating the environmental footprint of construction practices (Almusaed et al., 2024).

Materials such as cement and steel contributes upto 30% of a building's lifetime energy demand (Ding et al., 2004). Beyond carbon output, the extraction and production of these materials can lead to habitat loss, resource depletion, and soil degradation (Hammond & Jones, 2008).

The embodied energy of a material or activity is calculated as the product of its quantity and its specific energy intensity, typically expressed in megajoules per kilogram (MJ/kg):

Embodied Energy (EE) = Q x Embodied Energy per unit                                                                                             (4)

Where:

- EE = total embodied Energy (MJ/kg)

- Q = quantity used or consumed (kg)

- Embodied Energy per unit: Specific energy intensity for the material/process (MJ/kg)

Embodied energy per unit data for individual material was derived from recognized databases and peer-reviewed studies to ensure accuracy and consistency (Singh* et al., 2019; Varun et al., 2012).

By quantifying and analyzing embodied energy, practitioners can make informed decisions about construction materials prioritizing approaches that minimize environmental harm and resource consumption. This ultimately facilitates the selection of sustainable alternatives and encourages continuous improvement in building practices.

## Case Study

The case study focuses on the Parvati Girls' Hostel at the National Institute of Technology (NIT), Hamirpur. The region falls under the 'subtropical highland' category of the Köppen climate classification ('Köppen-Geiger Climate Classification', 2010). It experiences hot summers and cold winters. Solar radiation is available throughout the year, ranging from 3.45 kWh/m²/day in December to 7.42 kWh/m²/day in May. The hostel building was constructed in 1998 and has a total built-up area of 3,522 m². It accommodated 160 students in total. The building is organized into six blocks (A, B, C, D, E, and F), offering various accommodation options tailored to meet the diverse needs of its residents. Blocks D, E, and F feature 54 single-seated rooms, and Blocks A, B, and C comprise 36 triple-sharing rooms.

Apart from 90 residential rooms, there are nine common washrooms, a dining hall, common room facilities, guest rooms and administrative rooms. Figure 2 represents the ground floor plan of the building and Figure 3 represents the typical floor plan for the first and second floors. The external walls are constructed using stone masonry with a thickness of 300 mm. The internal partitions are built with 230 mm-thick brick. Fenestration includes external windows fitted with 4 mm-thick clear glass, supported by mild steel (MS) frames. Building does not include any external shading devices to allow unobstructed daylight penetration. Doors are positioned at a lintel height of 2,000 mm and are fabricated from wood, reinforced with MS angles for durability. Safety considerations are addressed through railings composed of MS channels, set at a standard height of 600 mm. Flooring throughout the building is finished in terrazzo.

The architectural plans and bill of quantities of the hostel were sourced from the Institute's Engineering Office, providing a detailed blueprint of the building design and materials used. Additionally, data on water and electricity consumption were acquired from the monthly expense bills maintained by the hostel office. These bills offered insights into their usage patterns, enabling a comprehensive analysis of resource consumption. The collected data were meticulously organized and validated to ensure accuracy and reliability for the study's objectives.

Fig. 2 Ground floor plan of the building

Fig. 3 Typical floor plan of Building

Transportation-related carbon emissions were quantified through calculations based on vehicle mileage, payload, trip frequency, and distance from the source. The total trips were calculated by dividing the material quantity by the vehicle payload. The diesel consumption is based on the total trips, where each trip consumes a set amount of diesel. Carbon emissions (kg CO₂- eq.) are calculated based on the diesel consumed. The emission factor for diesel is a constant that converts diesel consumption into carbon dioxide equivalents (B. Surekha et al., 2016). The source of the building materials, such as bricks, MS frames, etc., is Una (Himachal Pradesh), which is 67 km away from the site.

# RESULTS

A comprehensive assessment of embodied energy and carbon emissions was conducted for the primary construction materials used in the project. The results reveal significant variations in environmental impact across materials, reflecting differences in processing intensity, unit quantities, and emission factors.

### Pre-Occupancy stage (Construction stage)

Embodied energy during the pre-occupancy stage is due to the materials. The total embodied energy for the building materials is estimated to be approximately 10.93 million megajoules (MJ), as mentioned in Table 1. The most energy-intensive materials were brick and limestone, contributing 4.22 million MJ and 2.90 million MJ, respectively. This is attributed to their high embodied energy per unit mass and the magnitude of their usage. Reinforced Cement Concrete (RCC) and cement also contributed significantly, with total embodied energies of 1.78 million MJ and 1.25 million MJ, respectively. Despite their moderate per unit embodied energy, their bulk usage resulted in high cumulative energy demand.

Other materials with notable embodied energy contributions include vitrified tiles (113,057 MJ), MS grill (247,564 MJ), and glass panes (21,097 MJ). In contrast, materials such as paver blocks, granite, CPVC and PVC pipes, paints, and cement putty exhibited comparatively lower total energy impacts due to smaller usage volumes or lower energy intensities.

### Pre-occupancy stage: Construction stage

Carbon emissions during this stage arise primarily due to raw material extraction, manufacturing, and transportation. Heavy construction vehicles and equipment also contribute significantly to greenhouse gas emissions, but are not considered in the study. Energy-intensive processes like cement and steel production generate substantial amounts of CO₂, both directly from chemical reactions (e.g., calcination in cement) and indirectly from the use of fossil fuels.

The total carbon emissions associated with the building materials amount to 3.4 million kilograms of CO₂- eq., as mentioned in Table 1. Brick emerges as the highest emitter, responsible for 1.7 million kg CO₂- eq., which corresponds to over 50% of total emissions. This is a direct result of its high emission factor (0.48 kg CO₂- eq./kg) and substantial volume. RCC is the second-largest contributor, with emissions amounting approximately to 1.0 million kg CO₂-eq. owing to its high emission factor (410 kg CO₂- eq./Cu.m) and considerable usage.

Table 1 Materials with quantities, emissions, and embodied energy

| Materials | Quantity | Units | Emission factor (kg CO₂-eq./ unit) | Total emissions (kg CO₂- eq.) | Embodied Energy (MJ/Unit) | Total Embodied Energy (MJ) |
| --- | --- | --- | --- | --- | --- | --- |
| Earth Work | 9,032.9 | Cu.m | 0.02 | 181 | NA | NA |
| RCC | 2,447.8 | Cu.m | 410 | 1,003,610 | 730 | 1,786,894 |
| Cement | 272,272.0 | kg | 0.967 | 263,287 | 4.6 | 1,252,451.2 |
| Concrete | 3,696.0 | kg | 0.159 | 588 | 5.6 | 20,697.6 |
| Brick | 3,548,023.0 | kg | 0.48 | 1,703,051 | 2,141 | 4,220,167.92 |
| Limestone | 208,216.0 | kg | 0.015 | 3,123 | 34,860 | 2,903,489.4 |
| Paver Blocks | 24,024.0 | kg | 0.159 | 3,820 | 1.5 | 36,036 |
| Granite | 1,647.0 | kg | 0.04 | 66 | 5.908 | 9,730.476 |
| Terrazzo flooring | 4.8 | Cu.m | 237 | 1,127 | 2,450 | 11,760 |
| Vitrified Tiles | 20,555.9 | Sqm | 18.33 | 376,790 | 5.5 | 113,057.45 |
| Wood | 8,200.5 | kg | 1.5 | 12,301 | 10.8 | 88,565.4 |
| MS Grill | 5,894.4 | kg | 2.75 | 16,209 | 42 | 247,564.8 |
| Glass Panes | 817.7 | kg | 1.735 | 1,419 | 25.8 | 21,096.66 |
| CPVC Pipes | 971.7 | kg | 3.96 | 3,848 | 71.2 | 69,185.04 |
| PVC Pipes | 365.8 | kg | 3.23 | 1,181 | 80 | 29,264 |
| Paints | 762.3 | litre | 2.86 | 2,180 | 98.1 | 74,781.63 |
| Cement Putty | 9,287.0 | kg | 0.78 | 7,244 | 4.8 | 44,577.6 |
| Total |  |  |  | 3,400,025 |  | 10,929,319.2 |

Other significant sources of emissions include tiles (376,790 kg CO₂- eq.), cement (263,287 kg CO₂- eq.), and MS grill (16,209 kg CO₂- eq.). Glass panes and CPVC pipes also represent non-negligible emission sources due to their relatively high emission factors (1.735 and 3.96 kg CO₂- eq./kg, respectively), despite being used in lower quantities.

In contrast, materials such as limestone, granite, and wood contributed minimally to total carbon emissions, reflecting low emission factors.

Transporting these materials from production sites to construction locations adds to the carbon footprint. Together, these stages make construction materials a major source of carbon emissions. Transportation-related emissions were assessed based on the number of delivery trips, diesel consumption, and an emission factor of 3.14 kg CO₂- eq./ litre of diesel. 15.2 litre of diesel is consumed in 1 trip from Una to the site.

The total emissions due to transportation of the materials from Una to the site were estimated to 63,169 kg CO₂- eq. (Table 2). The highest contributor was Reinforced Cement Concrete (RCC) with 756 trips, consuming 11,491.2 litres of diesel, and emitting 36,427 kg CO₂- eq. Brick followed with 438 trips, consuming 6,559.7 litres of diesel, and emitting 21,105 kg CO₂- eq. Other notable materials included cement and tiles, contributing 1,638 kg CO₂- eq. and 1,976 kg CO₂- eq., respectively.

Table 2 Materials with emissions due to their transportation

| Materials | Quantity | Units | Trips | Diesel Consumption (in litres) | Emissions by transportation (kg CO₂- eq.) |
| --- | --- | --- | --- | --- | --- |
| Earth Work | 9,032.9 | Cu.m | NA | NA | NA |
| RCC | 2,447.8 | Cu.m | 756 | 11,491.2 | 36,427 |
| Cement | 272,272.0 | kg | 34 | 516 | 1,638 |
| Concrete | 3,696.0 | kg | 1 | 15.2 | 48 |
| Brick | 3,548,023.0 | kg | 438 | 6,559.7 | 21,105 |
| Limestone | 208,216.0 | kg | 27 | 394.7 | 1,253 |
| Paver Blocks | 24,024.0 | kg | 3 | 45.6 | 145 |
| Granite | 1647.0 | kg | 1 | 15.2 | 48 |
| Terrazzo flooring | 4.8 | Cu.m | 2 | 30.4 | 96 |
| Vitrified Tiles | 20,555.9 | Sqm | 41 | 623.2 | 1,976 |
| Wood | 8,200.5 | kg | 2 | 30.4 | 96 |
| MS Grill | 5,894.4 | kg | 1 | 15.2 | 48 |
| Glass Panes | 817.7 | kg | 1 | 15.2 | 48 |
| CPVC Pipes | 971.7 | kg | 1 | 15.2 | 48 |
| PVC Pipes | 365.8 | kg | 1 | 15.2 | 48 |
| Paints | 762.3 | litre | 1 | 15.2 | 48 |
| Cement Putty | 9,287.0 | kg | 2 | 30.4 | 96 |
| Total emissions |  |  |  |  | 63,169 |

Materials like glass, pipes, wood, paints, and concrete had minimal impact (≤96 kg CO₂- eq. each) due to fewer trips and smaller quantities. Although materials such as limestone and paver blocks were used extensively, their transportation emissions remained moderate. Earthwork had no associated transport emissions due to on-site excavation. The total emissions are 34,00,025 kg CO₂- eq., with an additional 63,169 kg CO₂- eq. from transportation. Major contributors to total emissions are RCC and Brick, comprising approximately 79% of the total emissions from building materials.

During occupancy, several mechanical processes run inside the building, consuming Energy from LPG, water, and electricity, thus releasing carbon or equivalent pollutants. Biological processes like respiration are also a major source of carbon dioxide emissions.

### Post Occupancy stage: Operational stage

LPG is used for cooking in the mess. The Annual consumption of LPG is 4,750 kg. Emissions due to 1kg of LPG are 3kg CO₂- eq. (Eggleston, 2006). Thus, the annual emission due to cooking gas is 14,250 kg CO₂- eq.

There are 160 students in the hostel and 175 in total, including mess workers, attendants, and others. Generally, an individual emits 1kg of carbon daily (Baste & Thakare, 2023). Thus, the annual average emission is 63,875 kg CO₂- eq.

Electricity use is the primary source of carbon dioxide emissions, playing a dominant role in fueling climate change (Li et al., 2025). In 2022, power generation accounted for 53% of total energy-related CO2 emissions in India (International Energy Agency, 2025). This highlights the significant role of the electricity production sector in the country's overall emissions profile. Electricity sourced from the national grid powers all operations within the building, including the functioning of air conditioners, computers, heavy machinery, and lighting systems.

The emission factor for 1kWh is 0.921 (Varun et al., 2012). Figure 4 represents the monthly electricity consumption (use of geyser, ceiling fans, lights, iron, hair styling appliances, etc.) and emissions for 2022, 2023, and 2024.

Peak electricity consumption is seen in April, attributed to the simultaneous use of geysers and fans. In 2022, the lowest consumption was recorded in March, primarily due to reduced occupancy resulting from COVID-19 restrictions.

Fig. 4 Electricity consumption and its emissions

In contrast, the minimum consumption for 2023 and 2024 occurred in July, coinciding with the semester break vacation, which led to fewer users. The total electricity consumption for the years 2022, 2023, and 2024 is 98,835 kWh, 117,390 kWh, and 107,444 kWh, respectively. The total emissions due to electricity for the years 2022, 2023, and 2023 are 91,027.04 kg CO₂- eq., 108,116.2 kg CO₂- eq., and 98,955.92 kg CO₂- eq. respectively.

The study investigates the greenhouse gas (GHG) emissions linked to the water supply system. The water supply system includes the extraction, treatment, and distribution of potable water (Lahmouri et al., 2019). The calculations are based on the monthly user data provided by the hostel. The emission factor for water consumption varies by region due to differences in supply systems and energy sources. For greater accuracy, this research references a study from Dehradun, which provides a locally derived emission factor based on the city's specific water supply and energy usage. Using this region-specific data ensures the assessment reflects the actual environmental impact of water use in that context. Water production including extraction, treatment, and delivery to storage contributes to the GHG emissions. The related emission factor for water consumption is 0.00112 kg CO₂- eq. per liter of water (Medha, 2020). The total annual water consumption amounts to 7,814,070 liters, resulting in an associated emission of 8,751.75 kg CO₂- eq.

### Overall Emissions

Over a span of 50 years, the building's total GHG emissions can be categorized into embodied and operational emissions. Pre-occupancy emissions for the entire life span refer to the cumulative emissions due to the preparation and transportation of building materials and construction-related activities. The total is calculated to be 3,463,194 kg CO₂- eq., accounting for 27% of emissions.

Post-occupancy emission for building's life span includes the emissions due to major building operations, viz cooking, water, and electricity consumption. In this study, the operations considered are electricity usage, water consumption, users' exhalation, and LPG consumption, amounting to 186,242 kgCO₂- eq annually and 9,312,100 kgCO₂- eq in the entire life span. Thus, 73% of the total lifespan emissions are due to operational stage (Fig. 5). Also, maximum weightage in emissions in the operational stage is due to user's exhalation and electricity, which is 87% of the total in the operational stage (Fig.6).

Total emissions for the life span include pre-occupancy and post-occupancy emissions, which are calculated to be 12,775,294 kgCO₂- eq.

Fig. 5 Distribution of total emissions in pre- occupancy and post- occupancy

Fig. 6 Share of Emissions due to different operations in Post- Occupancy phase

# RECOMMENDATIONS

The LCA quantifies the carbon emissions generated by a building throughout its entire life cycle. Given that the building considered for study is already constructed and operational, the emissions that have already occurred cannot be reversed. While achieving absolute zero carbon emissions is not feasible, it is possible to attain net-zero carbon through compensatory measures.

Consequently, selecting materials with lower embodied energy becomes critical for sustainable design (Kibert, 2016). Furthermore, integrating sustainable practices during the pre-construction phase, such as energy-efficient machinery and waste reduction strategies, can significantly lower the embodied energy associated with construction activities (Zuo & Zhao, 2014). By adopting a comprehensive approach to embodied energy, stakeholders can enhance the sustainability of the built environment, ultimately contributing to climate change mitigation and resource conservation.

To identify pathways for sustainable construction practices, it is crucial to assess the environmental impact of buildings. Since construction processes vary based on factors like building type, climate, topography, location, and soil conditions, tailored solutions are needed. For instance, sloping roofs are commonly used in hilly regions to protect buildings from heavy rain and snow. By adopting this site-specific approach, key construction and operational areas can be identified, and targeted actions can be implemented.

## Electricity

The annual average consumption of electricity in the building is 107,889 kWh. As mentioned, the source of electricity here is a thermal power plant. The associated emission factor of thermal sources and renewable sources (PV panels) is 921g CO₂- eq.per kWh and 35g CO₂- eq. per kWh, respectively (Varun et al., 2012). The difference between them is 886g CO₂- eq. per kWh. Therefore, the implementation of solar photovoltaic panels for electricity generation can reduce carbon emissions by 95,589.65 kg CO₂- eq. annually which accounts to 96 % of the emissions due to electricity.

## Cooking gas

Biogas has been identified as the most effective alternative fuel, exhibiting the minimal adverse environmental impacts throughout its entire life cycle (Kumar et al., 2024). The building consumes an annual average of 4,750 kg of LPG for cooking, with an emission factor of 3 kg CO₂- eq., while Biogas has a lower emission factor of 0.3 kg CO₂- eq. Although LPG consumption cannot be entirely replaced, installing a biogas plant can reduce emissions and can also improve waste management efficiency. Past studies have worked on the methods of estimating food wastage from hostel mess. In 2017, food wastage at IIT Mandi averaged 27g and 32g per capita per meal in two messes, amounting to 118g per capita daily. At IIT Bombay, a 2011 waste audit showed an average daily mess hall waste of over 950 kg, with per capita waste ranging from 100g to 160g, averaging 130g (Dash A, 2015).

At Parvati Girls' Hostel, with an average food waste of 124g per capita daily and a total capacity of 160 people, the total waste generated is 19.84 kg per day. According to Pathak et al. (2022), 1 kg of kitchen waste can produce 0.3 m³ of Biogas, resulting in 5.952 m³ of Biogas daily and 178.56 m³ monthly in the hostel. Since 1 m³ of Biogas is equivalent to 0.46 kg of LPG, biogas production would replace 82.14 kg of LPG monthly and, thus 821 kg annually. Thus, by using this alternate, annual carbon emissions can come down to 12,031 kg CO₂- eq. from 14,250 kg CO₂- eq. by using LPG gas. The biogas plant can reduce 2,219 kg CO₂- eq. (15.56%) of emissions annually in terms of cooking gas (considering the current waste production in the kitchen) as mentioned in Table 3.

Table 3 Comparison of the LPG and the Combination of LPG and biogas for cooking gas

|  | LPG | LPG+ Biogas |
| --- | --- | --- |
| Annual average consumption (in kg) | 4,750 | 3,928.6 (LPG) + 821.4(Biogas) |
| Annual emissions (in kg CO₂- eq.) | 14,250 | 12,031.4 |

## Carbon sequestration by trees

The process of carbon uptake and storage involves the absorption of atmospheric carbon dioxide by vegetation through photosynthesis. In this process, trees and plants release oxygen while retaining carbon within their biomass. Fossil fuels, which originate from ancient biomass, also act as long-term carbon reservoirs until their combustion releases the stored carbon back into the atmosphere (UNFCCC, 2010).

Thakare in his study presented that after the full growth of 765 trees, 367.81 tons of carbon were sequestered per year (Baste & Thakare, 2023). This gives a rate of approximately 0.481 tons of carbon per fully grown tree per year and 24 tons of carbon per fully grown tree in 50 years. Trees found in the area are pine, oak, deodar, etc. The average carbon sequestration of pine is 0.44 tons per year, as per a study (Pant & Tewari, 2013). Following is the detailed calculation of trees required to carbon sequestration required after change in cooking gas and electricity source.

Step 1:

Molecular weight of CO2 = 44 g/mol

(Carbon = 12, Oxygen = 16 x 2; 12+32 =44)

Molecular weight of Carbon = 12

Fraction of Carbon in CO2 = 12/44 = 0.2727

Therefore, 1 kg CO2 x 12/44 = 0.2727 of Carbon

Step 2:

Total emissions through the lifetime of building after recommendations = (Materials) + (transportation of materials to the site) + (Cooking gas as Biogas+ LPG) + (Electricity through solar panels) + (Water consumption) + (User’s Exhalation)

= 3,400,025 + 63,169 + 601,570 + 188,806 + 437,587.5 + 3,193,750

= 7,884,907.5 kg CO₂- eq.

Which is equals to 2,149,585.7 kg of Carbon or 2,149.59 tons of Carbon

Step 3:

1 tree = 0.481 tons/ year

Over 50 years = 24 tons of Carbon / tree

Step 4:

Total number of trees = 2,149.59 / 24

=89.54 (approximately 90) trees

Thus, 90 fully grown trees are required to sequester the carbon emitted by the building in the entire life span in PGH. This approach helps offset or potentially neutralize the building's total carbon emissions over its lifecycle.

## Comparison between existing case and recommended case

Figure 9 presents a comparison of annual carbon emissions during the operational phase for the existing case and the recommended case, across electricity, cooking gas, water usage, and user exhalation, along with carbon sequestration by trees, shown as a negative value.

In the existing case, total operational emissions are substantially higher, dominated by electricity and user exhalation, which together account for the 87% of emissions. Water usage and cooking gas also contribute, though to a lesser extent which is 13%.

The recommended strategies demonstrate a significant reduction in net emissions. While emissions from electricity, and cooking gas persist, they are considerably lower than the existing case indicating improved energy and resource efficiency. Notably, the inclusion of carbon sequestration, depicted as a large negative value (Refer Fig. 9), offsets a substantial portion of the operational emissions. This sequestration, attributed to strategic planting of trees or green cover, shifts the overall carbon balance in favour of a more sustainable outcome. The data underscores the effectiveness of the recommended interventions in reducing net annual emissions, primarily through demand-side efficiency measures and natural carbon offset mechanisms.

Fig. 8 Emissions during operational phase for Existing case and recommended case

For buildings that are already constructed, emission reduction efforts must focus on the operational phase, which accounts for 73% of total emissions. This study was conducted on an existing, operational building, but similar methods could also be applied during the design phase once the construction materials and operational plans are finalized.

One effective strategy to reduce operational emissions is transitioning to renewable energy sources, such as installing solar photovoltaic (PV) panels to replace conventional electricity. Similarly, using Biogas as a substitute for LPG in cooking can significantly lower emissions. While emissions from human exhalation are unavoidable, they can be offset through carbon sequestration measures like planting trees. By integrating these strategies, this research aims to pave the way for more sustainable and environmentally responsible building practices.

# CONCLUSION

This study provides a comprehensive LCA associated with the Parvati Girls' Hostel at NIT Hamirpur, offering an evidence-based understanding of the environmental burden imposed by both the construction and operation of a residential building in a subtropical highland climate. The primary objective was to quantify and analyse the embodied energy, material- and transport-based emissions, and post-occupancy emissions to identify actionable pathways toward a low-carbon built environment.

The analysis revealed that over a 50-year life span, the building is responsible for a total of 12.77 million kg CO₂ equivalent, with emissions split across the construction phase (27%) and operational phase (73%). Operational emissions were dominated by electricity usage and human respiration, contributing 87% of annual emissions during building occupancy. Construction materials such as brick, RCC, and cement emerged as major contributors to both embodied energy and emissions, primarily due to their high usage volumes and energy-intensive production processes. Additionally, transportation of materials added a notable yet secondary layer of emissions, totalling 63,169 kg CO₂- eq.

Another finding is that 34% of post-occupancy emissions are linked to human exhalation, emphasizing the importance of passive ventilation strategies to maintain indoor air quality. Furthermore, the study demonstrates that the total lifetime emissions of the building can potentially be offset by planting 90 fully grown pine trees, offering a nature-based solution for carbon sequestration. The embodied energy of construction materials is also significant, with steel and brick being the major contributors.

This research contributes a replicable, data-driven methodology for evaluating sustainability in institutional residential buildings. It can inform architects, engineers, and policymakers seeking to implement net-zero or low-emission strategies in both new and retrofitted structures. Moreover, it aligns with India's commitment to achieve net-zero emissions by 2070, as announced at COP26, and supports global climate goals under the Paris Agreement and the UN Sustainable Development Goals (SDGs).

As a way forward, the study recommends expanding research on next-generation materials such as geopolymer concrete, hempcrete, recycled steel, biobased flooring like cork and bamboo. It also encourages to integrate smart metering and real time monitoring for energy optimization. The study also recommends to leverage decentralized renewable systems, such as hydrogen fuel cells and hybrid microgrids. With advancements in transportation, production, and resource extraction, new possibilities for reducing the carbon footprint across a building's lifecycle can be realized.

In conclusion, this work demonstrates that holistic assessment and intervention strategies can not only mitigate environmental impacts but also pave the way toward climate-resilient and resource-efficient buildings. As climate change accelerates, such analytical tools and localized actions will be essential for transforming the built environment into a catalyst for ecological balance and sustainability.

ACKNOWLEDGEMENTS

The authors extend sincere gratitude to the administration and management of Parvati Girls' Hostel, NIT Hamirpur, for their cooperation and assistance during data collection and site-related investigations.

Funding

This research received no external funding.

Author Contribution

Monika Garg was responsible for the conceptualization of the study, development of the methodology, simulation work, data curation, analysis, visualization, and preparation of the original draft. Dr. Swechcha Roy contributed through supervision, validation of results, critical review, and editing of the manuscript, as well as providing guidance on research design and overall project administration. All authors have read and approved the final version of the manuscript.

Data Availability

The authors declare that all data generated or analyzed during this study are included in this paper.

# Declarations

Competing Interests The authors declare no competing interests.

# REFERENCE

- Adeyeye, D., Olusola, A., Orimoloye, I. R., Singh, S. K., & Adelabu, S. (2023). Carbon footprint assessment and mitigation scenarios: A benchmark model for GHG indicator in a Nigerian University. Environ. Dev. Sustain., 25(2), 1361–1382. https://doi.org/10.1007/s10668-021-02098-1

- Almusaed, A., Yitmen, I., Myhren, J. A., & Almssad, A. (2024). Assessing the Impact of Recycled Building Materials on Environmental Sustainability and Energy Efficiency: A Comprehensive Framework for Reducing Greenhouse Gas Emissions. Buildings, 14(6), 1566. https://doi.org/10.3390/buildings14061566

- Ardente, F., Beccali, M., Cellura, M., & Mistretta, M. (2008). Building energy performance: A LCA case study of kenaf-fibres insulation board. Energy Build., 40(1), 1–10. https://doi.org/10.1016/j.enbuild.2006.12.009

- Arena, A. P., & de Rosa, C. (2023). Life cycle assessment of energy and environmental implications of the implementation of conservation technologies in school buildings in Mendoza—Argentina. Build. Environ., 38, 359–368.

- Asdrubali, F., Baldassarri, C., & Fthenakis, V. (2013). Life cycle analysis in the construction sector: Guiding the optimization of conventional Italian buildings. Energy Build., 64, 73–89. https://doi.org/10.1016/j.enbuild.2013.04.018

- Ashutosh Pathak, Harshad Patil, Sandesh Patil, Varun Patil, & Prof. Firdos Khan. (2022). A Review on Bio Methane Production using Kitchen Waste. Int. J. Eng. Res. Technol. IJERT, 11(4).

- B. Surekha, M. N. HEGDE, & K. S. JAGADISH. (2016). ENERGY AND BUILDING MATERIALS. IASET, 5(5), 13–24.

- Baste, P., & Thakare, Ar. H. (2023). Optimization for Carbon Footprint in an Institutional Campus. Ecol. Environ. Conserv., 29(01), 175–181. https://doi.org/10.53550/eec.2023.v29i01.028

- Ben-Alon, L., Loftness, V., Harries, K. A., & Cochran Hameen, E. (2021). Life cycle assessment (LCA) of natural vs conventional building assemblies. Renew. Sustain. Energy Rev., 144, 110951. https://doi.org/10.1016/j.rser.2021.110951

- Blengini, G. A. (2009). Life cycle of buildings, demolition and recycling potential: A case study in Turin, Italy. Build. Environ., 44(2), 319–330. https://doi.org/10.1016/j.buildenv.2008.03.007

- Brandon, P. S., & Lombardi, P. L. (2011). Evaluating sustainable development in the built environment (2. ed). Wiley-Blackwell.

- Cabeza, L. F., Rincón, L., Vilariño, V., Pérez, G., & Castell, A. (2014). Life cycle assessment (LCA) and life cycle energy analysis (LCEA) of buildings and the building sector: A review. Renew. Sustain. Energy Rev., 29, 394–416. https://doi.org/10.1016/j.rser.2013.08.037

- Chau, C. K., Leung, T. M., & Ng, W. Y. (2015). A review on Life Cycle Assessment, Life Cycle Energy Assessment and Life Cycle Carbon Emissions Assessment on buildings. Appl. Energy, 143, 395–413. https://doi.org/10.1016/j.apenergy.2015.01.023

- Chen, X., Shi, P., & Wang, H. (2025). Life cycle environmental impact assessment of bridges based on comparison of different construction schemes. Environ. Dev. Sustain. https://doi.org/10.1007/s10668-024-05895-6

- Chou, J.-S., & Yeh, K.-C. (2015). Life cycle carbon dioxide emissions simulation and environmental cost analysis for building construction. J. Clean. Prod., 101, 137–147. https://doi.org/10.1016/j.jclepro.2015.04.001

- Dash A. (2015). Food for thought. Insight: IIT Bombay’s Student Media Body.

- Ding, W., Cai, Z., & Wang, D. (2004). Preliminary budget of methane emissions from natural wetlands in China. Atmos. Environ., 38(5), 751–759. https://doi.org/10.1016/j.atmosenv.2003.10.016

- Dong, W., Kong, F., He, T., Liu, M., Wu, J., & Zhang, L. (2024). Enhancing performance and reducing carbon emissions in sprayed concrete using green silica-lignin admixture and recycled aggregates. Low-Carbon Mater. Green Constr., 2(1), 28. https://doi.org/10.1007/s44242-024-00055-5

- Dong, Y., Liu, P., & Hossain, Md. U. (2023). Life Cycle Sustainability Assessment of Building Construction: A Case Study in China. Sustainability, 15(9), 7655. https://doi.org/10.3390/su15097655

- Eggleston, H. S. (Ed.). (2006). 2006 IPCC guidelines for national greenhouse gas inventories. Institute for Global Environmental Strategies.

- Endo, Y., & Takamura, H. (2021). Evaluation of Life-Cycle Assessment Analysis: Application to Restoration Projects and New Construction in Alpine Climate, Japan. Sustainability, 13(7), 3608. https://doi.org/10.3390/su13073608

- Fan, F., Zhu, S., Huang, J., Xu, X., & Xu, F. (2025). Life cycle assessment of carbon emissions and reduction potential exploration of new building complexes in Universities: A case study in China. Int. J. Life Cycle Assess., 30(4), 621–637. https://doi.org/10.1007/s11367-025-02429-8

- Figueiredo, K., Hammad, A. W. A., & Haddad, A. N. (2024). Life cycle sustainability assessment applied in the built environment. In Materials Selection for Sustainability in the Built Environment (pp. 243–265). Elsevier. https://doi.org/10.1016/B978-0-323-95122-7.00012-5

- Finnveden, G., Hauschild, M. Z., Ekvall, T., Guinée, J., Heijungs, R., Hellweg, S., Koehler, A., Pennington, D., & Suh, S. (2009). Recent developments in Life Cycle Assessment. J. Environ. Manage., 91(1), 1–21. https://doi.org/10.1016/j.jenvman.2009.06.018

- Goh, C. S., Chong, H.-Y., Jack, L., & Mohd Faris, A. F. (2020). Revisiting triple bottom line within the context of sustainable construction: A systematic review. J. Clean. Prod., 252(10), 119884. https://doi.org/10.1016/j.jclepro.2019.119884

- Gore, C., & Gore, E. (1999). Knowledge management: The way forward. Total Qual. Manag., 10(4–5), 554–560. https://doi.org/10.1080/0954412997523

- Guinee, J. B. (2002). Handbook on life cycle assessment operational guide to the ISO standards. Int. J. Life Cycle Assess., 7(5), 311, BF02978897. https://doi.org/10.1007/BF02978897

- Hammond, G. P., & Jones, C. I. (2008). Embodied energy and carbon in construction materials. Proc. Inst. Civ. Eng. - Energy, 161(2), 87–98. https://doi.org/10.1680/ener.2008.161.2.87

- Hansen, R. N., Rasmussen, F. N., Ryberg, M., & Birgisdóttir, H. (2023). A systematic review of consequential LCA on buildings: The perspectives and challenges of applications and inventory modelling. Int. J. Life Cycle Assess., 28(2), 131–145. https://doi.org/10.1007/s11367-022-02126-w

- Hao, J. L., Cheng, B., Lu, W., Xu, J., Wang, J., Bu, W., & Guo, Z. (2020). Carbon emission reduction in prefabrication construction during materialization stage: A BIM-based life-cycle assessment approach. Sci. Total Environ., 723, 137870. https://doi.org/10.1016/j.scitotenv.2020.137870

- Hasik, V., Escott, E., Bates, R., Carlisle, S., Faircloth, B., & Bilec, M. M. (2019). Comparative whole-building life cycle assessment of renovation and new construction. Build. Environ., 161, 106218. https://doi.org/10.1016/j.buildenv.2019.106218

- Hauschild, M. Z., Rosenbaum, R. K., & Olsen, S. I. (Eds). (2018). Life Cycle Assessment: Theory and Practice. Springer International Publishing. https://doi.org/10.1007/978-3-319-56475-3

- India CO2 emissions. (2024). [Worldometer]. https://www.worldometers.info/co2-emissions/india-co2-emissions/

- International Energy Agency. (2025). [Https://www.iea.org/countries/india/electricity].

- Ioppolo, G., Cucurachi, S., Salomone, R., Shi, L., & Yigitcanlar, T. (2019). Integrating strategic environmental assessment and material flow accounting: A novel approach for moving towards sustainable urban futures. Int. J. Life Cycle Assess., 24(7), 1269–1284. https://doi.org/10.1007/s11367-018-1494-0

- ISO. (2006). ISO 14040 (2006).

- Jackson, T. (2009). Prosperity without Growth (0 edn). Routledge. https://doi.org/10.4324/9781849774338

- Kibert, C. J. (2016). Sustainable Construction: Green Building Design and Delivery (1st ed). John Wiley & Sons, Incorporated.

- Kofoworola, O. F., & Gheewala, S. H. (2008). Environmental life cycle assessment of a commercial office building in Thailand. Int. J. Life Cycle Assess., 13(6), 498–511. https://doi.org/10.1007/s11367-008-0012-1

- Köppen-Geiger Climate Classification. (2010). In B. Warf, Encyclopedia of Geography. SAGE Publications, Inc. https://doi.org/10.4135/9781412939591.n671

- Kumar, P., Upadhyay, E., Kannan, K., & Yadav, A. (2024). Evaluation of cooking practices in India to achieve SDGs 7.1.2. Phys. Chem. Earth Parts ABC, 136, 103774. https://doi.org/10.1016/j.pce.2024.103774

- Lahmouri, M., Drewes, J. E., & Gondhalekar, D. (2019). Analysis of Greenhouse Gas Emissions in Centralized and Decentralized Water Reclamation with Resource Recovery Strategies in Leh Town, Ladakh, India, and Potential for Their Reduction in Context of the Water–Energy–Food Nexus. Water, 11(5), 906. https://doi.org/10.3390/w11050906

- Li, X., Xiao, J., Zhu, L., Zhu, D., Song, X., Liu, Y., & Hua, W. (2025). Assessing carbon emissions of the innovative renovation project of Yihe Bridge on Beijing Road. Low-Carbon Mater. Green Constr., 3(1), 9. https://doi.org/10.1007/s44242-025-00071-z

- Liang, S., Gu, H., Bergman, R., & Kelley, S. S. (2020). Comparative life-cycle assessment of a mass timber building and concrete alternative. Wood Fiber Sci., 52(2), 217–229. https://doi.org/10.22382/wfs-2020-019

- Lovelock, J. (2009). The vanishing face of gaia: A final warning. Basic Books.

- Mathumitha, R., Rathika, P., & Manimala, K. (2024). Intelligent deep learning techniques for energy consumption forecasting in smart buildings: A review. Artif. Intell. Rev., 57(2), 35. https://doi.org/10.1007/s10462-023-10660-8

- Medha. (2020). A Study on Energy use in the Water Supply and Treatment in Dehradun, India. Roorkee Water Conclave 2020.

- Milić, I., & Bleiziffer, J. (2024). Life cycle assessment of the sustainability of bridges: Methodology, literature review and knowledge gaps. Front. Built Environ., 10, 1410798. https://doi.org/10.3389/fbuil.2024.1410798

- Ministry of Environment, Forest and Climate Change. (2023, August 3). Net zero emissions target [Gov.in]. https://www.pib.gov.in/PressReleaseIframePage.aspx?PRID=1945472

- Moncaster, A. M., & Song, J.-Y. (2012). A comparative review of existing data and methodologies for calculating embodied energy and carbon of buildings. Int. J. Sustain. Build. Technol. Urban Dev., 3(1), 26–36. https://doi.org/10.1080/2093761x.2012.673915

- Österbring, M., Mata, É., Thuvander, L., & Wallbaum, H. (2019). Explorative life-cycle assessment of renovating existing urban housing-stocks. Build. Environ., 165, 106391. https://doi.org/10.1016/j.buildenv.2019.106391

- Pant, H., & Tewari, A. (2013). Carbon Sequestration Potential of Chir Pine (Pinus roxburghii. Sarg) Forest on Two Contrasting Aspects in Kumaun Central Himalaya between 1650-1860 m Elevation. Appl. Ecol. Environ. Sci., 1(6), 110–112. https://doi.org/10.12691/aees-1-6-2

- Petrovic, B., Myhren, J. A., Zhang, X., Wallhagen, M., & Eriksson, O. (2019). Life Cycle Assessment of Building Materials for a Single-family House in Sweden. Energy Procedia, 158, 3547–3552. https://doi.org/10.1016/j.egypro.2019.01.913

- Rabani, M., Madessa, H. B., Ljungström, M., Aamodt, L., Løvvold, S., & Nord, N. (2021). Life cycle analysis of GHG emissions from the building retrofitting: The case of a Norwegian office building. Build. Environ., 204, 108159. https://doi.org/10.1016/j.buildenv.2021.108159

- Rees, W. E. (2009). The ecological crisis and self-delusion: Implications for the building sector. Build. Res. Inf., 37(3), 300–311. https://doi.org/10.1080/09613210902781470

- Rezaei, F., Bulle, C., & Lesage, P. (2019). Integrating building information modeling and life cycle assessment in the early and detailed building design stages. Build. Environ., 153, 158–167. https://doi.org/10.1016/j.buildenv.2019.01.034

- Santos, R., Costa, A. A., Silvestre, J. D., Vandenbergh, T., & Pyl, L. (2020). BIM-based life cycle assessment and life cycle costing of an office building in Western Europe. Build. Environ., 169, 106568. https://doi.org/10.1016/j.buildenv.2019.106568

- Schneider-Marin, P., & Lang, W. (2020). Environmental costs of buildings: Monetary valuation of ecological indicators for the building industry. Int. J. Life Cycle Assess., 25(9), 1637–1659. https://doi.org/10.1007/s11367-020-01784-y

- Shahana Y. Janjua, Prabir K. Sarker, & Wahidul K. Biswas. (2019). A Review of Residential Buildings’ Sustainability Performance Using a Life Cycle Assessment Approach. J. Sustain. Res., 1(1). https://doi.org/10.20900/jsr20190006

- Shams, S., Mahmud, K., & Amin, Md. A. (2011). A comparative analysis of building materials for sustainable construction with emphasis on CO<SUB align="right">2 reduction. Int. J. Environ. Sustain. Dev., 10(4), 364. https://doi.org/10.1504/ijesd.2011.047767

- Singh*, R. R., Kumar, R. V., & Durai, T. N. P. (2019). Assessment of Embodied Energy for an Institutional Building. Int. J. Recent Technol. Eng. IJRTE, 8(3), 3151–3158. https://doi.org/10.35940/ijrte.c4657.098319

- Torabi, M., & Evins, R. (2024). Towards net-zero carbon buildings: Investigating the impact of early-stage structure design on building embodied carbon. Int. J. Life Cycle Assess. https://doi.org/10.1007/s11367-024-02287-w

- Ullah, Z., Nasir, A. R., Alqahtani, F. K., Ullah, F., Thaheem, M. J., & Maqsoom, A. (2023). Life Cycle Sustainability Assessment of Healthcare Buildings: A Policy Framework. Buildings, 13(9), 2143. https://doi.org/10.3390/buildings13092143

- UNFCCC. (2010). https://unfccc.int/resource/cd_roms/na1/ghg_inventories/english/8_glossary/Glossary.htm

- United Nations Environment Programme (UNEP). (2021). Global Status Report for Buildings and Construction: Towards a Zero-emission, Efficient and Resilient Buildings and Construction Sector.

- Varun, Sharma, A., Shree, V., & Nautiyal, H. (2012). Life cycle environmental assessment of an educational building in Northern India: A case study. Sustain. Cities Soc., 4, 22–28. https://doi.org/10.1016/j.scs.2012.03.002

- Wang, J., Wu, H., Duan, H., Zillante, G., Zuo, J., & Yuan, H. (2018). Combining life cycle assessment and Building Information Modelling to account for carbon emission of building demolition waste: A case study. J. Clean. Prod., 172, 3154–3166. https://doi.org/10.1016/j.jclepro.2017.11.087

- Wang, Y. (2023). Exploring the importance of sustainability in the construction  industry. Appl. Comput. Eng., 9(1), 312–318. https://doi.org/10.54254/2755-2721/9/20230011

- World Bank: Washington, DC, USA. (2021). World Bank. Total Greenhouse Gas Emissions (kt of CO2 Equivalent).

- Yang, X., Hu, M., Wu, J., & Zhao, B. (2018). Building-information-modeling enabled life cycle assessment, a case study on carbon footprint accounting for a residential building in China. J. Clean. Prod., 183, 729–743. https://doi.org/10.1016/j.jclepro.2018.02.070

- Zuo, J., & Zhao, Z.-Y. (2014). Green building research–current status and future agenda: A review. Renew. Sustain. Energy Rev., 30, 271–281. https://doi.org/10.1016/j.rser.2013.10.021