Buffering Techniques to Enhance the Multimedia Streaming Quality over Ad Hoc Network

ABSTRACT

Multimedia streaming over Ad Hoc networks has recently gained popularity because Ad Hoc networks are a highly scalable, self-starting, self-managing and self-healing infrastructure-free network technology. In Ad Hoc networks, data packets are routed from source to destination through intermediate nodes using a distributed network technology with multiple hops. At each intermediate node, data packets experience different buffering delays before being forwarded to the next node. The intermediate nodes make a random number of attempts before successfully forwarding the data packets to the next node. As a result, some data packets reach the destination earlier than the previous packets, some data packets are discarded and some data packets reach the destination later than the subsequent packets, disrupting the sequence of packets at the destination. Especially, when multimedia streaming is based on the User Datagram Protocol (UDP), stale data packets are discarded so that the achievable net throughput from the transport layer to the application layer decreases, resulting in lower quality multimedia streaming. To address these issues, this paper proposes two novel buffering algorithms: Static Reordering Buffer (SRB) and Improved Reordering Buffer (IRB) algorithms. The performances of the proposed buffering algorithms are evaluated using QualNet 6.1 network simulator to enhance the quality of multimedia streaming.

Keywords: Ad hoc networks, Buffering algorithms, Multimedia streaming, Quality of Service, Simulation

#### INTRODUCTION

The emergence and widespread use of mobile devices with advanced wireless technologies such as Bluetooth and Wi-Fi are increasingly being used to access the ubiquitous internet and multimedia services. Ad Hoc network technology has become the de facto standard for connecting billions of wireless devices over infrastructure-free network architecture. Over the years, Ad Hoc networks have gained acceptance in various fields such as environmental monitoring, file sharing, wireless printing, healthcare, transportation and military operations because they are low-cost, easy to deploy, scalable, self-starting and self-healing. The proliferation of multimedia streaming over Ad Hoc network is growing exponentially and has initiated novel applications such as wireless surveillance, voice calls, video conferencing, on-demand video streaming, interactive gaming, disaster monitoring and public safety services [1][2][3]. These applications demand high network bandwidth, reliable data transmission, lower end-to-end delay and jitter performances.

Ad Hoc networks consist of multiple nodes that cooperate with each other to route data to the destination using distributed multi-hop technology. There exist multiple paths for forwarding data from the source to the destination node. Each path may differ in the number of intermediate nodes that receive and buffer data packets and buffers them when the route to the destination node is unavailable, network resources are scarce, during transmission retries, the link fails, etc., until they are forwarded to the next node [4][5]. Therefore, the data packets experience a random end-to-end delay before reaching the destination node [6]. This variance in end-to-end delay and jitter causes some data packets to reach the destination node earlier than the preceding packets, some data packets to reach the destination node later than the succeeding packets, and some of the data packets to be discarded by the network before they reach the destination node, causing the packets to arrive at the destination node out of sequence [7]. As a result, the throughput performance and Quality of Service (QoS) for multimedia applications decrease, which is undesirable for multimedia streaming [8].

To address these issues, this paper proposes two new reordering buffers for the transport layer. In the Static Reordering Buffer (SRB), received data packets are buffered at the transport layer for a static duration of time and sent to the application layer after the data packets are reordered. Although packet reordering reduces packet loss, all data packets experience high end-to-end delay and jitter due to the buffering process. To minimize the end-to-end delay, the Improved Reordering Buffer (IRB) has been proposed, in which only the data packets that are not in the correct order are stored in the buffer until they are reordered and sent to the application layer. The proposed algorithms are implemented in the QualNet 6.1 network simulator and their performance has been studied and the results are presented in this paper.

#### RELATED WORK

Multimedia communication over Ad Hoc networks is an increasing trend due to ease of deployment, low cost compared to infrastructure networks. However, Ad Hoc arrangement of nodes require establishment of route dynamically with multiple nodes and out-of-sequence arrival of packets is an increasingly common phenomenon on the Ad Hoc networks. Further out-of-sequence delivery problem may be mitigated using end node solutions and using network-centric solutions which have received increased attention of many researchers and network vendors. Authors in [9] investigate the problem of packet reordering over internet and define a metric called Reorder Buffer-occupancy Density (RBD) for capturing reordering in a stream and to measure resources required to recover from such reordering. In [10], a theoretical foundation for analysis of the impact on packet reordering on sensor network performance is established considering the analytical relationships for variation of Reorder Density (RD) and Reorder Buffer-occupancy Density (RBD) as packet streams pass through re-sequencing buffers. Further the model presented in [10] provides an overall perspective of the impact of re-sequencing buffers and allows for buffer allocation to meet bounds related to packet reordering. In [11], authors use simulation studies to confirm the existence of the stochastic compensation effect to reduce packet reordering with randomizes packet scheduling at source into multiple paths of random transmission delays and its considerable influence on the application performance using simulation studies. Authors in [12] demonstrate the catastrophic effect of packet reordering on the performance of the TCP flavours using simulation studies. Authors of paper [13] conclude that parallel processing in the network processor and the allocation scheme for the transmit buffer also adversely impacts packet ordering. Also [14] explores different transmit buffer allocation schemes namely, contiguous, strided, local and global which reduces the packet retransmission to 24%. In [15], authors propose a receiver-initiated duty cycling MAC protocol called reordering-passive MAC (RP-MAC) for wireless sensor networks to achieve high energy efficiency and network throughput with lower end-to-end delay for heavy traffic or low duty cycle. Paper [16] proposes a cross layer approach between network layer and transport layer to identify the dropped and reordered packets for TCP in MANETs. The proposed work in [16] identifies the dropped or reordered packet and the source retransmits only the dropped but not reordered packets and the unnecessary retransmissions are reduced.

#### OVERVIEW OF MULTIMEDIA STREAMING OVER AD HOC NETWORKS

Multimedia streaming applications demands higher bandwidth and lower packet loss for uninterrupted services. To minimize bandwidth consumption and latency during streaming, raw multimedia data is compressed with codecs before transmission. In particular, Constant Bit Rate (CBR) codecs are used for multimedia compression in Ad Hoc networks because they can generate a suitable target bit rate to maximize the use of limited bandwidth [17]. In addition, most multimedia applications prefer transmission control protocols based on User Datagram Protocol (UDP) because these protocols require only a small channel bandwidth for data transmission compared to Transmission Control Protocols-Internet Protocol (TCP-IP).

The conventional services based on the UDP transport mechanism use the packet sequence number to indicate the freshness of the packets, duplication and sequence detection when the packets are received at the destination [18]. At the beginning of the transmission service, the source sets the sequence number of the first data packet to zero and for the following data packets the sequence number is incremented by one. These packets forwarded by the source may take different paths to the destination, resulting in increased variation in end-to-end delay, packet discard and causing packet discrepancy at the receiving end. To avoid this discrepancy, the destination node maintains the Expected Packet Sequence (EPS) Number or the sequence number of the last packet received. After receiving each data packet at the destination, the transport layer compares the EPS value with the sequence number of the data packet. If the EPS value is less than or equal to the sequence number of the received data packet, the received data packet is considered valid and rendered to the application layer, otherwise it is discarded as a duplicate or stale packet [18]. After each successful reception of a data packet, the EPS value at the destination is updated to the sequence number of the received packet plus one to indicate the next data packet expected from the source and the same process is repeated until the end of the service (Figure 1). Even if packets are forwarded to the correct destinations, they may be discarded due to incorrect sequence numbers, resulting in a reduction in packet delivery and throughput at the application layer in Ad Hoc networks. To address this issue, this paper proposes algorithms such as Static Reordering Buffer (SRB) and Improved Reordering Buffer (IRB) to improve the throughput performance and thereby enhance the QoS of multimedia applications in Ad Hoc networks.

Start

Is

received data packet sequence number ≥ EPS

?

No

Send data packet to Application Layer

Discard received data packet

Yes

EPS = Sent data packet

sequence number + 1

Start

Is

received data packet sequence number ≥ EPS

?

No

Send data packet to Application Layer

Discard received data packet

Yes

EPS = Sent data packet

sequence number + 1

Figure 1: Flowchart of Conventional transport layer service

#### PROPOSED STATIC REORDERING BUFFER (SRB) ALGORITHM

The proposed Static Reordering Buffer (SRB) algorithm uses a priority queue-based buffer at the transport layer to reduce the probability of packet loss due to packet discrepancies by using the following control parameters at the destination node:

- Expected Packet Sequence (EPS): The EPS value is a sequence number of the packet to be received.

- Buffer Retention Timer (BRT): The BRT expires periodically according to a static interval. Shortly after its expiration, the data packets are sent from the transport layer buffer to the application layer.

- Priority Queue: The received data packets are inserted temporarily in the priority queue according to the sequence number of the packet before being sent to the application layer.

Assign EPS to 0, Initialize BRT to static time interval

Start

Is

received data packet sequence number ≥ EPS

?

No

Insert data packet according to its sequence number into priority queue

Is

BRT expired

?

Send all sorted data packets queued in priority queue to application layer

EPS = Last sent data packet

sequence number + 1

Reinitialize BRT to static time interval

Discard received data packet

Yes

No

Yes

Assign EPS to 0, Initialize BRT to static time interval

Start

Is

received data packet sequence number ≥ EPS

?

No

Insert data packet according to its sequence number into priority queue

Is

BRT expired

?

Send all sorted data packets queued in priority queue to application layer

EPS = Last sent data packet

sequence number + 1

Reinitialize BRT to static time interval

Discard received data packet

Yes

No

Yes

Figure 2: Flowchart of proposed Static Reordering Buffer (SRB) algorithm

In the proposed SRB mechanism, when the first data packet is received at the destination node, the transport layer resets the EPS value to zero, initializes the BRT for a static time interval and inserts the received data packet into the priority queue according to the packet sequence. Then, the subsequent received data packets with a packet sequence number greater than or equal to the EPS value are inserted into the priority queue at the appropriate position according to their packet sequence number and the received data packets with a packet sequence number less than the EPS value are discarded. This process is repeated until the BRT expires. When the BRT expires, the sequentially ordered data packets in the priority queue are rendered to application layer, the BRT is reinitialized to a static time interval, and the EPS value is updated to the last sent packet sequence number plus one to indicate the next expected data packet. This process is repeated until the end of the service (Figure 2). Thus, in the proposed SRB algorithm, the possibility of packet drop reduces due to re-sequencing the out of sequence data packets to improve throughput performance and QoS for multimedia applications. On the other hand, queuing latency may increase because all received data packets remain in the priority queue until the BRT expires. To address this issue, an enhancement to the SRB algorithm has been proposed in Improved Reordering Buffer (IRB) algorithm.

#### IMPROVED REORDERING BUFFER (IRB) ALGORITHM

In the proposed Improved Reordering Buffer (IRB) algorithm, the transport layer initializes the control parameters such as EPS and BRT similar to the SRB algorithm.

Assign EPS to 0, Initialize BRT to static time interval

Start

Is

sequence number of first data packet in priority queue == EPS

?

Yes

Insert data packet into priority queue according to its sequence number

Send received data packet to application layer

No

Is

received data packet sequence number > EPS

?

Discard received data packet

Is

BRT expired

?

Send all sorted data packets queued in priority queue to application layer

EPS = Last sent data packet

sequence number + 1

Reinitialize BRT to static time interval

No

Yes

EPS = EPS + 1

Yes

No

Is

received data packet sequence number == EPS

?

Retrieve and sent data packet to application layer

No

Yes

Assign EPS to 0, Initialize BRT to static time interval

Start

Is

sequence number of first data packet in priority queue == EPS

?

Yes

Insert data packet into priority queue according to its sequence number

Send received data packet to application layer

No

Is

received data packet sequence number > EPS

?

Discard received data packet

Is

BRT expired

?

Send all sorted data packets queued in priority queue to application layer

EPS = Last sent data packet

sequence number + 1

Reinitialize BRT to static time interval

No

Yes

EPS = EPS + 1

Yes

No

Is

received data packet sequence number == EPS

?

Retrieve and sent data packet to application layer

No

Yes

Figure 3: Flowchart of proposed Improved Reordering Buffer (IRB) algorithm

However, in this proposed IRB algorithm, the received data packets with the sequence number corresponding to the EPS value are directly rendered to the application layer without storing them in the priority queue. The received data packets with sequence number higher than the EPS value are inserted into the priority queue at the corresponding position according to their packet sequence number. The retained packets in the priority queue are sent to the application layer as soon as the sequence number of the data packet is equal to the EPS value or after the BRT expires. Meanwhile, the EPS value is incremented by one after sending each data packet to the application layer. Once the BRT has expired, the remaining data packets retained in the priority queue are sent to the application layer by the transport layer. In addition, the BRT is reinitialized to a static time interval and EPS value is set to the sequence number of the last data packet sent plus one. The proposed IRB mechanism significantly reduces the queuing delay and improves the QoS for multimedia applications as shown in figure 3.

#### SIMULATION AND RESULTS

In this scenario, performance of the proposed SRB and IRB algorithms are evaluated and compared with the AODV routing protocol using QualNet 6.1 network simulator. The scenario designed for the simulation studies includes nodes arranged in a grid with the inter node distance of 49, 81, 121, 169 and 225 nodes. Two ray path loss model with constant shadowing of mean 4dB has been selected to carry out the simulation studies. The remaining simulation parameters considered for simulation studies are listed in Table 1.

Table 1. Simulation Parameters

| Parameter | Value |
| --- | --- |
| Area | 2500m x 2500m |
| Simulation Time | 30 seconds |
| Number of nodes | 49, 81, 121, 169 and 225 |
| Node Placement | Grid |
| Traffic type | CBR |
| Buffering Techniques | SRB, IRB and without buffer |
| Radio type | IEEE 802.11g |
| Channel frequency | 2.412 GHz |
| Path loss model | Two Ray |
| Shadowing model | Constant |

Initially, for the 49-node scenario (Figure 4), we consider a 1Mbps Constant Bit Rate (CBR) link to evaluate the performance of the SRB algorithm by setting the buffer retention period to 125ms. To introduce the out-of-sequence arrival of data packets at the destination node, a random propagation delay with a maximum variance of 30ms is introduced at the air interface of the source node while data packets are transmitted to destination during the simulation process. The performance metrics such as throughput, aggregate messages received, average delay and jitter are recorded. Similar simulation studies are performed by increasing the buffer retention period to 250ms and 500ms. Also, the simulation studies are repeated by considering the scenarios with node density of 81, 121, 169 and 225 nodes. Similar simulation studies are repeated by changing the CBR data rate to 2Mbps and 3Mbps. A similar simulation procedure is also performed for the proposed IRB algorithm. To evaluate the performance of conventional transport layer mechanism, a similar procedure is repeated without varying the BRT time interval.

Figure 4: Snapshot of simulation scenario with 49 nodes

Figures 5, 6, 7 and 8 shows the throughput, aggregate messages received, average delay and jitter performance of the proposed SRB, IRB and conventional transport layer mechanisms with increasing node density and data rate for different BRT expiration intervals. Figures 9 and 10 depict the number of data packets in the priority queue during simulation study of the proposed SRB and IRB algorithms respectively with different BRT intervals for different node densities.

In this simulation study, a random propagation delay is introduced at the air interface of the source node to deliver out-of-sequence data packets to the destination node. In conventional transport layer service, the destination node discards all data packets with a sequence number lower than EPS, resulting in a decrease in the throughput and aggregate message received performance (Figures 5 and 6). On the other hand, in the proposed SRB algorithm at the transport layer, the received data packets that are greater than or equal to EPS are stored and sorted for the duration of BRT using a priority queue before being sent to the application layer. With this proposed reordering process, the loss of data packets at destination node due to out-of-sequence delivery is reduced, improving the throughput and aggregate message received performance of SRB algorithm (Figure 5 and 6).

In contrast, if the sequence number of the received data packet matches the EPS value, the IRB sends data packets directly to the application layer without storing them in the priority queue. Only the data packets with a sequence number greater than the EPS value are retained in priority queue until a suitable data packet with a sequence number matching the EPS value arrives or for the duration of the BRT. Therefore, the number of data packets rendered to application layer is slightly higher for IRB compared to the SRB mechanism, resulting in improved throughput and aggregate message received performance of IRB algorithm (Figures 5 and 6). From Figures 5 and 6, it is evident that as the BRT expiration interval increases, the throughput and aggregate message received performance of proposed SRB and IRB algorithms increase because longer BRT intervals reduce out-of-sequence packet drop [19].

Figure 5: Throughput Performance of SRB algorithm, IRB algorithm and conventional mechanism without buffer for increasing node density and BRT interval for 2Mbps data rate

Figure 6: Aggregate Messages Received Performance of SRB algorithm, IRB algorithm and conventional mechanism without buffer for increasing node density and BRT interval for 2Mbps data rate

From Figure 7, it is evident that delay performance in the conventional transport layer is better than that of the proposed SRB and IRB mechanisms. This is because in conventional transport layer, the received data packets that match the EPS value are directly sent to the application layer without queuing and processing overhead. Whereas, in proposed SRB algorithm, all sorted data packets are retained in priority queue until the BRT expires, which increases the number of packets occupied in the priority queue. Therefore, there is a large queuing delay and processing overhead for each data packet, resulting in an increase in average delay performance compared to the IRB algorithm [19][20]. In contrast, the IRB algorithm holds back a smaller number of data packets whose sequence number is greater than EPS value and sends them to application layer immediately after the arrival of proper data packet with expected sequence number in accordance with EPS value or after the BRT expires. Therefore, the number of packets queued in the priority queue decreases, which in turn decreases queuing delay and processing overheads leading to improved average delay performance of IRB algorithm (Figure 7). Figure 7 also shows that the average delay performance of SRB and IRB algorithms increases as the BRT expiration interval increases, because increasing the BRT interval increases the packets retention time of packets in a priority queue, resulting in a higher average packet delay.

Figure 7: Average delay performance of SRB algorithm, IRB algorithm and conventional mechanism without buffer for increasing node density and BRT interval for 2Mbps data rate

Figure 8: Average jitter performance of SRB algorithm, IRB algorithm and conventional mechanism without buffer for increasing node density and BRT interval for 2Mbps data rate

From Figure 8, it is evident that the jitter performance of the conventional transport service is higher than that of the proposed SRB and IRB algorithms. This is because the received data packets that are not in the correct order are discarded at a random time, which leads to an increase in the variance of the inter-packet interval or jitter performance. It can also be seen from Figure 8 that the jitter performance of SRB algorithm is higher than that of the IRB algorithm. This is because data packets retained in the priority queue experience a high variance in queuing delay until the BRT expires, which increases the jitter performance compared to the IRB algorithm (Figure 7). In the IRB algorithm, on the other hand, data packets with sequence numbers that match EPS are sent directly to application layer and only the data packets that do not match the sequence are retained in priority queue, reducing the variance in queuing delay and processing overhead, resulting in a reduction in jitter performance (Figure 7). Figure 8 also shows that the jitter performance of SRB and IRB algorithms increases as the BRT expiration interval increases, since the data packets are routed through a larger number of intermediate nodes.

Figure 9 (a,b): Throughput performance of SRB algorithm, IRB algorithm and conventional mechanism without buffer for increasing data rates and Buffer Retention period for (a) 81 and (b) 169 node density

Figure 10(a,b): Aggregate Messages Received Performance of SRB algorithm, IRB algorithm and conventional mechanism without buffer with increasing data rates and Buffer Retention period for (a) 81 and (b) 169 node density

Figure 11 (a, b): Average delay performance of SRB algorithm, IRB algorithm and conventional mechanism without buffer with increasing data rates and Buffer Retention period for (a) 81 and (b) 169 node density

Figure 12 (a, b): Average jitter Performance of SRB algorithm, IRB algorithm and conventional mechanism without buffer with increasing data rates and Buffer Retention period for (a) 81 and (b) 169 node density

Figures 9(a, b), 10(a, b), 11(a, b), and 12(a, b) show the throughput, aggregate messages, average delay and jitter performance of the proposed SRB algorithm, IRB algorithm and conventional transport services with increasing data rate and BRT expiration interval for node densities of 81 and 169 nodes. Figures 9(a, b) and 10(a, b) depicts that the throughput and aggregate messages received performances of both the proposed SRB algorithm, IRB algorithm and the conventional transport layer services increase with increasing data rate as a larger number of data packets are routed to the destination. Figures 11(a, b) and 12(a, b) show that the average delay and jitter performance of the proposed SRB algorithm, IRB algorithm and the conventional transport layer services increases with increasing data rate. Since, network congestion and co-channel interference increase the number of retransmissions, thereby increasing the transmission delay [21]. In addition, the number of data packets that do not arrive at the destination in the correct order increases queuing overheads.

#### CONCLUSION

In Ad Hoc networks, the performance of each link varies continuously over time due to variation in hop count, buffer length, traffic density, route discovery process, etc., which results in packet mismatch as data packets do not arrive at the destination node in the correct order. In conventional transport layer service, due to the packet discrepancy, data packets are discarded which reduces the achievable throughput performance of Ad Hoc network thereby reducing the QoS of multimedia applications. To address this issue, Static Reordering Buffer (SRB) and Improved Reordering Buffer (IRB) algorithms are proposed for the transport layer service in this paper. The proposed SRB and IRB algorithms take into account out-of-sequence data packets and arrange them in a priority queue before sending them to the application layer, thereby enhancing throughput and QoS for multimedia applications. Simulation results show that the delay and jitter performance of IRB algorithm is better than that of the SRB algorithm due to its low processing and queuing delay.

###### REFERENCES

- Patrick Stuedi and Gustavo Alonso, "Wireless Ad Hoc VoIP", MNCNA '06 ,2007

- J. Karlsson; H. Li; J. Eriksson, "Real-time video over wireless ad-hoc networks", 14th International Conference on Computer Communications and Networks, 2005.

- Sadaf Momeni, Bernd E Wolfinger, "Availability evaluations for IPTV in VANETs with different types of access networks ", EURASIP Journal on Wireless Communications and Networking, 2014

- Kamal Mistry, Sanjay Srivastava, R. B. Lenin," Buffer Aware Routing in Interplanetary Ad Hoc Network ", COMSNETS'09 Proceedings of the First international conference on COMmunication Systems and NETworks, 2009

- Thomas D. Lagkas, Georgios I. Tsiropoulos, "Bandwidth Allocation in Cooperative Wireless Networks: Buffer Load Analysis and Fairness Evaluation", Elsevier Physical Communication, 2011.

- Baban A. Mahmood, D. Manivannan, "GRB: Greedy Routing Protocol with Backtracking for Mobile Ad Hoc Networks", arXiv:1804.01133 [cs.NI], April 2009

- Xiaoming Zhou and Piet Van Mieghem, "Reordering of IP Packets in Internet"

- White Paper, "Media Quality and Network Connectivity Performance in Skype for Business Online", Microsoft

- Nischal M. Piratla, Anura P. Jayasumana, Abhijit A. Bare, Tarun Banka, “Reorder buffer-occupancy density and its application for measurement and evaluation of packet reordering”, 10.1016/j.comcom.2007.03.001, Computer Communications, Elsevier,30 (2007) 1980–1993

- Raghunandan M. Narasiodeyar, Anura P. Jayasumana, “Improvement in packet-reordering with limited re-sequencing buffers: An analysis”, 38th Annual IEEE Conference on Local Computer Networks, 21-24 Oct. 2013, ISSN(p): 0742-1303, DOI: 10.1109/LCN.2013.6761274

- Korzun, D.G., Kuptsov, D., Gurtov, A, “A comparative simulation study of deterministic and stochastic strategies for reduction of packet reordering in multipath data streaming”, International Journal of Simulation: Systems, Science and Technology, 2016.

- Yasin W., Ibrahim H., Abdul Hamid N.A.W., Udzir N.I, “Performance Analysis of Transport Control Protocol Flavours in the Existence of Packet Reordering Phenomena”, Digital Enterprise and Information Systems. DEIS 2011. Communications in Computer and Information Science, vol 194. Springer, Berlin, Heidelberg, DOI-https://doi.org/10.1007/978-3-642-22603-8_50, ISBN(p):978-3-642-22602-1, ISBN(O):978-3-642-22603-8

- S. Govind, R. Govindarajan and J. Kuri, "Packet Reordering in Network Processor", 2007 IEEE International Parallel and Distributed Processing Symposium, Rome, 2007, pp. 1-10. doi: 10.1109/IPDPS.2007.370287

- Ka-Cheong Leung, Chengdi Lai, Victor O. K. Li, Daiqin Yang, “A packet-reordering solution to wireless losses in transmission control protocol”, Wireless Networks, October 2013, Volume 19, Issue 7, pp 1577–1593

- Q. Hu, Q. Tian & Z Tang, “RP-MAC: A Passive MAC Protocol with Frame Reordering for Wireless Sensor Networks”, International Journal of Wireless Information Networks, March 2013, Volume 20, Issue 1, pp 74–80 https://doi.org/10.1007/s10776-012-0184-2

- Dodda Sunitha, AithaNagaraju, Gugulothu Narsimha, “Cross-layer based routing protocol and solution to packet reordering for TCP in MANET”, 2017, DOI 10.1007/s10586-017-1179-7

- Pooja J Kotian, P Vaishnavi, Shifana Begum, “Review on Data Traffic in Real Time for MANETS”, International Research Journal of Engineering and Technology (IRJET), Vol.6(11), Nov 2017

- White paper, H. Schulzrinne, GMD Fokus, S. Casner," RTP: A Transport Protocol for Real-Time Applications Audio-Video Transport Working Group”, IETF, Jan. 1996

- Syed Humair Ali, Syeda Areeba Nasir, Sameer Qazi, "Impact of Router Buffer Size on TCP/UDP Performance", Proceedings of 2013 3rd IEEE International Conference on Computer, Control & Communication.

- Sarfraz Ahmed Soomro, M. Mujtaba Shaikh, Abdul Fattah Chandio, Nasreen Nizamani, Ehsan Ali Buriro," Impact of Buffer Size on UDP Performance for Real-Time Video Streaming Application ", IJCSNS International Journal of Computer Science and Network Security, VOL.18 No.6, June 2018, p. 157-160

- Pan Li, Yuguang Fang, Jie Li, "Throughput, Delay, and Mobility in Wireless Ad Hoc Networks", publication in the IEEE INFOCOM 2010 proceedings This paper was presented as part of the main Technical Program at IEEE INFOCOM 2010, IEEE