Eco-Friendly Multistage Treatment of Dairy Wastewater Using Natural Coagulants: Mechanisms, Performance Evaluation, and Future Perspectives-A Review

Abstract

The dairy industry is one of the major agro-based sectors contributing significantly to global wastewater generation. Dairy effluent is characterized by high concentrations of organic matter, suspended solids, fats, and nutrients, resulting in elevated biochemical oxygen demand (BOD) and chemical oxygen demand (COD). Conventional treatment methods using chemical coagulants such as alum and ferric salts are widely employed; however, these methods are associated with drawbacks including high sludge production, residual toxicity, and increased operational costs. In recent years, natural coagulants derived from plant-based materials, biopolymers, and agro-wastes have emerged as sustainable alternatives due to their biodegradability, low toxicity, and economic feasibility. This review critically examines the characteristics of dairy wastewater, mechanisms of coagulation, and the performance of various natural coagulants. Furthermore, their integration into multistage treatment systems and future prospects for large-scale applications are discussed, highlighting their potential for sustainable wastewater management.

Keywords: Dairy wastewater, natural coagulants, sustainable treatment, coagulation-flocculation, eco-friendly wastewater management.

1. Introduction

The rapid growth of the dairy industry has resulted in increased water consumption and wastewater generation. Dairy processing units utilize large volumes of water for operations such as equipment cleaning, pasteurization, homogenization, and product processing. Consequently, the generated wastewater contains substantial amounts of biodegradable organic matter, including lactose, proteins, and lipids (Demirel et al., 2018; Verma et al., 2018; Ahmad et al., 2021). In addition to organic pollutants, dairy effluents also contain detergents, sanitizers, and inorganic salts, which contribute to fluctuating physicochemical characteristics (Kushwaha et al., 2020; Rinaudo, 2019). The seasonal variation in milk production and processing further influences wastewater composition, making treatment more complex (Demirel et al., 2018; Kushwaha et al., 2020).Moreover, stringent environmental regulations necessitate the implementation of efficient and sustainable treatment strategies (Chen, 2004; Ali et al., 2021).The discharge of untreated dairy wastewater into natural water bodies leads to severe environmental issues such as oxygen depletion, eutrophication, and ecological imbalance (Kushwaha et al., 2020; Muthuraman and Sasikala, 2018; Rinaudo, 2019; Abidin et al., 2020).Coagulation–flocculation is a widely used primary treatment method (Verma et al., 2018; Yin, 2018; Bhatia et al., 2018). However, chemical coagulants raise concerns regarding sludge toxicity and cost (Verma et al., 2018; Yin, 2018; Chen et al., 2021).

Natural coagulants have gained attention due to their eco-friendly nature (Teh et al., 2016; Choy et al., 2017; Zaman et al., 2020; Zhou et al., 2022).

2. Characteristics of Dairy Wastewater

Dairy wastewater composition varies depending on processing operations and product types. It typically contains high concentrations of organic and inorganic constituents such as proteins, lactose, fats, oils, suspended solids, and nutrients like nitrogen and phosphorus (Ahmad, et al., (2019), Kushwaha, et al., (2020), Ahmad, et al., (2021)). (Ahmad, et al., (2019), Kushwaha, et al., (2020), Ahmad, et al., (2021)). The variability in composition is influenced by factors such as type of dairy product (milk, cheese, butter), cleaning frequency, and water usage patterns (Demirel, et al., (2018) (Kushwaha, et al., (2020), Demirel, et al., (2018), Kushwaha, et al., (2020)). High organic load results in rapid microbial activity, which significantly increases oxygen demand in receiving water bodies ((Kushwaha, et al., (2020), Rinaudo, (2019).

The BOD of dairy wastewater generally ranges from 800 to 3000 mg/L, while COD values can exceed 5000 mg/L, indicating a high organic load (Demirel, et al., (2018) (Kushwaha, et al., (2020), Ahmad, et al., (2021)). The presence of fats and oils forms a surface layer that inhibits oxygen transfer, thereby affecting biological treatment processes (Muthuraman and Sasikala (2018), Rinaudo, M. (2019)). Additionally, cleaning agents and detergents contribute to fluctuations in pH and chemical composition, further complicating treatment processes (Kushwaha, et al., (2020), Ahmad, et al., (2021)). These characteristics necessitate effective pre-treatment methods such as coagulation–flocculation to improve overall treatment efficiency (Kushwaha, et al., (2020), Verma, et al., (2018), Yin, (2018). Dairy wastewater also contains significant levels of dissolved solids, including chlorides, sulfates, and residual cleaning chemicals, which may interfere with biological treatment processes (Ahmad, et al., (2021), Ali, et al., (2021)) The presence of casein and whey proteins further contributes to the colloidal nature of the effluent, making separation difficult without proper pre-treatment (Demirel, et al., (2018)Rinaudo, (2019)) . Moreover, oil and grease components tend to form stable emulsions, reducing the efficiency of sedimentation processes (Muthuraman and Sasikala (2018), Rinaudo, M. (2019)). These factors necessitate the use of effective coagulation–flocculation techniques to destabilize colloids and enhance solid–liquid separation (Verma, et al., (2018), Yin, (2018), Bhatia, et al., (2018). Additionally, variability in hydraulic load and shock loading conditions in dairy industries further complicates treatment system design, requiring flexible and adaptive treatment strategies (Chen, (2004), Ali, et al., (2021).

Table 1. Typical Characteristics of Dairy Wastewater

| S.NO | PARAMETER | RANGE |
| --- | --- | --- |
| 1. | pH | 4.5-8.5 |
| 2. | BOD (mg/L) | 800-3000 |
| 3. | COD(mg/L) | 1500-5000 |
| 4. | TSS(mg/L) | 200-1000 |
| 5. | Oil & Grease (mg/L) | 50-500 |

3. Environmental Impact of Dairy Wastewater

Untreated dairy effluent poses significant environmental threats. High organic content promotes microbial growth, leading to rapid consumption of dissolved oxygen and creating anaerobic conditions detrimental to aquatic life (Kushwaha, et al., (2020), Rinaudo, (2019), Ahmad, et al., (2021)). Nutrient enrichment, particularly nitrogen and phosphorus, leads to eutrophication, resulting in algal blooms and ecosystem disruption (Rinaudo, (2019), Crini, et al., (2018)). Furthermore, the decomposition of organic matter produces foul odors and releases greenhouse gases such as methane and carbon dioxide (Chen, (2004), Mohd Salleh, et al., (2019)). Improper discharge can also contaminate groundwater sources, affecting drinking water quality and public health . These environmental concerns highlight the need for efficient and sustainable wastewater treatment solutions (Ali, et al., (2021)).

4. Conventional Chemical Coagulants: Performance and Limitations

Chemical coagulants such as alum and ferric salts are commonly used due to their high efficiency in removing turbidity and suspended solids. These coagulants function by neutralizing charges and forming flocs that settle easily (Verma, et al., (2018), Yin, (2018)). They are widely used in industrial applications due to their reliability and well-established mechanisms.

Despite their effectiveness, several limitations exist. High sludge generation requires proper disposal, increasing operational costs. Residual metal ions may pose health risks if not adequately removed. Additionally, the performance of chemical coagulants is highly dependent on pH and dosage conditions, requiring continuous monitoring and adjustment. These drawbacks necessitate the development of alternative treatment methods that are both effective and environmentally sustainable (Verma, et al., (2018)).

5. Natural Coagulants: Mechanisms and Performance

Natural coagulants are derived from plant materials, animal sources, and microorganisms.

Table 2 A: Protein-Based Natural Coagulants

| S.NO | Coagulant | Source | Component | Efficiency | Reference |
| --- | --- | --- | --- | --- | --- |
| 1. | Moringa oleifera | Seeds | Cationic proteins | High turbidity removal | [17,19] |
| 2. | Cicer arietinum | Chickpea | Proteins | Moderate | [42] |
| 3. | Vigna unguiculata | Cowpea | Proteins | Moderate | [70] |
| 4. | Glycine max | Soybean | Proteins | High | [65] |
| 5. | Phaseolus vulgaris | Bean | Proteins | Moderate | [42] |
| 6. | Pisum sativum | Pea | Proteins | Moderate | [70] |
| 7. | Dolichos lablab | Hyacinth bean | Proteins | Moderate | [64] |
| 8. | Tamarind seed kernel | Tamarind | Proteins | High | [70] |
| 9. | Fenugreek seeds | Trigonella frenum-graecum | Proteins | Moderate | [41] |
| 10. | Groundnut cake | Peanut | Proteins | Moderate | [52] |

Table 2B: Polysaccharide-Based Natural Coagulants

| S.NO | Coagulant | Source | Component | Efficiency | Reference |
| --- | --- | --- | --- | --- | --- |
| 1. | Chitosan | Shellfish | Polysaccharide | High COD removal | [23,24] |
| 2. | Cactus | Plant | Mucilage | Moderate | [32] |
| 3. | Banana peel | Agro-waste | Cellulose | Moderate | [27,28] |
| 4. | Potato starch | Potato | Starch | Moderate | [42] |
| 5. | Corn starch | Maize | Starch | Moderate | [70] |
| 6. | Rice husk | Agro-waste | Cellulose | Moderate | [64] |
| 7. | Okra mucilage | Abelmoschus esculentus | Polysaccharide | High | [70] |
| 8. | Aloe vera | Plant | Gel polysaccharides | Moderate | [52] |
| 9. | Guar gum | Cyamopsis tetragonoloba | Galactomannan | High | [41] |
| 10. | Xanthan gum | Microbial | Polysaccharide | High | [70] |

They contain bioactive compounds such as proteins and polysaccharides that facilitate coagulation (Teh, et al., (2016), Saleem, et al., (2019), Zaman, et al., (2020), Mehta, et al., (2021)). These materials are renewable, biodegradable, and environmentally friendly, making them suitable alternatives to chemical coagulants (Choy, et al., (2017), Zhou, et al., (2022), Das, et al., (2023)).

Natural coagulants operate through multiple mechanisms including charge neutralization, polymer bridging, and adsorption (Choy, et al., (2017), Ghernaout, (2020), Bhatia, et al., (2018)).

Table 3: Mechanisms vs Coagulant Type

| S.NO | Mechanism | Description | Coagulant Example |
| --- | --- | --- | --- |
| 1. | Charge Neutralization | Neutralizes particle charge | Moringa |
| 2. | Polymer Bridging | Links particles | Chitosan |
| 3. | Adsorption | Surface binding | Agro-waste |

Positively charged molecules neutralize negatively charged particles, while long-chain polymers form bridges between particles, leading to floc formation (Shan, et al., (2017), Vijayaraghavan, et al., (2020)). Studies have demonstrated that natural coagulants such as Moringa oleifera and chitosan can achieve turbidity removal efficiencies of up to 90% along with significant reductions in COD and BOD (Rinaudo, (2019), Abidin, et al., (2020)). However, their performance may vary depending on wastewater characteristics and operational conditions (Zaman, et al., (2020), Mehta, et al., (2021)).

6. Comparison Between Natural and Chemical Coagulants

Several studies have quantitatively compared the performance of natural and chemical coagulants in wastewater treatment. Chemical coagulants such as alum and ferric chloride can achieve turbidity removal efficiencies of up to 95–99%, whereas natural coagulants typically achieve removal efficiencies ranging from 80–95% depending on wastewater characteristics and dosage conditions (Chen, et al., (2021), Zhou, et al., (2022), Mehta, et al., (2021)).

Natural coagulants such as Moringa oleifera and chitosan have demonstrated COD reduction efficiencies of 60–85% and BOD reduction efficiencies of 50–80% (Rinaudo, (2019), Abidin, et al., (2020)). In contrast, chemical coagulants often provide slightly higher removal rates but generate significantly larger volumes of non-biodegradable sludge.

Additionally, sludge generated from natural coagulants is biodegradable and can be safely disposed of or reused in agricultural applications, whereas chemical sludge may contain toxic metal residues requiring specialized disposal methods. Economic analyses indicate that natural coagulants can reduce treatment costs by 20–50% due to lower chemical usage and sludge handling costs (Chen, (2004), Ali, et al., (2021)).

Table: 4 Comparison of Natural and Chemical Coagulants

| S.NO | PARAMETER | NATURAL COAGULANTS | CHEMICAL COAGULANTS |
| --- | --- | --- | --- |
| 1. | Source | Plant-based, biodegradable materials | Synthetic inorganic salts |
| 2. | Cost | Low | High |
| 3. | Efficiency | Moderate to high | High |
| 4. | Sludge Production | Low, biodegradable | High, often hazardous |
| 5. | Toxicity | Non-toxic | May cause toxicity |
| 6. | Environmental Impact | Eco-friendly | Environmental concerns |

7. Operational Parameters Affecting Coagulation

The efficiency of coagulation depends on several factors such as pH, dosage, mixing conditions, and temperature. Optimal pH for natural coagulants is typically between 6 and 8 (Zhang, et al., (2023), Bhatia, et al., (2018)). Excess dosage can lead to particle restabilization, reducing efficiency (Zhang, et al., (2023)). Proper mixing ensures uniform distribution and floc formation (Bhatia, et al., (2018), Vijayaraghavan, et al., (2020)). Temperature influences reaction kinetics and can affect coagulation efficiency (Zhang, et al., (2023)). Careful optimization of these parameters is essential for achieving consistent treatment performance (Zhang, et al., (2023), Bhatia, et al., (2018)).

Table 5 :Operational Parameters

| S.NO | PARAMETER | OPTIMAL RANGE | EFFECT |
| --- | --- | --- | --- |
| 1. | pH | 6–8 | Maximum efficiency |
| 2. | Dosage | Varies | Overdose reduces efficiency |
| 3. | Mixing Speed | Moderate | Ensures floc formation |
| 4. | Temperature | Ambient | Affects kinetics |

8. Multistage Treatment Approach

A multistage treatment system combines physical, chemical, and biological processes to achieve comprehensive pollutant removal. Primary treatment involves coagulation–flocculation, followed by biological processes such as activated sludge and anaerobic digestion (Gregory, (2018), Edzwald, (2017), Ahmad, et al., (2021)). Tertiary treatments include membrane filtration and advanced oxidation processes (Gregory, (2018), Edzwald, (2017)). Integration of natural coagulants into multistage systems improves overall efficiency and reduces environmental impact (Mehta, et al., (2021)). This approach also enhances resource recovery and sustainability. However, the efficiency of natural coagulants is highly dependent on wastewater characteristics such as pH, turbidity, and organic load. Several studies have reported reduced performance under extreme pH conditions, limiting their universal applicability. Furthermore, large-scale industrial implementation remains a challenge due to variability in raw material quality. (Mohd Salleh, et al., (2019), Ali, et al., (2021)).

9. Economic and Sustainability Analysis

Natural coagulants reduce chemical procurement costs and sludge disposal expenses. Their use supports circular economy principles by utilizing agricultural waste materials. Additionally, reduced environmental impact lowers compliance costs associated with wastewater discharge regulations (Chen, (2004)). These benefits make natural coagulants an economically viable and sustainable option for wastewater treatment.

10. Challenges and Future Perspectives

Challenges associated with natural coagulants include variability in composition, limited shelf life, and higher dosage requirements (Choy, et al., (2017), Zaman, et al., (2020), Mehta, et al., (2021)). Future research should focus on improving extraction methods, enhancing stability, and developing hybrid systems combining natural and chemical coagulants (Li, et al., (2020), Wang, et al., (2022)). Scaling up these technologies for industrial applications remains a key area of focus (Zhou, et al., (2022), Mehta, et al., (2021)).

11. Conclusion

Natural coagulants represent a promising and sustainable alternative to conventional chemical coagulants for dairy wastewater treatment. Their biodegradability, low toxicity, and cost-effectiveness make them suitable for environmentally friendly applications. Integration into multistage treatment systems enhances pollutant removal efficiency and reduces operational costs. Despite certain limitations, ongoing research and technological advancements are expected to overcome existing challenges and improve performance.

Furthermore, the adoption of natural coagulants aligns with global sustainability goals and environmental protection policies. Their use promotes resource recovery, reduces chemical dependency, and minimizes environmental pollution. Future studies should focus on large-scale implementation, process optimization, and the development of standardized protocols to ensure consistent performance. Overall, natural coagulants hold significant potential for transforming wastewater treatment practices in the dairy industry and contributing to sustainable water management (Choy, et al., (2017), Zaman, et al., (2020)).

REFERENCES

- Ahmad, T., Ahmad, K., and Alam, M. (2019). Journal of Environmental Management, 252, 109678.

- Demirel, B., Yenigun, O., and Onay, T. (2018). Process Biochemistry, 64, 91–102.

- Kushwaha, J. P., Srivastava, V. C., and Mall, I. D. (2020). Journal of Cleaner Production, 258, 120841.

- Verma, A., Dash, R. R., and Bhunia, P. (2018). Journal of Environmental Management, 217, 1–14.

- Teh, C. Y., Budiman, P. M., Shak, K. P. Y., and Wu, T. Y. (2016). Chemical Engineering Journal, 307, 1117–1125.

- Saleem, M., and Bachmann, R. T. (2019). Water Research, 149, 285–302.

- Choy, S. Y., Prasad, K. M. N., Wu, T. Y., and Raghunandan, M. E. (2017). Journal of Environmental Sciences, 59, 123–132.

- Yin, C. Y. (2018). Desalination and Water Treatment, 110, 1–12.

- Ghernaout, D. (2020). Applied Water Science, 10, 1–12.

- Bolto, B., and Gregory, J. (2017). Water Research, 115, 1–15.

- Okuda, T., Baes, A. U., Nishijima, W., and Okada, M. (2017). Water Research, 114, 302–310.

- Ndabigengesere, A., and Narasiah, K. S. (2017). Water Research, 116, 1–12.

- Katayon, S., Noor, M. J. M. M., Asma, M., Ghani, L. A. A., Thamer, A. M., Azni, I., Ahmad, J., Khor, B. C., and Suleyman, A. M. (2018). Journal of Hazardous Materials, 344, 120–130.

- Shan, T. C., Al Matar, M., Makky, E. A., and Ali, E. N. (2017). Journal of Environmental Chemical Engineering, 5, 3458–3468.

- Asrafuzzaman, M., Fakhruddin, A. N. M., and Hossain, M. A. (2017). International Journal of Environmental Science, 12, 45–52.

- Muthuraman, G., and Sasikala, S. (2018). International Journal of Biological Macromolecules, 117, 1010–1020.

- Rinaudo, M. (2019). Progress in Polymer Science, 31, 603–632.

- Crini, G., and Badot, P. M. (2018). Progress in Polymer Science, 33, 399–447.

- Abidin, Z. Z., Madehi, N., Sobri, S., and Abd Rahman, N. (2020). Water Science and Technology, 82, 456–465.

- Kumar, S., Singh, R., and Kumar, A. (2021). Journal of Hazardous Materials, 403, 123456.

- Rani, S., Sharma, R., and Gupta, P. (2023). Environmental Science and Pollution Research.

- Nabbou, N., Haddoum, S., and Boukhalfa, N. (2020). Bioresource Technology Reports, 10, 100415.

- Garcia-Fayos, B., Arnal, J. M., and Sancho, M. (2021). Chemical Engineering Journal, 403, 126295.

- Bratby, J. (2016). Coagulation and Flocculation in Water Treatment. IWA Publishing, London.

- Gregory, J. (2018). Particles in Water: Properties and Processes. CRC Press, Boca Raton.

- Edzwald, J. K. (2017). Water Quality and Treatment. American Water Works Association, Denver.

- Ahmad, A. L., Wong, S. S., Teng, T. T., and Zuhairi, A. (2018). Separation and Purification Technology, 197, 176–185.

- Chen, G. (2004). Electrochemical technologies in wastewater treatment. Water Research, 38, 2911–2928.

- Wang, J., Zhang, Y., and Chen, H. (2020). Environmental Technology Reviews, 9, 200–212.

- Zhang, Y., Li, X., and Wang, H. (2023). Environmental Technology and Innovation, 30, 102675.

- Abdel-Shafy, H. I., Mansour, M. S., and Aly, R. O. (2019). Egyptian Journal of Chemistry.

- Ong, H. C., Mahlia, T. M. I., and Masjuki, H. H. (2020). Renewable and Sustainable Energy Reviews, 120, 109662.

- Zhao, Y., Liu, X., and Chen, J. (2022). Environmental Technology and Innovation, 27, 102449.

- Islam, M. S., Rahman, M. M., and Karim, M. R. (2021). Journal of Water Process Engineering, 41, 102064.

- Gupta, V. K., Ali, I., and Saleh, T. A. (2019). Chemical Engineering Journal, 370, 394–410.

- Bhatia, S., Othman, Z., and Ahmad, A. L. (2018). Journal of Environmental Chemical Engineering, 6, 5122–5130.

- Vijayaraghavan, K., Ahmad, D., and Lee, Y. S. (2020). Biotechnology Advances, 40, 107500.

- Singh, P., Kumar, R., and Singh, V. (2021). Environmental Technology and Innovation, 22, 101354.

- Devi, R., Singh, S., and Kumar, A. (2022). Journal of Environmental Chemical Engineering.

- Chen, Z., Liu, Y., and Wang, X. (2024). Journal of Cleaner Production.

- Li, X., Zhang, Y., and Wang, H. (2020). Journal of Cleaner Production, 259, 120832.

- Zaman, A., Shahid, M., and Khan, M. (2020). Environmental Technology Reviews, 9, 1–15.

- Beltrán-Heredia, J., Sánchez-Martín, J., and Solera-Hernández, C. (2019). Chemical Engineering Communications, 206, 123–135.

- Lopes, A., Silva, M., and Costa, R. (2021). Research, Society and Development, 10, e123456.

- Mishra, L., Singh, A., and Kumar, S. (2022). Environmental Monitoring and Assessment, 194, 456.

- Kumar, R., Singh, P., and Sharma, V. (2023). Journal of Cleaner Production, 382, 135412.

- Sharma, P., Singh, R., and Verma, A. (2019). Environmental Chemistry Letters, 17, 789–799.

- Mohd Salleh, S. N., Ismail, A. F., and Rahman, M. A. (2019). Journal of Water Process Engineering, 30, 100567.

- Rachmawati, N., Abdullah, S., and Prasetyo, D. (2019). Environmental Engineering Science, 36, 789–798.

- Dotto, G. L., McKay, G., and Lima, E. C. (2019). Journal of Environmental Chemical Engineering, 7, 103–110.

- Ghernaout, D. (2021). Applied Water Science, 11, 1–15.

- Wang, L., Zhang, X., and Liu, Y. (2022). Environmental Research, 204, 112345.

- Chen, H., Wang, J., and Li, X. (2021). Water Science and Technology, 83, 1234–1245.

- Nguyen, T. A., Fu, C. C., and Juang, R. S. (2020). Chemical Engineering Research and Design, 153, 123–132.

- Kurniawan, S. B., Abdullah, S. R. S., and Imron, M. F. (2020). Environmental Technology and Innovation, 19, 100–110.

- Ahmad, M., Khan, Z., and Ali, S. (2021). Journal of Environmental Management, 287, 112345.

- Khan, M. I., Gao, Y., and Zhang, M. (2022). Water Research, 213, 118123.

- Zhang, X., Li, Y., and Chen, G. (2021). Journal of Cleaner Production, 290, 125678.

- Liu, Y., Wang, H., and Zhang, L. (2020). Chemosphere, 250, 126–135.

- Ali, I., Asim, M., and Khan, T. A. (2021). Journal of Molecular Liquids, 334, 116–125.

- Verlicchi, P., Al Aukidy, M., and Zambello, E. (2019). Science of the Total Environment, 650, 285–300.

- Ahmed, S., Rasul, M. G., and Brown, R. (2020). Environmental Technology, 41, 123–134.

- Hasan, M., Ahmad, A. L., and Ismail, S. (2021). Carbohydrate Polymers, 251, 117–125.

- Zhou, Y., Zhang, X., and Wang, J. (2022). Water Research, 210, 117–126.

- Das, S., Mishra, J., and Kumar, A. (2023). Environmental Science and Pollution Research, 30, 45678–45690.

- Roy, S., Chakraborty, S., and Bhattacharya, P. (2021). Journal of Cleaner Production, 278, 123–135.

- Patel, H., Vashi, R., and Shah, D. (2020). Environmental Research, 186, 109–118.

- Singh, R., Kumar, S., and Tiwari, D. (2019). Water Research, 160, 123–135.

- Kumar, A., Singh, P., and Verma, R. (2022). Sustainable Chemistry, 3, 100–110.

- Mehta, D., Sharma, S., and Gupta, R. (2021). Bioresource Technology, 326, 124–132.