Traditional parboiling process improvement  through combined hydrothermal treatments in SriLanka.

# ABSTRACT

Investigation was fabricated to produce quality parboiled rice of two improved Sri Lankan rice varieties Ambalanthota rice (At 362), Bathalagoda rice (Bg 366) and one traditional rice variety “Adakari” at laboratory scale Paddy parboiling process plant  . Investigate the   different parboiling treatments combinations such as duration of soaking, and three steaming methods as Pressurized-Soaking Steam (PSS), Un-soaked Steam (USS) and Soak Steam (SS) were investigated on their quality standards in terms of milling and cooking quality standards, which refer as Head Rice Yield (HR) during milling and resistance starch formation (RS) during cooking and keeping Raw paddy rice as control. Among all combined parameters investigated in the milling yield analysis the process through improved streaming method of Pressure-soaked steam (PSS) was recorded as highest head rice yield as (90 ± 5%) which was achieved as higher milling yield than the other traditional steaming methods investigated, which achieved for 2 days of soaking duration (48 hrs.) and 30 minutes of steaming. All other compared steaming methods (USS and SS) achieved moderate head rice yield as 70 ± 5%. Rice cooked with excess water with duration of 30 minutes in electric rice cooker.  Texture analysis was performed by as texture analyzer after cooking under this process, variety (Adakari) performed better than other paddy variety in the formation of resistant starch under the pressurized soaked steam (PSS) process, in this texture investigation, which was yield a higher texture analyser outcome (10-12) N than other methods investigated. Pressure-soaked steam cooked samples show higher texture analyser outcome (N) therefore pressure-soaked steam parboiling method was performed better in terms of quality parboiled rice than other investigated treatments. Non-parboiled sample (control) was yield  a lower texture analyser outcome (6-7) N than parboiled rice samples tested. Statistical treatments column that we have a statistically significant interaction at the p < .0001 level, there was no statistically significant difference in between variety (p = 0.759), but there were statistically significant differences between steaming methods (p < .005).

Keywords:  Paddy soaking, steaming & drying, dehusking, milling and  head rice & texture analysis.

## 1.Background

Rice (Oryza sativa L.) is one of the world's most important food crops, second only to wheat in terms of annual production for human consumption. It is the primary source of nutrition for more than half of the world's population (Dutta, 2014).. The parboiling method accounts for a significant portion of the global rice processing industry. The requisite hardness is imparted to the grain when paddy is parboiled properly. Before milling, paddy undergoes a hydrothermal treatment known as parboiling. Different parboiling techniques have traditionally been followed and scientifically developed for the preparation of parboiled rice (Fofana et al., 2011).  Parboiling results in higher milling recovery, more transparent kernels, and increased swelling, when cooked to the perfect softness ((Islam et al., 2001).Soaking paddy in cold water for 24–72 hours, until the kernels are saturated, is the most common way to complete the three stages of parboiling – soaking, steaming, and drying. To obtain 80 percent gelatinized starch, the soaked paddy is boiled for 1 hour at 100℃. Finally, during sun-drying the boiled paddy is dried to a moisture content of 14 percent (Miah et al., 2002a). The amylose, resistant starch (RS) and their relative changes with total starch (TS) in different varieties of rice from Northern Sri Lanka with the different traditional cooking methods were determined (Vasanthy, 2020).

## Objective of the study

In this study, to investigate the quality improvements on head rice yield and resistant starch formation  by the  treatments of soaking duration and steaming method of selected traditional and improved paddy varieties.

# MATERIALS AND METHOD

### 2.1 Location

Improved Paddy samples were collected from Paranthan seed farm (Government) and traditional paddy variety  in the region of Kilinochchi farm (Private) commonly cultivated varieties in Kilinochchi district.. Research work was carried at the Department of Agricultural Engineering, Faculty of Agriculture, University of Jaffna, SriLanka. Milling and further analysis of texture by extrusion after cooking, which were carried out at the laboratory of Pot-Harvest Technology division for Both raw (unparboiled) and parboiled rice.

2.2 Materials

Two improved Srilankan rice varieties Ambalanthota red rice (At 362), Bathalagoda white rice (Bg 366) and one traditional rice variety “Addakari” with red pericarp were used in this study. The purity percentage of the three varieties was 99% and the samples have been harvested one month before analysis. Parboiling process was initiated with the raw paddy samples were soaked in water at room temperature (30℃) with the intervals of 1-3 days and then steaming was practiced in three different ways, Pressurized Soak Steam (PSS), Un-soaked Steam (USS) and Soak Steam (SS). which practices under the improved and traditional way of steaming in an electric steam cooker (Panasonic SR-MP 300K). The steamed paddy was dried in an electric conventional oven (Memmert UN-110) at 60˚C for 3 hrs for the safe milling analysis (Behera, G. et, al. 2018). The hardness analysis was performed by an extrusion test to test the internal strength of the rice kernel after cooking. Parboiled rice was cooked with excess water with duration of 30 minutes in electric rice cooker (SR-UH 36 P-W). Hardness analysis was done at 30˚C, with a 10mm screw  displacement and the speed of 30 rpm (Meresa, A. et al. 2020)

2.3  Physical properties of paddy variety tested

The measurements were taken  in triplicate. Length (L), thickness (T) and width (W) of the milled rice kernels were measured using a micrometer. Size and shape was classified according to the method described by Cruz and Kush (2000).

| Variety Characteristics | Attakari | BG 366 | At 362 |
| --- | --- | --- | --- |
| 1000 grain weight (g) | 25.4 ± 0.3 | 24.8 ± 0.2 | 25.7 ± 0.5 |
| Grain shape | Long Cylinder | Intermediate Bold | Long Medium |
| Raw milling recovery (%) | 75.6 ± 0.4 | 73.4 ± 0.6 | 68.2 ± 0.3 |
| Raw head rice yield (%) | 70.2 ± 0.5 | 67.4 ± 0.3 | 55.6 ± 0.4 |

Table 1 Physical properties of selected paddy variety tested

2.4 Parboiling treatments

2.4.1 Experimental setup

Parboiling was done in three important steps. Such as soaking (steeping), steaming, and drying. Three different steaming methods were implemented in this study. Paddy varieties of “At 362”, “Bg 366”, and “Adakari” were soaked in water at ambient temperature (30℃) for up to 72 hours (1-3 days). Paddy grains were then steamed in electric steamer (Panasonic SR-MP 300K) for 30-40 min until husk split off (5.0±0.5%).

2.4.2 Soaked steam parboiling (SS)

A 500g of paddy sample was put into the steamer, and water was added 1.5 times higher than the paddy volume and started heating up to 30-40 min. After steaming, paddy was kept for tempering, then oven-dried for 3 hours at 60℃ (Wimberly, 1983).

2.4.3 Pressure-soaked steam parboiling (PSS)

A 500g of paddy sample was put into the pressure steam cooker, and water was added 1.5 times higher than the paddy volume. Paddy grains were steamed under pressure at 1.5 psi (121℃) for 30-40 min until their hulls split by using the pressure steamer. After steaming, paddy was kept for tempering, then oven-dried for 3 hours at 60℃ (Otegbayo, Osamuel, et al., 2004).

2.4.4 Un-soaked steam parboiling (USS)

A 500g of paddy sample was placed on wire mesh of steamer, and water was added 1.5 times of paddy volume (v/v) into cooker, then wire mesh in steamer was kept above the water. After steaming for 30-40 min, paddy was kept for tempering, and then oven-dried for 3 hours at 60℃ (Juliano, 1993).

## 2.5 Analysis of milling yield and head rice percentage

### 2.5.1 Milling yield determination

All the parboiled samples 500 grams were de-husked by using a Satake paddy husker at 100 r.p.m.(Satake Engineering Co.Ltd). Then all the de-husked samples were winnowed by electric conventional fan (1500W) to remove the husk and foreign materials. Then milled paddy was measured and separate the head rice.  It was done by using a head rice separator (Satake Engineering Co.Ltd).

Degree of milling (%) = weight of  milled riceweight of raw paddy %

#### 2.5.2 Head rice percentage

Kernels that should be at least three-fourths of the original kernel length were considered as head rice (USDA, 2005). After separation head rice was weighed by using a weighing balance of analytical grade (Satake Engineering Co.Ltd) .Milling yield determination was carried out by using the IRRI formula.  (Fofana et al., 2011).

Head rice (%) =weight of head riceweight of milled rice %

## 2.6 Analysis of extrusion texture quality

Rice Grain Hardness

The hardness of grain after parboiling without re-cooking was determined as the rice grain hardness. This analysis was performed by using Kiya Grain Hardness Tester Instrument. Ten grains of black rice sample were tested. The needle was set displacement to 10 mm. The grain was pressed by using tester spindle until it back. The needle would show the hardness value in N. This analysis was performed by using Kiya Grain Hardness Tester Instrument. . The needle was set displacement to 10 mm. The cooked grain was pressed by using tester spindle until it back. The needle would show the hardness value in Newton (N).The running temperature of the Extruder was 30˚C and the speed was 530 rpm.  Rice kernels and  water were placed in an automated electric rice cooker (Satake Engineering Co.Ltd) with different rice-to-water weight ratio.

# RESULTS AND DISCUSSION

Steaming time decreases, when soaking duration goes up in all three paddy varieties in all steaming methods in parboiling process. Figure 1, shows the relationship between steaming time (min) and the soaking duration (days) of three different methods of steaming in parboiling process. Considerably Pressure-Soaked Steam Parboiling takes lesser time of steaming compared to other investigated steaming methods. Figure 2, shows the changes of  moisture percentage with drying duration. Moisture percentage was decreasing with drying duration increased. Paddy kernels it takes 3 hours of time when it was under oven drying at 60˚C ttemperature  for moisture percentage of kernels should be less than 12% for safe milling.

Fig 1;  Changes steaming  with soaking time

SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling, OSP - Open steam Parboiling and Without steaming as control.

Fig 1;  Changes steaming  with soaking time

SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling, OSP - Open steam Parboiling and Without steaming as control.

Fig:1 Steaming duration (mints) with Soaking duration in Days of  Parboiling

SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling, OSP - Open steam Parboiling and Without steaming as control.

Fig:1 Steaming duration (mints) with Soaking duration in Days of  Parboiling

SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling, OSP - Open steam Parboiling and Without steaming as control.

Fig:2 Drying duration with moisture content (Wb) in hrs of  Raw Paddy variety in parboiling.

Fig:2 Drying duration with moisture content (Wb) in hrs of  Raw Paddy variety in parboiling.

3.1 Milled head rice yield

Milled head rice percentage increases with soaking duration increases in all three varieties in all steaming treatments  investigated in parboiling process. According to the figures 3, 4 & 5 shows increment of  soaking duration in parboiling process  yield a higher milled head rice percentage than control (Raw Paddy). Above figures reveled that the relationship between milled head rice yield percentage and the soaking duration in days of three different paddy varieties in three different steaming methods.  How-ever rate of such increase in pressure-soaked steam parboiling is higher (90%) than other investigated steaming methods. All other compared steaming methods (USS & SS) achieved moderate head rice yield as (70±5)%. Non-parboiled samples (Raw rice) showed  lower milling recovery  yield (65 ±5%) than parboiled rice samples investigated.

Fig: 3 Effect of parboiling treatment on yield of  Head rice % of variety At 362

SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling,

OSP - Open steam Parboiling and Without steaming as control.

Fig: 3 Effect of parboiling treatment on yield of  Head rice % of variety At 362

SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling,

OSP - Open steam Parboiling and Without steaming as control.

Fig:4 Effect of parboiling treatment on yield of  Head rice % of variety Bg 366.

SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling,

OSP - Open steam Parboiling and Without steaming as control.

Fig:4 Effect of parboiling treatment on yield of  Head rice % of variety Bg 366.

SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling,

OSP - Open steam Parboiling and Without steaming as control.

Fig: 5 Effect of parboiling treatment on yield of  Head rice % of variety Adakari.

SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling,

OSP - Open steam Parboiling and Without steaming as control.

Fig: 5 Effect of parboiling treatment on yield of  Head rice % of variety Adakari.

SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling,

OSP - Open steam Parboiling and Without steaming as control.

3.2 Hardness expressed in terms of texture analyser (N)

The rice kernal had high resistant starch content after cooking texture analyser outcome (Nutan-N) outcome is high. Figure 6, 7 & 8 shows the relationship between texture analyser outcome (N) and the soaking time of three different paddy varieties in different steaming methods. Hardness of all three varieties increases  with soaking duration in all steaming methods. Control (Non-steamed) sample shows a lower texture analyser outcome (N) than parboiled cooked rice samples and pressure-soaked steam (PSS) cooked rice sample shows a higher texture analyser outcome (10-12N) than other methods investigated. Pressure-soaked steam cooked samples show higher texture analyser outcome (N) therefore pressure-soaked steam parboiling method was performed better in terms of cooking quality as whole parboiled rice than other investigated treatments. Non-parboiled sample (control) was yield  a lower texture analyser outcome (6-7N) than parboiled rice samples tested.

Fig: 6  Effect of parboiling treatment on yield  of extrusion % of variety At 362 .

SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling

OSP - Open steam Parboiling and Without steaming as control.

: 6 Extrusion % with soaking time (days)

Fig: 6  Effect of parboiling treatment on yield  of extrusion % of variety At 362 .

SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling

OSP - Open steam Parboiling and Without steaming as control.

: 6 Extrusion % with soaking time (days)

Fig: 7  Effect of parboiling treatment on yield of extrusion % of variety Adakari.

SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling

OSP - Open steam Parboiling and Without steaming as control.

Fig: 7  Effect of parboiling treatment on yield of extrusion % of variety Adakari.

SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling

OSP - Open steam Parboiling and Without steaming as control.

Fig: 8 Effect of parboiling treatment on yield of  extrusion % of variety Bg 366 SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling, OSP - Open steam Parboiling and Without steaming as control.

Fig: 8 Effect of parboiling treatment on yield of  extrusion % of variety Bg 366 SSP - Soaked Steam Parboiling, PSSP - Pressure Soaked Steam Parboiling, OSP - Open steam Parboiling and Without steaming as control.

3.3 Statistical Analysis  ANOVA -Two Factor with replication

The particular rows we are interested in  the " soaking days "," Steaming methods" and " Interaction “and these are highlighted above table 3.3. These rows inform us whether our independent variables and their interaction have a statistically significant effect on the dependent variable, you can see from the "Sig." column that we have a statistically significant interaction at the p < .0001 level. We can see from the table above that there was no statistically significant difference in between variety (p = 0.759), but there were statistically significant differences between steaming methods (p < .005).

| Table 3.3 Anova: Two-Factor With Replication |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SUMMARY Total | soaking days | PSS(high) | SSP (medium) | OSP (low) | CONTROL | Total |
| Count | 9 | 9 | 9 | 9 | 9 |  |
| Sum | 18 | 672 | 647 | 636 | 580 |  |
| Average | 2 | 74.67 | 71.88 | 70.66 | 64.44 |  |
| Variance | 0.75 | 7 | 1.61 | 8 | 2.78 |  |
| ANOVA |  |  |  |  |  |  |
| Source of Variation | SS | df | MS | F | P-value | F crit |
| Sample (Soaking) | 43.2 | 2 | 21.6 | 6.39474 | 0.00486 | 3.31583 |
| Columns (Steaming) | 34205.68 | 4 | 8551.422 | 2531.67 | 1.8E-37 | 2.68963 |
| Interaction | 16.578 | 8 | 2.072 | 0.61349 | 0.75942 | 2.26616 |
| Within | 101.333 | 30 | 3.38 |  |  |  |
| Total | 34366.8 | 44 |  |  |  |  |

CONCLUSIONS

Different parboiling treatments practice were investigated as soaking duration and steaming methods in three varieties of red pericarp paddy, were compared on the quality impact on milling yield and resistance starch formation after cooking. In  parboiling treatments combinations such as duration of soaking (1 – 3 hours) , and three steaming methods as Pressurized-Soaking Steam (PSS), Un-soaked Steam (USS) and Soak Steam (SS).  All Investigated parameters Pressure-soaked steam (PSSP) was recorded as higher head rice yield as (90 ± 5%) with 2 days of soaking duration (48 hrs.) in all three steam treatments, which was achieved as higher than the other steaming method investigated. Other steaming methods (USS & SS) yield moderate head rice yield as (70±5)%. Texture analysis was performed by as texture analyzer after cooking under this process, variety (Adakari) performed better than other paddy variety in the formation of resistant starch under the Pressurized soaked steam (PSS) process, in this texture investigation, which was yield a higher texture analyser outcome (10-12) N than other methods investigated. Pressure-soaked steam cooked samples show higher texture analyser outcome (N) therefore pressure-soaked steam parboiling method was performed better in terms of quality parboiled rice than other investigated treatments. Non-parboiled sample (control) was yield  a lower texture analyser outcome (6-7) N than parboiled rice samples tested. Statistical treatments column that we have a statistically significant interaction at the p < .0001 level, there was no statistically significant difference in between variety (p = 0.759), but there were statistically significant differences between steaming methods (p < .005).

# REFERENCES

[1] Alexandre, N. ‘Effect of Parboiling Technique on the Nutritional Quality of Rice’, Global Journal of Nutrition & Food Science, 2(5) 2020, pp. 1–13. doi: 10.33552/gjnfs.2020.02.000548.

[2]  Behera, G. and Sutar, P. P.‘A comprehensive review of mathematical modeling of paddy parboiling and drying: Effects of modern techniques on process kinetics and rice quality’, Trends in Food Science and Technology,  2018, 75, pp. 206–230. doi: 10.1016/j.tifs.2018.03.015.

[3] Bello,M., Abubakar and Lawan, I.  ‘Rice Parboiling Practices and Technologies in Kano’, Journal of Engineering and Technology, 10(2), 2015 pp. 52–59.

[4] Cruz, N.D. and Khush, G.S. Rice Grain Quality Evaluation Procedures. In: Singh, R.K., Singh, U.S. and Khush, G.S., Eds., Aromatic Rices, Oxford and IBH Publishing Co Pvt. Ltd, New Delhi, 2000, 15-28.

[5] Darandakumbura, H. D. K. et al. ‘Effect of Processing Conditions and Cooking Methods on Resistant Starch, Dietary Fiber and Glycemic Index of Rice’, Tropical Agricultural Research, 24(2),  2013 pp. 163–174.

[6] Fofana, M. et al. ‘Effect of improved parboiling methods on the physical and cooked grain characteristics of rice varieties in Benin’, International Food Research Journal, 18(2), 2011, pp. 715–721.

[7] Islam, R., Shimizu, N. and Kimura, T. ‘Quality Evaluation of Parboiled Rice with Physical Properties’, Food Science and Technology Research, 7(1),  2001,  pp. 57–63. doi: 10.3136/fstr.7.57.

[8] Issah, M. H., Appiah, F. and Dery Ninfaa, A.  ‘Effect of Parboiling on the Quality of Processed Degan Rice (Oryza Spp)’, International Journal of Sciences: Basic and Applied Research (IJSBAR) International Journal of Sciences: Basic and Applied Research, 24(3) 2015, pp. 153–163.

[9] Juliano, B. O.‘A Simplified Assay for Milled-Rice Amylose’, Cereal Science Today, 16(October 1971, pp. 334–360.

[10] Meresa, A. et al. ‘Effect of Parboiling Conditions on Physical and Cooking Quality of Selected Rice Varieties’, International Journal of Food Science, 2020. doi: 10.1155/2020/8810553.

{11}Kannapot Kaewsorn et al. Evaluation of Precision and Sensitivity of Back Extrusion Test for Measuring Textural Qualities of Cooked Germinated Brown Rice in Production Process. Foods 2023, 12(16), 3090; https://doi.org/10.3390/foods12163090.

[12] Vasanthy, A.; Printhajini, P.A. study on changes in amylose and resistant starch contents of selected rice varieties by traditional cooking methods. Tropical Agriculture Research and Extension. 2022, 25(1) 46-58.