Fuzzy-controlled Power-aware Proactive-acknowledgement Based Broadcasting (FP2B) For Mobile Ad Hoc Networks

Anuradha Banerjee

Dept. of Computer Applications

Kalyani Govt. Engg. College

West Bengal, India

anuradha79bn@gmail.com

Abstract—Network-wide broadcasting is a fundamental operation for mobile ad hoc networks. In broadcasting, a source node sends a message to all other nodes in the network. Under ordinary flooding procedure, each node transmits the broadcast message to all of its 1-hop downlink neighbors, i.e. all nodes residing within its radio-range. Receiving the broadcast message all those downlink neighbors reply with an acknowledgement. Since in an ad hoc network a node may have multiple uplink neighbors, in ordinary flooding procedure, a node is supposed to receive the broadcast message from all those uplink neighbors and send acknowledgement to all of them, generating huge message contention and collision. This is popularly referred to as the broadcast storm problem. The present article is focused to remove the broadcast redundancy within 2-hop neighborhood and beyond, as much as possible by prioritizing the 1-hop downlink neighbors of a node. Priority of a 1-hop downlink neighbor of a node ni increases if it is equipped with a large number of 1-hop downlink neighbors, large radio-range, high remaining battery power and very small number of uplink neighbors closer to the broadcast source than ni. ni waits a predefined amount of time to receive proactive acknowledgements from the 1-hop downlink neighbors having less priority. If it does not receive acknowledgement from those downlink neighbors within the waiting time, it sends the broadcast message to them. A fuzzy controller named Priority Assignor (PA)   is embedded in every node that determines the priority of a 1-hop downlink neighbor. Simulation results firmly establish that the proposed protocol FP2B produces high broadcast delivery ratio at much lesser message cost, compared to other state-of-the-art broadcast algorithms.

Keywords-Ad hoc network, broadcast redundancy, flooding, priority assignor, proactive acknowledgement.

# Introduction

A mobile ad hoc network is a wireless network that is self-organized with many mobile nodes. No static infrastructure such as a wired backbone is available. All nodes are free to move around and the network topology may change frequently. Due to limited transmission range of wireless network interface, nodes are required to forward messages for those located outside the radio coverage, thereby forming a multi-hop network. Possible applications include emergency rescue in disaster situations, communication between mobile robots, exchanging information in the battlefield etc. [1-5]. Broadcast is useful in delivering messages to users with unknown location or group of users whom the source need not exactly know. Broadcast plays an important role in routing, network management etc. Many on-demand or reactive routing protocols (dynamic source routing (DSR) [2], ad hoc on-demand distance vector routing (AODV) [3], on-demand multicast routing protocol (ODMRP) [4] etc.) rely on broadcast to discover a route between two nodes or to update group status and multicast routes. Broadcast is also a viable candidate for multicast in ad hoc networks with rapid changing topology.

A density based innovative flooding (DBF) algorithm is proposed in [6]. In this algorithm, each node forwards a message based on its neighbor density and neighbor density of its previous node from which the broadcasted message. In a cluster of loosely couples nodes with few intermediate nodes as neighbors, the probability of forwarding the broadcasted message will be high. On the other hand, if a node is having high density of neighbors, then there will be lots of chances of packet collision at that point. Density based flooding tries to avoid that situation by assigning low priority at that point. The article in [7] proposes a tree based broadcast (TBB) method that maintains a spanning tree in the network. The algorithm is fully distributed, decentralized and resource-efficient. Broadcast operation is performed using a tree by forwarding the message not to all

neighbors, but only those neighbors in the tree structure. Since the tree is acyclic, each message is received only once by each node, giving two advantages over the existing methods. Firstly it is needless to store the previous broadcasts in order to avoid endless multiplications of broadcast messages along a cycle of links. Only the originator of a broadcast message needs to store it and pay attention to whether its broadcast was successful or not if it is of great importance. Secondly, it is very economical considering how many times a broadcast message should be forwarded.

A reliable broadcast (RB) method is proposed in [8], which combines area based and neighbor-based technique of broadcast. Each node gains knowledge of neighbors and maintains neighbor list. The algorithm calculates the relative position of the nodes with respect to broadcast source node. The nodes that are farthest from the source rebroadcasts next. The algorithm tries to minimize the number of rebroadcasts by intermediate nodes and thus reduces message cost. Reference [9] proposes a method for reduction of broadcast traffic (RBT) in mobile ad hoc networks. It focuses on the fact that communication links in ad hoc networks break frequently due to node mobility. As the nodes move, a node receiving a packet on the boundary of communication range of a transmitter node is allowed to drop the packet, as the receiver may soon move out of the radio range of the transmitter. To approximate the distance between receiver and transmitter, receiver signal strength information is used.

The proposed algorithm FP2B eliminates much more redundancy than these algorithms. It considers the then topological situation of the network up to 2-hop neighborhoods of a node (in terms of the number and positions of uplink and downlink neighbors’ w.r.t. the geographical locations of the broadcast source) , the radio-ranges and residual energy of involved nodes (this is very important from the point of view of ad hoc networks because there is no point to send the broadcast messages to the nodes which are about to be inoperable). All these greatly improve the network throughput by reducing the message cost through tackling the broadcast storm problem.

# Overview of FP2B

According to the study of discharge curve of the batteries heavily used in ad hoc networks, at least 40% of total charge is required to remain in operable condition, the range from 40% to 60% is satisfactory, 60% to 80% is good whereas the next higher range is considered to be more than sufficient [13]. A node with residual energy 40%-60% will be termed as operational whereas a node with residual energy more than 60% will be called power-efficient in rest of the article. In FP2B, a node nj drops a message immediately after receiving it from an uplink neighbor ni at time t, provided its residual energy at that time is less than 40%. Otherwise nj tries to find out a pair of 1-hop uplink neighbor and 1-hop downlink neighbors (ni, nk) of nj at time t s.t. njDi(t), nkDi(t) and nkDj(t) where Di(t) denotes the set of 1-hop downlink neighbors of ni at time t. In this situation, nj won’t send the broadcast message to nk because nk has automatically received that after ni has sent the message. If all the 1-hop downlink neighbors of nj at time t, are also 1-hop downlink neighbors of some uplink neighbor nl of nj at that time s.t. nj has received the broadcast message from nl, then nj wont forward the broadcast message after receiving it. This eliminates the redundancy between 1-hop and 2-hop downlink neighborhood of a node. But if there exists at least one 1-hop downlink neighbor of nj that is not the 1-hop downlink neighbor of any uplink neighbor of nj, then nj will need to prioritize those 1-hop downlink neighbors. This is performed by a fuzzy controller named priority assignor (PA). Priority of a node nk which is a downlink neighbor of nj, depends upon its number of 1-hop downlink neighbors, radio-range, residual battery power, total number of power-efficient uplink neighbors (residual energy is more than 60%) and the number of power-efficient uplink neighbors that are closer to the broadcast source. nj waits a predefined amount of time to receive proactive acknowledgements from the 1-hop downlink neighbors having less priority. If it does not receive acknowledgement from those downlink neighbors within the waiting time, it sends the broadcast message to them. The waiting time is equal to the amount of time that is required by nj to forward (Mj-1) number of messages where Mj is the number of locations in message queue of nj.

# Input Parameters of PA of na

Here it is assumed that na is an uplink neighbor of ni at time t that is evaluating the priority of ni at that time. Below I describe the input parameters of the PA of na.

## Residual Energy Index

The residual energy index i(t) of ni at time t is defined as,

i(t) =1 – ei(t)/Ei                                                             (1)

Where ei(t) and Ei indicate the consumed battery power of ni at time t and the maximum battery capacity of the same node, respectively. It is clear from the formulation in (1) that i(t) ranges between 0 and 1. Values of it close to 1 increase the priority of ni as a downlink neighbor. It indicates that if ni receives the broadcast message, then it is well-equipped from the point of view of battery power to forward it to its 1-hop downlink neighbors.

## Radio-range Index

Assuming that Rmin and Rmax denote the minimum and maximum possible radio-ranges of the network,  radio-range index rr(i) of node ni is given by,

rr(i) = (Ri-Rmin+1) / (Rmax-Rmin+1)                                 (2)

where Ri is the radio-range of node ni. From (2), it is evident that rr(i) ranges between 0 and 1. As Ri approaches Rmax, rr(i) approaches 1. 1 is added to the denominator in (2) to avoid 0 value in the denominator when Rmax=Rmin. In order to maintain the fractional nature of radio-range index, 1 is added to the numerator also. High radio-range index of a node denotes that it has got high encapsulating capability. So, the number of its 1-hop downlink neighbors may increase any time.

## Downlink Cardinality Index

The downlink cardinality index i(t) of ni at time t is defined as,

i(t) =(1- 1/(|Di(t)|+1)) exp  (1/(|Di(t)|+1))                      (3)

From the formulation in (3), i(t) ranges between 0 and 1. Please note that 1 is added to | Di(t)| to avoid 0 value in the denominator when | Di(t)|=0. Values of i(t) close to 1 increase the priority of ni as a downlink neighbor. It indicates that if ni receives the broadcast message, then it may send that to a huge number of 1-hop downlink neighbors.

## Uplink Cardinality Index

The uplink cardinality index i(t) of ni at time t is defined as,

i(t) = 1-(f1i(t)  f2i(t))                                                  (4)

f1i(t) = (1-1/(|Ui(t)|+1))exp(1/(|Ui(t)|+1))

f1i(t) = (1-1/(|Ui(t)|+1))

Where Ui(t) is the set of power-efficient uplink neighbors of ni at time t and Ui(t) is a subset of Ui(t) containing those uplink neighbors of ni at time t which are closer to the broadcast source than na. 1 is added to both |Ui(t)| and |Ui(t)| to avoid 0 value in the denominator in (5) and (6) when |Ui(t)| and |Ui(t)| are 0. From the formulation in (4), i(t) ranges between 0 and 1. Values of it close to 0 decrease the priority of ni as a downlink neighbor of na. It indicates that for ni, na is not the only uplink neighbor from where it can receive broadcast message at time t, several other uplink neighbors are there. Priority of ni decreases even more if some of those uplink neighbors are closer to the broadcast source than na.

# Rule Bases of PA

The range divisions of the parameters of PA are shown in table 1.

TABLE 1: RANGE DIVISION OF PARAMETERS

| Name of parameter | Range division | Fuzzy premise variables |
| --- | --- | --- |
|  | 0-0.40 | A1 |
|  | 0.40-0.60 | A2 |
|  | 0.60-0.80 | A3 |
|  | 0.80-1.00 | A4 |
| rr, , | 0-0.25 | A1 |
|  | 0.25-0.5 | A2 |
|  | 0.5-0.75 | A3 |
|  | 0.75-1.00 | A4 |

Table 2 shows the fuzzy composition of  and rr producing a temporary variable t1.  is given more weight because high residual energy is indispensable for smooth operation of a node. Table 3 combines t1 and  generating another temporary output t2. The fuzzy combination of t2 and  produces the ultimate output X of PA. If X=A3 or X=A4, then that downlink neighbour directly receives the broadcast message. Otherwise, the uplink neighbours wait for a specific amount of time (this time is equal to the time of receiving acknowledgement from the farthest downlink neighbour) before sending the broadcast message.

TABLE 2: FUZZY COMBINATION OF  AND rr PRODUCING t1

| rr | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
| A1 | A1 | A1 | A2 | A2 |
| A2 | A1 | A1 | A2 | A3 |
| A3 | A1 | A2 | A3 | A4 |
| A4 | A1 | A2 | A3 | A4 |

TABLE 3: FUZZY COMBINATION OF t1 AND  PRODUCING t2

| t1 | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
| A1 | A1 | A2 | A3 | A3 |
| A2 | A1 | A2 | A3 | A4 |
| A3 | A1 | A2 | A3 | A4 |
| A4 | A1 | A2 | A3 | A4 |

TABLE 4: FUZZY COMBINATION OF t2 AND  PRODUCING X

| t2 | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
| A1 | A1 | A2 | A3 | A4 |
| A2 | A1 | A2 | A3 | A4 |
| A3 | A1 | A2 | A3 | A4 |
| A4 | A1 | A2 | A3 | A4 |

# Simulation Results

I evaluate the performance of FLB, using the network simulator ns-2 [10]. Except FP2B, I implement the protocols tree-based broadcasting (TBB), reduction of broadcast traffic (RBT) approach, and density-based flooding (DBF). The simulation parameters are shown in table 5. The simulation metrics are broadcast cost (total no. of messages transmitted by all nodes in the network / (total no. of messages generated by broadcast sources  total no. of nodes in the network)) and packet delivery ratio ((total no. of data packets actually delivered / total no. of data packets actually transmitted)  100). These are measured with respect to the total number of nodes in the network and total number of broadcast sources. The graphical results are illustrated in figures 1, 2, 3 and 4. When the measurements are performed with respect to the number of nodes, the number of broadcast cost is kept constant at 10. Similarly, when the measurement is performed with respect to the broadcast sources, the number of nodes is kept constant at 200.

TABLE 5:  SIMULATION PARAMETERS

| Name of parameter | Value |
| --- | --- |
| Network area | 500 m 500 m |
| Number of nodes | 50-300 |
| Transmission range | 10 m -50 m in first ten runs, 30 m – 100 m in second ten runs 10 m – 100 m in last ten runs |
| Packet size | 128 bytes |
| MAC layer | IEEE 802.11g |
| Traffic type | Constant bit rate |
| Mobility model | Random waypoint |
| Bandwidth | 1-4 mbps in first ten runs, 2-7 mbps in second ten runs and 3-10 mbps in last ten runs |
| Simulation time | 1000 seconds |

Figure 1: Broadcast cost vs number of nodes

Figure 2: Data packet delivery ratio vs number of nodes

Figure 3: Broadcast cost vs number of broadcast sources

Figure 4: Data packet delivery ratio vs number of broadcast sources

Since our proposed scheme FP2B is power-aware, load-balanced and much more rigorous in decreasing redundancy than all other competitor protocols, the broadcast cost is much lesser in it than others, both with respect to the number of nodes and number of broadcast sources. Less message cost generates less signal contention and collision increasing the data packet delivery ratio.

It is seen from figure 2 that for all the protocols, the data packet delivery ratio increases with increase in the number of nodes due to better network connectivity until the network gets overloaded or saturated with nodes. But since FP2B is more efficient than others in confronting the broadcast storm problem, it suffers least from the node overload as well as message overload due to a huge number of broadcast sources.

# Conclusion

This paper presents a new approach for efficient broadcasting in mobile ad hoc networks. The proposed protocol called FP2B is power and topology aware (up to 2 hops) and very efficiently reduces the cost of messages in the network by increasing the broadcast throughput. Design of the fuzzy controller Priority Assignor is based on real life observations and heuristics and it intelligently tackles the broadcast storm.

References:

[1] Anuradha Banerjee, Paramartha Dutta, “A Survey of Unicast Routing Protocols in Mobile Ad Hoc Networks”, International Journal of Advances in Science and Technology, vol. 2, no. 10, 2010

[2] D David B. Johnson. “Routing in Ad Hoc Networks of  Mobile Hosts”. Proceedings of the Workshop on Mobile Computing Systems and Applications, pp. 158–163, IEEE Computer Society, Santa Cruz, CA, December 1994

[3] Ian D. Chakeres et. Al, “AODV routing protocol implementation design”, In Proceedings of 2nd International Workshop on Wireless Ad Hoc Networking, Notification (WWAN), March 2005

[4] Lee Sung-Ju, Su W., M. Gerla, “On-demand Multicast Routing Protocol (ODMRP) for Ad Hoc Networks”, June 1999

[5] Ho. C, Obraczka K, Tsudil G., “Flooding for Reliable Multicast in Multi-hop Ad Hoc Networks”, In Proceedings of 3rd International Workshop on Discrete Algorithms and Methods for Mobile Computing and Communications (DIAL M’99), August 1999

[6] N. Karthikeyan, V. Palanisamy, K. Duraiswamy, “Optimum Density Based Model for probabilistic Flooding Protocol in Mobile Ad Hoc Network”, European Journal of Scientific Research, vol. 39 no.4, pp. – 577-588, 2010

[7] L. Tan, X. Zhan, J. Lie, F. Zhao, “A Novel Tree-based Broadcast Algorithm for Wireless Ad Hoc Networks”, International Journl of Wireless and Mobile Computing, vol. 1 no. 2, pp. 156 – 162, 2006

[8] S.V.M.G. Bavithiraja, R. Radhakrishnan, “A New Reliable Broadcasting Method in Mobile Ad Hoc Networks”, International Journal of Computer Science and Network Security, vol. 9, no. 4, pp. 340-349, 2009

[9] A. Shukla, “On the Reduction of Broadcast Traffic in Mobile Ad Hoc Networks”, In Proceedings of the International Conference on Wireless Communications, Networking and Mobile Computing, September 2007

[10] http://isi.edu/nsnam/ns

[11] Anuradha Banerjee, Paramartha Dutta, Fuzzy Controlled Adaptive Geocasting in Mobile Ad Hoc Networks, International Journal of Information Technology, vol. 14 no.2, 2008

[12] Anuradha Banerjee, Paramartha Dutta, Fuzzy Controlled Source Initiated Multicasting (FSIM) in Ad Hoc Networks, Lecture Notes in Computer Science, 2010, vol. 5966/2010, pp. 55-71, doi: 1007/978-3-642-11659-9_5

[13] Anuradha Banerjee, Paramartha Dutta, Binary Location Search-based Scalable Routing Protocol For Ad Hoc Networks, International Journal of Computer Networks and Communications (IJCNC) vol. 2 no. 5, September 2010

[14] Anuradha Banerjee, Paramartha Dutta, Reputation-based Attack Resistant Cooperation Stimulation (RACS) for Mobile Ad Hoc Networks, International Journal of Artificial Intelligence and Applications, vol. 1 no. 3, 2010

[15] Anuradha Banerjee, Paramartha Dutta, Fuzzy Controlled Route Discovery For Mobile Ad Hoc Networks, vol. 2 no. 6, International Journal of Engineering, Science and Technology, June 2010, pp. 2337-2346

[16] Anuradha Banerjee, Paramartha Dutta, Fuzzy Controlled Adaptive and Intelligent Route (FAIR) Selection in Mobile Ad Hoc Networks, European Journal of Scientific Research (impact factor: 0.736), vol. 45 no. 3, September 2010

[17] Anuradha Banerjee, Paramartha Dutta, Fuzzy Controlled Localized Route-Repair (FLRR) for On-demand Routing Protocols in Ad Hoc Networks, International Journal of Computer Applications, vol. 7 no. 6, pp. 1169-1444, September 2010

[18] Anuradha Banerjee, Paramartha Dutta, Link Stability and Node Energy Conscious Local Route-Repair Scheme for Mobile Ad Hoc Networks, American Journal of Applied Sciences (Impact factor: 0.223) , vol. 7(8), pp. 1129-1138, 2010

[19] Anuradha Banerjee, Paramartha Dutta, A Survey of Unicast Routing Protocols in Ad Hoc Networks, International Journal of Advances in Science and Technology, vol. 1 no. 3, 2010

[20] Anuradha Banerjee, A Survey of Broadcast Routing Protocols in Mobile Ad Hoc Networks, International Journal of Advances in Science and Technology, vol. 1 no.2, 2010

[21] Anuradha Banerjee, Paramartha Dutta, A Survey of Multicast Routing Protocols For Ad Hoc Networks, International Journal of Engineering, Science and Technology, vol. 2 no. 10, October 2010,pp.5594-5604

[22] Anuradha Banerjee, Paramartha Dutta, An Efficient Method For Tracking Nodes (EMTN) In Mobile Ad Hoc Networks, International Journal of Computer Applications, vol. 11 no. 2, December 2010,pp.38-41

[23] Anuradha Banerjee, Subhankar Ghosh, Paramartha Dutta, Fuzzy-controlled Energy Efficient Weight-based Two Hop Clustering For Multicast Communication in Mobile Ad Hoc Networks, Fuzzy and Neural Computing Conference, Vishakhapatnam, Dec. 19-21, 2011

[24] Anuradha Banerjee, Paramartha Dutta, Fuzzy-controlled Multi-hop Adaptive Clustering (FMAC) For Mobile Ad Hoc Networks, Scientific Research and Essays vol.7 no. 2(impact factor: 0.445), 2012

[25] Anuradha Banerjee, Paramartha Dutta, Fuzzy-controlled Load-balanced Broadcasting (FLB) In Clustered Mobile Ad Hoc Networks, International Journal of Computer Science Issues (IJCSI - impact factor: 0.242), vol. 9, no. 1, January 2012

[26] Paramartha Dutta, Anuradha Banerjee, Fuzzy –controlled Power-aware Multicast Routing (FPMR) For Mobile Ad Hoc Networks, Procedia Technology (Elsevier), vol. 4, pp-38-49, 2012

[27] Anuradha Banerjee, Fuzzy-controlled Intelligent Multicast Routing (FIMR) Protocol For Mobile Ad Hoc Networks, Advanced Science Letters (impact factor: 1.253) vol.19 no. 1, pp.42-47, 2013

[28] Anuradha Banerjee, Subhankar Ghosh, Paramartha Dutta, Experience Based Energy efficient Reactive Routing Protocol (EXERP) for Mobile Ad-hoc Networks, Arabian Journal of Science and Engineering (Springer), vol. 39 no. 2, 2013

[29] Anuradha Banerjee, Paramartha Dutta, Alternative Node Based Energy Depletion and Expected Residual Lifetime Balancing Method For Mobile Ad Hoc Networks, International Journal of Advanced Networking and Applications, vol. 5 no. 2, pp. 1886-1892, 2014

[30] Anuradha Banerjee, Paramartha Dutta, Delay-efficient, Energy and Velocity-conscious Non-preemptive Scheduler for Mobile Ad Hoc Networks, International Journal of Advanced Networking and Applications, vol. 5 no. 4, pp. 2002-2010, 2014

[31] Anuradha Banerjee, Cost Effective Route Discovery (CERD) For Mobile Ad Hoc Networks, International Journal of Advance Research in Science and Engineering, Vol. 04, Issue 01, march 2015

[32] Anuradha Banerjee, FESA: Fuzzy-controlled Energy-efficient Selective Allocation and Reallocation of Tasks Among mobile Robots, International Journal of Advance Research in Science and Engineering, Vol. 04, Issue 01, march 2015

[33] Anuradha Banerjee, Fuzzy-controlled Rebroadcasting Based on 2-hop Downlink Neighborhood Information (FR-2N) In Mobile Ad hoc Networks, International Journal of Applied Engineering Research (Scopus), vol. 10 no. 81, pp. 114-120 2015

[34] Anuradha Banerjee, Paramartha Dutta, Cost-Effective routing Protocols Based on 2-hop Neighborhood Information (2NI) in Mobile Ad Hoc Networks, International Journal of Applied Networking and Applications, vol. 7 issue 3, pp. 2771-2778, 2015

[35] Anuradha Banerjee, Paramartha Dutta Abu Sufian: “Fuzzy Controlled Scheduling on Real Time Data Packet (FSRP) in Mobile Ad-hoc Network”, International Journal of Computer Science and Mobile Computing, Vol. 5, Issue. 5, pg- 507-5013, May-2016

[36] A. Sufian, A. Banerjee, P. Dutta, “Survey of Various Real time and Non Real time Scheduling algorithms in Mobile Ad-hoc Networks” International Conference on Industry Interactive Innovations in Science, Engineering and Technology (I3SET-2K16), Proceedings published in Springer series of Lecture Notes in Networks and Systems (LNNS)

[37] Abu Sufian, Anuradha Banerjee and Paramartha Dutta: “Fuzzy-controlled Scheduling of Route-Request Packets (FSRR) in Mobile Ad Hoc Networks”, Indian Journal of Science and Technology, Vol 9(43), DOI: 10.17485/ijst/2016/v9i43/104384, November 2016

[38] Anuradha Banerjee, Paramartha Dutta and Abu Sufian: “Fuzzy Route Switching for Energy Preservation (FEP) in Ad Hoc Networks”, Indian Journal of Science and Technology, Vol 9(43), DOI: 10.17485/ijst/2016/v9i43/104383, November 2016

[39] Anuradha Banerjee, Paramartha Dutta and Abu Sufian: “EMR-PL: Energy-efficient multipath routing based on link life prediction in ad hoc networks”, Journal of Information and optimization Science (Taylor and Francis), vol. 39, issue 1, 2018