Real-time Air Pollution Monitoring and AQI Prediction System:

Environmental Intelligence with IoT-Based Approach and Machine Learning

Prof. Nitin Goyal1 , Shorya Chandokia2 , Abdul Rahman3 Ateek Saifi4 Tushar Sharma5

∗Department of Computer Science,R.D. Engineering College, Ghaziabad, India

Email: nitin.cs@rdec.in, shorya022ai0070@rdec.in, abdul022ai0005@rdec.in, ateek022ai0020@rdec.in, tushar022ai0078@rdec.in

Abstract—Air pollution is one of the most signifi-cant health concerns on earth, and the World Health Organization believes that 7 million premature deaths happen annually due to air quality. In this paper, the author is going to provide an elaborate, deploy-able system architecture that incorporates IoT sensor networks, real-time data processing, and machine learning advanced algorithms to monitor and predict air quality. It is made of distributed low-cost sensor nodes, 5G/4G cellular communication infrastructure, cloud-based data processing pipelines, and LSTM-GRU hybrid neural networks to predict AQI.

24 months of performance analysis of 47 urban monitoring stations indicates the probability of mak-ing 24-hour AQI predictions with accuracy of 91.3 percent with RMSE of 12.8µg/m3 for PM2.5 concen-

- Introduction

- Background and Global Context

One of the most pressing environmental issues of the 21 st century, air pollution is the reason of about 7 million untimely deaths each year due to ambient air or domestic air pollution or both [1]. The economic cost is also quite impressive: air pollution costs the world economy about $5 trillion a year in healthcare spending and loss of productivity. This issue is also complicated by the lack of a proper monitoring network in developing areas.

Particulate matter (PM2.5 and PM10) and gaseous pollutants (NO , SO , O ) are often at hazardous

tration. Compared to classical ARIMA approaches,

2	2	3

-tration . Compared to classical ARIMA approaches, it is demonstrated that it has a 18% improvement and 12% improved compared to single LSTM models. Some of the features of the system include real-time alerts, health advisory services, and regulatory compliance reporting. Scalability analysis aids the confirmation of linear increase of costs (O(n)) with density of sensor network which allows cost-effective deployment over geographical areas. The work is useful in modernizing environmental monitoring in-frastructure, and in evidence-based policy formulation of airlines.

Index Terms—Air Quality Index, IoT Sensors, time series prediction, real-time monitoring, machine learn-ing, environmental monitoring, sensor,networks, time-series forecasting.

levels in urban centers during periods of pollution. Conventional surveillance takes a form of sparse reference stations with the developed world having an average of 1 reference station per 5,000–10,000 km2 and the developing countries having none at all.

- Limitations of Existing Systems

Conventional air quality monitoring systems face several critical limitations:

  - Spatial Coverage Gaps: Reference-grade in-struments cost $40,000–$150,000 per unit, limiting deployment. Most countries have fewer than 100 monitoring stations.

  - Temporal Resolution: Data availability ranges from 1–24 hours post-measurement, reducing actionability for real-time public health inter-ventions.

  - Data Accessibility: 73% of monitoring data globally is not publicly accessible in standard-ized formats.

  - Cost Barriers: Annual maintenance and cali-bration costs exceed $20,000 per station.

Recent advances in low-cost IoT sensors, com-bined with 5G infrastructure deployment and ma-chine learning algorithms, present unprecedented opportunities to address these limitations.

- Research Objectives

Key objectives: (1) design distributed IoT net-work with 91% cost reduction; (2) develop en-semble LSTM-GRU achieving 91.3% categorical accuracy; (3) create production-ready platform; (4) demonstrate O(n) scalability. Contributions: inte-grated system architecture, ensemble methodology, and validation across 47 stations.

- Literature Review

- Air Quality Standards and Health Impacts

The Air Quality Index (AQI) provides a stan-dardized communication mechanism between mon-itoring agencies and the public [1]. The EPA AQI ranges from 0 to 500, with six health impact cate-gories based on National Ambient Air Quality Stan-dards (NAAQS). Recent epidemiological studies demonstrate significant mortality risks associated with prolonged exposure to fine particulate matter:

standards correlates with increased hospital admis-sions, emergency room visits, and long-term chronic diseases. A comprehensive deployment of real-time monitoring systems has been demonstrated to re-duce pollution-related health incidents by 18-22% through timely public warnings and adaptive traffic management interventions [7].

- IoT Sensor Technologies

The emergence of low-cost IoT sensors has rev-olutionized environmental monitoring capabilities [2]. A recent meta-analysis of 127 studies reports correlation coefficients (R2) with reference instru-ments across multiple pollutant types:

These high correlation coefficients demonstrate that modern low-cost sensors are sufficiently ac-curate for regulatory compliance applications [5]. However, sensor calibration and maintenance re-main critical. Studies indicate temporal drift of 8–15% monthly for electrochemical sensors and 12–20% for optical sensors, necessitating regular calibration protocols. The integration of multiple sensors with complementary operating principles (optical, electrochemical, and thermal) improves overall measurement reliability and enables cross-validation for anomaly detection. Additionally, co-location with reference-grade instruments during calibration campaigns establishes baseline accuracy metrics essential for post-deployment quality assur-ance [6].

- ML Methods

Time-series forecasting methods have evolved significantly over the past decade. Classical ARIMA methods serve as important baselines: achieving

∆Mortality = 0.55%  per 10 µg/m3 PM2.5 increase RMSE of 18–25 µg/m3 with 72–78% categorical

(1)

The relevance of real-time air quality monitoring to the decision-making of people is of critical importance due to this relationship. Current air quality standards recommend PM2.5 concentrations remain below 15 µg/m3 for 24-hour averages and

5 µg/m3 for annual averages [13]. The World Health Organization guidelines are even stricter, recommending 15 µg/m3 annually and 37.5 µg/m3 for 24-hour exposure. Non-compliance with these

accuracy. Nevertheless, ARIMA makes the assump-tions of linear relationships and stationarity, and as such, it cannot be applicable to the more complex dynamics of air pollution due to meteorology, traffic patterns, and atmospheric changes in the boundary layer.

These limitations are overcome by the Long Short-Term Memory (LSTM) networks which model temporal dependencies and non-linear pat-terns. Recent applications have 12–18 µg/m3 RMSE

with 85–89% categorical accuracy [9]. GRUs have similar performance and inference times 30–40% times faster with lower memory footprints, and they are well-suited to edge deployment applications [8]. Ensemble methods are a new paradigm in fore-casting, which involves using predictions of het-erogeneous models by adaptively weighting them. Recent publications show 3–8% performance im-provement using ensemble technique [15]. The most important benefit is the model diversity: hybrid LSTM-GRU ensembles represent complementary factors of air quality dynamics. Also, Transformer-based architectures are promising in terms of long-range dependencies in multi-day pollution episodes, but they demand more data and more computational

resources.

Cloud

Edge

IoT

Layered Cloud-Edge IoT Architecture for Air Quality Monitoring

APPLICATION LAYER

Web Dashboard I Mobile App I Admin Portal

BUSINESS LOGIC LAYER

Prediction Service I Alert Service I Analytics Service

DATA PROCESSING LAYER

Apache Kafka I Spark Streaming I Feature Engine

DATA STORAGE LAYER

lnfluxDB I PostgreSQL I Redis Cache

COMMUNICATION LAYER

LTE Gateway I LoRaWAN Gateway I MQTTIHTTP

SENSOR LAYER

PM Sensors I Gas Sensors I Reference lnstruments I GPSIMeteo

RESTI

GraphQL

gRPC

SQLI NoSQL

MQTTIHTTP

- System Architecture and Design

- Overall System Architecture

The proposed system employs a layered cloud-edge architecture designed for scalability, reliability, and real-time responsiveness as illustrated in Fig-ure 2. The system comprises seven primary layers with clear separation of concerns. This architecture was designed following microservices principles to enable independent scaling of components based on load requirements [10].

- Hardware Components

1) Sensor Node Specifications: Each monitor-ing station comprises specialized sensors with the specifications provided in Table ??. The component selection balances cost, accuracy, and reliability requirements [7]:

Reference-Grade Station: 4 units at $260,000 each for ground truth calibration and post-deployment validation. These reference stations em-ploy advanced optical particle counters and elec-trochemical gas analyzers with factory calibration traceability to national standards.

Network Cost Analysis: (43 × $720) + (4 ×

$260,000) = $1,071,000 vs. reference-only network

of $12.22M. This is a saving of **91% of costs** and preservation of measurement reliability with hybrid calibration techniques [11]. The cost benefit will allow it to deploy in developing countries

Fig. 1. Layered cloud-edge system architecture for real-time air quality monitoring. The seven layers enable separation of concerns: sensors perform edge processing, communication gateways provide protocol translation and buffering, data pro-cessing pipelines enable real-time feature computation, storage layers provide tiered access patterns, business logic implements forecasting and alerting, and application layer provides user interfaces. The architecture can be deployed on heterogeneous IoT, 5G, and cloud infrastructure [3].

and underserved areas to overcome spatial coverage gaps that do not allow evidenced based policy formulation on air quality.

- Software Stack

The entire technology stack is described in Ta-ble III :

- Data Collection and Quality Management

- Temporal Resolution and Sampling Strategy

Frequencies of data collection are adjusted cau-tiously to provide a balance between calculation and content of information [12].Sampling rates of PM2.5 and PM10 the gas-phase pollutants (NO2, SO2, O3, CO) (at 1-minute), meteorological variables (temperature, humidity, pressure, wind speed) (at 10-minute frequency) and GPS positions (at 1 hour frequency) will be used to validate spatial and tem-poral scales respectively. This stratified sampling randomization scheme provides a way of capturing the pollution events at reasonable time scales and it

TABLE III

System Technology Stack

Layer	Technology Function	Key Bene-

fit

Sensor	Arduino	Edge process- Low over-Firmware C++	ing	head

- Spatial outlier detection: Determination of the stations with abnormal level of pollution in comparison with the neighbors.

The completeness of network-wide data is 96.2% upon automated quality control, and the majority of missing data can be explained by the planned peri-

CommunicaMtiQonTT

over	TLS 1.3

Ingestion Apache

Kafka v3.2

Lightweight protocol

Distributed broker

Pub-sub model

1M+

events/sec

ods of maintenance and temporary communication

disabilities. Interpolation of gaps in data is done by local interpolation or model-based imputation instead of deletion to maintain continuity of time.

Stream Process-ing Time-Series DB

Apache Spark v3.3

InfluxDB v2.4

Real-time pipeline

Data persistence

Native ML library

Optimized queries

C. Multi-Tier Storage Architecture

The storage architecture has three distinct tiers that are optimized based on the access patterns as well as being cost effective [13]:

- Hot Tier (<100 ms latency, 7-day retention):

Relational PostgreSQL Reference

ACID com-

Data with 1-minute resolution are placed in

DB ML

Frame-work Backend API

v14 TensorFlow v2.10

FastAPI v0.95

data

Model train-ing

REST

endpoints

pliance Production deployment

3–7A˜ —

faster

the InfluxDB storage as raw data and up-dated live on a dashboard and alerts. This level allows sub-second response times when responding to a query, a requirement critical to active incident response.

Frontend React.js

v18

Mobile	React  Na-

tive

Web	Component-

dashboard	based iOS/Android	Cross-

platform

- Warm Tier (100 ms–1 sec latency, 2-year retention): Saves 1-hour aggregated data in PostgreSQL with statistical summaries (min,

OrchestratioKnubernetes

v1.26

Cloud

Container mgmt

Auto-scaling

max, mean, std dev, percentiles). Favors the long-term trend analysis and identification of

AWS (EC-

S/EKS)

Infrastructure  Global reli-

ability

the seasonal patterns.

- Cold Archive (1–5 minute latency): Com-pressed Annual summaries storage in cloud

is able to utilize the wireless bandwidth limitations in the cellular networks.

- Real-Time Validation and Quality Assurance

Verification pipelines are run instantly with laten-cies through Apache Spark streaming of less than 5 seconds [8]. Quality control procedures entail :

  - Range checks: Flagging of measurements which are out of physical range (e.g., PM2.5

¿ 500 µg/m³ will indicate sensor failure);

object stores (AWS S3) to act as regulatory legal, compliance, and reference.

- Machine Learning for AQI Prediction

  - AQI Calculation Methodology

Calculation based on the EPA-standard AQI transforms the measured concentrations of pollu-tants into the dimensionless indices that allow com-municating them to the public [14]:

    - Rate-of-change filtering: Identification of un-realistic variation (e.g., PM2.5 change ¿ 100

I = AQIhi − AQIlo ×(C

p	BPhi − BPlo

p −BPlo)+AQIlo

(2)

µg/m³ over 30 minutes);

    - Cross-sensor correlation: Testing ratios of pollutants (e.g., consistency of the ratio be-tween NO2 to NOx );

where Ip is the sub-index of pollutant p, Cp is

value of the measured concentration, and BP is EPA specified breakpoints. Each pollutant has distinct breakpoints calibrated to health effect thresholds.

Final AQI determination represents the maximum sub-index:

yGRU (t) = GRU(x1:t)  with hidden dimension = 96

AQIfinal = max(IPM 2.5, IPM 10, INO2, ISO2, IO3, ICO )

(3)

(5)

This ”maximum is responsible pollutant” ap-proach ensures that any single pollutant exceed-ing standards triggers elevated AQI classification. Categorical mapping follows EPA guidelines: 0–50 (Good, Green), 51–100 (Moderate, Yellow), 101–

150 (USG, Orange), 151–200 (Unhealthy, Red),

201–300 (Very Unhealthy, Purple), 300+ (Haz-ardous, Maroon).

  - Feature Engineering for Temporal Forecasting

We construct 30-dimensional input vectors cap-turing multiple aspects of air quality dynamics [15]:

- Lagged pollutant concentrations: History at 1, 6, 24, and 168-hour lags enabling capture of both rapid and seasonal patterns

- Temporal features: Hour-of-day (sine/cosine encoding), day-of-week, day-of-year, holiday indicators

- Meteorological variables: Temperature, hu-midity, pressure, wind speed, wind direction, precipitation

- Interaction terms: PM-temperature, NO2-wind speed products capturing pollutant-meteorology coupling

- Rolling statistics: 6-hour and 24-hour rolling

means and standard deviations of pollutants

yBi−LST M (t) = BiLSTM(x1:t)  with hidden dimension = 64

(6)

Each architecture operates on normalized input sequences of 168 time steps (one week) to cap-ture intra-week periodicity. Dropout regularization (p=0.3) prevents overfitting. The hybrid approach leverages complementary advantages: LSTM excels at capturing long-range dependencies, GRU offers computational efficiency, and bidirectional LSTM incorporates future context useful for offline analy-sis.

Total network parameters: ∼85,000; GPU memory requirement: ∼340 MB per batch (batchsize=32).

- Adaptive Ensemble Weighting

The ensemble combines predictions through adaptive weighting mechanism:

4

yˆensemble(t) = X wi(t) · yˆi(t)	(7)

i=1

Weights are updated every 6 hours based on individual model performance on rolling validation windows:

- Derived	features:  Atmospheric  stability

proxies, mixing height estimates, ventilation

exp(−λ · RMSEi(t − 6h))

i

4

j=1

exp(−λ · RMSEj(t − 6h))

w (t) = :E

(8)

indices

All features undergo z-score normalization with training set statistics to ensure numerical stability during neural network training.

  - LSTM-GRU Hybrid Ensemble Architecture

The ensemble combines four complementary neural network architectures optimized through ex-tensive hyperparameter search:

with decay parameter λ = 0.5. This mechanism automatically down-weights underperforming mod-els, improving robustness to model-specific failure modes. For instance, if winds shift unexpectedly affecting local pollution transport, the bidirectional LSTM incorporating future context automatically gains higher weight.

E. Model Training and Validation

All models undergo rigorous 5-fold cross-validation on an 8,760-hour test set spanning 4

yLST M (t) = LSTM(x1:t)  with hidden dimension = m12o8nths during December (winter pollution episode)

(4)	and June (summer ozone episode). During training,

the parameter λ is tuned to balance bias-variance tradeoff. As shown in Figure 3, the ensemble method achieves 91.3% categorical accuracy with

12.8 µg/m3 RMSE, representing 29.7% improve-ment over ARIMA baseline and 12% improvement over individual LSTM models.

Figure 4 demonstrates a representative 24-hour forecast with 95% confidence intervals com-puted through Quantile Regression Forest post-processing. The shaded uncertainty band (mean width: 28.4 µg/m3) reflects model prediction vari-ance and is crucial for risk-based decision-making by public health authorities. The system exhibits satisfactory pollution dynamic capturing such as morning rush-hour maximums, afternoon mixing height ventilation impacts, and evening stagnation [11].

- Experimental Results and

Performance Evaluation

- Deployment Scenario

We implemented the system in 47 monitoring stations that were spread in three large metropoli-tan areas within a 24-month period of assessment (January 2023 to December 2024). These nodes consisted of 43 cost-effective IoT nodes and 4 reference-grade ones in the network ground truth stations. Meteorological sources of data consisted of automated weather stations that were in co-location with the reference instruments.

- Forecast Accuracy Metrics

Complete model analysis uses several measures of errors that reflect various facets of forecast qual-ity [12]:

The ensemble achieves categorical accuracy of 85.6%, which is, in 24-hour forecasts, able to identify categories of AQI correctly ¿85% of the time, offers reasonable advice to help people make the right decisions in relation to public health. The 33.6% higher RMSE on top of ARIMA (14.2 vs 21.4 µg/m3) is statistically significant (p ¡ 0.001) and statistically signifies the significant enhance-ment over the baseline techniques, especially at the times of moderate pollution.

TABLE IV

Forecast Performance Across Ensemble and

Baseline Methods

Method		RMSE	MAE Accuracy Deployment (µg/m3) (µg/m3)	(%)	Cost ($K)

| ARIMA | 21.4 | 15.6 | 72.8 | 12,220 |
| --- | --- | --- | --- | --- |
| Baseline |  |  |  |  |
| Prophet | 19.2 | 13.8 | 78.3 | 12,220 |
| LSTM | 15.8 | 11.4 | 82.6 | 1,095 |
| GRU | 15.3 | 11.1 | 83.2 | 1,070 |
| BiLSTM | 14.7 | 10.6 | 84.1 | 1,090 |
| Ensemble | 14.2 | 10.3 | 85.6 | 1,071 |

There is anticipated seasonal variation in per-formance, where winter 16.8 µg/m3 and summer

12.3 µg/m3 are the mean winter and summer RMSE respectively (greater prediction error is observed in the winter because of stagnation events). Event-based analysis shows 73.5% recognition of pollu-tion episode onsets ¿48 hours or less, which offers moderate evidence of proactive measures on behalf of the population.

- Scalability Analysis and

Deployment Considerations

- Computational Scalability

The network density of the system architecture shows linear computational complexity O(n) [10]. This is a vital feature that allows cost effective geographic expansion:

Total Cost = c0 + n × cper node	(9)

where c0 is a fixed cost of infrastructure (cloud services, dashboards) and cper node is the cost per monitoring station of incremental costs. In our case, adding every station ran up the average total infrastructure cost by $721 during our deployment within budget targets.

The throughput of messages using Kafka brokers scaling is linear: there are 47 stations, which pro-duce about 8,640 messages per day per pollutant (1-

minute resolution) , or ∼52,000 messages in total

per day. Storm events of 200,000 messages/hour can

be sustained without degradation. Latency The end-to end real-time processing in real time is always fixed at ¡500 ms independent of network size.

- System Reliability and Redundancy

Field deployment showed that the system had average availability of 97.8% per 24 months. Crit-ical maintenance (0.8% downtime) was done on a quarterly basis to update the firmware and sensor recalibration. The average of the unplanned outages was 1.4% and were below the plan . Mainly was the unavailability of network 4G in some geographical locations and some hardware failures that necessi-tate distant diagnosis.

Redundancy mechanisms mitigated impact of

Cloud

Edge

IoT

Layered Cloud-Edge IoT Architecture for Air Quality Monitoring

APPLICATION LAYER

Web Dashboard I Mobile App I Admin Portal

BUSINESS LOGIC LAYER

Prediction Service I Alert Service I Analytics Service

DATA PROCESSING LAYER

Apache Kafka I Spark Streaming I Feature Engine

DATA STORAGE LAYER

lnfluxDB I PostgreSQL I Redis Cache

COMMUNICATION LAYER

LTE Gateway I LoRaWAN Gateway I MQTTIHTTP

SENSOR LAYER

PM Sensors I Gas Sensors I Reference lnstruments I GPSIMeteo

RESTI

GraphQL

gRPC

SQLI NoSQL

MQTTIHTTP

component failures [13]:

- Dual sensors for critical pollutants (PM2.5 sensor + backup) enable continued operation during sensor drift

- Distributed Kafka brokers prevent single points of failure in data ingestion

- TensorFlow Lite model quantization enables fallback inference on edge microcontroller by-passing cloud connectivity during outages

- Localized memory buffering (12-hour capac-ity) permits temporary cloud service unavail-ability

- Operational Costs

Long-term operational expenditure analysis demonstrates economic viability across diverse deployment scenarios:

Fig. 2. Layered cloud-edge system architecture for real-time air quality monitoring. The seven layers enable separation of concerns: sensors perform edge processing, communication gateways provide protocol translation and buffering, data pro-cessing pipelines enable real-time feature computation, storage layers provide tiered access patterns, business logic implements forecasting and alerting, and application layer provides user interfaces. This architecture supports deployment across hetero-geneous IoT, 5G, and cloud infrastructure and is optimized for fault tolerance and elastic scalability [3].

- Conclusions

This research demonstrates the practical feasibil-ity of integrating Internet of Things (IoT) technol-ogy with machine learning for real-time air qual-ity monitoring. The proposed seven-layer cloud-edge architecture successfully achieves 97.8% sys-tem availability with sub-500 millisecond end-to-end latency across 47 geographically distributed monitoring stations, providing a reliable founda-

Annual OpEx = Hardware Replacement+Personnel+CtiloonudfoSr ernvviicreosnmental monitoring systems [3]. The

(10)

Per-station annual operational costs averaged

$2,340, dominated by personnel costs for monthly maintenance visits (55%) and cloud infrastructure (30%). Hardware replacements averaged $240 an-nually (sensor wear-out and component failures). This demonstrates cost advantages compared to reference-grade station OpEx ($24,000 annually) while providing moderate improvements in spatial resolution [7].

ensemble learning methodology combining LSTM and GRU architectures demonstrates improvements in predictive performance, achieving 85.6% cate-gorical accuracy and 14.2 µg/m³ root mean square error, representing a 33.6% improvement over tradi-tional ARIMA methods and meaningful gains over single-model approaches, while adaptive weighting mechanisms improve robustness to model-specific limitations [11]. Economically, the system can save the system about 90% of the cost relative to

Model Performance Comparison for 24-Hour AQI Forecast

RMSE (µg/m3)

Category Accuracy (o)

18.2

91.3%

16.5

87.6%

14.3

13.1

85.4%

12.8

81.2%

78.3%

25

20

RMSE (µg/m³)

15

10

5

0

ARIMA	Prophet	LSTM	TCN	Ensemble

Forecasting Model

100

95

Category Accuracy (%)

90

85

80

75

70

provides a balance between access latency and storage economics [8]. The accuracy of event-based analysis in forecasting pollution events longer than 48 hours is 73.5% which can support the decision-making process by the population in a moderate fashion. The system is already functioning in three metropolitan areas. Future studies will target the extension of deployment to new areas, adding better deep learning structures, and deploying federated learning models to enhance models in a system of different jurisdictions.

Fig. 3. Model performance comparison showing RMSE, MAE, and categorical accuracy for 24-hour AQI forecasting across ARIMA, Prophet, LSTM, GRU, BiLSTM, and Ensemble meth-ods (n=8,760 hours, test set spanning 4 months). The ensemble method achieves superior performance through adaptive weight-ing of complementary model architectures, reducing RMSE by 40.2% compared to ARIMA baseline.Error bars are used to denote 95% confidence intervals, which are 5-fold cross-validation [10].

24-Hour AQI Forecast with Ensemble Model and Confidence Intervals

RMSE: 12.8±1.5 µg/m³

µg/m³ 91.3%

MAE: 8.6

Accuracy:

)

(35 µg/m3)

.

(±28.4 µg/m3

dence Interval sured PM Forecast Guideline

95% Confi Actual Mea Ensemble WHO 24-h

140

Acknowledgment

The authors appreciate the collaboration of the municipal environmental protection agencies and the data sharing assistance of the operators of environmental monitoring stations around the metropolitan area. Special acknowledgement is the credit of the research assistants that carried out field calibration and validation experiments. En-vironments The Environmental Science Research

PM . Concentration (µg/m3)

120

100

80

60

40

20

00:00

02:00

04:00

06:00

08:00

10:00

12:00

14:00

16:00

18:00

20:00

22:00

0

Hour of Day

Fund (Grant No. ESR-2022-1847) and the Office of Research Development.

References

- V. W. Tsai et al., “Global, regional, and national disability-adjusted life-years (DALYs) for 315 diseases and injuries,” The Lancet, vol. 403, no. 10438, pp. 1949–2023, 2024.

- Alphasense Ltd., “Alphasense Electrochemical Sensor

Fig. 4. Example 24-hour PM2.5 forecast with ensemble model predictions (blue line) and 95% confidence intervals (shaded region). Observations from reference instruments are shown as red dots. The shaded uncertainty band (mean width: 28.4 µg/m3) reflects model prediction variance and is computed through Quantile Regression Forest post-processing. This case study demonstrates the model’s ability to capture both rapid pollution onset during evening stagnation events and daytime ventilation patterns controlled by atmospheric mixing height dynamics [11].

reference-grade monitoring networks and achieves acceptable spatial resolution coverage, and per-station capital cost of $720 and yearly operational costs of $2,340, which allows broader deployment throughout developing countries and unserved ar-eas in the developing world [7]. The geographic expansion based on network density is enabled by the linear computational complexity of O(n), which does not need a significant redesign of infrastructure to achieve, and the multi-tier storage architecture

Datasheets,” OEM Sensor Product Guide, v2.12, 2024.

- GSMA Intelligence, “5G Rollout Status and Coverage Report,” London, 2024.

- LoRa Alliance, “LoRaWAN Deployment Statistics 2024,” San Jose: Lora Alliance, 2024.

- D. Thierry et al., “Performance Evaluation of LoRaWAN for Air Quality Sensor Networks,” Int. J. Environ. Sci. Technol., vol. 21, no. 3, pp. 1573–1588, 2024.

- Cradlepoint, “NB-IoT Deployment Report 2024,” Los Al-tos, CA, 2024.

- P. Polastre et al., “Design and Evaluation of NB-IoT for Environmental Monitoring,” IEEE Commun. Mag., vol. 62, no. 2, pp. 78–85, 2024.

- Confluent, “Kafka in Production: Deployment Patterns 2024,” San Francisco: Confluent, 2024.

- Databricks, “Apache Spark 3.4: Performance and Scalabil-ity Benchmarks,” 2024.

- Amazon Web Services, “AWS Kinesis Best Practices and Performance Tuning,” Technical Documentation, 2024.

- V. Zaichkin et al., “Time-Series Database Performance Benchmarks,” Proc. VLDB, vol. 16, no. 13, 2024.

- Timescale Inc., “TimescaleDB Performance at Scale,” Technical Whitepaper, 2024.

- Gartner, “Magic Quadrant for Cloud Infrastructure and Platform Services,” Report ID G00706049, 2024.

- Statista, “Cloud Market Share Statistics 2024,” Hamburg, 2024.

- IDC, “Cloud Infrastructure Market Share Analysis,” Boston, 2024.