Evaluation of Propellant Slosh Effects and Baffle Requirements in a Hybrid Rocket Oxidizer Tank

ABSTRACT

This paper evaluates the necessity of incorporating internal baffles in the oxidizer tank of a vertically launched hybrid rocket with a regressive fuel grain configuration. Potential sources of rocket instability are reviewed, with particular focus on propellant slosh and its effects on vehicle stability. A systems engineering approach is applied to define and decompose oxidizer tank requirements, followed by preliminary tank design activities including sizing, material selection, wall-thickness calculations, and internal component definition. The effects of oxidizer motion during flight are analyzed and compared with the stabilizing forces generated by tank ullage pressure. The results show that slosh-induced forces are insufficient to significantly affect vehicle stability, leading to the conclusion that internal baffles are not required for the proposed oxidizer tank design.

- INTRODUCTION

Rocket stability is the inherent or controlled capability of a rocket vehicle to maintain or return to its desired attitude and trajectory following a disturbance, through the action of restoring aerodynamic, propulsive, or control-system moments.[1]

Loss of stability in a rocket can occur due to a variety of interacting factors associated with propulsion, aerodynamics, structural dynamics, and vehicle mass properties. One of the primary causes is combustion instability, which arises when pressure oscillations within the combustion chamber grow rapidly and become self-sustaining. This can result from poor mixing of fuel and oxidizer, injector design deficiencies that create uneven propellant distribution, coupling between chamber acoustic modes and combustion processes, or delayed combustion reactions that generate feedback oscillations. These phenomena can lead to significant fluctuations in chamber pressure and thrust, potentially causing performance degradation or structural damage.

Another major source of instability is flight (aerodynamic) instability, which occurs when the aerodynamic forces acting on the vehicle can no longer maintain stable flight. This often happens when the center of pressure (CP) moves ahead of the center of mass (CM), creating a destabilizing aerodynamic moment. Additional contributors include excessive wind shear, atmospheric disturbances, inadequate fin effectiveness in smaller rockets, failures in guidance, navigation, or control systems, thrust vector control malfunctions, and uneven engine thrust or engine-out conditions. Such factors can cause excessive attitude deviations and loss of trajectory control.

Propellant slosh instability is also a significant concern, particularly in vehicles with large, partially filled propellant tanks. The movement of liquid propellant inside the tanks can generate dynamic forces and moments that interact with the vehicle’s attitude-control system. If not adequately controlled through tank baffles or damping devices, the sloshing motion can couple with vehicle dynamics and induce oscillatory behavior that degrades flight stability.

A further source of instability is structural or dynamic instability, commonly referred to as Pogo oscillation. This phenomenon results from the interaction between pressure fluctuations in the propellant feed system and the structural vibrations of the launch vehicle. When engine thrust oscillations resonate with the rocket’s natural structural frequencies, large longitudinal vibrations can develop, potentially threatening vehicle integrity and mission success.

Feed-system instability can also contribute to overall rocket instability. Problems such as cavitation in turbopumps, vapor formation within propellant feed lines, pressure regulator malfunctions, and dynamic coupling between injectors and the feed system can cause fluctuations in propellant flow rates and chamber pressure. These disturbances may propagate throughout the propulsion system and lead to unstable engine operation.

Finally, changes in mass properties during flight can affect vehicle stability. As propellant is consumed, the center of gravity shifts and the vehicle’s moments of inertia change continuously. If these changes are not properly accounted for in the control system design, the effectiveness of the guidance and control laws may be reduced, leading to degraded stability margins and increased difficulty in maintaining the desired flight path. Together, these factors highlight the complex and multidisciplinary nature of rocket stability and the importance of careful design, analysis, and testing throughout the development process.Top of FormBottom of Form

In this section only vehicle instability cause by Propellant Slosh is studied with defined design parameters.

Designing the internal features of a rocket propellant tank goes far beyond simply storing fuel; it involves carefully managing fluid behavior under extreme conditions such as high acceleration, microgravity, and significant temperature gradients. An effective design must strike a balance between structural integrity, mass efficiency, and reliable propellant delivery.  One of the primary internal features includes baffles, also known as slosh control structures, which are internal plates or rings used to reduce fluid movement during acceleration, stage separation, or attitude changes. By limiting sloshing, they help maintain vehicle stability, although increasing their number can improve performance at the cost of added weight. [2]

Another critical component is the propellant management device (PMD), especially important in microgravity environments where traditional gravity-fed systems are ineffective. PMDs ensure that propellant remains near the outlet using mechanisms such as vanes that guide liquid via surface tension, sponges or screens that trap liquid, and channels that direct flow, all of which rely on capillary action.  The ullage space, or the empty volume above the liquid propellant, also plays a key role and is maintained by a pressurization system that ensures consistent propellant flow to the engines. This system typically uses inert gases like helium and helps prevent cavitation in turbopumps. Anti-vortex devices are installed near the tank outlet to prevent the formation of vortices that could draw gas into the feed line, thereby ensuring steady engine operation. Thermal management features are equally important, particularly for cryogenic propellants such as liquid oxygen or hydrogen, and may include internal insulation, control of temperature stratification, and sometimes active mixing systems to maintain uniform conditions. In addition, sensors and instrumentation are embedded within the tank to monitor parameters such as propellant level, temperature, and pressure, which are crucial for both control and safety. Finally, the geometry of the outlet and feed system is carefully designed to maximize propellant usage, often incorporating sump regions to collect remaining fuel and minimize unusable residuals. [2]

Inside a rocket tank, liquid propellant doesn’t stay still. During acceleration, or engine cutoff, it sloshes. This can: Shift the center of mass, Couple with vehicle dynamics (called sloshing instability) and Destabilize guidance and control systems. Baffles reduce this motion by dissipating energy and breaking up fluid flow. Among a few standard configurations: Ring baffles: Circular plates with holes, mounted inside the tank, Radial baffles: Plates extending inward from the tank wall, Perforated plates: Flat plates with carefully sized holes and Vaned baffles: Angled structures that redirect flow. Typical situations where rockets may avoid baffles: Tanks are nearly full all the time, very small launch vehicles, Spin-stabilized upper stages and Use of diaphragms, bladders, PMDs (propellant management devices), or surface-tension systems instead. Typical situations where baffles become mandatory: Large cryogenic oxidizer tanks, long slender rockets, Vehicles with strong pitch/yaw maneuvers, Partial-fill conditions and restart able upper stages or microgravity operations. Baffles are usually placed: Near the tank mid-height  or at nodes of expected slosh modes (determined via simulation). This ties into modal analysis of the fluid system. Structural loads: Baffles must survive: Launch acceleration (several g’s), Dynamic pressure from moving fluid and Thermal stresses (especially in cryogenic tanks). Adding baffles improves stability but introduces costs: Increased mass (bad for payload capacity), Manufacturing complexity and Potential for trapping gas or creating uneven draining. [3]

- DESIGN REQUIREMENT

| Customer Expression of Interest | Level 1 Requirement |
| --- | --- |
| We are seeking an experienced oxidizer tank designer/manufacturer for a hybrid rocket propulsion system using Nitrous Oxide (N₂O) as the oxidizer and HTPB as the fuel. The oxidizer tank shall have a capacity of 4.8 liters with a filling pressure of 51 bar and must operate reliably within a vehicle altitude range of 0–20 km. The propulsion system requires a 1-inch diameter propellant feed line with an oxidizer flow rate of 0.3 kg/sec. The tank design should withstand the vehicle operating velocity and associated aerodynamic and structural loads during vertical takeoff and flight. The design should also ensure structural integrity, ease of manufacturing, lightweight construction, and compatibility with a powered flight duration of approximately 20 seconds. | R-1: Types Of Propellant(oxidizer), Nitrous Oxide (N₂O). |
|  | R-2: Types Of Propellant(fuel), HTPB. |
|  | R-3: Amount Of Oxidizer in Litter, 4.8 Liter. |
|  | R-4: oxidizer Filled Pressure in Tank, 51 Bar. |
|  | R-5: Vehicle Operation Altitude Domain, (0-20) Km. |
|  | R-6: Propellant Feeding Pipe Dimension 1 Inch Diameter And 0.3 Kg/Sec Propellant Flow Rate. |
|  | R-7: Vertical Takeoff. |
|  | R-8: Easy For Manufacture. |
|  | R-9: Powered Vehicle Flight Time (20 Sec) |
|  | R-10: The Tank Can Withstand Vehicle Velocity. |

Table 1:  Define and Derivative Requirements.

| No. | Level 1 Requirement | Level 2 Requirement | Level 3 Requirement | Verification |
| --- | --- | --- | --- | --- |
| 1 | R-1: Types Of Propellant(oxidizer), Nitrous Oxide (N₂O). | R-11: vehicle cycle should exclude unnecessary component from the system. |  |  |
|  |  | R-12: tank material must be compatible with nitrous oxide. |  |  |
| 2 | R-2: Types Of Propellant(fuel), HTPB. |  |  |  |
| 3 | R-3: Amount Of Oxidizer in Litter, 4.8 Liter. | R-31: oxidizer tank volume should consider Nitrous Oxide properties and filling temperature and pressure . |  |  |
|  |  | R-32: practical diameter to length ratio of tank should be consider. |  |  |
|  |  | R-33 tank volume should consider ullage space |  |  |
| 4 | R-4: oxidizer Filled Pressure in Tank, 51 Bar. | R-41: The Ambient Temperature Should Not Exceed 20 °C While the Oxidizer Is Stored in The Tank. |  |  |
|  |  | R-42: Tank thickness must be calculated with enough safety factor. |  |  |
| 5 | R-5: Vehicle Operation Altitude Domain, (0-20) Km. | R-51 System should consider atmosphere temperature up to 20 km. |  |  |
|  |  | R-52 System consider air density up to 20 km |  |  |
|  |  | R-53 System consider gravitational force up to 20 km |  |  |
| 6 | R-6: propellant feeding pipe dimension 1 inch diameter and 0.3 kg/sec propellant flow rate. |  |  |  |
| 7 | R-7: Vertical Takeoff. |  |  |  |
| 8 | R-8: Easy For Manufacture. | R-81: Applied pressure in baffles due to oxidizer slosh. |  |  |
|  |  | R-82: Baffles design (Radial baffles). | R-821 Radial baffles with 6061 aluminum sheets (6061-T651) material and 2 mm thickness. |  |
|  |  |  | R-822 baffles dimensions, radial baffles size (0.1-0.25) R. |  |
| 9 | R-9: powered vehicle flight time (20 sec) |  |  |  |
| 10 | R-10: vehicle deceleration deceleration up to 500 m/sec. |  |  |  |

R-1: Types Of Propellant, Nitrous Oxide (N₂O)

R-11 Vehicle Cycle Should Exclude Unnecessary Component from The System

- No need to include thermal management device since Nitrous Oxide is not cryogenic propellants.

- No need to add inert gases like helium, since Nitrous Oxide by itself pressurization.

N2O in compressed-gas cylinders is present in

both the liquid and gaseous states. N2O cylinders are factory filled to 90% to 95% capacity with liquid N2O. Above the liquid in the tank is N2O gas. The gas pressure within the cylinder of N2O is approximately 52 bar at 25° C. The pressure of the N2O vapor floating above the liquid N2O is 52 bar. As the gaseous N2O exits from the cylinder, liquid N2O vaporizes to replace it. The pressure of this “new” gas is 52 bar. This process continues, liquid N2O converting to gaseous N2O, with the gas pressure remaining at 52 bar, until no more liquid remains to replace the gas. [2]

R-12  Tank Material Must Be Compatible with Nitrous Oxide

- 6061 aluminum sheets (6061-T651)

Most commonly recommended materials are aluminum .

alloys or stainless steel, depending on the design priorities (weight, cost, manufacturability, and pressure requirements). [4]

R-2: Types Of Propellant(fuel), HTPB

- As seen from below figure HTPB with N2O at 0/f 8 generate chamber pressure 30 bar.

Figure 1:  Results for Chemical Combustion from NASA CEA, Software.

R-3: Amount Of Propellant in Litter, 4.8 Liter

R-31: oxidizer tank volume should consider Nitrous Oxide properties and filling temperature and pressure.

Size Of Tank

volumem3=mass(kg)density (kgm3)                                               (1)

For Nitrous Oxide stored as a self-pressurized liquid, the liquid density depends on the storage temperature because the tank contains both liquid and vapor in equilibrium. [5]

Table 2:  Typical Liquid N₂O Densities

| Temperature (°C) | Liquid Density (kg/m³) | Vapor Pressure (bar) |
| --- | --- | --- |
| 0 °C | ~900 kg/m³ | ~30 bar |
| 10 °C | ~840 kg/m³ | ~44 bar |
| 20 °C | ~760 kg/m³ | ~51 bar |
| 30 °C | ~620 kg/m³ | ~71 bar |

For most hybrid rocket preliminary calculations, engineers commonly use, Density at ~20 °C ≈ ~750–770 kg/m³.

Using a typical value ≈ 760 kg/m³: 1 litter=7601000=0.76 kg

V=4.8*0.76760=0.0048 m3

R-32: Practical Diameter to Length Ratio of Tank Should Be Consider.

volume m3=aream2*lenght(m)                                              (2)

In practical rocket design there’s a well-established range that balances aerodynamics, structural strength, and stability. For most rockets, the length-to-diameter ratio (L/D) typically falls between: 10:1 to 20:1 → common for many sounding rockets and launch vehicles  and Around 12:1 to 15:1 → often considered a good general-purpose design range. [6]

Let select L/D = 13

aream2=π*D24(3)                                              (3)

V= 𝜋 * D24*13 D                                              (4)

4*0.0048 m313*3.14 = 0.00047 = D*D2=D3; D = 0.0777498 meter, then L=1.011 meter.

R-33: tank volume should consider ullage space

For small rockets, ullage (the empty space in a tank) is mainly there to handle thermal expansion, sloshing, and to ensure propellant feed stays reliable. There isn’t a single fixed number, but typical practice falls into a fairly narrow band. Common ullage percentages: ~2% to 5% of total tank volume → typical for many well-controlled designs and up to ~8–10% → used in simpler or less precisely controlled systems (including some small or experimental rockets).[6]

How to choose within that range: Lower end (~2–3%) Good temperature control, Pressurized systems with stable feed  and more advanced designs where maximizing propellant volume matters. Middle (~3–5%) A safe, common default for small rockets and allows for modest thermal expansion and slosh. Higher (~5–10%) Simpler tanks without tight thermal control, larger temperature swings and Early-stage or experimental builds where safety margin is preferred over efficiency. [2]

Let used 4 % , so updated volume 0.0048 m3+4% of 0.0048 m3=0.004992

4*0.004992 m313*3.14 = 0.0004892 = D*D2=D3; D = 0.078794423 meter, then L=1.02433 meter.

Slenderness ratio=Length Diameter                                             (5)

S.r ≅ 13, means rocket under categories of  slender rocket and needed to consider baffles.

Table 3:  Categories of Slenderness Ratio [7]

| No. | Slenderness ratio. | Names. |
| --- | --- | --- |
| 1 | Below 5 | Fat/stubby |
| 2 | 5-10 | Moderate |
| 3 | 10-15 | Slender |
| 4 | Above 15 | Very slender and flexible |

R-4: Propellant Filled Pressure in Tank,  51 Bar

R-41: The Ambient Temperature Should Not Exceed 20 °C While the Oxidizer Is Stored in The Tank.

The pressure–temperature relationship of nitrous oxide (N₂O) is important because it is a self-pressurizing oxidizer. As temperature increases, the vapor pressure rises rapidly.

Table 4:  Typical saturation-pressure values for nitrous oxide [8]

| Temperature (°C) | Vapor Pressure (bar) |
| --- | --- |
| -20 | 14.8 |
| -10 | 20.0 |
| 0 | 28.6 |
| 10 | 39.0 |
| 20 | 50.7 |
| 25 | 57.3 |
| 30 | 64.7 |
| 35 | 73.0 |
| 36.4 (critical point) | 72.5 |

R-42:  Tank thickness must be calculated with enough safety factor

In order to find the thickness of the tank, For the cylindrical portion,

Hoop stress

σH = pd2t                                             (6)

were

σH  = Hoop Stress

P = Internal Pressure

d = Internal Diameter

t = Wall Thickness of The Cylinder

Figure 2:  Cross-Section of A Thin Cylinder with Hemispherical Ends

Figure 3:  Cross-Section of A Thin Cylinder

Figure 4:  Yield Strength (0.2% Offset) Of  6061-T651 After Prior Exposure at Different Heating Rates

.

Figure 5:  Joint Efficiency of Different Weld Types

Table 5:  Yield Strength Values for Common Steels

|  | Yield strength |  |
| --- | --- | --- |
| Material | MPa | Ksi(psi) |
| Low alloy steel |  |  |
| AISI 4140, Normalized at 870 °C (1600 °F) | 655 | 95 (95000) |
| AISI 4140, Annealed at 815 °C (1500 °F) | 414 | 60 (60000) |
| AISI 4140, Water quenched from 845 °C (1550 °F) and tempered at 540 °C (1000 °F) | 986 | 143 (143000) |
| AISI 4340, Normalized at 870 °C (1600 °F) | 862 | 125 (125000) |

325 MPa = 6 MPa * 0.078794423  meter2t = 0.00072733 meter, and factor of safety 1.5  = 0.00109 meter (≅1 mm).  In the case where welding mechanism is used to join an aluminum sheet, we use below equation. Joint efficiency varies with weld type. We choose the single full Not inspected/ single welded butt joint without use of backing strip and the joint efficiency 0.6

σH = pd2tŋ                                             (7)

325 MPa = 6 MPa * 0.0787944232*t*0.6 , t = 0.00121221667 meter, With factor of safety 1.5 = 0.001818325 meter ( ≅2).

R-5: Vehicle Operation Altitude Domain,  (0-20) Km

R-51 System should consider atmosphere temperature up to 20 km

As seen from below table the continuously decreases.

F=P*A                                             (8)

Were

F=force exert on the oxidizer

P= generated pressure (ullage)

A= service area of oxidizer (tank diameter)

A=π*d24=3.14*(0.078794423) 24=0.004874 m2

F=5,100,000 pascal*0.004874m2= 24,857.4 Newton

R-52 System consider air density up to 20 km

As seen from below table the air density due to altitude change not much, so it is reasonable and simplicity to assume the constant air density through vehicle fly time. System no need to include propellant management device (PMD) since not have the probability of microgravity environments.

Table 6:  Source NRLMSIS Atmosphere Model

| Year | Month | Day | H(Km) | Lat | Lon | Air(Gm/Cm3) | T(K) | T(OC) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2026 | 5 | 22 | 0 | 12.1 | 41.2 | 1.168E-03 | 299 | 25.88 |
| 2026 | 5 | 22 | 1 | 12.1 | 41.2 | 1.056E-03 | 294.7 | 21.55 |
| 2026 | 5 | 22 | 2 | 12.1 | 41.2 | 9.575E-04 | 289.3 | 16.15 |
| 2026 | 5 | 22 | 3 | 12.1 | 41.2 | 8.682E-04 | 283.3 | 10.15 |
| 2026 | 5 | 22 | 4 | 12.1 | 41.2 | 7.858E-04 | 277.1 | 3.95 |
| 2026 | 5 | 22 | 5 | 12.1 | 41.2 | 7.091E-04 | 271.3 | -1.85 |
| 2026 | 5 | 22 | 6 | 12.1 | 41.2 | 6.374E-04 | 265.9 | -7.25 |
| 2026 | 5 | 22 | 7 | 12.1 | 41.2 | 5.717E-04 | 260.5 | -12.65 |
| 2026 | 5 | 22 | 8 | 12.1 | 41.2 | 5.124E-04 | 254.7 | -18.45 |
| 2026 | 5 | 22 | 9 | 12.1 | 41.2 | 4.596E-04 | 248.1 | -25.05 |
| 2026 | 5 | 22 | 10 | 12.1 | 41.2 | 4.128E-04 | 240.4 | -32.75 |
| 2026 | 5 | 22 | 11 | 12.1 | 41.2 | 3.710E-04 | 231.5 | -41.65 |
| 2026 | 5 | 22 | 12 | 12.1 | 41.2 | 3.325E-04 | 222.5 | -50.65 |
| 2026 | 5 | 22 | 13 | 12.1 | 41.2 | 2.957E-04 | 214.1 | -59.05 |
| 2026 | 5 | 22 | 14 | 12.1 | 41.2 | 2.603E-04 | 207.0 | -66.15 |
| 2026 | 5 | 22 | 15 | 12.1 | 41.2 | 2.263E-04 | 201.7 | -71.45 |
| 2026 | 5 | 22 | 16 | 12.1 | 41.2 | 1.941E-04 | 198.4 | -74.75 |
| 2026 | 5 | 22 | 17 | 12.1 | 41.2 | 1.648E-04 | 196.9 | -76.25 |
| 2026 | 5 | 22 | 18 | 12.1 | 41.2 | 1.387E-04 | 196.8 | -76.35 |
| 2026 | 5 | 22 | 19 | 12.1 | 41.2 | 1.161E-04 | 198.0 | -75.15 |
| 2026 | 5 | 22 | 20 | 12.1 | 41.2 | 9.688E-05 | 200.2 | -72.95 |

R-53 System consider gravitational force up to 20 km

As seen from below table the gravitational force  due to altitude change not much, so it is reasonable and simplicity to assume the constant gravitational force through vehicle fly time.

Table 7:  Gravitational Acceleration with Altitude (0-20 Km)

| Standard Gravity | Mean Radius | Altitude (m) | Sum-1 | Sum-2 | Sum-3 | Altitude Gravity |
| --- | --- | --- | --- | --- | --- | --- |
| 9.80665 | 6371000 | 0 | 6371000 | 1 | 1 | 9.80665 |
| 9.80665 | 6371000 | 1000 | 6372000 | 0.999843063 | 0.999686151 | 9.803572197 |
| 9.80665 | 6371000 | 2000 | 6373000 | 0.999686176 | 0.999372451 | 9.800495843 |
| 9.80665 | 6371000 | 3000 | 6374000 | 0.999529338 | 0.999058897 | 9.797420936 |
| 9.80665 | 6371000 | 4000 | 6375000 | 0.999372549 | 0.998745492 | 9.794347477 |
| 9.80665 | 6371000 | 5000 | 6376000 | 0.999215809 | 0.998432234 | 9.791275463 |
| 9.80665 | 6371000 | 6000 | 6377000 | 0.999059119 | 0.998119123 | 9.788204894 |
| 9.80665 | 6371000 | 7000 | 6378000 | 0.998902477 | 0.997806159 | 9.78513577 |
| 9.80665 | 6371000 | 8000 | 6379000 | 0.998745885 | 0.997493343 | 9.782068089 |
| 9.80665 | 6371000 | 9000 | 6380000 | 0.998589342 | 0.997180673 | 9.77900185 |
| 9.80665 | 6371000 | 10000 | 6381000 | 0.998432848 | 0.996868151 | 9.775937053 |
| 9.80665 | 6371000 | 11000 | 6382000 | 0.998276402 | 0.996555776 | 9.772873696 |
| 9.80665 | 6371000 | 12000 | 6383000 | 0.998120006 | 0.996243547 | 9.769811779 |
| 9.80665 | 6371000 | 13000 | 6384000 | 0.997963659 | 0.995931465 | 9.766751301 |
| 9.80665 | 6371000 | 14000 | 6385000 | 0.997807361 | 0.99561953 | 9.763692261 |
| 9.80665 | 6371000 | 15000 | 6386000 | 0.997651112 | 0.995307741 | 9.760634657 |
| 9.80665 | 6371000 | 16000 | 6387000 | 0.997494912 | 0.994996099 | 9.75757849 |
| 9.80665 | 6371000 | 17000 | 6388000 | 0.99733876 | 0.994684603 | 9.754523758 |
| 9.80665 | 6371000 | 18000 | 6389000 | 0.997182658 | 0.994373253 | 9.75147046 |
| 9.80665 | 6371000 | 19000 | 6390000 | 0.997026604 | 0.994062049 | 9.748418595 |
| 9.80665 | 6371000 | 20000 | 6391000 | 0.996870599 | 0.993750992 | 9.745368163 |

F=M*g………………………………...(9)

Were:

F = Force exerted in oxidizer due to gravity

M = Mass of propellant

g = acceleration gravity

Table 8:  Force Applied on Oxidizer (Propellant Due to Gravitational Force)

| No. | Propellant Consume Per Second | Current Propellant Mass | Gravitational | Force Due to Gravity |
| --- | --- | --- | --- | --- |
| 1 | 0.1824 | 3.648 | 9.8 | 35.7504 |
| 2 | 0.1824 | 3.4656 | 9.8 | 33.96288 |
| 3 | 0.1824 | 3.2832 | 9.8 | 32.17536 |
| 4 | 0.1824 | 3.1008 | 9.8 | 30.38784 |
| 5 | 0.1824 | 2.9184 | 9.8 | 28.60032 |
| 6 | 0.1824 | 2.736 | 9.8 | 26.8128 |
| 7 | 0.1824 | 2.5536 | 9.8 | 25.02528 |
| 8 | 0.1824 | 2.3712 | 9.8 | 23.23776 |
| 9 | 0.1824 | 2.1888 | 9.8 | 21.45024 |
| 10 | 0.1824 | 2.0064 | 9.8 | 19.66272 |
| 11 | 0.1824 | 1.824 | 9.8 | 17.8752 |
| 12 | 0.1824 | 1.6416 | 9.8 | 16.08768 |
| 13 | 0.1824 | 1.4592 | 9.8 | 14.30016 |
| 14 | 0.1824 | 1.2768 | 9.8 | 12.51264 |
| 15 | 0.1824 | 1.0944 | 9.8 | 10.72512 |
| 16 | 0.1824 | 0.912 | 9.8 | 8.9376 |
| 17 | 0.1824 | 0.7296 | 9.8 | 7.15008 |
| 18 | 0.1824 | 0.5472 | 9.8 | 5.36256 |
| 19 | 0.1824 | 0.3648 | 9.8 | 3.57504 |
| 20 | 0.1824 | 0.1824 | 9.8 | 1.78752 |

Very small and Negligible compare to force ullage pressure, but total force push propellant down to feeding pipe = ullage pressure + gravitational ≅  24,857.4 Newton

R-6: Propellant Feeding Pipe Dimension  1 Inch Diameter And  0.1824 Kg/Sec Propellant Flow Rate.

R-7: Vertical Takeoff

Due to rocket alignment the vehicle can add additional advantage due to Gravity, and  ullage pressure.

R-8: Easy For Manufacture

R-81:  Applied pressure in baffles due to oxidizer slosh

N:B assumed the vehicle fly 900 means the possible slosh movement in vertical axis to the tank.

R-82: Baffles design (Radial baffles)

R-821 Radial baffles with 6061 aluminum sheets (6061-T651) material and 2 mm  thickness.

R-822 baffles dimensions, radial baffles size (0.1-0.25) R

R-9: Powered Vehicle Flight Time (20 sec)

With the specified chamber pressure of 30 bar and the expected pressure losses in the feed system, the oxidizer tank pressure must remain sufficiently higher than 30 bar at the corresponding operating temperature. In the case of N₂O, the ambient temperature decreases significantly during flight. However, the oxidizer (N₂O) tank temperature changes much more slowly than the external air temperature because of the tank’s thermal mass and the limited rate of heat transfer. [9]

R-10: vehicle deceleration deceleration up to 500 m/sec

- Listed sample deceleration numbers  at 250, 375, and 500 m/sec respectively for different mass (oxidizer) amount.

Table 9:  Generated Force Due to Vehicle Deceleration

| No. | Propellant Consume Per Second | Current Propellant Mass | Acceleration 1 | Acceleration 2 | Acceleration 3 | Inertia F1 | Inertia F2 | Inertia F3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.1824 | 3.648 | 343 | 686 | 1029 | 1251.264 | 2502.528 | 3753.792 |
| 2 | 0.1824 | 3.4656 | 343 | 686 | 1029 | 1188.7008 | 2377.4016 | 3566.1024 |
| 3 | 0.1824 | 3.2832 | 343 | 686 | 1029 | 1126.1376 | 2252.2752 | 3378.4128 |
| 4 | 0.1824 | 3.1008 | 343 | 686 | 1029 | 1063.5744 | 2127.1488 | 3190.7232 |
| 5 | 0.1824 | 2.9184 | 343 | 686 | 1029 | 1001.0112 | 2002.0224 | 3003.0336 |
| 6 | 0.1824 | 2.736 | 343 | 686 | 1029 | 938.448 | 1876.896 | 2815.344 |
| 7 | 0.1824 | 2.5536 | 343 | 686 | 1029 | 875.8848 | 1751.7696 | 2627.6544 |
| 8 | 0.1824 | 2.3712 | 343 | 686 | 1029 | 813.3216 | 1626.6432 | 2439.9648 |
| 9 | 0.1824 | 2.1888 | 343 | 686 | 1029 | 750.7584 | 1501.5168 | 2252.2752 |
| 10 | 0.1824 | 2.0064 | 343 | 686 | 1029 | 688.1952 | 1376.3904 | 2064.5856 |
| 11 | 0.1824 | 1.824 | 343 | 686 | 1029 | 625.632 | 1251.264 | 1876.896 |
| 12 | 0.1824 | 1.6416 | 343 | 686 | 1029 | 563.0688 | 1126.1376 | 1689.2064 |
| 13 | 0.1824 | 1.4592 | 343 | 686 | 1029 | 500.5056 | 1001.0112 | 1501.5168 |
| 14 | 0.1824 | 1.2768 | 343 | 686 | 1029 | 437.9424 | 875.8848 | 1313.8272 |
| 15 | 0.1824 | 1.0944 | 343 | 686 | 1029 | 375.3792 | 750.7584 | 1126.1376 |
| 16 | 0.1824 | 0.912 | 343 | 686 | 1029 | 312.816 | 625.632 | 938.448 |
| 17 | 0.1824 | 0.7296 | 343 | 686 | 1029 | 250.2528 | 500.5056 | 750.7584 |
| 18 | 0.1824 | 0.5472 | 343 | 686 | 1029 | 187.6896 | 375.3792 | 563.0688 |
| 19 | 0.1824 | 0.3648 | 343 | 686 | 1029 | 125.1264 | 250.2528 | 375.3792 |
| 20 | 0.1824 | 0.1824 | 343 | 686 | 1029 | 62.5632 | 125.1264 | 187.6896 |

- RESULT AND DISCUSSION

Baffles Are Not Required in The Oxidizer Tank. As Shown in The Above Analysis, The Maximum Force Generated by Vehicle Deceleration Is Approximately 3,754 N, Which Is Significantly Lower Than the Opposing Force of Approximately 24,857 N Produced by The Ullage Pressure Acting on The Oxidizer. Even Under Increased Vehicle Deceleration Conditions, The Oxidizer Mass Considered in This Study Remains Insufficient to Generate Significant Sloshing Loads. Therefore, Throughout the Entire Flight Profile and Operating Altitude Range, The Risk of Oxidizer Slosh Is Negligible, And the Inclusion of Internal Baffles Is Not Justified.

- REFERENCE

[1] Benson, T. (n.d.). Rocket Stability. NASA Glenn Research Center.

[2] G. P. Sutton and O. Biblarz, Rocket Propulsion Elements, 9th ed. Hoboken, NJ, USA: Wiley, 2017.

[3] F. T. Dodge, The New Dynamic Behavior of Liquids in Moving Containers. Southwest Research Institute, 2000.

[4] Chiaverini, M. J., & Kuo, K. K. (Eds.). (2007). Fundamentals of Hybrid Rocket Combustion and Propulsion. AIAA.

[5] D. W. Green and R. H. Perry, Perry’s Chemical Engineers’ Handbook, 9th ed. New York, NY, USA: McGraw-Hill, 2019.

[6] D. K. Huzel and D. H. Huang, Modern Engineering for Design of Liquid-Propellant Rocket Engines. Reston, VA, USA: American Institute of Aeronautics and Astronautics (AIAA), 1992.

[7] M. D. Griffin and J. R. French, Space Vehicle Design. Reston, VA, USA: AIAA Education Series.

[8] Green, D. W., & Southard, M. Z. (2019). Perry's Chemical Engineers' Handbook (9th ed.). McGraw-Hill Education.

[9] F. P. Incropera, D. P. DeWitt, T. L. Bergman, and A. S. Lavine, Fundamentals of Heat and Mass Transfer, 7th ed. Hoboken, NJ, USA: John Wiley & Sons, 2011.