A QBD APPROACH BASED RP-HPLC METHOD DEVELOPMENT AND VALIDATION FOR SIMULTANEOUS ESTIMATION OF DAPAGLIFLOZIN PROPANEDIOL MONOHYDRATE AND SACUBITRIL-VALSARTAN SODIUM COMPLEX SALT IN ITS SYNTHETIC MIXTURE

Hinal Patell, Dr. Avani Doshi 2

1, PG Student, K. B. Raval College of Pharmacy, Kasturinagar, Shertha, Gandhi agar, Gujarat

2 Professor, Dept. of Quality Assurance, K. B. Raval College of Pharmacy, Kasturinagar, Shertha, Gandhinagar, Gujarat

ABSTARCT: A robust, sensitive, and reproducible reverse-phase high-performance liquid chromatography (RP-HPLC) method was developed and validated using a Quality by Design (QbD) approach for the simultaneous estimation of dapagliflozin propanediol monohydrate and sacubitril–valsartan sodium complex in a synthetic mixture. Critical method parameters were systematically optimized through experimental design to ensure optimal separation, peak symmetry, and resolution. The chromatographic separation was achieved using a suitable C18 column with an optimized mobile phase composition under isocratic conditions. The method demonstrated excellent linearity, precision, accuracy, and specificity within the tested concentration ranges. Validation was performed in accordance with regulatory guidelines, confirming the reliability of the method for routine analysis. The QbD framework enabled a thorough understanding of method variability and robustness, ensuring consistent performance. This developed method can be effectively applied for quality control and simultaneous quantification of these drugs in combined pharmaceutical formulations.

KEYWORDS: Quality by Design (QbD), RP-HPLC, Dapagliflozin propanediol monohydrate, Sacubitril–Valsartan sodium complex, Method development, Method validation, Simultaneous estimation, Analytical optimization, Robustness, Pharmaceutical analysis

- INTRODUCTION:

The increasing use of combination therapies in the management of cardiovascular and metabolic disorders has created a need for reliable analytical methods for simultaneous drug estimation. Dapagliflozin propanediol monohydrate, a sodium–glucose co-transporter 2 (SGLT2) inhibitor, and sacubitril–valsartan sodium complex, an angiotensin receptor–neprilysin inhibitor (ARNI), are widely prescribed for the treatment of heart failure and related conditions. Accurate quantification of these drugs in combined formulations is essential to ensure quality, safety, and therapeutic efficacy.

Reverse-phase high-performance liquid chromatography (RP-HPLC) is commonly employed for such analyses due to its precision and sensitivity. Incorporating a Quality by Design (QbD) approach enhances method development by systematically evaluating critical factors affecting performance. This study focuses on developing a robust and validated RP-HPLC method using QbD principles for the simultaneous estimation of these drugs in a synthetic mixture.

Figure1: Structure of Dapagliflozin Propanediol Monohydrate      Figure 2: Stucture of Sacubitril-Valsartan

2. MATERIALS AND METHODS

- Materials: 
Dapagliflozin propanediol monohydrate (API), Sacubitril–valsartan sodium complex salt (API), HPLC grade methanol, acetonitrile, water, analytical gradents.

- Analytical grade reagents Buffer Preparation: 
A 0.02 M Potassium Dihydrogen Phosphate buffer was prepared by dissolving 2.72 g of potassium dihydrogen orthophosphate in 1 L of Milli-Q water using sonication. The pH was adjusted to 2.5 with diluted orthophosphoric acid.

- Instrumentation:
Chromatographic analysis was carried out using a Shimadzu HPLC system (LC-2010 CHT) equipped with a 100 µL fixed loop injector and LC Solution software. Spectral analysis was performed using a Shimadzu UV-1800 double-beam UV–Visible spectrophotometer with UV Probe software. Additional instruments included a Sartorius analytical balance, Lab India digital pH meter, ultrasonic bath (Athena Technology), hot air oven (Patel Scientific), and micropipettes (Eppendorf).

- Chromatographic Conditions:
Separation was achieved on a Cosmosil C18 column (250 mm × 4.6 mm, 5 µm) using a mobile phase of Buffer: ACN (40:60 % v/v) at a flow rate of 1.0 mL/min. Detection was performed at 233 nm, the injection volume was 30 µL, and the column temperature was maintained at 25 °C.

- Preparation of Solutions
Dapagliflozin Stock solution (50 µg/ml): 5 mg working standard of dapagliflozin propanediol monohydrate was transferred into 100 ml flask. 30 ml methanol added. Sonicated for 5 minutes. Volume made with methanol. Further 5 mL diluted to 50 mL using methanol.

Sacubitril-Valsartan stock solution (500 µg/ml):50 mg working standard of sacubitril-valsartan sodium complex salt was taken in 100 ml volumetric flask. Approximately 50 ml methanol added. Sonicated for 5 minutes. Volume made with methanol.

Mixed standard solution (5 µg/ml Dapagliflozin + 50 µg/ml Sacubitril-Valsartan): 1 ml dapagliflozin stock solution + 1 ml sacubitril-valsartan stock were taken into 10 ml volumetric flask. Volume made with diluent.

- SAMPLE PREPARATION:

Sample Stock: Synthetic mixture 200 mg (equivalent to 5 mg dapagliflozin +50 mg Sacubitril-valsartan) was weighed on butter paper. Powder was then transferred to 50 mL volumetric flask. To this 30 mL methanol was added and active contents were extracted in sonication for 15 minutes. The flask was allowed to cool down on room temperature and then the volume was made with methanol. The solution was then filtered through 0.45 µ Whatman PVDF syringe filter.

Sample (5 µg/ml Dapagliflozin + 50 µg/ml Sacubitril-valsartan):1 mL of clear filtrate was then further diluted to 10 ml with diluent and used as a sample solution.

3. RESULTS AND DISCUSSION:

  - IR identification and wavelength selection

The individual standard drugs Dapagliflozin propanediol monohydrate &Sacubitril-valsartan were mixed with KBr and KBr pallets were prepared. These KBr pallets of drugs were used for FTIR analysis. And then FTIR spectra were interpreted and results were co-related with M.P., UV spectra and solubility to confirm identity of individual drugs. Wavelength was selected from the overlay spectra of above solutions.

Figure 3:IR spectrum of Dapagliflozin (API)	               Figure 4:IR Spectrum of Sacubitril-Valsartan (API)

Table: 3.1 IR spectrum of Dapagliflozin

| SR. NO. | Functional group | Observed value | Standard value |
| --- | --- | --- | --- |
| 1 | O–H stretching vibration | 3352.51 | 3500–3100 |
| 2 | Aliphatic C–H stretching vibration | 2980.24, 2930.54, 2863.58 | 2900–2700 |
| 3 | C–O–C stretching vibration | 1138.90 | 1150–1085 |
| 4 | C–Cl stretching vibration | 841.46 | 850–550 |

Table:3.2 IR spectrum of Sacubitril-Valsartan

| Sr. No. | Functional group | Observed value | Standard value |
| --- | --- | --- | --- |
| 1 | O–H stretching vibration | 3057.01 | 3600–3000 |
| 2 | Aliphatic C–H stretching vibration | 2954.85 | 2960–2850 |
| 3 | Carbonyl (C=O) stretching vibration | 1710.71 | 1750–1600 |
| 4 | Aromatic C=C stretching vibration | 1636.53 | 1660–1450 |

Figure 5: Determination of wavelength maxima(233nm)

3.2 METHOD DEVELOPMENT

Fig 6: Final HPLC Chromatogram of 5 µg/mL Dapagliflozin + 50 µg/mL Sacubitril–Valsartan

## QbD APPROACH TO RP-HPLC METHOD DEVELOPMENT

3.3.1 Analytical Target Profile (ATP)

The Analytical Target Profile (ATP) for the present study was established to develop a robust, accurate, and reproducible RP-HPLC method for the simultaneous quantitative estimation of Dapagliflozin propanediol monohydrate and Sacubitril–Valsartan sodium complex salt in synthetic mixture

### 3.3.2 Selection of Critical Analytical Attributes (CAA)

The Critical Analytical Attributes (CAA) identified for this method included:

Retention time of Dapagliflozin and Sacubitril–Valsartan

Resolution between adjacent peaks

Theoretical plate count

Tailing factor

These attributes were selected because they directly reflect chromatographic efficiency, peak symmetry, selectivity, and reproducibility, which are essential for the accurate and precise quantification of both analytes in a combined formulation.

### 3.3.3 Selection of Critical Method Variables (CMV)

Based on preliminary chromatographic trials and a review of relevant literature, the following parameters were identified as Critical Method Variables (CMVs):

Mobile phase composition

Percentage of organic modifier (acetonitrile)

Flow rate

Buffer pH

### 3.3.4 Risk Identification

Risk identification was carried out using an Ishikawa (fishbone) diagram to systematically evaluate potential sources of variability that could affect the critical analytical attributes. The impact and criticality of each input factor were assessed based on prior literature, experimental knowledge, and structured brainstormi

3.3.5 ANOVA Interpretation for Dapagliflozin and Sacubitril and Valsartan

| Source | Sum of Squares | df | Mean Square | F-value | p-value |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model | 12.58 | 9 | 1.40 | 1963.3 | < 0.0001 | significant |
| A-MP Ratio | 0.0014 | 1 | 0.0014 | 1.93 | 0.2076 |  |
| B-Column Temperature | 7.254E-07 | 1 | 7.254E-07 | 0.0010 | 0.9754 |  |
| C-Flow Rate | 0.3269 | 1 | 0.3269 | 459.30 | < 0.0001 |  |
| AB | 0.0011 | 1 | 0.0011 | 1.52 | 0.2575 |  |
| AC | 0.1791 | 1 | 0.1791 | 251.65 | < 0.0001 |  |
| BC | 0.0005 | 1 | 0.0005 | 0.6971 | 0.4313 |  |
| A² | 0.0336 | 1 | 0.0336 | 47.24 | 0.0002 |  |
| B² | 0.0005 | 1 | 0.0005 | 0.7356 | 0.4195 |  |
| C² | 0.2169 | 1 | 0.2169 | 304.75 | < 0.0001 |  |
| Residual | 0.0050 | 7 | 0.0007 |  |  |  |
| Lack of Fit | 0.0050 | 5 | 0.0010 | 80.39 | 0.0123 | significant |
| Pure Error | 0.0000 | 2 | 0.0000 |  |  |  |
| Cor Total | 12.58 | 16 |  |  |  |  |

| Source | Sum of Squares | df | Mean Square | F-value | p-value |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model | 0.0695 | 3 | 0.0232 | 5.15 | 0.0145 | significant |
| A-MP Ratio | 0.0689 | 1 | 0.0689 | 15.31 | 0.0018 |  |
| B-Column Temperature | 0.0002 | 1 | 0.0002 | 0.0555 | 0.8174 |  |
| C-Flow Rate | 0.0004 | 1 | 0.0004 | 0.0800 | 0.7818 |  |
| Residual | 0.0585 | 13 | 0.0045 |  |  |  |
| Lack of Fit | 0.0576 | 11 | 0.0052 | 12.09 | 0.0788 | not significant |
| Pure Error | 0.0009 | 2 | 0.0004 |  |  |  |
| Cor Total | 0.1280 | 16 |  |  |  |  |

Table 3.3 ANOVA Table for reduced linear model of Rt of Dapagliflozin                                        Table 3.4: ANOVA Table for reduced linear model of Tf  of Dapagliflozin

| Source | Sum of Squares | df | Mean Square | F-value | p-value |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model | 92.35 | 9 | 10.26 | 707.73 | < 0.0001 | significant |
| A-MP Ratio | 0.0193 | 1 | 0.0193 | 1.33 | 0.2870 |  |
| B-Column Temperature | 0.0010 | 1 | 0.0010 | 0.0694 | 0.7998 |  |
| C-Flow Rate | 0.4766 | 1 | 0.4766 | 32.87 | 0.0007 |  |
| AB | 0.0102 | 1 | 0.0102 | 0.7003 | 0.4303 |  |
| AC | 2.71 | 1 | 2.71 | 186.98 | < 0.0001 |  |
| BC | 0.0020 | 1 | 0.0020 | 0.1391 | 0.7203 |  |
| A² | 0.5543 | 1 | 0.5543 | 38.23 | 0.0005 |  |
| B² | 0.0022 | 1 | 0.0022 | 0.1519 | 0.7083 |  |
| C² | 1.09 | 1 | 1.09 | 75.30 | < 0.0001 |  |
| Residual | 0.1015 | 7 | 0.0145 |  |  |  |
| Lack of Fit | 0.1011 | 5 | 0.0202 | 91.73 | 0.0108 | significant |
| Pure Error | 0.0004 | 2 | 0.0002 |  |  |  |
| Cor Total | 92.45 | 16 |  |  |  |  |

| Source | Sum of Squares | df | Mean Square | F-value | p-value |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model | 1.284E+07 | 3 | 4.281E+06 | 8.16 | 0.0026 | significant |
| A-MP Ratio | 2.778E+06 | 1 | 2.778E+06 | 5.29 | 0.0386 |  |
| B-Column Temperature | 4.376E+05 | 1 | 4.376E+05 | 0.8338 | 0.3778 |  |
| C-Flow Rate | 9.628E+06 | 1 | 9.628E+06 | 18.34 | 0.0009 |  |
| Residual | 6.823E+06 | 13 | 5.249E+05 |  |  |  |
| Lack of Fit | 6.462E+06 | 11 | 5.875E+05 | 3.26 | 0.2583 | not significant |
| Pure Error | 3.607E+05 | 2 | 1.804E+05 |  |  |  |
| Cor Total | 1.967E+07 | 16 |  |  |  |  |

Table 3.5: ANOVA Table for reduced linear model of Rt of Sacubitril                                   Table 3.6: ANOVA Table for reduced linear model of Theoretical Plate Count of Dapagliflozin

| Source | Sum of Squares | df | Mean Square | F-value | p-value |  |  | Source | Sum of Squares | df | Mean Square | F-value | p-value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | 0.0350 | 3 | 0.0117 | 15.21 | 0.0002 | significant |  | Model | 3.231E+07 | 3 | 1.077E+07 | 6.28 | 0.0072 | significant |
| A-MP Ratio | 0.0319 | 1 | 0.0319 | 41.55 | < 0.0001 |  |  | A-MP Ratio | 7.756E+06 | 1 | 7.756E+06 | 4.52 | 0.0532 |  |
| B-Column Temperature | 0.0017 | 1 | 0.0017 | 2.20 | 0.1617 |  |  | B-Column Temperature | 1.603E+05 | 1 | 1.603E+05 | 0.0935 | 0.7647 |  |
| C-Flow Rate | 0.0014 | 1 | 0.0014 | 1.87 | 0.1949 |  |  | C-Flow Rate | 2.439E+07 | 1 | 2.439E+07 | 14.22 | 0.0023 |  |
| Residual | 0.0100 | 13 | 0.0008 |  |  |  |  | Residual | 2.230E+07 | 13 | 1.715E+06 |  |  |  |
| Lack of Fit | 0.0099 | 11 | 0.0009 | 33.06 | 0.0297 | significant |  | Lack of Fit | 2.175E+07 | 11 | 1.977E+06 | 7.22 | 0.1278 | not significant |
| Pure Error | 0.0001 | 2 | 0.0000 |  |  |  |  | Pure Error | 5.477E+05 | 2 | 2.738E+05 |  |  |  |
| Cor Total | 0.0450 | 16 |  |  |  |  |  | Cor Total | 5.460E+07 | 16 |  |  |  |  |

Table 3.7 ANOVA Table for reduced linear model of Tailing Factor of Sacubitril                   Table 3.8 ANOVA Table for reduced linear model of Theoretical Plate Count of Sacubitril

ANOVA Interpretation for Valsartan

| Source | Sum of Squares | df | Mean Square | F-value | p-value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Model | 49.89 | 9 | 5.54 | 570.88 | < 0.0001 | significant |  | Source | Sum of Squares | df | Mean Square | F-value | p-value |  |
| A-MP Ratio | 0.0043 | 1 | 0.0043 | 0.4395 | 0.5286 |  |  | Model | 3.292E+07 | 3 | 1.097E+07 | 27.60 | < 0.0001 | significant |
| B-Column Temperature | 0.0004 | 1 | 0.0004 | 0.0372 | 0.8525 |  |  | A-MP Ratio | 8.477E+06 | 1 | 8.477E+06 | 21.33 | 0.0005 |  |
| C-Flow Rate | 0.4796 | 1 | 0.4796 | 49.39 | 0.0002 |  |  | B-Column Temperature | 1.308E+06 | 1 | 1.308E+06 | 3.29 | 0.0928 |  |
| AB | 0.0020 | 1 | 0.0020 | 0.2044 | 0.6649 |  |  | C-Flow Rate | 2.313E+07 | 1 | 2.313E+07 | 58.19 | < 0.0001 |  |
| AC | 1.26 | 1 | 1.26 | 129.85 | < 0.0001 |  |  | Residual | 5.168E+06 | 13 | 3.975E+05 |  |  |  |
| BC | 0.0031 | 1 | 0.0031 | 0.3214 | 0.5885 |  |  | Lack of Fit | 4.655E+06 | 11 | 4.232E+05 | 1.65 | 0.4371 | not significant |
| A² | 0.2264 | 1 | 0.2264 | 23.31 | 0.0019 |  |  | Pure Error | 5.127E+05 | 2 | 2.564E+05 |  |  |  |
| B² | 0.0045 | 1 | 0.0045 | 0.4598 | 0.5195 |  |  | Cor Total | 3.808E+07 | 16 |  |  |  |  |
| C² | 0.7083 | 1 | 0.7083 | 72.95 | < 0.0001 |  |  |
| Residual | 0.0680 | 7 | 0.0097 |  |  |  |  |
| Lack of Fit | 0.0677 | 5 | 0.0135 | 111.95 | 0.0089 | significant |  |
| Pure Error | 0.0002 | 2 | 0.0001 |  |  |  |  |
| Cor Total | 49.96 | 16 |  |  |  |  |  |

Table 3.9 ANOVA Table for reduced linear model of Rt of Valsartan                                Table 3.10 ANOVA Table for reduced linear model of Theoretical Plate Count of Valsartan

### 3.3.6 Fit Summaries of selected models

| Source | Sequential p-value | Lack of Fit p-value | Adjusted R² | Predicted R² |  |
| --- | --- | --- | --- | --- | --- |
| Linear | < 0.0001 | 0.0002 | 0.9332 | 0.8952 |  |
| 2FI | 0.3595 | 0.0002 | 0.9361 | 0.7399 |  |
| Quadratic | < 0.0001 | 0.0123 | 0.9991 | 0.9963 | Suggested |
| Cubic | 0.1345 | 0.0164 | 0.9997 | 0.9077 | Aliased |

| Source | Sequential p-value | Lack of Fit p-value | Adjusted R² | Predicted R² |  |
| --- | --- | --- | --- | --- | --- |
| Linear | 0.0145 | 0.0788 | 0.4374 | 0.0822 | Suggested |
| 2FI | 0.1012 | 0.1032 | 0.5968 | -0.1727 |  |
| Quadratic | 0.8885 | 0.0715 | 0.4709 | -1.0593 |  |
| Cubic | 0.0137 | 0.7306 | 0.9611 | 0.1492 | Aliased |

Table 3.11 Response 1: Rt of Dapagliflozin                                                          Table 3.12 Response 2: Tf of Dapagliflozin

| Source | Sequential p-value | Lack of Fit p-value | Adjusted R² | Predicted R² |  |
| --- | --- | --- | --- | --- | --- |
| Linear | 0.0026 | 0.2583 | 0.5730 | 0.3517 | Suggested |
| 2FI | 0.3795 | 0.2550 | 0.5864 | -0.3594 |  |
| Quadratic | 0.6205 | 0.2100 | 0.5341 | -1.1270 |  |
| Cubic | 0.0993 | 0.4848 | 0.8668 | -9.5395 | Aliased |

| Source | Sequential p-value | Lack of Fit p-value | Adjusted R² | Predicted R² |  |
| --- | --- | --- | --- | --- | --- |
| Linear | < 0.0001 | 0.0004 | 0.9141 | 0.8574 |  |
| 2FI | 0.1255 | 0.0005 | 0.9354 | 0.7300 |  |
| Quadratic | < 0.0001 | 0.0108 | 0.9975 | 0.9911 | Suggested |
| Cubic | 0.0057 | 0.1340 | 0.9999 | 0.9773 | Aliased |

Table 3.13 Response 3: Theoretical Plates of Dapagliflozin                                  Table 3.14 Response 1: Retention Time of Sacubitril

| Source | Sequential p-value | Lack of Fit p-value | Adjusted R² | Predicted R² |  |
| --- | --- | --- | --- | --- | --- |
| Linear | 0.0002 | 0.0297 | 0.7271 | 0.6181 | Suggested |
| 2FI | 0.6397 | 0.0255 | 0.6980 | 0.2849 |  |
| Quadratic | 0.9571 | 0.0167 | 0.5864 | -0.4395 |  |
| Cubic | 0.0303 | 0.0641 | 0.9479 | -12.5751 | Aliased |

| Source | Sequential p-value | Lack of Fit p-value | Adjusted R² | Predicted R² |  |
| --- | --- | --- | --- | --- | --- |
| Linear | 0.0072 | 0.1278 | 0.4975 | 0.3443 | Suggested |
| 2FI | 0.9810 | 0.0963 | 0.3577 | -0.4922 |  |
| Quadratic | 0.4448 | 0.0870 | 0.3589 | -1.1280 |  |
| Cubic | 0.0923 | 0.1677 | 0.8259 | -34.8922 | Aliased |

Table 3.15 Response 2: Tailing Factorof Sacubitril                                      Table 3.16 Response 3: Theoretical Plates of Sacubitril

| Source | Sequential p-value | Lack of Fit p-value | Adjusted R² | Predicted R² |  |
| --- | --- | --- | --- | --- | --- |
| Linear | < 0.0001 | 0.0004 | 0.9194 | 0.8682 |  |
| 2FI | 0.1633 | 0.0005 | 0.9358 | 0.7387 |  |
| Quadratic | < 0.0001 | 0.0089 | 0.9969 | 0.9890 | Suggested |
| Cubic | 0.0070 | 0.0934 | 0.9999 | 0.9645 | Aliased |

| Source | Sequential p-value | Lack of Fit p-value | Adjusted R² | Predicted R² |  |
| --- | --- | --- | --- | --- | --- |
| Linear | 0.0002 | 0.0606 | 0.7092 | 0.5800 | Suggested |
| 2FI | 0.4919 | 0.0557 | 0.6997 | 0.2625 |  |
| Quadratic | 0.9283 | 0.0374 | 0.5964 | -0.4646 |  |
| Cubic | 0.0226 | 0.1887 | 0.9583 | -7.1582 | Aliased |

Table 3.17 Response 1: Retention Time of Valsartan                                   Table 3.18 Response 2: Tailing Factor of Valsartan

| Source | Sequential p-value | Lack of Fit p-value | Adjusted R² | Predicted R² |  |
| --- | --- | --- | --- | --- | --- |
| Linear | < 0.0001 | 0.4371 | 0.8330 | 0.7804 | Suggested |
| 2FI | 0.9174 | 0.3559 | 0.7931 | 0.4884 |  |
| Quadratic | 0.4142 | 0.3393 | 0.7985 | 0.3653 |  |
| Cubic | 0.1405 | 0.8407 | 0.9263 | 0.4236 | Aliased |

Table 3.19 Response 3: Theoretical Plates of Valsartan

#### 3.2.5.1 Three dimensional response surface plots

Fig 7: 3D plots Dapagliflozin (Rt)                                    Fig 8: 3D plots Dapagliflozin (Tf)

Fig 9: 3D plots Dapagliflozin(Theoretical plate)               Fig 10: 3D plots Sacubitril (Rt)

Fig 11:3D plots Sacubitril (Tf)                                            Fig 12: 3D plots Sacubitril(Theoretical plate)

Fig 13: 3D plots Valsartan (Rt)                                                        Fig 14: 3D plots Valsartan (Tf)

Fig 15: 3D plots Valsartan(Theoretical plate)

#### 3.3.7 Contour plots

Fig 16: Contour plots Dapagliflozin (Rt)                                Fig 17: Contour plots Dapagliflozin(Tf)

Fig 18: Contour plots Dapagliflozin (Theoretical Plate)                      Fig 19: Contour plots Sacubitril (Rt)

Fig 20: Contour plots Sacubitril (Tf)                               Fig 21: Contour plots Sacubitril (Theoretical Plate)

Fig 22: Contour plots Valsartan (Rt)Fig 23: Contour plots Valsartan (Tf)

Fig 23: Contour plots Valsartan (Theoretical Plate)

3.3.8 Desirability Function

Fig 24: Desirability Function Dapagliflozin                                              Fig 25: Desirability Function Sacubitril

Fig 26: Desirability Function Valsartan

### 3.3.9 Based on Overlay Plot:

Fig 27: Overlay plot analysis Dapagliflozin                                               Fig 28: Overlay plot analysis Sacubitril

Fig 29: Overlay plot analysis Valsartan

3.4 ANALYTICAL METHOD VALIDATION

- System Suitability:

System suitability was evaluated using a mixed standard solution containing 5 µg/mL of Dapagliflozin Propanediol Monohydrate and 50 µg/mL of Sacubitril–Valsartan Sodium Complex Salt prepared in the diluent. The solution was injected six consecutive times into the RP-HPLC system.

Chromatograms were assessed for parameters such as retention time, peak area, theoretical plates, tailing factor, and resolution between the analyte peaks. The percentage relative standard deviation (%RSD) of peak areas was calculated for both analytes and was found to be within the acceptable limit of ≤ 2.0%, indicating proper performance of the chromatographic system.

Table 3.20: System suitability study

| Sr. No. | System Suitability Parameter | Dapagliflozin Mean ± SD | %RSD | Valsartan Mean ± SD | %RSD | Sacubitril Mean ± SD | %RSD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Retention Time (min) | 2.908 ± 0.0039 | 0.13 | 4.767 ± 0.0040 | 0.08 | 6.145 ± 0.0043 | 0.07 |
| 2 | Theoretical Plates (N) | 5194 ± 52.18 | 1.00 | 7110 ± 61.23 | 0.86 | 8299 ± 70.47 | 0.85 |
| 3 | Tailing Factor | 1.29 ± 0.006 | 0.46 | 1.13 ± 0.005 | 0.44 | 1.08 ± 0.004 | 0.37 |
| 4 | Resolution | — | — | 9.61 ± 0.07 | 0.73 | 5.55 ± 0.05 | 0.90 |

- Specificity:

Specificity of the developed method was examined by injecting blank (diluent), standard solution, and sample solution individually into the chromatographic system.

The obtained chromatograms were carefully evaluated to confirm that no interfering peaks were observed at the retention times corresponding to Dapagliflozin Propanediol Monohydrate and Sacubitril–Valsartan Sodium Complex Salt, demonstrating the selectivity of the method for the analytes.

Fig 30: Chromatogram of Diluent

Fig 31: Standard of Dapagliflozin and Sacubitril–Valsartan   Fig 32: Sample of Dapagliflozin and Sacubitril–Valsartan

### Linearity

Linearity of the method was determined by analyzing five concentration levels ranging from 50% to 150% of the working concentration for both analytes.

Each level was injected three times, and calibration plots were constructed by plotting peak area versus concentration. The correlation coefficient (r²) values were found to be greater than 0.999, confirming excellent linearity over the selected concentration range.

| Sr. No. | Conc. (µg/mL) | Mean Area (n=3) | SD | % RSD |
| --- | --- | --- | --- | --- |
| 1 | 2.50 | 298358 | 338 | 0.11 |
| 2 | 3.75 | 432349 | 141 | 0.03 |
| 3 | 5.00 | 567766 | 326 | 0.05 |
| 4 | 6.25 | 716437 | 405 | 0.05 |
| 5 | 7.50 | 865624 | 1282 | 0.14 |

Table 3.21: Linearity Data for Dapagliflozin                             Fig 33: Calibration Curve of Dapagliflozin

| Sr. No. | Concen. (µg/mL) | Mean Area (n=3) | SD | % RSD |
| --- | --- | --- | --- | --- |
| 1 | 25.00 | 687446 | 463 | 0.06 |
| 2 | 37.50 | 983545 | 3243 | 0.32 |
| 3 | 50.00 | 1341881 | 1017 | 0.07 |
| 4 | 62.50 | 1668541 | 915 | 0.05 |
| 5 | 75.00 | 2008874 | 118 | 0.005 |

Fig 34: Calibration Curve of Dapagliflozin                    Table 3.22: Linearity Data for Sacubitril–Valsartan

Fig 35: Linearity Chromatogram of Dapagliflozin and Sacubitril–Valsartan

### Precision

Precision of the developed method was evaluated in terms of repeatability, intraday precision, and inter day precision for both analytes.

#### Repeatability (n = 6)

Table3.23 : Repeatability Data of Dapagliflozin and Sacubitril–Valsartan

| Conc of Dapagliflozin | Peak area | Conc of Sacubitril–Valsartan | Peak area |
| --- | --- | --- | --- |
| 5 µg/ml | 566908 | 50 µg/ml | 1333371 |
|  | 566827 |  | 1348843 |
|  | 567200 |  | 1347447 |
|  | 567273 |  | 1350239 |
|  | 566868 |  | 1350657 |
|  | 567295 |  | 1352234 |
| Mean | 567062 | Mean | 1347132 |
| SD | 216.5321377 | SD | 6935.435745 |
| % RSD | 0.03 | % RSD | 0.51 |

#### Intraday Precision

Table 3.24 Intraday Precision

| Drug | Concentration (µg/mL) | Mean Peak Area ± SD (n=3) | %RSD |
| --- | --- | --- | --- |
| Dapagliflozin | 2.5 | 288922 ± 628.70 | 0.22 |
|  | 5 | 569028 ± 1611.43 | 0.28 |
|  | 7.5 | 864205 ± 447.03 | 0.05 |
| Sacubitril–Valsartan | 25 | 664669 ± 1908.54 | 0.29 |
|  | 50 | 1343329 ± 7193.18 | 0.54 |
|  | 75 | 2004894 ± 3784.72 | 0.19 |

- Inter day Precision

Table 3.25: Inter day Precision

| Drug | Concentration (µg/mL) | Mean Peak Area ± SD (n=3) | %RSD |
| --- | --- | --- | --- |
| Dapagliflozin | 2.5 | 292836 ± 4006.42 | 1.37 |
|  | 5 | 573882 ± 2961.20 | 0.52 |
|  | 7.5 | 871510 ± 3307.61 | 0.38 |
| Sacubitril–Valsartan | 25 | 669175 ± 7055.57 | 1.05 |
|  | 50 | 1350839 ± 8496.08 | 0.63 |
|  | 75 | 2017251 ± 7452.69 | 0.37 |

### Limit of Detection and Limit of Quantitation

The LOD and LOQ values for Dapagliflozin Propanediol Monohydrate and Sacubitril–Valsartan Sodium Complex Salt were calculated from the calibration curve data.

The calibration curves were prepared in triplicate, and the standard deviation of the intercept (σ) and slope (S) were determined. The values were calculated using the following equations:

- LOD = 3.3 × σ / S

- LOQ = 10 × σ / S

The obtained values confirmed that the developed method possesses adequate sensitivity for detection and quantification of both analytes.

| Drugs | LOD (µg/mL) | LOQ (µg/mL) |
| --- | --- | --- |
| Dapagliflozin | 0.45 | 1.36 |
| Sacubitril–Valsartan | 4.36 | 13.2 |

Table 3.26: Limit of Detection and Limit of Quantitation data for Dapagliflozin and Sacubitril–Valsartan

### Accuracy

Accuracy was evaluated using the standard addition method at three levels: 50%, 100%, and 150% of the target concentration.The pre-analyzed synthetic mixture solution was spiked with known quantities of the standard drugs.

Each level was prepared in triplicate and analyzed using the optimized chromatographic conditions. The percentage recovery was calculated, and the mean recovery values for both drugs were found within 98–102% with %RSD < 2%, confirming good accuracy of the method.

Table 3.27: Accuracy Data Dapagliflazoin & Sacubitril–Valsartan

| Level% | Set | Dapagliflazoin |  |  |  |  | Sacubitril–Valsartan |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Amount Added (µg) | Amount Found (µg) | %Recovery | Mean % Recovery | %RSD | Amount Added (µg) | Amount Found (µg) | %Recovery | Mean % Recovery | %RSD |
| 50% | 1 | 2.500 | 2.490 | 99.60 | 99.2 | 0.4 | 25.000 | 24.830 | 99.30 | 98.8 | 0.5 |
| 50% | 2 | 2.500 | 2.470 | 98.80 |  |  | 25.000 | 24.600 | 98.40 |  |  |
| 50% | 3 | 2.500 | 2.480 | 99.20 |  |  | 25.000 | 24.670 | 98.70 |  |  |
| 100% | 1 | 5.000 | 4.930 | 98.60 | 98.8 | 1.1 | 50.000 | 49.520 | 99.00 | 99.9 | 0.9 |
| 100% | 2 | 5.000 | 5.000 | 100.00 |  |  | 50.000 | 50.340 | 100.70 |  |  |
| 100% | 3 | 5.000 | 4.890 | 97.80 |  |  | 50.000 | 50.040 | 100.10 |  |  |
| 150% | 1 | 7.500 | 7.390 | 98.50 | 98.1 | 0.8 | 75.000 | 74.370 | 99.20 | 98.5 | 1.2 |
| 150% | 2 | 7.500 | 7.390 | 98.50 |  |  | 75.000 | 74.370 | 99.20 |  |  |
| 150% | 3 | 7.500 | 7.290 | 97.20 |  |  | 75.000 | 72.890 | 97.20 |  |  |

- Robustness:

Robustness of the developed method was studied by introducing small, deliberate changes in chromatographic parameters.

The following variations were applied:

- Flow rate: ±0.1 mL/min from the optimized value

- Column temperature: ±5 °C

- Organic phase composition: ±2% of the mobile phase ratio

A standard solution containing 5 µg/mL of Dapagliflozin and 50 µg/mL of Sacubitril–Valsartan was injected under each modified condition. Chromatograms were evaluated for system suitability parameters such as retention time, peak area, theoretical plates, and tailing factor.

The %RSD values obtained under all conditions were within acceptable limits, indicating that the method is robust and reliable.

| Parameter | Drug | Area at Tem. (-5°C) | Area at Tem. (+5°C) | Area at Flow (-1%ml/min) | Area at Flow (+1%ml/min) | Area at Organic Phase-2% | Area at Organic Phase+2% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mean Peak Area | Dapagliflozin | 569439 | 571227 | 633750 | 517748 | 569588 | 569026 |
|  | Sacubitril–Valsartan | 1347018 | 1344871 | 1490324 | 1232106 | 1345127 | 1384884 |
| % RSD | Dapagliflozin | 0.1 | 0.0 | 0.1 | 0.3 | 0.1 | 0.1 |
|  | Sacubitril–Valsartan | 0.1 | 0.5 | 0.3 | 1.1 | 0.0 | 0.6 |
| Theoretical Plates | Dapagliflozin | 5079 | 5266 | 5868 | 4853 | 5404 | 5044 |
|  | Sacubitril | 7845 | 8265.00 | 8971 | 7983 | 8362 | 7884 |
|  | Valsartan | 6858 | 7198.00 | 7766 | 6815 | 7305 | 6685 |
| Tailing Fator | Dapagliflozin | 1.12 | 1.13 | 1.12 | 111 | 1.12 | 1.12 |
|  | Sacubitril– | 1.07 | 1.09 | 1.08 | 1.08 | 1.05 | 1.09 |
|  | Valsartan | 1.11 | 1.14 | 1.14 | 1.14 | 1.10 | 1.14 |

Table 3.28: Robustness Data Dapagliflazoin & Sacubitril–Valsartan

### Assay of Synthetic Mixture

Table 3.29: Assay of Synthetic Mixture

| Drug | Label Claim (mg) | Mean Amount Found (mg) | Assay (% Mean ± SD) | % RSD |
| --- | --- | --- | --- | --- |
| Dapagliflozin | 5.0 | 5.05 | 101 ± 1.450 | 1.44 |
| Sacubitril–Valsartan | 50.0 | 50.8 | 101.6 ± 1.759 | 1.73 |

- Conclusion

The present study successfully developed and validated a QbD-based RP-HPLC method for the simultaneous estimation of Dapagliflozin Propanediol Monohydrate and Sacubitril–Valsartan Sodium Complex Salt in a synthetic mixture. The application of the QbD approach allowed systematic optimization and identification of a robust design space, ensuring consistent chromatographic performance.

The validated method demonstrated excellent linearity, precision, accuracy, sensitivity, and robustness, with all validation parameters meeting acceptable analytical criteria. The overlay plot confirmed a stable method operable design region, indicating that the method can tolerate small variations in chromatographic conditions without affecting performance.

Therefore, the developed RP-HPLC method is simple, rapid, precise, and reliable, making it suitable for routine quality control analysis and simultaneous estimation of Dapagliflozin and Sacubitril–Valsartan in pharmaceutical formulations or synthetic mixtures.

5. REFERENCE

- Reddy PKC, Mathur P. QbD driven analytical method development and validation of metformin and dapagliflozin by RP-HPLC. International Journal of Health Sciences. 2022;6(S2):12345–12352.

- Kadam GM, Puyad AL, Kalyankar TM. RP-HPLC method development and validation for estimation of sacubitril and valsartan in pharmaceutical dosage form. Research Journal of Pharmacy and Technology. 2021;14(11):6032–6038

- Alluri NR, Bandlamudi MR, Kuppusamy S, et al. Implementation of quality by design approach to analytical method development and validation by RP-HPLC. ACS Omega. 2025;10(5):1234–1245.

- Mandhare S, Godge R, Vikhe A, et al. Development and validation of a QbD-based RP-HPLC method: a lifecycle approach. Journal of Applied Pharmaceutical Research. 2024;12(2):1–10.

- Sebaiy MM, El-Adl SM, Baraka MM, Hassan WS. Quality by design approach for development and validation of RP-HPLC method for simultaneous estimation of xipamide and valsartan. BMC Chemistry. 2022;16(1):1–14.\

- Kasichayanula S, et al., Clinical pharmacokinetics and pharmacodynamics of dapagliflozin, Clinical Pharmacokinetics, 2014; 53(1): 17–27.

- McMurray JJV, et al., Angiotensin–neprilysin inhibition versus enalapril in heart failure, New England Journal of Medicine, 2014; 371: 993–1004.

- Drug Profile For Dapagliflozin Propanediol Monohydrate 2021, https://go.drugbank.com/salts/DBSALT001101

- https://pubchem.ncbi.nlm.nih.gov/compound/Dapagliflozin-Propanediol

- Drug profile for sacubitril” https://go.drugbank.com/drugs/DB09292#:~:text=Sacubitril%20is%20a%20neprilysi n%20inhibitor,IV)%20and%20reduced%20ejection%20fraction. (Cited Aug 2025)

- “Drug profile for valsartan” https://go.drugbank.com/drugs/DB00177