A MANUALLY OPERATED SEED DRILL FOR SMALL SCALE FARMERS.

Rotimi Rufus DINRIFO1* and Olanrewaju Akintunde TANIMOLA2

1,2 Department of Agricultural and Biosystems Engineering, Lagos State University of   Science and Technology, Ikorodu, Lagos Nigeria

1*Corresponding author e-mail : rdinrifo@yahoo.com , ORCID:  0000-0002-1385-0173

ABSTRACT.

The objective of this work was to design and construct a manually operated seed drill for small scale farmers, particularly those cultivating small acreages in urban and peri-urban areas. The machine is constructed using readily available materials and standard mechanical components. The low-cost equipment is drawn behind a human operator and operates via a ground- wheel-driven chain and sprocket system that transmits motion to a seed metering mechanism. The drill works best in tilled soils where coverage of the sown seed is easier. The drill was tested with the planting of corn seeds. The efficiency of the machine was 72% and the depth of sowing was influenced by the degree of tilth and moisture content of the soil. With no requirement for fuel or electricity, this innovation offers a sustainable, affordable, and user-friendly solution to enhance agricultural productivity, especially in resource-constrained settings.

1.0 Introduction

Sowing is one of the most important agricultural operations for raising crops (Akhta et al., 2006;  Morgan et al., 2024; Dennett, 2024). The basic objective of sowing is to put the seed and, sometimes, fertilizer and other agrochemicals at desired depth and spacing, and thereafter cover the seed with soil. In many countries, especially the less developed ones, sowing is generally carried out manually (by hand). Sowing by hand consumes time, often causes fatigue and backache due to the long hours required for the careful hand-metering of seeds (Borbale, et al., 2021). Mechanization plays a vital role in the reduction of the stress (Bamigboye & Mofolasayo, 2006). However, most of the sowing machines available in the market today, especially the tractor-mounted ones, though effective are costly and require high maintenance. They are unaffordable to small scale farmers in Nigeria. There is therefore the need to develop machines that are cheap and affordable (Pingali, 2007); Sims & Kienzle (2017) and Huang & Zou (2018)). Manually operated seed drills fit into this category.

A lot of work has been reported by various investigators relating to manual or animal drawn seed drills for sowing. Singh et al., (2006) assessed a manually operated seed drills for farm women. Modifications were made as per ergonomical data of the subjects, to existing seed drills. The physiological workload requirement was reduced by 23% and 33.9% in pulling and pushing modes of operation with the modified unit of CIAE seed drill as compared to its original unit. Reddy (2015) developed a four-row bullock drawn groundnut planter and as tested for winter-summer sowing (irrigated condition) at Agricultural Research Station, Anantapur during the years2010-11 and 2011-12 seasons. During evaluation, the field capacity of the planter was 0.15 ha/h at the average speed of operation 2.57 km/h. The field efficiency of the planter was 67%. It can maintain recommended seed rate of 50 kg/ha with row to row and seed to seed spacing of 22.5 cm and 10 respectively. Kalay (2015) presented the design and fabrication of a manually operated single row multi crop planter. The seed metering device was designed to be interchangeable to allow for the different varieties and types of seeds. The work of Shambhu & Thakur (2019) was based on the laboratory and field performance evaluation of manual seed drill for sowing jute and tiny seeds.

Several manually operated seed drills have resulted from engineering research at universities and polytechnics in Nigeria (Adekanye & Akande (2015); Soyoye, et al., (2016); Ochin, et al., (2019);  Ali (2019) and Sedara et al., (2020)). Various "push-type" and "jab" planters have been fabricated and sold in Nigeria for small-scale maize and bean farming. These researchers often focus on locally sourced materials and specific smallholder farmer needs. Different farmers at different places tend to have different requirements.

Lagos State, like many other modern cities is growing rapidly (Auwalu & Bello,2023; Onilude & Vaz, 2021). It has been estimated that Lagos residents consume N5 trillion worth of food annually, the bulk of which are sourced from other states in the hinterland of Nigeria or imported from outside the country (Ajayi, 2021). The Lagos State Government has therefore set itself the ambitious target of 40% sufficiency in food production, up from the present 20% (LSMAFS , 2024). The Lagos urban and peri-urban farmers are characteristically small land holders; because of land availability constraints The present design is specifically tailored towards the need of the urban and peri-urban low scale farmers. This will significantly reduce the drudgery involved in their farming activities.

2. METHODOLOGY / MACHINE DESIGN

The objectives set to be met by this design include thatt he machine should be operated entirely by human power; use locally available, low-cost materials; ensure ease of operation and maintenance and achieve adjustable, but regular seed spacing .

2.1 Material selection and requirements of components

Materials with appropriate engineering properties were used for different components of the seed drill as the drill will be subjected to varying loads, torque and frictional effects. Angle iron was selected as the material suitable for the frame of the seed drill based on its strength and rigidity properties. The hopper is trapezoidal on the inside and having an external cuboidal shape. This shape was informed by the type and location of the seed metering mechanism. Galvanised steel sheet was used for the hopper and the thickness is 3mm. The handle is used to provide the push force from a human operator to move the seed drill from one point to another during sowing operation. Galvanized steel pipe was used for the handle because of its ability to resist corrosion and its durability. The metering mechanism is a major component in a seed drill. It picks required numbers of seed and delivers them into the soil through the chute at required depth created by furrow openers, A fluted roller was used to build the metering device. The Furrow opener opens the soil and seeds falling through the chute will be dropped into the furrow, Angle iron was selected for fur opener and the furrow closer. The details are provided in Table 1

| Component | Function | Materials |
| --- | --- | --- |
| Frame | Provides structural support and houses all components | Angle iron |
| Seed Hopper | Funnel shape, trapezoidal, to direct seeds into the metering device at the base | Made from galvanized steel sheet |
| Handle | To move the machine from a one point to another during sowing operation. | Galvanized steel pipe |
| Seed Metering Mechanism | To control the seed rate during sowing | A fluted roller, PVC |
| Furrow Openers | Designed to create a clean V-shaped furrow | made of hardened steel |
| Ground Wheel | rubber tread for traction | rubber |
| Transmission chain | Chain and sprocket system to reduce speed and increase torque for precise seed drop | steel |
| Seed Tubes | guiding seeds from metering outlet to furrow | Flexible PVC pipes |
| Covering Blades | Two small angled blades attached behind furrow openers to cover seeds with soil. | Steel |

2.2 Design Calculations

An average human performing sustained, manual labour can generate a useful power output of approximately 75–100 Watts (about 0.1 to 0.13 horsepower). According to Campbell (1990) and Stanley (2003):

HP=0.35-0.092×log t

where t is time spent working in minutes

On the average, humans can work on the field 2-4 hours providing 0.13 to 0.16 hp continuously. So, if we take working time to be four hours then the power developed by a human on the seed drill can be estimated as:

HP=0.35-0.092×log 240 = 0.13 hp.

Studies ( Sinha et al., 2019, Chethan & Krishnan 2017) indicated that forward speed of workers on specialized, small-scale machinery (e.g. drills, planters) show optimal performance often occurring in the range of 1.4 to 2.5 km/h. We use the operating speed 2.km/h, then

We know that, HP = {Pull (kg) × Speed (m/sec)}/ 75, then  pull (force)  delivered to the chain drive is:

HP = forceX speed of machine m/s75

The operating speed of the machine is 2.5 km/h (0.7 m/s). Then the force

force =0.13×750.7   13.93N

Speed of ground wheel (Nw),

Nw=specdofmach∈e(s)×100π×60....      (2)

Nw=41.66×1003.142×60 =22,11 rpm

Torque on ground wheel (Tw), N.m  (Sharma& Mukesh,2010)

Tw=KwWt   Rw       -  3

where,

Kw=coefficient of rolling resistance (0.3 for the metallic wheel)

Wt=active weight of the machine (50 kg) and

Rw= the radius of ground wheel (16.5 cm).

TW=0.3×50×0.16  =7.68Nm

Determination of maximum bending moment on the shaft

Power is transmitted to the machine by the chain drive system determines the bending moment of the shaft . The force transmitted to the shaft was determined using the theorem of the chain drive system (Sharma & Mukesh, 2010). So, the load on the chain or chain load (Q) is given by:

Q=K1Pt

where,

KI =coefficient of chain (1.15 for the mild steel)

Pt =push force of the chain.

Q=1.15×13.92 =16.008N

Now, the chain drive is working at an angle φ (35°) with the horizontal. Therefore, equivalent chain load on the machine was calculated as

Qv=Q sin Ꞷ	....(5)

=16.008×sin35 ==9.18

The overhung of the wheel is 15 cm, and that the sprocket is 5cm. Total weight of machine was determined as 50 kg. Then, the weight on one wheel is 20kg

Bending moment (Mb) on the shaft given by the chain drive system

Mb=(20×0.15)+(9.18×0.05)  =3.459kgm

Hence: Equivalent bending  momem=mb2+mo2

Where Mt =Tw  =3.4592+0.992 =4.439 kgm

The drill was designed to be operated manually to make it cost effective

Power is transmitted from the wheel to the seed metering wheel through pintle chain. Power (HP) transmitted in manual seed drill is low. A chain sprocket system was introduced which have two sprockets (small sprocket has 18 teeth and large sprockets has 48 teeth). The chain length is calculated by the following equation (Share Mukesh,2010).

M=2cp+Z1+Z22+(Z2-Z1)22πp

m=number of chain links

C=centre to centre distance between two sprocket mm.

P=is the chain pitch, mm

zl and z2 are the number of teeth in the driver sprocket and driven sprocket respectively.

M=2×27010+48+182+(48-18)22×3.142×10=81

Length of the chain(mm)

Lc=m×p (3.26)

where.

Lc=chain length, mm

m =number of chain links

p  =chain pitch, mm

Lc=137×10=1370mmor 1.370m

The sprockets

D=psin(180N)....(8)

where, p=chain pitch

chr of sprocket

N=Number Of teeth

D=20sin⁡(180/16) =102.5 mm

During operation, the seed drill is pushed forward using the handle, the ground wheel rotates, driving the chain-sprocket transmission. The chain drive transmit power to the seed metering mechanism at the bottom of the hopper /seed tank. The seeds are fed into the machine through the hopper and then moved by gravity into the metering chamber. The device releases two to three seeds at a time. The delivery from the seed metering system is connected to the pipe carrying the seeds to the furrow made by the furrow- openers, and the furrow is thereafter covered by the covering device. Figure 1 shows a sketch of the machine

Figure 1: The Seed drill.

3.0  Performance Evaluation

The prototype was tested on a prepared loam soil plot (0.1 hectare) with maize seeds

The machine was tested on three (3) different fields, each of approximately 100 m2 square respectively. The moisture content of each of the fields was determined using the gravimetric oven method. The resistance to the movement of the seed drill while in operation was also estimated

4.0 Results and Discussion

4.1 Seed Sowing performance

Table 1 shows the data obtained during the field test. The figures given were average figures from the three fields.

Table 1: Results from the field tests

| Parameter | Result |
| --- | --- |
| Sowing Rate | 0.042 ha/hour |
| Seed Usage | 4.8 kg /hour |

Sowing depth					  4.2 cm

Sowing distance				  30 cm

The rate at which the machine sows the seeds was 4.8 kg/hr . The distance of sowing was 30cm while the hopper of the machine can contain 3kg of seed (maize).

4.2 Effects of Moisture content on the drill’s operations

The recorded moisture content of the three fields was: 13.3%, 14% and 12.5% wet basis respectively. It was found that the moisture content of the field affected the depth at the which the machine sows the seeds. This could be attributed to the fact that a loose moist soil is easier to open than a dryer one. The furrow opener push force generally increases with a decrease in soil moisture content. This also affect the pushing force needed to operate the planter.  The moisture content of the soil plays a critical role in determining the pushing force (draft) required to operate a seed drill. Rashidi et all., (2013) reported that lowering moisture content increases the draft force needed, as dry soil is firmer and harder for furrow openers to penetrate.

5.0	Conclusion

A manually operated seed drill has been successfully constructed with the use of available local materials. Its performance evaluation was also carried out. The machine will go a long way in increasing the sowing efficiency of sowing, will help in sowing seed at uniform distance and in reducing drudgeries encountered in the traditional method of sowing. Its affordability and fuel independence make it particularly suitable for smallholder farmers, contributing to sustainable agricultural intensification.

REFERENCES

Adekanye, T. A., & Akande, A. M. (2015). Development and evaluation of a manual multi-crop planter for peasant farmers. Elixir Agriculture, 35095-35101.

Akhtar, M., Cheema, M. S., Jamil, M., & Ali, L. (2006). Effect of time of sowing on some important characters of wheat, Triticum aestivum genotypes. Journal of Agricultural Research (JAR)., 44(4), 255-259.

Ali, M. R. (2019). Design and development of a multi-crop manual seed drill. Agricultural Engineering International: CIGR Journal, 21(3), 51-60.

Auwalu, F. K., & Bello, M. (2023). Exploring the Contemporary Challenges of Urbanization and the Role of Sustainable Urban Development: A Study of Lagos City, Nigeria. Journal of Contemporary Urban Affairs, 7(1), 175-188.

Bamigboye O A & Mofolasayo A (2006): Performance evaluation of a two-row okra planter. Agricultural Engineering International: CIGR Journal Manuscript PM 0662 vol 8

Bandhiya, R. D., Vadher, A. L., Parmar, N. B., & Mehta, T. D. (2016) : Development and evaluation of manually operated seed drill for bajra crop. International Journal of Science, Environment and Technology, 5(6), 4630-4637.

Borbale, M. K., Pohekar, A. U., Nistane, J. R., Nichit, N. S., Yesankar, T. I., & Kaware, A. G. (2021) : Study of traditional method of weeding and fertilizing and their drawbacks. International Research Journal of Engineering and Technology, 8(3), 236-242.

Campbell E. (1990): Influence of spacing time of sowing and nitrogen fertilizer on the yield of wheat . Indian Journal of Agronomy 15: 251-253

Chethan CR, Krishnan DA (2017): Dynamic push–pull strength data generation for agricultural workers to develop manual dry land weeders current science. 2017; 8:25.

Dennett, M. D. (2024) : Effects of sowing date and the determination of optimum sowing date. In Wheat (pp. 123-140). CRC Press.

Huang, M., & Zou, Y. (2018) : Integrating mechanization with agronomy and breeding to ensure food security in China. Field Crops Research, 224, 22-27.

Kalay Khan G (2015): Optimization of design and operational parameters of a pneumatic seed metering device for planting cotton seeds. Biosystems Engineering 94(4): 429-438

LSMAFS (2024) : The Lagos State Government is planning for Lagos Agriculture  and Food Systems (LSMAFS) -Roadmap 2.0  Lagos State Ministry of Agriculture and Food Systems  – available at https.//lasagric.com.

Stanley J (2003): Human Power and the Watt. Physics 162, Spring Term 2003). https://hypertextbook.com/facts/2003/WeiLiangMok.shtml.

Morgan, K., McDonald, G., Allen, R., Faulkner, M., & Trengove, S. (2024 ): Evaluating the importance of sowing rate, depth and time of sowing on emergence and yield–wheat and canola. Hart Trial Results  2024, University of Adelaide , 119.-124

Ochin, N. G., Adewumi, A. A., Faleye, T., Adeyi, A. M., Willoughby, F. A., & Ehiosun, P. (2019) : Development and performance evaluation of single row manual seed planter. Journal of Agricultural and Crop Research, 7(3), 31-37.

Onilude, O. O., & Vaz, E. (2021) : Urban sprawl and growth prediction for Lagos using GlobeLand30 data and cellular automata model. Sci, 3(2), 23.

Pingali, P. (2007) : Agricultural mechanization: adoption patterns and economic impact. Handbook of agricultural economics, 3, 2779-2805.

Priya Sinha, SV Jogdand, Shubham Sinha and Shambhu Singh (2019): Journal of Pharmacognosy and Phytochemistry 2019; 8(6): 2488-2492 )

Rashidi, M., Najjarzadeh, I., Jaberinasab B., Mohammad S E & Fayyazi M. (2013): Effect of Soil Moisture Content, Tillage Depth and Operation Speed on Draft Force of Moldboard Plow. Middle-East Journal of Scientific Research 16 (2): 245-249, 2013 ISSN 1990-9233 © IDOSI Publications, 2013 DOI: 10.5829/idosi.mejsr.2013.16.02.11675

Reddy L (2015): Development of a push type seed drill for sowing maix=ze in Rwanda

Sedara, A., Ibrahim, Y., Manuwa, S., & Sedara, O. (2020): Development of a low-cost seed drill for sowing amaranth seed. World Acad. J. Eng. Sci, 7(3), 39-45.

Shambhu V B &Thakur A K (2019):  Laboratory and field performance of manual seed drill for sowing jute and tiny seeds. Indian Journal of Agricultural Sciences89 (1): 129–32, January 2019/

Sharma A  & Mukesh E (2010) : A textbook on Farm machinery Design principles and problems. Jain brothers. New Delhi. Second Edition

Sims, B., & Kienzle, J. (2017): Sustainable agricultural mechanization for smallholders: what is it and how can we implement it?. Agriculture, 7(6), 50.

Singh P S, Laxman P G, Agarwai N (2006): Ergonomic Evalaution of manually operated seed drill for farm women. Journal of Agricultural Engineering (India) 43(1): 42-48

Soyoye, B. O., Ademosun, O. C., & Olu-Ojo, E. O. (2016): Development of a manually operated vertical seed-plate maize planter. Agricultural Engineering International: CIGR Journal, 18(4), 70-80.