## A Machine Learning Based Multimodal Framework for continuous health monitoring using wearable and cough data

Karthick Rajapandiyan1, Sridhar L 2

1Department of computer science Engineering, National Institute of Technical Teachers' Training and Research, Taramani, Chennai, India, 600113

2Department of computer science Engineering, SRM institute of Science and Technology, Kattankulathur, Chennai, India, 603203

Email: karthickris@gmail.com ;  laksridhar@gmail.com

#### ABSTRACT

The contemporary digital health environment features an abundance of applications and wearable technologies capable of monitoring a wide array of physiological parameters, including cardiac activity, energy expenditure, sleep architecture, stress levels, and critical vital signs such as ECG, SpO₂, and body temperature. However, specialized instruments, exemplified by LCM for respiratory symptom surveillance like cough, typically function in isolation from broader health tracking ecosystems. This technological proliferation has paradoxically created a significant lacuna in the integration of these disparate data sources into a cohesive, publicly accessible, and user-friendly platform. This paper presents a novel framework designed to address this challenge by employing Generative AI (GenAI) to synchronize data from discrete cough monitoring tools with wearable health device datasets. Through GenAI's capacity for multimodal data analysis, the system can discern intricate patterns and correlations between respiratory symptoms and other physiological metrics, thereby facilitating the early detection of nascent health conditions. The proposed solution is engineered to demystify health data interpretation for non-expert users by generating personalized, localized summaries and actionable insights. This integrated approach not only augments the accuracy of health assessments but also empowers individuals to exercise informed agency over their well-being. The framework holds substantial promises for advancing chronic disease management, expediting illness detection, bolstering preventive care, and informing post-care decisions, fostering a unified, intelligent, adaptive, and accessible paradigm for health monitoring [21]

Keywords: Cough pattern analysis, Gen-AI Multimodal health data, Smart LCM cough monitoring, Smart health ecosystem, Human-centered health interfaces

# 1. INTRODUCTION

The proliferation of wearable health devices and mobile health applications has enabled continuous monitoring of diverse physiological parameters, yet the lack of integration across platforms limits their potential for holistic health management. Cough monitoring, a critical indicator for respiratory health, remains siloed from broader health data streams. Integrating these modalities using Generative AI (GenAI) offers a pathway to unified, intelligent health prediction and personalized care [2][3][9][20]

Recent advances in GenAI have demonstrated its ability to model complex, multimodal biomedical data, including physiological waveforms, imaging, and electronic health records, for disease prediction and patient stratification [2][3][8][9][15][20]. AI-driven wearable devices have shown promise in early detection of chronic diseases, mental health monitoring, and real-time health risk assessment,  but challenges remain in data fusion, privacy, and user interpretability [3][9][10][11][15][16][20]. Cough monitoring tools, leveraging AI for accurate event detection, have achieved high sensitivity and specificity, yet are rarely integrated with other health data sources [4][13][19].

The proposed system synchronizes cough monitoring (e.g., via smartphone or wearable sensors) with other physiological data streams using GenAI models capable of multimodal data fusion. This enables the identification of correlations between respiratory symptoms and broader health metrics, supporting early detection of conditions such as respiratory infections or exacerbations of chronic diseases [2][3][9][10][19]. Personalized, actionable health summaries are generated for users, enhancing accessibility and empowering informed decision-making [1][2][11][12]

# 2. RESEARCH METHODOLOGY

The proposed paper takes a structured approach to developing an AI-driven framework. This section presents the overall system architecture, strategies for data collection and pre-processing, the design of the machine learning model and the methods employed for medical assessment. Each report has been carefully crafted so that key problems, such as chronic diseases, are addressed. The results, client issue density and patterns are considered, and a summary is suggested.

Smart, AI-driven cough and health monitoring Designed as an intelligent health monitoring assistant, this system uses AI-based models to detect cough patterns and assess respiratory health. It provides predictive insights based on real-time and historical health data. The architecture comprises three core components:

The based on the research   methodology to correlate wearable health data (e.g., heart rate, SpO₂, activity) with cough detection signals for symptom prediction is a multimodal machine learning (ML) fusion framework that integrates heterogeneous data sources (sensor and audio) for synchronized inference and disease symptom modeling.

Recommended Machine Learning Methodologies:

- Multimodal Data Fusion Framework for Health Condition Prediction

- Deep Multimodal Networks with Transfer Learning

- Out-of-Distribution (OOD) Robust Learning

- Hybrid Ensemble Learning for Disease and Symptom Prediction

Multimodal Data Fusion Framework for Health Condition Prediction

Based on recent research and empirical results, we adopted a multimodal data fusion framework for our dataset analysis. This approach integrates data from wearable sensors and cough-detection systems to enhance the accuracy of health condition prediction. Studies have shown that multimodal fusion can improve predictive performance by up to 15% compared to unimodal models.

Multimodal fusion can be implemented at various levels which is described here:

- Data-level fusion involves the direct integration of raw sensor signals, such as combining accelerometer readings with audio amplitude from cough recordings.

- Feature-level fusion concatenates extracted features from different modalities—for example, Mel-frequency cepstral coefficients (MFCCs) derived from cough sounds and heart rate variability (HRV) metrics from wearable devices.

- Decision-level fusion aggregates predictions from independently trained models using ensemble techniques, such as stacked architecture combining LSTM and CNN classifiers.

Among these, feature-level fusion using deep neural networks consistently demonstrates superior correlation accuracy in symptom analysis, making it the preferred strategy for our framework.

## 2.1 Data Acquisition and Preprocessing

The Leicester Cough Monitor (LCM) continuously records ambient audio via a microphone and employs a semi-automated algorithm to detect and quantify individual cough events. Its analysis pipeline includes advanced filtering techniques to eliminate background noise and non-cough sounds, ensuring high-fidelity cough detection.

To augment the dataset and improve model generalization, Generative Adversarial Networks (GANs) are employed to synthesize realistic cough audio samples. These synthetic samples simulate diverse acoustic profiles across age groups, environments, and health conditions. By combining LCM’s precise event detection with GAN-driven data augmentation, the system enhances training diversity and robustness for downstream machine learning tasks such as symptom classification and health risk prediction.

Health telemetry data is collected from microphones, wearable sensors, and mobile devices to capture attributes such as cough frequency, intensity, acoustic signature, respiratory rate, blood pressure, heart rate, oxygen saturation levels, ambient conditions, and user-reported symptoms. Preprocessing involves denoising audio signals, normalizing biometric metrics and engineering features that reflect temporal and physiological variations. Synthetic data is generated using Generative Adversarial Networks (GANs) to simulate diverse cough scenarios and enhance model generalization across age groups, environments, and health conditions.

Data Preprocessing phase focuses on ensuring data integrity and consistency. It involves systematic data cleaning to remove duplicate entries, manage missing values, and correct inconsistencies. Additionally, signal normalization and noise reduction techniques are applied, and all biometric signals are standardized to maintain consistency across diverse sensor inputs.

## 2.2 User interface:

A dashboard built with Streamlit app enables users and healthcare providers to visualize cough trends, symptom progression, and risk assessments. The interface supports interactive health tracking, report uploads, and real-time alerts. Embedded AI conversational agent functionality enables intuitive interaction, allowing users to query their health status and receive AI-driven recommendations for preventive care or medical consultations in user’s preferred language. This platform provides an accessible interface for both patients and healthcare professionals to monitor and analyze various health indicators.

An embedded AI conversational agent facilitates natural language interaction. This agent enables users to query their health status, interpret system-generated recommendations, and receive context-sensitive guidance for preventive care.

3. EXPERIMENTAL/NUMERICAL WORK

| index | Blood Oxygen Level (%) | Heart Rate (BPM) | Cough_detected | Severity | Risk |
| --- | --- | --- | --- | --- | --- |
| 0 | 100 | 40 | 0 | mild | No Risk |
| 1 | 100 | 40 | 0.0029 | mild | No Risk |
| 2 | 100 | 40 | 0.0088 | mild | No Risk |
| 3 | 100 | 40 | 0.0116 | mild | No Risk |
| 4 | 100 | 41.70989513 | 0.0122 | mild | No Risk |
| 5 | 100 | 43.74140206 | 0.013 | mild | No Risk |
| 6 | 100 | 43.85677975 | 0.0139 | mild | No Risk |
| 7 | 100 | 44.4371386 | 0.0155 | mild | No Risk |
| 8 | 100 | 47.2722567 | 0.0201 | mild | No Risk |
| 9 | 100 | 47.42860421 | 0.0236 | mild | No Risk |
| 10 | 100 | 48.6917189 | 0.0246 | mild | No Risk |
| 11 | 100 | 51.40996728 | 0.0261 | mild | No Risk |
| 12 | 100 | 53.3187863 | 0.0294 | mild | No Risk |
| 13 | 100 | 53.93020624 | 0.0306 | mild | No Risk |
| 14 | 100 | 54.86000605 | 0.0307 | mild | No Risk |
| 15 | 100 | 56.22934455 | 0.0361 | mild | No Risk |
| 16 | 100 | 58.28 | 0.0373 | mild | No Risk |
| 17 | 100 | 58.93977586 | 0.0391 | mild | No Risk |
| 18 | 100 | 59.06505478 | 0.0399 | mild | No Risk |
| 19 | 100 | 59.20747694 | 0.0425 | mild | No Risk |
| 20 | 100 | 59.55423802 | 0.0456 | mild | No Risk |
| 21 | 100 | 60.82606954 | 0.0482 | mild | No Risk |
| 22 | 100 | 61.15 | 0.0564 | mild | No Risk |
| 23 | 100 | 61.34892365 | 0.0576 | mild | No Risk |
| 24 | 100 | 61.68463511 | 0.0602 | mild | No Risk |
| 25 | 99.98256601 | 61.95016529 | 0.0632 | mild | No Risk |
| 26 | 99.92804064 | 62.4783902 | 0.0735 | mild | No Risk |
| 27 | 99.91706621 | 62.71943845 | 0.0776 | mild | No Risk |
| 28 | 99.86637886 | 63.47682463 | 0.0821 | mild | No Risk |
| 29 | 99.83937258 | 63.68 | 0.0884 | mild | No Risk |
| 30 | 99.7947542 | 63.77805795 | 0.0931 | mild | No Risk |
| 31 | 99.78959924 | 64.10446781 | 0.0953 | mild | No Risk |
| 32 | 99.72973954 | 64.42024701 | 0.0964 | mild | No Risk |
| 33 | 99.64606451 | 64.62704386 | 0.1068 | mild | No Risk |
| 34 | 99.62675299 | 64.79977848 | 0.115 | mild | No Risk |
| 35 | 99.51747431 | 65.22 | 0.1643 | mild | No Risk |
| 36 | 99.49580561 | 66.12078656 | 0.1701 | mild | No Risk |
| 37 | 99.41376032 | 66.62971237 | 0.2098 | mild | No Risk |
| 38 | 99.26840702 | 66.81680741 | 0.2216 | mild | No Risk |
| 39 | 99.25495076 | 67.23 | 0.2677 | mild | No Risk |
| 40 | 99.23093185 | 67.38743157 | 0.2798 | mild | No Risk |
| 41 | 99.22451077 | 67.4 | 0.3009 | mild | No Risk |
| 42 | 99.15265159 | 68.09020293 | 0.3147 | mild | No Risk |
| 43 | 99.1508077 | 68.10013872 | 0.3216 | mild | No Risk |
| 44 | 99.06763734 | 68.31010413 | 0.3349 | mild | No Risk |
| 45 | 99.04772547 | 68.48889181 | 0.3393 | mild | No Risk |
| 46 | 99.04517538 | 68.76188374 | 0.3576 | mild | No Risk |
| 47 | 99.02468691 | 68.84966038 | 0.3627 | mild | No Risk |
| 48 | 99.01491346 | 69.05858943 | 0.3747 | mild | No Risk |
| 49 | 98.99749752 | 69.23683246 | 0.4277 | mild | No Risk |
| 50 | 98.99650466 | 69.42479492 | 0.579 | mild | No Risk |
| 51 | 98.97829843 | 69.61567441 | 0.5799 | mild | No Risk |
| 52 | 98.97343165 | 70.0113134 | 0.5919 | mild | No Risk |
| 53 | 98.95516142 | 70.24380998 | 0.6224 | mild | No Risk |
| 54 | 98.92118099 | 70.44503588 | 0.6614 | mild | No Risk |
| 55 | 98.89437303 | 70.46467123 | 0.6762 | mild | No Risk |
| 56 | 98.80965025 | 70.54823343 | 0.6856 | mild | No Risk |
| 57 | 98.78567386 | 70.59301387 | 0.6911 | mild | No Risk |
| 58 | 98.77628474 | 70.81290843 | 0.7335 | mild | No Risk |
| 59 | 98.75711035 | 71.64829086 | 0.7511 | mild | No Risk |
| 60 | 98.69426497 | 72.33 | 0.7525 | mild | No Risk |
| 61 | 98.67071423 | 72.77180577 | 0.7811 | mild | No Risk |
| 62 | 98.66834393 | 72.91402277 | 0.8023 | mild | No Risk |
| 63 | 98.61610958 | 73.15646556 | 0.8053 | mild | No Risk |
| 64 | 98.58593454 | 73.519921 | 0.8109 | mild | No Risk |
| 65 | 98.58379676 | 73.85294067 | 0.824 | mild | No Risk |
| 66 | 98.57779006 | 74.10500253 | 0.8568 | mild | No Risk |
| 67 | 98.53219516 | 74.12758955 | 0.8648 | mild | No Risk |
| 68 | 98.5050674 | 74.14 | 0.8713 | mild | No Risk |
| 69 | 98.46340229 | 74.53213746 | 0.8749 | mild | No Risk |
| 70 | 98.44328703 | 74.86797764 | 0.8775 | mild | No Risk |
| 71 | 98.42435021 | 75.02968227 | 0.8792 | mild | No Risk |
| 72 | 98.37515943 | 75.16860351 | 0.886 | mild | No Risk |
| 73 | 98.2765583 | 75.21115232 | 0.8904 | mild | No Risk |
| 74 | 98.27067918 | 76.30138329 | 0.8937 | mild | No Risk |
| 75 | 98.24457119 | 76.3255203 | 0.8955 | mild | No Risk |
| 76 | 98.18899114 | 76.73815114 | 0.8996 | mild | No Risk |
| 77 | 98.14712188 | 76.84986596 | 0.9078 | mild | No Risk |
| 78 | 98.10010128 | 76.95659338 | 0.9089 | mild | No Risk |
| 79 | 98.05750152 | 77.06835233 | 0.9128 | mild | No Risk |
| 80 | 97.93977886 | 77.31 | 0.922 | mild | No Risk |
| 81 | 97.93118024 | 77.34780702 | 0.922 | mild | No Risk |
| 82 | 97.91523601 | 77.91 | 0.9301 | mild | No Risk |
| 83 | 97.78972645 | 77.93675733 | 0.933 | mild | No Risk |
| 84 | 97.7082761 | 77.96393073 | 0.9334 | mild | No Risk |
| 85 | 97.63093798 | 78.04729996 | 0.9349 | mild | No Risk |
| 86 | 97.61892653 | 78.17970894 | 0.938 | mild | No Risk |
| 87 | 97.61730184 | 78.22792335 | 0.9415 | mild | No Risk |
| 88 | 97.60522246 | 78.45025107 | 0.9437 | mild | No Risk |
| 89 | 97.55307153 | 79.06263167 | 0.9456 | mild | No Risk |
| 90 | 97.47398146 | 79.38203546 | 0.9472 | mild | No Risk |
| 91 | 97.47263459 | 79.61779162 | 0.9475 | mild | No Risk |
| 92 | 97.40257094 | 79.73192298 | 0.9504 | mild | No Risk |
| 93 | 97.37429828 | 79.76807352 | 0.9507 | mild | No Risk |
| 94 | 97.3630655 | 79.84141585 | 0.9531 | mild | No Risk |
| 95 | 97.28039806 | 79.86425437 | 0.9536 | mild | No Risk |
| 96 | 97.27278687 | 80.11309121 | 0.9547 | mild | No Risk |
| 97 | 97.24506118 | 80.24336481 | 0.958 | mild | No Risk |
| 98 | 97.22179634 | 80.29323472 | 0.9594 | mild | No Risk |
| 99 | 97.21894485 | 81.26723473 | 0.9609 | mild | No Risk |
| 100 | 97.17113365 | 81.28112872 | 0.9632 | mild | No Risk |
| 101 | 97.13166932 | 81.53346713 | 0.9664 | mild | No Risk |
| 102 | 97.09234007 | 81.73349679 | 0.9673 | mild | No Risk |
| 103 | 97.05295352 | 81.78254065 | 0.9674 | mild | No Risk |
| 104 | 96.92388342 | 81.90295748 | 0.9699 | mild | No Risk |
| 105. | 96.89421335 | 82.00624112 | 0.9712 | mild | No Risk |
| 106 | 96.88066821 | 82.56980161 | 0.9716 | mild | No Risk |
| 107 | 96.85734695 | 83.28 | 0.9729 | mild | No Risk |
| 108 | 96.81891253 | 84.03342652 | 0.9734 | mild | No Risk |
| 109 | 96.80491907 | 84.21830058 | 0.9743 | mild | No Risk |
| 110 | 96.76113261 | 84.54568993 | 0.9751 | mild | No Risk |
| 111. | 96.65776603 | 84.58662488 | 0.9775 | mild | No Risk |
| 112 | 96.47894032 | 84.67722962 | 0.9778 | mild | No Risk |
| 113 | 96.30256923 | 85.48782568 | 0.9779 | mild | No Risk |
| 114 | 96.22980353 | 85.84 | 0.9782 | mild | No Risk |
| 115 | 96.06816146 | 86.03063332 | 0.9794 | mild | No Risk |
| 116. | 96.05285348 | 86.22925548 | 0.9794 | mild | No Risk |
| 117 | 96.02709913 | 86.55 | 0.9794 | mild | No Risk |
| 118 | 95.98134261 | 86.95767523 | 0.9815 | mild | No Risk |
| 119 | 95.94128721 | 87.13591831 | 0.9818 | mild | No Risk |
| 120 | 95.91431916 | 87.20860676 | 0.982 | mild | No Risk |
| 121. | 95.87135567 | 87.53 | 0.9822 | mild | No Risk |
| 122 | 95.85509586 | 87.62148625 | 0.9828 | mild | No Risk |
| 123 | 95.62932748 | 88.44 | 0.9828 | mild | No Risk |
| 124 | 95.60040644 | 88.768957 | 0.985 | mild | No Risk |
| 125 | 95.59658918 | 89.11832898 | 0.985 | mild | No Risk |
| 126 | 95.5513723 | 89.84118985 | 0.9854 | mild | No Risk |
| 127 | 95.50820974 | 90.76257818 | 0.987 | mild | No Risk |
| 128. | 95.39944002 | 90.92094065 | 0.9877 | severe | Risk |
| 129 | 95.38976001 | 91.49512214 | 0.9882 | severe | Risk |
| 130 | 95.37734529 | 91.92283492 | 0.9883 | severe | Risk |
| 131 | 95.3732375 | 92.6710236 | 0.9887 | severe | Risk |
| 132 | 95.16805157 | 92.85283684 | 0.99 | severe | Risk |
| 133 | 95.10799129 | 93.34033768 | 0.99 | severe | Risk |
| 134. | 95.07724753 | 93.72881219 | 0.9904 | severe | Risk |
| 135 | 95.06438019 | 95.12348223 | 0.9911 | severe | Risk |
| 136 | 95.00984608 | 95.37229037 | 0.9912 | severe | Risk |
| 137 | 94.86025082 | 96.06324243 | 0.9917 | severe | Risk |
| 138 | 94.80752628 | 96.08870081 | 0.992 | severe | Risk |
| 139 | 94.74385747 | 96.28593769 | 0.9926 | severe | Risk |
| 140 | 94.74363715 | 98.05924946 | 0.9941 | severe | Risk |
| 141 | 94.54069655 | 99.65307299 | 0.9943 | severe | Risk |
| 142. | 94.30245048 | 101.91 | 0.9944 | pseudocough | Risk |
| 143 | 94.20290978 | 103.59 | 0.9947 | pseudocough | Risk |
| 144 | 93.79926729 | 106.04 | 0.9948 | pseudocough | Risk |
| 145 | 93.72753354 | 107.61 | 0.9959 | pseudocough | Risk |
| 146 | 93.21914653 | 110.3743641 | 0.9961 | pseudocough | Risk |
| 147 | 92.94840735 | 115.0782881 | 0.9962 | pseudocough | Risk |
| 148. | 92.90617323 | 118.2949332 | 0.9968 | pseudocough | Risk |
| 149 | 92.6556727 | 147.8030523 | 0.9968 | pseudocough | Risk |
| 150 | 92.53219516 | 147.8030523 | 0.997 | pseudocough | Risk |
| 151 | 92.05285348 | 147.8030523 | 0.9973 | pseudocough | Risk |
| 152 | 91.25285348 | 147.8030523 | 0.9974 | pseudocough | Risk |
| 153 | 91.0765583 | 147.8030523 | 0.9977 | pseudocough | Risk |

Table 1:  Key dataset from wearable devices and isolated cough intensity measurement device

We can identify potential inferences and correlations between wearable data and cough events, as follows:

## 3.1 Heart Rate, Cough Detected, and Risk Co-relation.

The provided dataset clearly illustrates a strong and positive co-relation between `Cough_detected` values, `Heart Rate (BPM)`, and the ultimate `Risk` classification. As the `Cough_detected` metric increases, there is a consistent and noticeable upward trend in Heart Rate, which is directly associated with a shift from a "No Risk" to a "Risk" status.

Detailed Analysis:

No Risk Scenario (Low `Cough_detected`):

- In the initial entries where `Risk` is designated as "No Risk" (indices 0 through 127), `Cough_detected` values are relatively low, ranging from 0 to approximately 0.987.

- During this phase, `Heart Rate (BPM)` remains within a generally healthy range, starting at 40 BPM and gradually increasing to about 90.76 BPM.

- This indicates that when cough activity (as measured by `Cough_detected`) is minimal to moderate, the heart rate typically stays within expected non-risk parameters.

Transition to Risk (Increasing `Cough_detected` and `Heart Rate`):

- A significant and clear transition occurs around `index` 128. At this point, the `Cough_detected` value rises to 0.9877, and simultaneously, the `Risk` status changes to "Risk."

- Correspondingly, `Heart Rate (BPM)` shows a marked increase, jumping to 90.92 BPM and continuing to escalate. This suggests that a higher `Cough_detected` value acts as a critical threshold that, when crossed, signals an elevated risk and is accompanied by an increased heart rate.

Risk Scenario (High `Cough_detected` and Elevated `Heart Rate`):

- Once the `Risk` status is "Risk" (indices 128 through 153), both `Cough_detected` and `Heart Rate (BPM)` remain consistently elevated.

- Severity "severe": For `severity` classified as "severe" (indices 128-141), `Cough_detected` ranges from 0.9877 to 0.9943. Within this range, `Heart Rate (BPM)` is notably higher, ranging from 90.92 BPM to 99.65 BPM.

- Severity "pseudocough": As `severity` progresses to "pseudocough" (indices 142-153), `Cough_detected` values reach their highest, from 0.9944 to 0.9977. In this `severity` category, `Heart Rate (BPM)` experiences its most substantial elevations, starting at 101.91 BPM and peaking at 147.80 BPM.

The dataset strongly supports a direct and escalating co-relation among `Cough_detected`, `Heart Rate (BPM)`, and `Risk`. As the `Cough_detected` metric increases, `Heart Rate (BPM)` tends to rise, and these combined physiological changes are explicitly associated with an increased likelihood of being classified under "Risk." The data suggests that `Cough_detected` values above approximately 0.98, coupled with heart rates exceeding 90 BPM, are significant indicators of an elevated risk status, intensifying further with higher `Cough_detected` values and heart rates.

## 3.2 Blood Oxygen Level, Cough Detected, and Risk Co-relation.

Co-relation between Blood Oxygen Level (%), Cough Detected, and Risk

The provided dataset clearly illustrates a strong, inverse co-relation between `Blood Oxygen Level (%) ` and `Cough_detected`, which together are critical indicators of the `Risk` status. As `Cough_detected` values increase, `Blood Oxygen Level (%) ` tends to decrease, and this combined physiological shift is directly associated with the progression from a "No Risk" to a "Risk" state, and further into more severe categories.

Detailed Analysis:

"No Risk" Scenario (Low `Cough_detected`, High Blood Oxygen):

- For entries identified as "No Risk" (from index 0 to 127), `Cough_detected` values remain relatively low, ranging from 0 to approximately 0.987.

- During this period, `Blood Oxygen Level (%) ` is consistently high and stable, typically ranging from 100% down to roughly 95.5%.

- This indicates that when cough activity is minimal to moderate, the body's oxygen saturation remains within a healthy and stable range, signifying no immediate risk.

Transition to "Risk" (Increasing `Cough_detected`, Decreasing Blood Oxygen):

- A critical transition point is observed around `index` 128. Here, the `Cough_detected` value increases to 0.9877, and concurrently, the `Risk` status changes to "Risk."

- At this same point, `Blood Oxygen Level (%) ` shows a noticeable and sustained decline, dropping to 95.39944%. This drops below the 95.5% mark, combined with a higher `Cough_detected` value, serves as a clear signal of an elevated risk.

"Risk" Scenario (High `Cough_detected`, Low Blood Oxygen):

- Once the `Risk` status is established as "Risk" (from index 128 to 153), `Cough_detected` values continue to rise, and `Blood Oxygen Level (%) ` continues to decline further.

- "severe" Severity: Within the "severe" severity classification (indices 128-141), `Cough_detected` ranges from 0.9877 to 0.9943. Correspondingly, `Blood Oxygen Level (%) ` is notably lower, falling within the range of 95.39944% down to 94.54069655%.

- "pseudocough" Severity: As the severity progresses to "pseudocough" (indices 142-153), `Cough_detected` values reach their highest, from 0.9944 to 0.9977. In this most severe category, `Blood Oxygen Level (%) ` experiences its most significant decreases, dropping further from 94.30245048% to a low of 91.0765583%.

The data undeniably demonstrates a powerful inverse co-relation between `Blood Oxygen Level (%) ` and `Cough_detected`, which are directly linked to the `Risk` level. As the `Cough_detected` metric increases, `Blood Oxygen Level (%) ` decreases. This combined physiological shift—specifically, `Cough_detected` values exceeding approximately 0.98 and Blood Oxygen Levels falling below 95.5%—serves as a robust indicator of an elevated risk status. The severity of this risk escalates further as `Cough_detected` continues to rise and blood oxygen levels continue to drop, moving from "mild" to "severe" and then "pseudocough" categories

4. RESULTS AND IMPLICATIONS

Integrating cough monitoring with wearable health data via GenAI can improve the accuracy and timeliness of health assessments, facilitate chronic disease management, and support preventive care. Key challenges include ensuring data privacy, model interpretability, and user trust, as well as addressing technical barriers to seamless data integration [3][7][9][14][20]. Future work should focus on real-world piloting, standardization, and ethical governance to maximize clinical impact [7][14][20].

5. CONCLUSION

A GenAI-driven, unified health monitoring ecosystem that integrates cough and physiological data from wearables holds significant promise for advancing personalized, preventive healthcare. This approach can bridge current gaps in digital health, making comprehensive health insights accessible and actionable for all users.

REFERENCES

- Shetty, S., & K, P. (2025). Personalized Predictive Healthcare Using Machine Learning and Generative AI. 2025 International Conference on Artificial Intelligence and Data Engineering (AIDE), 867-872. https://doi.org/10.1109/aide64228.2025.10987453.

- He, R., Sarwal, V., Qiu, X., Zhuang, Y., Zhang, L., Liu, Y., & Chiang, J. (2024). Generative AI Models in Time-Varying Biomedical Data: Scoping Review. Journal of Medical Internet Research, 27.  https://doi.org/10.2196/59792.

- Liu, J., Cen, X., Yi, C., Wang, F., Ding, J., Cheng, J., Wu, Q., Gai, B., Zhou, Y., He, R., Gao, F., & Li, Y. (2025). Challenges in AI-driven Biomedical Multimodal Data Fusion and Analysis. Genomics, Proteomics & Bioinformatics, 23. https://doi.org/10.1093/gpbjnl/qzaf011

- Orlandic, L., Thevenot, J., Teijeiro, T., & Atienza, D. (2023). A Multimodal Dataset for Automatic Edge-AI Cough Detection. 2023 45th Annual International Conference of the IEEE Engineering in Medicine & Biology Society (EMBC), 1-7.  https://doi.org/10.1109/embc40787.2023.10340413

- Patidar, A., Gore, D., Inje, B., & Alhat, A. (2025). AI-Enhanced Depression Detection System: Integrating Multimodal Analysis for Early Diagnosis. 2025 International Conference on Computational, Communication and Information Technology (ICCCIT), 170-175.  https://doi.org/10.1109/icccit62592.2025.10928002

- Kajornkasirat, S., Sawangwong, C., Puangsuwan, K., Chanapai, N., Phutthamongkhon, W., & Puttinaovarat, S. (2025). Integrating AI-Driven Predictive Analytics in Wearable IoT for Real-Time Health Monitoring in Smart Healthcare Systems. Applied Sciences.  https://doi.org/10.3390/app15084400

- Reddy, S. (2024). Generative AI in healthcare: an implementation science informed translational path on application, integration and governance. Implementation Science : IS, 19.  https://doi.org/10.1186/s13012-024-01357-9

- Mohsen, F., Ali, H., Hajj, N., & Shah, Z. (2022). Artificial intelligence-based methods for fusion of electronic health records and imaging data. Scientific Reports, 12.  https://doi.org/10.1038/s41598-022-22514-4

- Shajari, S., Kuruvinashetti, K., Komeili, A., & Sundararaj, U. (2023). The Emergence of AI-Based Wearable Sensors for Digital Health Technology: A Review. Sensors (Basel, Switzerland), 23.  https://doi.org/10.3390/s23239498

- M, S., & Joshi, P. (2025). Early Prediction of Chronic Diseases Through Multimodal Data Fusion in Machine Learning: A Survey. 2025 7th International Conference on Intelligent Sustainable Systems (ICISS), 1108-1115.  https://doi.org/10.1109/iciss63372.2025.11076438

- Alzghaibi, H. (2025). Adoption barriers and facilitators of wearable health devices with AI integration: a patient-centred perspective. Frontiers in Medicine, 12.  https://doi.org/10.3389/fmed.2025.1557054

- Sirapangi, M., & Gopikrishnan, S. (2024). MAIPFE: An Efficient Multimodal Approach Integrating Pre-Emptive Analysis, Personalized Feature Selection, and Explainable AI. Computers, Materials & Continua.  https://doi.org/10.32604/cmc.2024.047438

- Shim, J., Kim, M., Lee, S., Kim,, S., Kwon, J., Song, C., Ahn, K., Kang, S., Park, H., Park, H., Kim, B., & Yang, M. (2023). An artificial intelligence algorithm‐based smartphone application for daily cough monitoring. Allergy, 78.  https://doi.org/10.1111/all.15632

- Chen, Y., & Esmaeilzadeh, P. (2024). Generative AI in Medical Practice: In-Depth Exploration of Privacy and Security Challenges. Journal of Medical Internet Research, 26.  https://doi.org/10.2196/53008

- Acosta, J., Falcone, G., Rajpurkar, P., & Topol, E. (2022). Multimodal biomedical AI. Nature Medicine, 28, 1773 - 1784.  https://doi.org/10.1038/s41591-022-01981-2.

- Kargarandehkordi, A., Li, S., Lin, K., Phillips, K., Benzo, R., & Washington, P. (2025)

Fusing Wearable Biosensors with Artificial Intelligence for Mental Health Monitoring: A Systematic Review. Biosensors, 15.   https://doi.org/10.3390/bios15040202

- Alasmari, S., AlGhamdi, R., Tejani, G., Sharma, S., & Mousavirad, S. (2025). Federated learning-based multimodal approach for early detection and personalized care in cardiac disease. Frontiers in Physiology, 16.  https://doi.org/10.3389/fphys.2025.1563185

- Prashanthi, B., Sharma, A., , M., , K., Riad, R., , A., & Chidambaranathan, S. (2024). AI-Driven Smart Health Monitoring System Using Wearable IoT Devices and Predictive Analytics. 2024 International Conference on IoT, Communication and Automation Technology (ICICAT), 952-956.  https://doi.org/10.1109/icicat62666.2024.10923489

- Ijaz, A., Nabeel, M., Masood, U., Mahmood, T., Hashmi, M., Posokhova, I., Rizwan, A., & Imran, A. (2022). Towards using Cough for Respiratory Disease Diagnosis by leveraging Artificial Intelligence: A Survey. ArXiv, abs/2309.14383  https://doi.org/10.1016/j.imu.2021.100832

- Chen, X., Xie, H., Tao, X., Wang, F., Leng, M., & Lei, B. (2024). Artificial intelligence and multimodal data fusion for smart healthcare: topic modeling and bibliometrics. Artif. Intell. Rev., 57, 91.  https://doi.org/10.1007/s10462-024-10712-7

- Karthick Rajapandiyan, Sridhar L. 2026. Vol. 6 No. Supplementary 1 (2026): Advancement in Management and Technology (AMT), Book of Abstracts,  https://doi.org/10.46977/amt.v06isupp1%20%20  : Page 48