EVALUATION OF ELECTROMAGNETIC FIELD EXPOSURE LEVEL FROM MOBILE PHONES AND ITS HEALTH RISKS

¹Akpolile A.F., ¹Agbajor G.K., ¹Omoriwhovo J.O., ²Adonuja J.A. and ¹Ukerun-Akpesiri A.A

¹Department of Physics, Delta State University, P.M.B 1, Delta State, Nigeria

²Department of Physics, Southern Delta University, Ozoro, Delta State, Nigeria

*Corresponding author: Amuofu J.A/ amuofuj@gmail.com

ABSTRACT

This study investigated the electromagnetic field (EMF) exposure and Specific Absorption Rate (SAR) levels associated with different mobile phone models operating at frequencies of 900 MHz and 2100 MHz. The objective was to assess the electric field strength, magnetic field strength, and power density emitted by these devices, and to determine the potential impact of mobile phone radiation on human health. Using an EMF Multi-Function Meter (GQ EMF-390V2 Electronics), measurements of the electric field (E), magnetic field (H), and power density (S) were obtained. The specific absorption rate (SAR) and health risk index (HRI) were computationally analyzed from dielectric and density data for seven (7) head tissues and the measured electric field over a range of popular mobile phone models and compared their EMF exposure levels against the safety limits set by the International Commission on Non-Ionizing Radiation Protection (ICNIRP). The study found that while most mobile phones complied with ICNIRP safety standards, significant differences in radiation emission were observed between models, with certain devices exceeding recommended SAR thresholds. The findings suggested that radiation exposure varies across phone designs and operating frequencies, with potential implications for long-term health, especially in sensitive tissue areas like the brain and cerebrospinal fluid. This study indicated the importance of adhering to EMF safety guidelines and encourages consumers to be aware of SAR values when selecting mobile devices, particularly those frequently used close to the head.

Key: Electromagnetic field exposure, Specific Absorption Rate, Health Risk Index, Head tissues, Mobile phones

INTRODUCTION
The rapid increase in smartphone ownership and use has solidified the significance of mobile phones in modern culture. Recent figures indicate that 6.6 billion persons, or 83.72% of the global population, own smartphones, a notable rise from 3.7 billion users (49.40%) in 2016 (Ahmad & Shaharun, 2023). In addition to conventional communication, mobilephones have features like internet access, Bluetooth, and digital information exchange, making them important (Akpolile & Ugbede, 2019). Nonetheless, their functionality is intrinsically dependent on electromagnetic field (EMF) radiofrequency (RF) radiation, which is the foundation of all wireless technologies (Ayinmode & Farai, 2013; Akpolile & Ugbede, 2019). 
Mobile communication depends on the exchange of data via radio waves, with networks reaching almost the whole world (Akbal et al., 2012). Although electromagnetic waves are pervasive, their potential health effects often escape public awareness (Damian, 2011; Buckus et al., 2014). Studies indicates that electromagnetic radiation (EMR) emitted by mobile phones and base station antennas may impact human health at the cellular level (Januševičienė & Venckus, 2011; Paulauskas & Klimas, 2011; Akeju et al., 2016). At radio frequencies, EMF enters the human body, with exposure quantified using the Specific Absorption Rate (SAR), a metric signifying energy absorbed per kilogram of tissue (Hillert et al., 2006; Zhang & Alden, 2011). 
Proximity to mobile phones during operation exposes users to substantial electromagnetic fields (Buckus et al., 2014).. The highest radiation intensity is seen within 1–10 cm from the antenna, with peak absorption occurring in epidermal layers at a depth of 1 cm (Baltrenas et al., 2011). Although radiation levels decrease dramatically during standby mode, proximity to base stations also modifies exposure, with shorter distances related to lower radiation levels (Buckus et al., 2014). The electromagnetic fields created by mobile phones, whether steady analog transmissions or digital-pulsed fields, interact with biological tissues. This interaction may generate molecular alterations and thermal impacts, depending on parameters such as radiation intensity, exposure time, and individual biological features (Grigoriev, 2010; Lin, 2002; Buckus et al., 2014). 
The increase in mobile phone usage has led to elevated levels of ambient electromagnetic pollution, prompting assessments of their safety (Mousa, 2011; Buckus et al., 2014; Ahmad & Shaharun, 2023). Dosimetric metrics, including power density (W/m²), electric field strength (V/m), magnetic field strength (A/m), and SAR, are important for measuring RF radiation exposure (Ayinmode & Farai, 2013; Osaigbovo & Isabona, 2018; Akpolile & Ugbede, 2019). Regulatory bodies such as the International Commission on Non-Ionizing Radiation Protection (ICNIRP), the Federal Communications Commission (FCC), and the Institute of Electrical and Electronics Engineers (IEEE) have established SAR limits to mitigate risks associated with tissue heating, particularly for frequencies between 3 kHz and 300 GHz (Zhang & Alden, 2011; Akpolile & Ugbede, 2019; Ahmad & Shaharun, 2023). 
Given the extensive usage of mobile phones and improvements in wireless technology, the public is increasingly exposed to RF radiation. This underscores the need of examining the RF properties of different phone manufacturers and determining SAR values in human tissues. Such studies are vital for understanding the possible health repercussions of cell phone use. Accordingly, this work intends to measure the electric field (E-field), magnetic field (H-field), and power density of regularly used mobile phones, as well as computationally analyse SAR values and health risk index in various tissues of the head.

### MATERIALS AND METHOD

Accurate dosimetric assessment of electromagnetic RF fields is essential for protecting humans from potential health hazards. Field measurements, which utilize electromagnetic RF survey meters or detectors, are typically preferred for on-the-spot and precise evaluations of field quantities at various points of interest. In this study, measurements of the electric field (E), magnetic field (H), and power density (S) were conducted on 34 mobile phones of various models and brands owned by individuals in Delta State, Nigeria. The measurements were performed using an EMF Multi-Function Meter (GQ EMF-390V2 Electronics) in an isolated unit to minimize interference from other RF sources. Background RF field levels were recorded for each measurement and subsequently subtracted to ensure accuracy. The recorded values were compared against internationally established standards for electromagnetic field exposure limits. To quantify the rate of RF radiation absorption in different head tissues, the Specific Absorption Rates (SAR) were calculated. The SAR values were derived using the measured E-field strength, along with the dielectric properties of the head tissues—relative permittivity (ε), electrical conductivity (σ), and density (ρ)—based on the formula:

SAR=σE2ρ							(1)

where σ, E and ρ indicates the electrical conductivity of the biological tissue in (S/m), Electric field intensity emitted by the mobile phone in (V/m) and the density of the biological tissue in  (kg/m3). Calculations were performed at two standard frequencies, 900 MHz and 2100 MHz, reflecting common operating frequencies for mobile phones. The study focused on analyzing the Specific Absorption Rate (SAR) in several head tissues, including the skin, fat, skull, dura mater, brain, cerebrospinal fluid (CSF), and muscle. The dielectric characteristics and densities of various tissues were acquired from Osaigbovo and Isabona (2018) and reported in Table 1. The health risk index (HRI), an important risk indicator, was computed in conformity with the criteria stated by Khurana et al. (2009), where the maximum permissible SAR value as per the International Commission on Non-Ionizing Radiation Protection (ICNIRP) is 2 W/kg over 10 g of tissue. To examine the correlations among electromagnetic frequency factors and their effect on SAR, a correlation matrix was developed. Computational analysis and data visualization were accomplished utilizing the Python programming language in the ANACONDA environment, notably using the Spyder development tool (Sivapriya et al., 2019). Python tools, such as Matplotlib and Seaborn, played a vital role in displaying massive datasets via maps, plots, and trend graphs, allowing effective understanding and transmission of findings. These technologies are essential for comprehending complicated data patterns and making educated choices in data science.

Table1. Dielectric Properties of the Tissues at 900 MHz and 2100 MHz (Osaigbovo and Isabona, 2018)

|  | 900 MHz |  | 2100 MHz |  | Density |
| --- | --- | --- | --- | --- | --- |
|  | εr | σ (S/m) | εr | σ (S/m) | ρ (kg/m3) |
| CSF | 68.6 | 2.41 | 66.76 | 3.15 | 1030 |
| DURA | 44.4 | 0.96 | 42.49 | 1.47 | 1030 |
| Brain | 45.80 | 0.77 | 43.05 | 1.31 | 1030 |
| Muscle | 55.90 | 0.97 | 54.04 | 1.57 | 1480 |
| Skin | 43.88 | 0.86 | 38.43 | 1.31 | 1010 |
| Skull | 20.80 | 0.34 | 15.28 | 0.51 | 1850 |
| Fat | 11.30 | 0.11 | 5.32 | 0.09 | 920 |

RESULTS

The study presents the measured values of electric field strength (E-field), magnetic field strength (H-field), and power density for various mobile phone models at different frequencies, along with the Specific Absorption Rate (SAR) for human head tissues as presented in Table 2 and 3 respectively. The results are categorized based on their operating radio frequencies (900 MHz, 1800 MHz, and 2100 MHz). Tables 4 outlines the ICNIRP recommended safety limits for E-field and H-field strengths, power density, and SAR values at the specified frequencies.

Table 2:  E-field, H-field, Power density values of other phone models and SAR to different tissues of the head at 900 MHz

| Phone Model | E (V/m) | H (A/m) | S (mW/m2) | SAR values of tissues in the head region (W/kg) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | CSF | DURA |  | Brain |  | Muscle | Skin | Skull | Fat |
| Infinix hot | 1.0 | 12.75 | 20.4553 | 0.002340 | 0.000932 |  | 0.000748 |  | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Infinix smart | 1.0 | 0.65 | 20.0925 | 0.002340 | 0.000932 |  | 0.000748 |  | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Infinix note | 1.0 | 0.60 | 0.002340 | 0.000932 | 0.001427 |  | 0.000748 |  | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Vivo y20 | 3.0 | 0.50 | 9.4710 | 0.021058 | 0.008388 |  | 0.006728 |  | 0.005899 | 0.007663 | 0.001654 | 0.001076 |
| Tecno pouvoir | 1.3 | 0.60 | 18.0530 | 0.003954 | 0.001575 |  | 0.001263 |  | 0.001108 | 0.001439 | 0.000311 | 0.000202 |
| Tecno spark | 1.0 | 0.60 | 12.1940 | 0.002340 | 0.000932 |  | 0.000748 |  | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Tecno camon | 1.0 | 0.60 | 12.9900 | 0.002340 | 0.000932 |  | 0.000748 |  | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Tecno F1 | 3.0 | 0.60 | 201.6000 | 0.021058 | 0.008388 |  | 0.006728 |  | 0.005899 | 0.007663 | 0.001654 | 0.001076 |
| Tecno POP | 2.0 | 0.85 | 4.7040 | 0.009359 | 0.003728 |  | 0.002990 |  | 0.002622 | 0.003406 | 0.000735 | 0.000478 |
| OPPO | 2.5 | 0.60 | 87.7650 | 0.014624 | 0.005825 |  | 0.004672 |  | 0.004096 | 0.005322 | 0.001149 | 0.000747 |
| Redmi | 1.0 | 0.50 | 92.6000 | 0.002340 | 0.000932 |  | 0.000748 |  | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Redmi note | 1.5 | 0.60 | 7.2110 | 0.005265 | 0.002097 |  | 0.001682 |  | 0.001475 | 0.001916 | 0.000414 | 0.000269 |
| Nokia 61T | 1.0 | 0.50 | 2.9320 | 0.002340 | 0.000932 |  | 0.000748 |  | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Itel x16pro | 0.7 | 0.60 | 1.6320 | 0.001147 | 0.000457 |  | 0.000366 |  | 0.000321 | 0.000417 | 0.000090 | 0.000059 |
| Blade Va Vita | 4.5 | 0.60 | 470.2000 | 0.047381 | 0.018874 |  | 0.015138 |  | 0.013272 | 0.017243 | 0.003722 | 0.002421 |
| Galaxy | 1.3 | 0.55 | 46.5450 | 0.003954 | 0.001575 |  | 0.001263 |  | 0.001108 | 0.001439 | 0.000311 | 0.000202 |
| Samsung galaxy | 1.0 | 0.53 | 32.5380 | 0.002340 | 0.000932 |  | 0.000748 |  | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Samsung | 1.0 | 0.50 | 11.2200 | 0.002340 | 0.000932 |  | 0.000748 |  | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Huawei P20 lite | 1.0 | 0.60 | 14.1400 | 0.002340 | 0.000932 |  | 0.000748 |  | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Iphone 6 | 2.0 | 0.50 | 2.9840 | 0.009359 | 0.003728 |  | 0.002990 |  | 0.002622 | 0.003406 | 0.000735 | 0.000478 |
| Iphone 6 plus | 2.0 | 0.60 | 11.3900 | 0.009359 | 0.003728 |  | 0.002990 |  | 0.002622 | 0.003406 | 0.000735 | 0.000478 |
| Iphone 6s | 1.0 | 0.16 | 7.1830 | 0.002340 | 0.000932 |  | 0.000748 |  | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Iphone 7 | 1.0 | 0.53 | 29.4680 | 0.002340 | 0.000932 |  | 0.000748 |  | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Iphone 7 plus | 1.0 | 0.60 | 6.1900 | 0.002340 | 0.000932 |  | 0.000748 |  | 0.001061 | 0.000851 | 0.000184 | 0.000120 |
| Iphone 8 plus | 1.7 | 0.53 | 19.5030 | 0.006762 | 0.002694 |  | 0.002160 |  | 0.001894 | 0.002461 | 0.000531 | 0.000346 |
| Iphone X | 1.2 | 0.53 | 18.8760 | 0.003369 | 0.001342 |  | 0.001077 |  | 0.000944 | 0.001226 | 0.000265 | 0.000172 |
| Iphone Xs max | 1.7 | 0.60 | 67.0680 | 0.006762 | 0.002694 |  | 0.002160 |  | 0.001894 | 0.002461 | 0.000531 | 0.000346 |
| Iphone 11 pro | 1.0 | 0.50 | 19.1505 | 0.002340 | 0.000932 |  | 0.000748 |  | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Iphone 11 pro max | 1.0 | 0.57 | 57.8600 | 0.002340 | 0.000932 |  |  | 0.000748 | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Iphone 12 | 1.0 | 0.60 | 21.2700 | 0.002340 | 0.000932 |  |  | 0.000748 | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Iphone 12 pro | 1.0 | 0.60 | 81.7700 | 0.002340 | 0.000932 |  |  | 0.000748 | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Iphone 12 pro max | 1.0 | 0.56 | 116.6500 | 0.002340 |  | 0.000932 |  | 0.000748 | 0.000655 | 0.000851 | 0.000184 | 0.000120 |
| Iphone 13 | 2.0 | 0.50 | 18.5300 | 0.009359 | 0.003728 |  |  | 0.002990 | 0.002622 | 0.003406 | 0.000735 | 0.000478 |

Table 3: E-field, H-field, Power density values of other phone models and SAR to different tissues of the head at 2100 MHz

| Phone Model | E (V/m) | H (A/m) | S (mW/m2) | SAR values to tissues in the head region (W/kg) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | CSF | DURA | Brain | Muscle | Skin | Skull | Fat |
| Infinix hot | 1.0 | 12.75 | 20.4553 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Infinix smart | 1.0 | 0.65 | 20.0925 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Infinix note | 1.0 | 0.60 | 5.1200 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Vivo y20 | 3.0 | 0.50 | 9.4710 | 0.027524 | 0.012845 | 0.011447 | 0.009547 | 0.011673 | 0.002481 | 0.000880 |
| Tecno pouvoir | 1.3 | 0.60 | 18.0530 | 0.005168 | 0.002412 | 0.002149 | 0.001793 | 0.002192 | 0.000466 | 0.000165 |
| Tecno spark | 1.0 | 0.60 | 12.1940 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Tecno camon | 1.0 | 0.60 | 12.9900 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Tecno F1 | 3.0 | 0.60 | 201.6000 | 0.027524 | 0.012845 | 0.011447 | 0.009547 | 0.011673 | 0.002481 | 0.000880 |
| Tecno POP | 2.0 | 0.85 | 4.7040 | 0.012233 | 0.005709 | 0.005087 | 0.004243 | 0.005188 | 0.001103 | 0.000391 |
| OPPO | 2.5 | 0.60 | 87.7650 | 0.019114 | 0.008920 | 0.007949 | 0.006630 | 0.008106 | 0.001723 | 0.000611 |
| Redmi | 1.0 | 0.50 | 92.6000 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Redmi note | 1.5 | 0.60 | 7.2110 | 0.006881 | 0.003211 | 0.002862 | 0.002387 | 0.002918 | 0.000620 | 0.000220 |
| Nokia 61T | 1.0 | 0.50 | 2.9320 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Itel x16pro | 0.7 | 0.60 | 1.6320 | 0.001499 | 0.000699 | 0.000623 | 0.000520 | 0.000636 | 0.000135 | 0.000048 |
| Blade Va Vita | 4.5 | 0.60 | 470.2000 | 0.061930 | 0.028900 | 0.025755 | 0.021481 | 0.026265 | 0.005582 | 0.001981 |
| Galaxy | 1.3 | 0.55 | 46.5450 | 0.005168 | 0.002412 | 0.002149 | 0.001793 | 0.002192 | 0.000466 | 0.000165 |
| Samsung galaxy | 1.0 | 0.53 | 32.5380 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Samsung | 1.0 | 0.50 | 11.2200 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Huawei P20 lite | 1.0 | 0.60 | 14.1400 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Iphone 6 | 2.0 | 0.50 | 2.9840 | 0.012233 | 0.005709 | 0.005087 | 0.004243 | 0.005188 | 0.001103 | 0.000391 |
| Iphone 6 plus | 2.0 | 0.60 | 11.3900 | 0.012233 | 0.005709 | 0.005087 | 0.004243 | 0.005188 | 0.001103 | 0.000391 |
| Iphone 6s | 1.0 | 0.16 | 7.1830 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Iphone 7 | 1.0 | 0.53 | 29.4680 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Iphone 7 plus | 1.0 | 0.60 | 6.1900 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Iphone 8 plus | 1.7 | 0.53 | 19.5030 | 0.008838 | 0.004125 | 0.003676 | 0.003066 | 0.003748 | 0.000797 | 0.000283 |
| Iphone X | 1.2 | 0.53 | 18.8760 | 0.004404 | 0.002055 | 0.001831 | 0.001528 | 0.001868 | 0.000397 | 0.000141 |
| Iphone Xs max | 1.7 | 0.60 | 67.0680 | 0.008838 | 0.004125 | 0.003676 | 0.003066 | 0.003748 | 0.000797 | 0.000283 |
| Iphone 11 pro | 1.0 | 0.50 | 19.1505 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Iphone 11 pro max | 1.0 | 0.57 | 57.8600 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Iphone 12 | 1.0 | 0.60 | 21.2700 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Iphone 12 pro | 1.0 | 0.60 | 81.7700 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Iphone 12 pro max | 1.0 | 0.56 | 116.6500 | 0.003058 | 0.001427 | 0.001272 | 0.001061 | 0.001297 | 0.000276 | 0.000098 |
| Iphone 13 | 2.0 | 0.50 | 18.5300 | 0.012233 | 0.005709 | 0.005087 | 0.004243 | 0.005188 | 0.001103 | 0.000391 |

Table 4: ICNIRP reference levels and recommended limits for general public exposure to time-varying E, H fields, Power density and SAR at different frequency (Akpolile and Ugbede, 2019)

| Frequency (MHz) | E (V/m) | H (A/m) | S (W/m2) | Whole-body average SAR (W/kg) | Localized SAR (head and trunk) (W/kg) | Localized SAR (limbs)(W/kg) |
| --- | --- | --- | --- | --- | --- | --- |
| 900 | 41.25 | 0.111 | 4.5 | 0.08 | 2.00 | 4.00 |
| 1800 | 58.34 | 0.157 | 9.0 | 0.08 | 2.00 | 4.00 |
| 2100 | 61.00 | 0.160 | 10.0 | 0.08 | 2.00 | 4.00 |

Figure 1 presents the correlation matrix analyzing the relationships between electromagnetic field (EMF) parameters, including electric field strength (E), magnetic field strength (H), radiofrequency (RF) power density, and specific absorption rate (SAR) in various head tissues, alongside associated health risk metrics at frequencies of 900 MHz and 2100 MHz. Figure 2 illustrates the RF power density across different phone models at 900 MHz and 2100 MHz. Figure 3 highlights the SAR values for various tissues by phone model at the same frequencies, while Figure 4 shows the corresponding risk metrics for phone models at 900 MHz and 2100 MHz.

Figure 1: Correlation Matrix of EMF Parameters and SAR values at 900 and 2100 MHz

Figure 2: Radio Frequency (RF) Power Density by Phone Model at 900 and 2100 MHz

Figure 3: SAR values for different Tissues by Phone models at  RF of 900 and 2100 MHz

Figure 4: Health risk Index by Phone Model at RF of 900 and 2100 MHz

### DISCUSSION

The study identifies the Blade Va Vita model as exhibiting the highest RF power density (470.2 mW/m²) and SAR values, particularly in cerebrospinal fluid (CSF) and brain tissues, as presented in Tables 2 and 3 and Figure 2. These findings suggest elevated electromagnetic exposure in this model, confirming the work of Gandhi et al. (2012), who reported that phones with higher power densities lead to greater energy absorption in tissues. Similarly, Yi et al. (2023) observed high SAR values in certain phone models can be due to the construction materials and design choices. In contrast, models like the iPhone 12 and Samsung Galaxy consistently demonstrated low RF power density and SAR values, indicative of effective design strategies to minimize electromagnetic exposure. These observations align with safety recommendations by the IEEE and ICNIRP (Yi et al., 2023). The study further highlighted that soft tissues with high water content, such as CSF and brain tissues, exhibit higher SAR values, while tissues like fat and skin show lower absorption rates as indicated in Figure 3. This pattern is consistent with the dielectric properties of tissues described by Gabriel et al. (1996), which indicate that tissues with higher conductivity and permittivity absorb more RF energy. The results emphasize the importance of tissue-specific exposure guidelines, particularly for vulnerable regions like the brain. Strong correlations were observed between electric field strength (E), RF power density, SAR, and health risk metrics, as depicted by the correlation matrix in Figure 1. These correlations reinforce findings from previous studies that electric fields and RF power densities significantly influence energy absorption and health risks. For instance, Iakovidis et al. (2022) emphasized the necessity of continuous RF-EMF monitoring to mitigate potential health risks. However, the weak correlation between magnetic field strength (H) and other parameters suggests that mitigation strategies should prioritize electric fields and RF power density. These findings align with Bonato et al. (2022), who highlighted similar observations in their evaluation of 5G networks. The presence of outliers, such as the Blade Va Vita model, and variability in SAR-to-threshold ratios across devices as shown in Figure 4 indicate inconsistencies in compliance with safety standards. Devices with higher SAR values, including the Blade Va Vita and Tecno F1, may pose increased health risks depending on usage duration and proximity to the user. These concerns are consistent with the studies by Hardell and Carlberg (2015) and Akpolile and Ugbede (2019), which linked high RF exposure to potential thermal effects and carcinogenic risks. These risks could be mitigated through the use of hands-free devices, appropriate phone casing accessories to absorb radiation energy, and adherence to ICNIRP guidelines, as emphasized by Bonato et al. (2022). Conversely, models with consistently low SAR values, such as the iPhone 12 and Samsung Galaxy, highlight compliance with rigorous SAR regulations, underscoring adherence to the safety standards set by ICNIRP.

### CONCLUSION

This study has been able to assess the variability in electromagnetic field (EMF) exposure levels and Specific Absorption Rates (SAR) for different mobile phone models at operational frequencies of 900 MHz and 2100 MHz. The measured electric field strength (E-field), magnetic field strength (H-field), and power density reveal significant differences across phone models, with some exceeding the ICNIRP recommended safety limits. Additionally, SAR values for various tissues (cerebrospinal fluid, dura, brain, muscle, skin, skull, and fat) indicate varying levels of radiation absorption, depending on the phone model and operational frequency. The results of this study indicated the need for strict regulatory measures to ensure mobile phones comply with international safety standards to minimize health risks. Consumers should be aware of SAR values when selecting mobile devices, particularly those frequently used close to the head. They are also advised to use hand-free devices and also employ proper phone casing accessories which have the potential of absorbing part of the radiation energy thereby reducing the exposure to radiation.

REFERENCES

- Ahmad, N. A., & Shaharun, P. F. (2023). SAR Measurement from Mobile Phone and Its Effect to Human Body. ELEKTRIKA-Journal of Electrical Engineering, 22(1), 63-69.

- Akbal, A., Kıran, Y., Şahin, A., Turgut Balik, D., & Balık, H. H. (2012). Effects of electromagnetic waves emitted by mobile phones on germination, root growth, and root tip cell mitotic division of Lens culinaris Medik. Polish Journal of Environmental Studies.

- Akeju, M., Aboyeji, O. S., Salau, H., & Ojo, J. (2016). Assessment of exposure to electromagnetic radiation from GSM antennas within and around two health facilities at the Obafemi Awolowo University, Ile-Ife, Nigeria. Ife Journal of Science, 18(4), 1053-1063.

- Akpolile, A. F., & Ugbede, F. O. (2019). Assessment of radiofrequency radiation levels of mobile phones and evaluation of specific absorption rate to tissues of human head layers. Federal University Wukari Trends in Science and Technology Journal, 4(3), 643-650.

- Ayinmode, B. O., & Farai, I. P. (2013). Measurement and method in radiofrequency radiation exposure assessments. The Pacific Journal of Science and Technology, 14(2), 110-118.

- Bahr, A., Dorn, H., & Bolz, T. (2006). Dosimetric assessment of an exposure system for simulating GSM and WCDMA mobile phone usage. Bioelectromagnetics: Journal of the Bioelectromagnetics Society, The Society for Physical Regulation in Biology and Medicine, The European Bioelectromagnetics Association, 27(4), 320-327.

- Baltrenas, P., Buckus, R., & Vasarevicius, S. (2011). Modelling of the computer classroom electromagnetic field. Elektronika ir elektrotechnika, 109(3), 75-80.

- Bednarek, K. (2010). Electromagnetic action of heavy-current equipment operating with power frequency. International journal of occupational safety and ergonomics, 16(3), 357-368.

- Bonato, M., Dossi, L., Gallucci, S., Benini, M., Tognola, G., & Parazzini, M. (2022). Assessment of human exposure levels due to mobile phone antennas in 5G networks. International Journal of Environmental Research and Public Health, 19(3), 1546.

- Buckus, R., Strukcinskiene, B., & Raistenskis, J. (2014). The assessment of electromagnetic field radiation exposure for mobile phone users. Vojnosanitetski pregled, 71(12).

- Damian, C. (2011). Foşalău С. Sources of indoor noise and options to minimize adverse human health effects. Environ Eng Manage J, 10(3), 393-400.

- Elechi, P., Orike, S., & Agugharam, P. (2019). Analysis and Evaluation of Specific Absorption Rate of GSM Signal in Port Harcourt, Nigeria. Journal of Engineering Research and Reports, 5(4), 1-10.

- Gabriel, S., Lau, R. W., & Gabriel, C. (1996). The dielectric properties of biological tissues: III. Parametric models for the dielectric spectrum of tissues. Physics in medicine & biology, 41(11), 2271.

- Gandhi, O. P., Morgan, L. L., De Salles, A. A., Han, Y. Y., Herberman, R. B., & Davis, D. L. (2012). Exposure limits: the underestimation of absorbed cell phone radiation, especially in children. Electromagnetic biology and medicine, 31(1), 34-51.

- Grigoriev, Y. (2010, April). Electromagnetic fields and the public: EMF standards and estimation of risk. In IOP Conference Series: Earth and Environmental Science (Vol. 10, No. 1, p. 012003). IOP Publishing.

- Hardell, L., & Carlberg, M. (2015). Mobile phone and cordless phone use and the risk for glioma–Analysis of pooled case-control studies in Sweden, 1997–2003 and 2007–2009. Pathophysiology, 22(1), 1-13.

- Hillert, L., Ahlbom, A., Neasham, D., Feychting, M., Järup, L., Navin, R., & Elliott, P. (2006). Call-related factors influencing output power from mobile phones. Journal of exposure science & environmental epidemiology, 16(6), 507-514.

- Iakovidis, S., Apostolidis, C., Manassas, A., & Samaras, T. (2022). Electromagnetic fields exposure assessment in Europe utilizing publicly available data. Sensors, 22(21), 8481.

- Institute of Electrical & Electronics Engineers (IEEE) (2005). Standard for Safety Levels with Respect to Human Exposure to Radiofrequency Electromagnetic Fields 3 kHz to 300 GHz. IEEE Std. C95.1., New York, USA.

- International Commission on Non-ionizing Radiation Protection (ICNIRP) (1988). Guidelines for limiting Exposure to Time-Varying Electric, Magnetic, and Electromagnetic fields (up to 300 GHz). Health Phys., 74: 494-522.

- Januševičienė, I., & Venckus, Z. (2011). The numerical modeling of nitrogen oxides and coal monoxide in the atmosphere, when applying phoenics programme. Journal of Environmental Engineering and Landscape Management, 19(3), 225-233.

- Khurana, V. G., Teo, C., Kundi, M., Hardell, L., & Carlberg, M. (2009). Cell phones and brain tumors: a review including the long-term epidemiologic data. Surgical neurology, 72(3), 205-214.

- Lin, J. C. (2002). Cellular mobile telephones and children. IEEE Antennas and Propagation Magazine, 44(5), 142-145.

- Mousa, A. (2011). Electromagnetic Radiation Measurements and Safety Issues of some Cellular Base Stations in Nablus. Journal of Engineering Science & Technology Review, 4(1).

- Osaigbovo, I. A., & Isabona, J. (2018). Current density and SAR analysis of biological tissues due to radiant electromagnetic waves from base station antennas. Int J Adv Res Phys Sci, 5, 15-23.

- Paulauskas, L., & Klimas, R. (2011). Modeling of the spread of motor transport noise in Šiauliai city. Journal of Environmental Engineering and Landscape Management, 19(1), 62-70.

- Sivapriya, J., Kumar, A., Sai, S. S., & Sriram, S. (2019). Breast cancer prediction using machine learning. International Journal of Recent Technology and Engineering (IJRTE), 8(4), 4879-4881.

- Yi, M., Wu, B., Zhao, Y., Su, T., & Chi, Y. (2023). Safety Assessment and Uncertainty Quantification of Electromagnetic Radiation from Mobile Phones to the Human Head. Applied Sciences, 13(14), 8107.

- Zhang, M., & Alden, A. (2011). Calculation of whole-body SAR from a 100 MHz dipole antenna. Progress In Electromagnetics Research, 119, 133-153.