AI for Climate-Resilient and Adaptive Infrastructure

- Abstract

Climate change is anticipated to exacerbate rainfall patterns, resulting in a heightened risk of pluvial flooding and subsequent disruptions to infrastructure systems. The development of effective adaptation strategies poses challenges due to the long-term and sequential nature of infrastructure investments, significant climate uncertainty, and the intricate interactions between environmental processes and constructed infrastructure.

This research introduces an Artificial Intelligence (AI)-based decision-support framework utilizing reinforcement learning (RL) for long-term climate adaptation planning. The framework is designed as an integrated assessment model that merges rainfall projections, flood modeling, and infrastructure impact evaluation. The RL-based methodology facilitates the learning of adaptive strategies that reconcile infrastructure investment and maintenance expenses with the costs of avoided damage.

The results suggest that AI-driven methodologies can pinpoint optimal spatial and temporal adaptation pathways amidst uncertain climate scenarios, fostering more resilient and economically viable infrastructure systems. This underscores the potential of reinforcement learning as a versatile and robust instrument for adaptive infrastructure planning in the face of climate uncertainty.

- Keywords

Artificial Intelligence, Climate Adaptation, Reinforcement Learning, Pluvial Flooding, Infrastructure Resilience

- Introduction

Between 1998 and 2017, floods represented 43% of all documented climate-related and geophysical disasters, impacting around 2 billion individuals globally. As climate change and global warming continue to intensify, it is anticipated that the frequency and severity of heavy precipitation and flooding incidents will further escalate. Such patterns have been noted in various regions, where extreme rainfall occurrences are increasingly becoming more frequent and severe. Therefore, it is crucial to mitigate the risks associated with these events to protect social, economic, and environmental systems.

Flooding presents considerable challenges to urban infrastructure systems due to their high density, interdependencies, and vulnerability to climate-related threats. These incidents can result in direct physical damage as well as indirect disruptions, which lead to heightened maintenance expenses, diminished system efficiency, and prolonged economic repercussions. For example, severe flooding incidents in urban areas have highlighted the susceptibility of infrastructure systems and the substantial costs linked to recovery and damage mitigation. As a result, it is vital to formulate strategies that not only address immediate effects but also bolster long-term resilience.

Successful adaptation necessitates that investments and interventions integrate both mitigation and adaptation advantages. This approach ensures that enhancements in long-term resilience are factored into cost-benefit analyses, facilitating more informed decision-making. Therefore, comprehending the scale of impacts and related costs is essential for enhancing infrastructure resilience and guaranteeing the safety and sustainability of urban settings.

Although current research emphasizes short-term adaptation strategies, long-term planning over extended periods (e.g., 50–100 years) remains relatively under-researched. This shortcoming diminishes the effectiveness of existing strategies, particularly in the face of uncertain and changing climate conditions. Bridging this gap requires sophisticated approaches capable of addressing these challenges.

4.  Background

4.1 Floods and Infrastructure

Floods are typically categorized according to their causes (such as precipitation, water accumulation, or the failure of technical systems), the severity of their impact, or their type (including river floods, flash floods, and coastal floods). In urban settings, flash floods arise when substantial amounts of water gather due to the limited capacity of soil to absorb it and the inadequacy of drainage or storage systems. These occurrences are frequently initiated by heavy rainfall or cloudbursts, which entail a significant volume of precipitation over a brief period. Such incidents can lead to considerable societal repercussions, encompassing physical damage and economic losses.

The effects of flooding on infrastructure can be generally divided into four categories: direct physical impacts (like structural damage), direct non-physical impacts (such as operational disruptions), indirect physical impacts (stemming from failures in interdependent systems), and indirect non-physical impacts (including diminished accessibility and interruptions in services). Grasping these categories and the mechanisms through which disruptions spread is vital for the development of resilient and adaptive infrastructure systems.

Numerous studies have investigated the economic consequences of flooding, often emphasizing direct costs such as repairs and reconstruction, alongside indirect costs linked to operational inefficiencies and service interruptions. These cost elements are crucial for conducting cost-benefit analyses related to infrastructure investments and adaptation strategies. Nevertheless, decision-making processes frequently emphasize short-term results, which restricts a thorough assessment of long-term risks and the benefits of adaptation.

Moreover, existing flood risk management strategies often neglect to integrate long-term climate forecasts. This oversight leads to approaches that may prove inadequate under future climate scenarios. Consequently, there is a pressing need for frameworks that take into account extended timeframes and the evolving nature of climate risks to bolster infrastructure resilience.

4.2 Artificial Intelligence for Climate Adaptation

Climate adaptation involves modifying systems in response to actual or anticipated climate impacts to mitigate damage and bolster resilience. Successful adaptation necessitates the evaluation of intricate trade-offs among costs, risks, and long-term advantages.

Artificial Intelligence (AI) offers robust tools to facilitate climate adaptation by promoting data-driven decision-making in complex and uncertain contexts. AI-driven frameworks can amalgamate various information sources, capture system dynamics, and assess trade-offs among diverse adaptation strategies. This fosters better alignment between scientific analysis and policy-making.

AI and machine learning methodologies have been utilized across numerous climate-related fields, employing techniques such as causal inference, computer vision, time-series analysis, and uncertainty quantification. These methods are especially beneficial in scenarios with limited data, as they can apply advanced strategies like transfer learning and reinforcement learning to derive significant insights. Reinforcement learning (RL), in particular, is adept at tackling complex, dynamic systems. It facilitates the discovery of optimal policies by learning from interactions with an environment and maximizing long-term rewards. In contrast to conventional optimization methods, RL can effectively manage non-linearity and uncertainty, rendering it suitable for long-term adaptation planning.

In comparison to other dynamic decision-making strategies, RL provides enhanced flexibility by continuously refining strategies based on new information and changing system conditions. Although current applications have showcased the potential of RL in climate-related challenges, many methods still concentrate on reactive measures rather than proactive, long-term adaptation strategies.

Moreover, certain existing frameworks encounter constraints such as limited decision flexibility and computational scalability.

5. Problem Formulation

The aim is to develop an adaptive policy that mitigates the effects of flooding on infrastructure systems through the use of reinforcement learning (RL). Utilizing the established policy, a series of adaptation measures can be formulated to lessen long-term consequences while considering uncertain future rainfall scenarios.

To depict adaptation measures and their effects, the system is structured as an Integrated Assessment Model (IAM). The IAM framework encompasses various components, including a rainfall projection model, a flood model, and an infrastructure impact assessment module. Together, these components facilitate the assessment of climate risks and the determination of optimal adaptation strategies.

5.1 Rainfall Projection Model

The initial element of the Integrated Assessment Model (IAM) is a rainfall projection model designed to simulate rainfall occurrences over an extended timeframe (2024–2100). Future rainfall statistics are evaluated under various climate scenarios, which encompass conservative, intermediate, and extreme conditions.

For each designated time period, rainfall distributions are formulated utilizing cumulative distribution functions (CDFs), with the assumption of stationarity within each time segment. Utilizing these distributions, rainfall events are sampled at each timestep and depicted as accumulated daily rainfall totals.

This probabilistic methodology facilitates the creation of rainfall scenarios that mirror potential future climate variability. The assumptions incorporated in the model may result in increased rainfall intensities, thereby illustrating conservative or worst-case scenarios. These generated rainfall events are utilized as inputs for subsequent flood modeling and infrastructure impact evaluations.

5.2 Flood Model

For every simulated rainfall event, the associated flooding behavior is modeled throughout the study area. Flood events can differ in intensity, ranging from minor water accumulation to severe extreme events triggered by heavy rainfall.

The flood modeling methodology employs high-resolution terrain and surface data to replicate water flow dynamics. The movement of water is influenced by elevation, surface characteristics, and natural drainage patterns. During each rainfall event, it is assumed that water is distributed uniformly across the area and subsequently redistributed based on terrain features.

As water gathers in low-lying regions, depressions fill until their capacity is surpassed, at which point overflow occurs, and water continues to flow downstream. This mechanism allows for the identification of flood-prone areas and the estimation of water depth levels throughout the infrastructure system.

The resulting flood depth data is then utilized to evaluate potential impacts on infrastructure, serving as a vital input for adaptive decision-making within the AI framework.

6. Methodological Validation on a Reduced Problem

The adaptation to climate change concerning pluvial flooding constitutes a sequential decision-making challenge characterized by profound uncertainty, where current investments have a significant impact on long-term economic and infrastructural results. The intricacy of this issue stems from unpredictable climate variations, non-linear system behaviors, and a vast array of potential adaptation strategies. Such complexities render conventional methods like dynamic programming and static optimization impractical due to their computational demands.

Reinforcement learning (RL) offers a robust framework to tackle this issue by facilitating the development of adaptive, state-dependent policies that maximize long-term cumulative rewards without the necessity for explicit transition probabilities.

To assess the effectiveness of the proposed framework, controlled experiments are performed on simplified problem instances. These reduced scenarios enable a direct comparison with well-established optimization methods while maintaining the core features of sequential decision-making amidst uncertainty.

6.1 Bayesian Optimization as a Baseline

Bayesian Optimization (BO) is a commonly utilized method for optimizing costly, black-box functions where evaluations are both computationally intensive and limited in quantity. It has proven effective in simulation-based models, especially in the optimization of climate and energy systems.

BO acts as an appropriate baseline for comparison with reinforcement learning (RL) methods. Nevertheless, its effectiveness diminishes in high-dimensional contexts due to the curse of dimensionality, which greatly escalates computational complexity.

To mitigate this issue, BO is employed solely on reduced problem instances where it remains computationally manageable. The aim of this comparison is not to demonstrate performance superiority, but to explore methodological distinctions, particularly regarding adaptivity and the management of sequential decisions.

In this research, BO is executed using a Gaussian process surrogate model featuring a Matérn 5/2 kernel and automatic relevance determination (ARD), which ensures a balance between flexibility and smoothness. Initial training is conducted with randomly generated feasible action plans, while optimization is directed through a batch acquisition strategy based on expected improvement. Convergence is established when no significant enhancement in reward is detected over successive iterations.

6.2 Simplified Problem Definition

In order to facilitate a comparison between Bayesian Optimization (BO) and Reinforcement Learning (RL) methodologies, two simplified problem instances have been established.

Experiment A: Optimization over a 5-year period across 10 spatial zones

Experiment B: Optimization over a 10-year period across 29 spatial zones

These simplified scenarios serve as condensed representations of the comprehensive problem, which encompasses long-term planning spanning multiple decades and broader spatial dimensions.

Both methodologies seek to determine optimal policy sets that reduce combined impacts and adaptation expenses. While BO approaches the issue as a static optimization challenge, the RL-based method conceptualizes it as a sequential decision-making process, thereby capturing temporal relationships and facilitating adaptive decision-making.

For the RL implementation, the previously defined environment is utilized, with a limited action space that aligns with the reduced temporal and spatial dimensions. This setup enables a fair and computationally manageable comparison between the two approaches.

Table 1 displays the overall reward achieved through Bayesian Optimization (BO) and the suggested Reinforcement Learning (RL)-based approach. The findings are presented as averages from three separate runs, including their respective standard deviations, for both reduced problem configurations (Experiment A and Experiment B) and across three climate scenarios (RCP2.6, RCP4.5, and RCP8.5).

| Exp. | Climate Scenario | BO (Mean ± Std) | RL (Mean ± Std) |
| --- | --- | --- | --- |
| A | RCP2.6 | -119.58 ± 0.25 | -118.84 ± 1.10 |
|  | RCP4.5 | -120.10 ± 0.38 | -119.01 ± 0.85 |
|  | RCP8.5 | -121.24 ± 0.30 | -119.23 ± 1.28 |
| B | RCP2.6 | -121.19 ± 1.45 | -117.87 ± 0.57 |
|  | RCP4.5 | -120.51 ± 1.71 | -117.09 ± 0.64 |
|  | RCP8.5 | -123.71 ± 1.42 | -119.88 ± 1.54 |

7. Results

Table 1 illustrates the overall impact, which is defined as the aggregation of flood damages and adaptation expenses, achieved through Bayesian Optimization (BO) and the proposed Reinforcement Learning (RL)-based method across three climate scenarios. The findings are presented for both simplified problem settings: Experiment A (5 years, 10 zones) and Experiment B (10 years, 29 zones).

In both experiments and across all climate scenarios, the RL-based method consistently demonstrates lower total impacts in comparison to the BO configuration. In Experiment A, RL surpasses BO by approximately 0.6% to 1.7%, which corresponds to reductions of around 73 million to 200 million DKK, depending on the climate scenario.

This performance advantage is further accentuated in Experiment B, where the extended planning horizon and larger spatial scale heighten the complexity of the problem. In this context, RL again consistently outperforms BO across all scenarios, with enhancements ranging from approximately 2.7% to 3.1%, translating to reductions between 332 million and 383 million DKK.

The widening performance gap from Experiment A to Experiment B underscores the efficacy of the RL-based approach in addressing higher-dimensional challenges. Specifically, it illustrates that adaptive, state-dependent policies become increasingly advantageous as the complexity and scale of climate-resilient infrastructure planning issues expand.

8. Conclusions

This research investigated the use of reinforcement learning in the context of long-term climate adaptation planning. The suggested multi-modular framework facilitates the development of adaptive policies that weigh investment costs against the benefits of reduced flood impacts, thereby improving the resilience of infrastructure systems in the face of evolving climate conditions.

The findings indicate that the developed policies create synchronized spatial and temporal adaptation pathways. These pathways aid in minimizing infrastructure damage while ensuring that investment costs remain stable and moderate over time.

Recent climate studies have underscored the necessity of creating models that consider various climate scenarios, incorporate uncertainty, and address distributional impacts. In this regard, future research should aim to expand the proposed framework to include probabilistic climate data, adaptive belief updating, and multi-objective formulations that explicitly reflect wider societal impacts, such as equity and well-being.

Moreover, aligning the framework with stakeholder-driven planning processes presents a promising avenue for enhancing its practical relevance, especially in meeting local planning requirements and context-specific adaptation strategies.

In summary, the reinforcement learning framework introduced in this study offers a scalable and adaptable method for facilitating climate adaptation policy. With additional improvements, it holds the potential to support robust, long-term adaptation pathways that remain effective across various climate scenarios.

## References

Balandat, M., Karrer, B., Jiang, D. R., Daulton, S., Letham, B., Wilson, A. G., & Bakshy, E. (2020). BoTorch: A framework for efficient Monte Carlo Bayesian optimization. Advances in Neural Information Processing Systems, 33.

Bowes, B. D., Tavakoli, A., Wang, C., Heydarian, A., Behl, M., Beling, P. A., & Goodall, J. L. (2021). Flood mitigation in coastal urban catchments using real-time stormwater infrastructure control and reinforcement learning. Journal of Hydroinformatics, 23, 529–547.

Cheong, S. M., Sankaran, K., & Bastani, H. (2022). Artificial intelligence for climate change adaptation. Wiley Interdisciplinary Reviews: Data Mining and Knowledge Discovery, 12, e1459.

Cuaran Bermudez, A. F., & Lundberg, L. (2015). Design of bioretention planters for stormwater flow control and removal of toxic metals and organic contaminants. Master’s thesis, Chalmers University of Technology.

Damberg, H. P. (2021). Increased cloudburst events in Denmark. Danish Meteorological Institute.

Danmarks Meteorologiske Institut (2011). Adaptation to the future climate in Denmark.

Danmarks Meteorologiske Institut (2023). Climate Atlas data.

Diakakis, M., Deligiannakis, G., Antoniadis, Z., Melaki, M., Katsetsiadou, N., Andreadakis, E., Spyrou, N., & Gogou, M. (2020). A flash flood impact severity scale for classification and mapping of flood impacts. Journal of Hydrology, 590, 125452.

Feng, K., Lin, N., Kopp, R. E., Xian, S., & Oppenheimer, M. (2025). Reinforcement learning–based adaptive strategies for climate change adaptation: Application to flood risk management. Proceedings of the National Academy of Sciences, 122, e2402826122.

Forrester, A., Sobester, A., & Keane, A. (2008). Engineering design via surrogate modelling: A practical guide. Wiley.

Frazier, P. I. (2018). A tutorial on Bayesian optimization. arXiv preprint arXiv:1807.02811.