Hydro-geophysical Mapping of Borehole Yield using Geophysical Method in Vandeikya Local Government Area, Benue State, Nigeria

Abstract

This study presents an integrated hydro-geophysical investigation of groundwater potential and borehole yield in Vandeikya Local Government Area, Benue State, Nigeria, using the Vertical Electrical Sounding (VES) technique with the Schlumberger array. Eighteen (18) VES stations were systematically distributed to ensure adequate spatial coverage of both upland and lowland terrains, characterized by moderately undulating topography that influences groundwater recharge and flow dynamics. Apparent resistivity values range from less than 50 Ωm to greater than 8000 Ωm, reflecting significant subsurface heterogeneity controlled by lithology, weathering, and moisture distribution. Low resistivity zones (<200 Ωm) indicate clay-rich or water-saturated formations, while high resistivity values (>1000 Ωm) suggest resistive materials such as fresh basement rocks. Geoelectrical interpretation reveals seven to nine subsurface layers, grouped into topsoil, weathered basement, fractured basement, and fresh basement. The weathered and fractured basement units form the principal aquifer system, with resistivity values of 46.5–749 Ωm and thicknesses ranging from 0.6 to 46.0 m. Areas such as Akar, Mbayongo, Achwa, and Ishan exhibit relatively thick overburden and enhanced groundwater storage potential. Dar-Zarrouk parameters show longitudinal conductance values of 0.001–0.731 mhos and transverse resistance values of 111 to >12,000 Ωm². High conductance values indicate moderate to good aquifer protective capacity, while high transverse resistance reflects significant transmissivity and groundwater yield potential. The results demonstrate that aquifer thickness, resistivity, and transmissivity are the primary controls on borehole productivity, confirming the effectiveness of the Schlumberger VES method for groundwater exploration in Basement Complex terrains.

Keywords: Vertical Electrical Sounding (VES) , Groundwater Potential , Basement Complex Terrain , Dar-Zarrouk Parameters  and Aquifer Characterization

## INTRODUCTION

Water is an indispensable natural resource that underpins human survival, agricultural productivity, industrial development, and environmental sustainability. In recent decades, increasing population pressure, rapid urbanization, climate variability, and widespread contamination of surface water bodies have significantly intensified reliance on groundwater resources, particularly in developing countries such as Nigeria (Offodile, 2002; Todd and Mays, 2005; Adelana & MacDonald, 2008; MacDonald et al., 2012; Ahmed et al., 2020). Groundwater is generally regarded as a more reliable and resilient source; however, its availability and sustainability are strongly controlled by subsurface geological and hydrogeological conditions (Taylor et al., 2013; Cuthbert et al., 2019).

In Basement Complex terrains, which underlie large parts of Nigeria, groundwater occurrence is inherently limited due to the negligible primary porosity of crystalline rocks. Consequently, groundwater is largely confined to zones of secondary porosity developed through weathering and fracturing processes (Olorunfemi and Fasuyi, 1993; Oseji et al., 2005; Wright et al., 2016). The spatial variability and discontinuity of these aquifer units often result in unpredictable borehole yields and high failure rates, especially where borehole siting is conducted without adequate subsurface investigation (Ako and Olorunfemi, 1989; MacDonald et al., 2021). This challenge is evident in many parts of Benue State, including Vandeikya Local Government Area, where numerous boreholes exhibit inconsistent performance.

To mitigate these challenges, geophysical methods, particularly electrical resistivity techniques—have been widely adopted for groundwater exploration. The Vertical Electrical Sounding (VES) method using the Schlumberger array has proven to be highly effective due to its cost-efficiency, operational simplicity, and ability to provide reliable information on subsurface layering and aquifer characteristics (Zohdy et al., 1974; Telford et al., 1990; Niwas & Singhal, 1981; Soupios et al., 2007; Emenike, 2018). Recent advances in hydrogeophysics further demonstrate the integration of resistivity data with hydrogeological parameters for improved aquifer characterization and yield prediction (Binley et al., 2015; Aizebeokhai & Oyeyemi, 2018).

Previous studies have demonstrated the applicability of VES in delineating aquifer zones and estimating hydrogeological parameters in Basement Complex environments (Olorunfemi and Fasuyi, 1993; Oseji and Okolie, 2011; Akinlalu et al., 2017). However, most existing studies focus primarily on qualitative aquifer delineation, with limited integration of quantitative parameters such as longitudinal conductance and transverse resistance for direct borehole yield prediction (Niwas & Singhal, 1981; Ekanem et al., 2020).

Furthermore, there remains a significant gap in the spatial mapping of borehole yield potential based on integrated hydro-geophysical parameters within Vandeikya and similar terrains. Many groundwater investigations do not adequately link resistivity-derived parameters with aquifer productivity indicators, resulting in suboptimal decision-making for borehole development (Oladapo et al., 2019; Olayinka & Olorunfemi, 2021). This underscores the need for a more robust, data-driven approach that combines geoelectrical interpretation with quantitative aquifer evaluation.

This study addresses these gaps by integrating detailed VES data with computed aquifer parameters to map borehole yield potential across Vandeikya LGA. The innovation of this research lies in the combined use of resistivity interpretation, aquifer thickness analysis, longitudinal conductance (protective capacity), and transverse resistance (transmissivity proxy) to delineate high yield groundwater zones. By establishing a clear relationship between these parameters and borehole productivity, the study advances beyond conventional groundwater exploration approaches and provides a more reliable framework for sustainable borehole siting in Basement Complex terrains.

The specific objectives are to: characterize the subsurface lithology using VES data, determine aquifer parameters controlling groundwater occurrence, and develop a spatial framework for mapping borehole yield potential within the study area. The outcomes of this study are expected to significantly improve groundwater exploration success rates and contribute to sustainable water resource management in Vandeikya and similar geological settings.

## MATERIALS AND METHODS

### Study Area Description

The study was conducted in Vandeikya Local Government Area (LGA), located in the southeastern part of Benue State, Nigeria, between latitudes 6°48′–7°12′N and longitudes 8°42′–9°06′E, covering an approximate area of 591 km². The area is predominantly agrarian, with major crops including yam, cassava, and rice.

Geologically, the area is underlain by Precambrian Basement Complex rocks, mainly granite, gneiss, and migmatite, which are characterized by negligible primary porosity. Groundwater occurrence is therefore controlled by secondary porosity resulting from weathering and fracturing (Olorunfemi and Fasuyi, 1993; Offodile, 2002) as presented in figure 1. The climate is tropical wet-and-dry, with annual rainfall ranging from 1200 to 1800 mm and temperatures between 24°C and 35°C, necessitating a high dependence on groundwater resources.

Figure 1: Location map of the study area

### Theoretical Background

#### Electrical Resistivity Method

The electrical resistivity method is based on the principle that subsurface materials exhibit varying resistance to the flow of electrical current depending on their composition, porosity, and fluid content (Telford et al., 1990). The method is governed by Ohm’s Law, expressed as presented in equation (1).

V=IR 											(1)

where V is the potential difference (V), I is the current (A), and R is the resistance (Ω).

For a homogeneous medium, the resistivity (ρ) is given by equation (2).

ρ=KΔVI 											(2)

where K is the geometric factor dependent on electrode configuration.

For the Schlumberger array, the geometric factor is defined as presented in equation (3)

K=π(AB)24MN 											(3)

where AB is the current electrode spacing and MN is the potential electrode spacing (Zohdy et al., 1974). The apparent resistivity (ρa) obtained represents an average resistivity of subsurface layers.

#### Groundwater Flow Theory

Groundwater movement in porous media is governed by Darcy’s Law as presented in equation (4)

Q=-KAdhdl 											(4)

where Q is the discharge, K is the hydraulic conductivity, A is the cross-sectional area, and dhdl is the hydraulic gradient (Fetter, 2001; Todd and Mays, 2005).

In Basement Complex terrains, groundwater occurs mainly within weathered and fractured zones, which serve as aquifers due to enhanced permeability and storage capacity (Olorunfemi and Fasuyi, 1993).

#### Dar-Zarrouk Parameters

Aquifer properties were further evaluated using Dar-Zarrouk parameters (Maillet, 1947; Henriet, 1976) as shown in equation (5) and (6).

S=i=1nhiρi 											(5)

T=i=1nhiρi 										(6)

where hi and ρi are the thickness and resistivity of the ith layer, respectively.

Longitudinal conductance (S) is an indicator of aquifer protective capacity, while transverse resistance (T) correlates with transmissivity and groundwater yield potential (Henriet, 1976; Olorunfemi and Fasuyi, 1993).

### Survey Design and Data Acquisition

A total of eighteen (18) VES stations were strategically established across the study area to capture geological variability, including upland and lowland regions as well as areas near existing boreholes. The survey design ensured adequate spatial coverage for reliable hydrogeophysical interpretation.

Field measurements were conducted using a resistivity meter (e.g., ABEM Terrameter) with the Schlumberger electrode configuration. Four electrodes were used: two current electrodes (C1 and C2) and two potential electrodes (P1 and P2).

The current electrode spacing (AB/2) was progressively increased from 1 m to a maximum of approximately 300–500 m to probe deeper subsurface layers. At each spacing, the injected current and resulting potential difference were measured, and the apparent resistivity was computed.

Quality control measures included repeat measurements at selected spacings, identification of noisy data, and documentation of possible sources of error such as cultural interference (power lines, metallic objects) and near-surface inhomogeneities.

### Data Processing and Interpretation

The acquired data were plotted as sounding curves of apparent resistivity versus electrode spacing on a log–log scale. Curve types (A, H, K, Q) were identified to provide preliminary insight into subsurface layering (Zohdy et al., 1974).

Quantitative interpretation was carried out using iterative curve matching and computer-based inversion software such as IPI2Win and WinResist. This process yielded layer resistivities and thicknesses, which were used to construct geoelectric sections and identify aquifer units.

Aquifer zones were delineated based on resistivity contrasts and thickness. Moderate resistivity values (typically 100–400 Ωm) with significant thickness were interpreted as water-bearing formations, while very low resistivity indicated clay-rich zones and high resistivity corresponded to fresh basement rocks.

### Estimation of Aquifer Parameters and Borehole Yield

Aquifer resistivity and thickness derived from VES interpretation were used to compute Dar-Zarrouk parameters (S and T). These parameters were employed as proxies for aquifer protective capacity and transmissivity, respectively (Henriet, 1976; Oseji and Okolie, 2011).

Borehole yield potential was estimated indirectly based on the relationship between resistivity, thickness, and transmissivity. Zones with moderate resistivity and high transverse resistance were classified as high-yield areas, while zones with low thickness and high resistivity were classified as low-yield areas.

Where available, existing borehole and pumping test data were used for calibration to improve the reliability of yield predictions. Yield classes were defined as low, moderate, and high based on inferred transmissivity and aquifer characteristics.

### Hydro-geophysical Mapping

All interpreted VES parameters, including aquifer depth, thickness, resistivity, longitudinal conductance, and transverse resistance, were integrated with geographic coordinates using GIS-based interpolation techniques.

Hydro-geophysical maps were generated to depict spatial variations in groundwater potential and borehole yield across the study area. These maps provide a basis for identifying suitable locations for sustainable groundwater development.

# RESULTS AND DISCUSSION

## Spatial Distribution of VES Stations

The spatial distribution of the Vertical Electrical Sounding (VES) stations (Table 1) indicates adequate coverage of the study area, encompassing both upland and lowland geomorphic units. Elevation ranges from approximately 137 m (Ihugh) to 270 m (Tsambe), reflecting a moderately undulating terrain. Such topographic variation plays a significant role in groundwater recharge and subsurface flow dynamics, as higher elevations typically serve as recharge zones while lower act as discharge or accumulation zones (Todd and Mays, 2005).

The relatively uniform spread of stations enhances the reliability of spatial interpolation used in generating resistivity and hydrogeophysical maps (Figures 2-7).

Table 1 presents the geographical distribution of the VES stations across the study area.

### Table 1. Description of VES Stations in the Study Area

| Station ID | Location | Latitude (N) | Longitude (E) | Elevation (m) |
| --- | --- | --- | --- | --- |
| VKY 1 | Vandeikya HQ | 5.74500 | 7.52000 | 180 |
| VKY 2 | Gbagbongom | 5.76550 | 7.48520 | 210 |
| VKY 3 | Mbakaange | 5.75580 | 7.51040 | 165 |
| VKY 4 | Hemde | 5.76155 | 7.46685 | 255 |
| VKY 5 | Ihugh | 5.75040 | 7.53525 | 137 |
| VKY 6 | Unde | 5.74607 | 7.51130 | 152 |
| VKY 7 | Kaanga | 5.72225 | 7.52792 | 170 |
| VKY 8 | Mbayongo | 5.77023 | 7.47132 | 204 |
| VKY 9 | Mbayongo II | 5.77920 | 7.50312 | 148 |
| VKY 10 | Tsambe | 5.77185 | 7.45642 | 270 |
| VKY 11 | Abwa | 5.76928 | 7.49707 | 148 |
| VKY 12 | Ishan | 5.74345 | 7.50350 | 170 |
| VKY 13 | Akar | 5.74963 | 7.54063 | 125 |
| VKY 14 | Aginde | 5.73445 | 7.47153 | 251 |
| VKY 15 | Agirgba | 5.71735 | 7.53594 | 162 |
| VKY 16 | Bako | 5.83278 | 7.43394 | 156 |
| VKY 17 | Achwa | 5.83653 | 7.41399 | 201 |
| VKY 18 | Agbile | 5.73505 | 7.48444 | 228 |

## Apparent Resistivity Characteristics

The apparent resistivity values (Table 2) exhibit significant lateral variation across the study area, ranging from less than 50 Ωm to values exceeding 8000 Ωm. This wide range reflects strong heterogeneity in subsurface lithology and hydrogeological conditions.

Low resistivity zones (<200 Ωm), observed in locations such as Gbagbongom, Abwa, and Mbayongo, are indicative of clay-rich materials and/or water-saturated formations. These materials typically possess low permeability but may act as protective layers against contamination. Conversely, high resistivity values (>1000 Ωm), recorded in Tsambe, Akar, and Agbile, suggest the presence of resistive formations such as fresh basement rocks or dry, compacted sands.

The spatial distribution of resistivity (Figure 2) highlights distinct conductive and resistive zones, confirming the heterogeneous nature of the Basement Complex terrain. These findings are consistent with established studies, where resistivity is primarily controlled by lithology, porosity, degree of weathering, and fluid saturation (Telford et al., 1990; Olorunfemi and Fasuyi, 1993).

Table 2: Apparent Resistivity Data

| Station ID | Min ρa (Ωm) | Max ρa (Ωm) | Mean ρa (Ωm) | Resistivity Class |
| --- | --- | --- | --- | --- |
| VKY 1 | <100 | >8000 | 2005 | Very High Range |
| VKY 2 | 50 | 500 | 200 | Low-Moderate |
| VKY 3 | 200 | 1200 | 600 | Moderate |
| VKY 4 | 180 | 950 | 400 | Moderate |
| VKY 5 | 150 | 700 | 350 | Moderate |
| VKY 6 | 200 | 800 | 420 | Moderate |
| VKY 7 | 120 | 600 | 300 | Moderate |
| VKY 8 | 46 | 500 | 220 | Low |
| VKY 9 | 60 | 650 | 280 | Low-Moderate |
| VKY 10 | 500 | >8000 | 2000 | High |
| VKY 11 | 70 | 500 | 250 | Low |
| VKY 12 | 60 | 450 | 230 | Low |
| VKY 13 | 100 | 1200 | 450 | Moderate |
| VKY 14 | 400 | 2000 | 900 | High |
| VKY 15 | 200 | 900 | 500 | Moderate |
| VKY 16 | 80 | 600 | 300 | Low-Moderate |
| VKY 17 | 110 | 1000 | 420 | Moderate |
| VKY 18 | 600 | >8000 | 2500 | Very High |

Figure 2:Apparent resistivity map of the study area

## Geoelectric Layering and Aquifer Geometry

Interpretation of the VES data (Table 3) reveals a multilayered subsurface structure comprising between seven and nine geoelectric layers. These layers are broadly classified into topsoil, weathered basement, fractured basement, and fresh basement.

The weathered and fractured basement units constitute the principal aquifer horizons. Aquifer resistivity values range from 46.5 Ωm to 749 Ωm, while aquifer thickness varies significantly from 0.6 m to 46.0 m. Areas such as Akar (46.0 m), Achwa (28.6 m), Mbayongo (34.0 m), and Ishan (24.0 m) are characterized by relatively thick overburden, suggesting enhanced groundwater storage capacity.

In contrast, locations including Vandeikya HQ, Kaanga, and Aginde exhibit very thin overburden (<1 m), indicating limited groundwater potential. The depth to basement ranges from 70 m to 180 m, with deeper basement zones generally associated with better-developed weathering profiles.

The spatial patterns observed in the aquifer resistivity (Figure 3), aquifer thickness (Figure 4), and depth-to-basement maps (Figure 5) further corroborate these interpretations. The variability in aquifer geometry reflects differential weathering and fracturing processes, which are key controls on groundwater occurrence in crystalline basement terrains (Oseji et al., 2005).

# Table 3: Geoelectric Layer & Aquifer Geometry

| Station ID | No. of Layers | Aquifer Resistivity (Ωm) | Aquifer Thickness (m) | Depth to Basement (m) | Lithology Interpretation |
| --- | --- | --- | --- | --- | --- |
| VKY 1 | 7 | 582 | 0.7 | 75 | Thin weathered layer |
| VKY 2 | 8 | 49.1 | 5.1 | 90 | Clayey/weathered |
| VKY 3 | 7 | 223 | 0.5 | 80 | Sandy/weathered |
| VKY 4 | 8 | 232 | 0.6 | 85 | Weathered basement |
| VKY 5 | 7 | 180 | 1.2 | 95 | Sandy clay |
| VKY 6 | 8 | 251 | 12.6 | 110 | Fractured basement |
| VKY 7 | 7 | 163 | 0.7 | 70 | Thin overburden |
| VKY 8 | 9 | 46.5 | 34.0 | 150 | Thick weathered zone |
| VKY 9 | 8 | 60 | 18.0 | 130 | Weathered/fractured |
| VKY 10 | 8 | 749 | 16.1 | 140 | Fractured basement |
| VKY 11 | 7 | 76.7 | 20.2 | 120 | Clayey aquifer |
| VKY 12 | 8 | 63.6 | 24.0 | 135 | Saturated zone |
| VKY 13 | 9 | 114 | 46.0 | 180 | Highly fractured |
| VKY 14 | 7 | 546 | 0.6 | 80 | Fresh basement |
| VKY 15 | 8 | 200 | 3.0 | 100 | Weathered layer |
| VKY 16 | 7 | 74.4 | 6.4 | 95 | Clayey sand |
| VKY 17 | 9 | 118 | 28.6 | 160 | Productive aquifer |
| VKY 18 | 8 | 601 | 14.0 | 140 | Fractured basement |

Figure 3:Aquifer resistivity map of the study area

Figure 4: Aquifer thickness map of the study area

Figure 5: Depth to basement map of the study area

Figure 6: Longitudinal conductance map of the study area

Figure 7: Transverse resistance map of the study area

## Dar-Zarrouk Parameters and Aquifer Protective Capacity

The computed Dar-Zarrouk parameters (Table 4) provide further insight into aquifer characteristics and groundwater potential.

Longitudinal conductance (S), which reflects the protective capacity of overburden materials, ranges from 0.001 to 0.731 mhos. High S values recorded in Mbayongo, Ishan, Abwa, and Akar indicate moderate to high protective capacity, suggesting reduced vulnerability to surface contamination. Conversely, low S values in Vandeikya HQ and Aginde indicate poor protective cover and increased susceptibility to contamination (Henriet, 1976).

Transverse resistance (T), a proxy for transmissivity, varies widely from 111 Ωm² to over 12,000 Ωm². High T values observed in Tsambe, Agbile, and Akar indicate zones of high transmissivity and significant groundwater yield potential. These results are consistent with previous findings that associate high transverse resistance with productive aquifer systems (Oseji and Okolie, 2011).

The spatial distribution of longitudinal conductance (Figure 6) and transverse resistance (Figure 7) clearly delineates zones of varying groundwater potential and aquifer protection.

# Table 4: Dar-Zarrouk Parameters

| Station ID | Location | ρ (Ωm) | h (m) | S (mhos) | T (Ωm²) | Protective Capacity | Transmissivity Class |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VKY 1 | Vandeikya HQ | 582 | 0.7 | 0.001 | 407 | Poor | Low |
| VKY 2 | Gbagbongom | 49.1 | 5.1 | 0.103 | 250 | Moderate | Low |
| VKY 3 | Mbakaange | 223 | 0.5 | 0.002 | 111 | Poor | Low |
| VKY 4 | Hemde | 232 | 0.6 | 0.003 | 139 | Poor | Low |
| VKY 6 | Unde | 251 | 12.6 | 0.050 | 308 | Moderate | Moderate |
| VKY 7 | Kaanga | 163 | 0.7 | 0.004 | 114 | Poor | Low |
| VKY 8 | Mbayongo | 46.5 | 34.0 | 0.731 | 1581 | High | High |
| VKY 10 | Tsambe | 749 | 16.1 | 0.021 | 12058 | Moderate | Very High |
| VKY 11 | Abwa | 76.7 | 20.2 | 0.263 | 1550 | High | High |
| VKY 12 | Ishan | 63.6 | 24.0 | 0.377 | 1526 | High | High |
| VKY 13 | Akar | 114 | 46.0 | 0.403 | 5244 | High | Very High |
| VKY 14 | Aginde | 546 | 0.6 | 0.001 | 164 | Poor | Low |
| VKY 16 | Bako | 74.4 | 6.4 | 0.086 | 474 | Moderate | Moderate |
| VKY 17 | Achwa | 118 | 28.6 | 0.242 | 3374 | High | High |
| VKY 18 | Agbile | 601 | 14.0 | 0.023 | 8414 | Moderate | Very High |

## Integrated Hydro-geophysical Interpretation

The integration of resistivity data, geoelectric parameters, and Dar-Zarrouk indices reveals that groundwater potential in the study area is highly variable and structurally controlled.

Areas such as Akar, Tsambe, Achwa, Ishan, and Abwa are characterized by: moderate resistivity values (indicative of saturated weathered/fractured zones), significant aquifer thickness, high transverse resistance, and moderate to high longitudinal conductance.

These conditions collectively indicate high transmissivity, good aquifer protection, and strong potential for sustainable groundwater development.

In contrast, areas such as Vandeikya HQ, Kaanga, and Aginde exhibit: Thin overburden, low longitudinal conductance, and low transverse resistance.

Such conditions are indicative of poor aquifer development and low borehole success rates.

## Implications for Borehole Development

The borehole yield classification (Table 5) confirms that aquifer productivity is primarily controlled by the thickness and hydraulic properties of the weathered/fractured basement.

Zones classified as having excellent to high groundwater potential (e.g., Akar, Tsambe, Agbile, Achwa, and Ishan) are recommended for borehole development due to their high transmissivity and storage capacity.

Conversely, areas with thin overburden and low transmissivity (e.g., Vandeikya HQ and Aginde) are less suitable for groundwater exploitation and may require alternative water supply strategies.

These findings align with previous studies in Basement Complex terrains, which emphasize the importance of secondary porosity (fracturing and weathering) in controlling groundwater occurrence (Olorunfemi and Fasuyi, 1993; Offodile, 2002).

Table 5: Borehole Yield Potential

| Station ID | Location | Aquifer Thickness (m) | Resistivity (Ωm) | T (Ωm²) | Yield Class | Groundwater Potential |
| --- | --- | --- | --- | --- | --- | --- |
| VKY 1 | Vandeikya HQ | 0.7 | 582 | 407 | Low | Poor |
| VKY 2 | Gbagbongom | 5.1 | 49.1 | 250 | Low | Low |
| VKY 6 | Unde | 12.6 | 251 | 308 | Moderate | Moderate |
| VKY 8 | Mbayongo | 34.0 | 46.5 | 1581 | High | High |
| VKY 10 | Tsambe | 16.1 | 749 | 12058 | Very High | Excellent |
| VKY 11 | Abwa | 20.2 | 76.7 | 1550 | High | High |
| VKY 12 | Ishan | 24.0 | 63.6 | 1526 | High | High |
| VKY 13 | Akar | 46.0 | 114 | 5244 | Very High | Excellent |
| VKY 14 | Aginde | 0.6 | 546 | 164 | Low | Poor |
| VKY 17 | Achwa | 28.6 | 118 | 3374 | High | High |
| VKY 18 | Agbile | 14.0 | 601 | 8414 | Very High | Excellent |

# CONCLUSION

This study employed the Vertical Electrical Sounding (VES) method using the Schlumberger array to evaluate groundwater potential in Vandeikya Local Government Area, Benue State. The results reveal a highly heterogeneous subsurface typical of Basement Complex environments, with resistivity variations reflecting differences in lithology, degree of weathering, and saturation conditions.

Geoelectrical interpretation identified between seven and nine subsurface layers, grouped into topsoil, weathered/fractured basement, and fresh basement. The weathered and fractured basement units constitute the principal aquifer system, with resistivity values ranging from 46.5 to 749 Ωm and thicknesses between 0.6 m and 46.0 m. Aquifer zones characterized by moderate resistivity and significant thickness were identified as the most productive.

The analysis of Dar-Zarrouk parameters further highlighted variations in aquifer protective capacity and transmissivity. High longitudinal conductance values indicate zones of good protective cover, while elevated transverse resistance values correspond to high groundwater yield potential. Notably, locations such as Akar, Tsambe, Achwa, and Agbile exhibit favourable hydro-geophysical conditions for sustainable groundwater development.

Spatial analysis shows that the central and northwestern parts of the study area are the most promising for groundwater exploration due to thicker overburden and well-developed weathered/fractured zones. In contrast, areas with thin overburden and high resistivity values correspond to poor groundwater potential.

Overall, the study demonstrates that aquifer thickness, resistivity, and transmissivity are the primary controls on groundwater occurrence and borehole yield in the area. It also confirms the effectiveness of the Schlumberger VES technique as a reliable tool for groundwater exploration and hydrogeophysical characterization in Basement Complex terrains.

REFERENCES

Adelana, S. M. A., & MacDonald, A. M. (2008). Applied groundwater studies in Africa. International Association of Hydrogeologists.

Ahmed, K., Shahid, S., Nawaz, N., & Wang, X. (2020). Groundwater sustainability under climate change in developing countries. Science of the Total Environment, 721, 137–148.

Aizebeokhai, A. P., & Oyeyemi, K. D. (2018). Application of geoelectrical resistivity methods for groundwater exploration: A review. Hydrogeology Journal, 26(1), 1–20.

Akinlalu, A. A., Afolabi, O. O., & Olorunfemi, M. O. (2017). Geoelectric investigation of groundwater potential in crystalline basement terrain. Applied Water Science, 7, 2343–2353.

Ako, B. D., & Olorunfemi, M. O. (1989). Geoelectric survey for groundwater in the newer basalt of Vom Plateau State, Nigeria. Journal of Mining and Geology, 25(1), 247–250.

Binley, A., Hubbard, S. S., Huisman, J. A., Revil, A., Robinson, D. A., Singha, K., & Slater, L. D. (2015). The emergence of hydrogeophysics for improved understanding of subsurface processes. Water Resources Research, 51, 3837–3866.

Cuthbert, M. O., Gleeson, T., Moosdorf, N., Befus, K. M., Schneider, A., Hartmann, J., & Lehner, B. (2019). Global patterns and dynamics of climate–groundwater interactions. Nature, 572, 230–234.

Ekanem, A. M., Essien, U. C., & Ushie, F. A. (2020). Evaluation of aquifer protective capacity and groundwater potential using resistivity method. Arabian Journal of Geosciences, 13, 1–15.

Emenike, E. A. (2018). Geophysical methods for groundwater exploration in crystalline basement terrains. Environmental Earth Sciences, 77, 1–12.

Fetter, C. W. (2001). Applied hydrogeology (4th ed.). Prentice Hall.

Henriet, J. P. (1976). Direct applications of Dar Zarrouk parameters in groundwater surveys. Geophysical Prospecting, 24(2), 344–353. https://doi.org/10.1111/j.1365-2478.1976.tb00931.x

MacDonald, A. M., Bonsor, H. C., Dochartaigh, B. É. Ó., & Taylor, R. G. (2012). Quantitative maps of groundwater resources in Africa. Environmental Research Letters, 7(2), 024009.

MacDonald, A. M., Taylor, R. G., & Bonsor, H. C. (2021). Groundwater and resilience to climate change. Philosophical Transactions of the Royal Society A, 379, 20200175.

Maillet, R. (1947). The fundamental equations of electrical prospecting. Geophysics, 12(4), 529–556.

Niwas, S., & Singhal, D. C. (1981). Estimation of aquifer transmissivity from Dar-Zarrouk parameters in porous media. Journal of Hydrology, 50, 393–399.

Offodile, M. E. (2002). Groundwater study and development in Nigeria (2nd ed.). Mecon Geology & Engineering Services Ltd.

Oladapo, M. I., Mohammed, M. Z., Adeoye-Oladapo, O. O., & Adetola, B. A. (2019). Hydrogeophysical evaluation of groundwater potential. Journal of African Earth Sciences, 150, 1–12.

Olayinka, A. I., & Olorunfemi, M. O. (2021). Advances in electrical resistivity methods for groundwater exploration in Nigeria. Nigerian Journal of Physics, 33(1), 1–15.

Olorunfemi, M. O., & Fasuyi, S. A. (1993). Aquifer types and the geoelectric/hydrogeologic characteristics of part of the central Basement Terrain of Nigeria. Journal of African Earth Sciences, 16(3), 309–317. https://doi.org/10.1016/0899-5362(93)90051-Q

Oseji, J. O., Asokhia, M. B., & Okolie, E. C. (2005). Determination of groundwater potential in Obiaruku and environs using surface geoelectric sounding. Environmentalist, 25(1), 17–23.

Oseji, J. O., & Okolie, E. C. (2011). Geoelectric investigation of the aquifer characteristics and groundwater potential in Kwale, Delta State, Nigeria. Nigerian Journal of Physics, 22(2), 107–116.

Soupios, P. M., Vallianatos, F., & Vafidis, A. (2007). Integration of geophysical methods for groundwater detection. Journal of Applied Geophysics, 62, 1–13.

Taylor, R. G., Scanlon, B., Döll, P., Rodell, M., Van Beek, R., Wada, Y., Longuevergne, L., et al. (2013). Groundwater and climate change. Nature Climate Change, 3, 322–329.

Telford, W. M., Geldart, L. P., & Sheriff, R. E. (1990). Applied geophysics (2nd ed.). Cambridge University Press.

Todd, D. K., & Mays, L. W. (2005). Groundwater hydrology (3rd ed.). Wiley.

Wright, E. P., Burgess, W. G., & Edmunds, W. M. (2016). Basement aquifers in Africa. Geological Society Special Publications.

Zohdy, A. A. R., Eaton, G. P., & Mabey, D. R. (1974). Application of surface geophysics to ground water investigations. U.S. Geological Survey.