Design and Evaluation of Data Centric Approach to Machinery Monitoring and Failure Prognosis Using Analytical Dashboards

Khushi Chabhadiya1

1School of Engineering, P P Savani University, Surat, India

Khushichabhadiya96@gmail.com

Tasneem Kagzi2

1School of Engineering, P P Savani University, Surat, India tasneem.kagzi@ppsu.ac.in

Abstract-

The increasing complexity of industrial machinery and the growing demand for uninterrupted operation have driven significant research interest in condition-based maintenance (CBM), machinery diagnostics, and prognostics. This paper presents a comprehensive review of methodologies, techniques, and technologies employed in machinery diagnostics and prognostics over the past two decades. We systematically examine signal processing methods, machine learning approaches, physics-based models, and hybrid frameworks for fault detection, identification, and remaining useful life (RUL)  estimation.  Key  application  domains  including  rotating  machinery,  bearings,  gears,  cutting  tools,  and multi-component   systems   are   surveyed.   A   data-centric   analytical   dashboard-PAM   (Predictive   Analysis   & Maintenance)-is presented as a practical realization of these frameworks, enabling real-time fleet health monitoring, RUL visualization, failure prediction, and maintenance scheduling through an interactive web-based interface. The review  also  identifies  open  challenges  such  as  data  scarcity,  domain  adaptation,  interpretability,  and  real-time deployment constraints. Finally, future research directions are discussed with emphasis on deep learning, digital twins, transfer learning, and edge computing integration.

Keywords: condition-based maintenance, machinery diagnostics, prognostics, remaining useful life, fault detection, signal processing, machine learning, rotating machinery, analytical dashboard.

I. INTRODUCTION

Industrial machinery forms the backbone of modern manufacturing, power generation, and transportation infrastructure. Equipment failures lead not only to costly unplanned downtime but also to potential safety hazards and environmental consequences. Traditional time-based or corrective maintenance strategies are increasingly inadequate for meeting the reliability demands of complex, high-value assets. Condition-based maintenance (CBM) has emerged as a paradigm shift, enabling maintenance decisions driven by the actual health state of equipment rather than fixed schedules or reactive responses [1].

CBM relies on continuous or periodic monitoring of physical parameters-such as vibration, temperature, acoustic emissions, oil debris, and electrical signatures-to assess machinery health, detect incipient faults, and forecast remaining useful life (RUL). The integration of signal processing, machine learning, and physics-of-failure modeling has transformed CBM from a reactive strategy into a predictive and even prescriptive approach [2][3].

The field has witnessed accelerating growth fueled by advances in sensor technology, industrial Internet of Things (IIoT), edge computing, and deep learning. The expanding availability of publicly accessible datasets, such as the NASA prognostics data repository [6],  has  further  catalyzed  research  activity.  This  paper  both  surveys  the  methodological  landscape  and  presents  a  practical implementation in the form of the PAM dashboard-a data-centric analytical tool for real-time machinery health management.

TABLE I -Systematic Comparison ofMaintenance Paradigms

Strategy

Reactive

Time-Based

CBM

Predictive (ML/AI)

Prescriptive

Trigger

Failure

Fixed Schedule

Monitored State

Predicted RUL

Optimized decision

Cost Implication

Very High

Medium-High

Medium

Low

Very Low

Downtime Risk

Very High

Medium

Low

Very Low

Minimal

Data Required

None

Calendar only

Sensor streams

Historical + real-time

Full fleet + context

II. CONDITION-BASED MAINTENANCE FRAMEWORK

1

A. Architecture and Process Flow

A  CBM  system  typically comprises  five  functional layers: data  acquisition, data  processing, health  state assessment,  prognostic reasoning,  and  maintenance  decision  support  [1][4].  The  data  acquisition  layer  captures  multi-modal  sensor  signals  including accelerometers, thermocouples, current transformers, and acoustic emission transducers. The processing layer filters, segments, and transforms raw signals into informative feature representations. The health assessment layer classifies machinery state and detects anomalies, while the prognostic layer estimates degradation trajectories and RUL. The decision support layer translates prognostic outputs into actionable maintenance recommendations [3].

B. Sensing and Data Acquisition

Vibration analysis remains the most widely employed sensing modality for rotating machinery diagnostics, owing to its sensitivity to structural defects in bearings, gears, shafts, and rotors [5]. Accelerometers mounted at bearing housings capture broadband vibration signals, from which characteristic defect frequencies can be extracted [6][12]. Complementary modalities include acoustic emission for early crack detection, motor current signature analysis (MCSA) for induction motor faults, and oil debris analysis for gear and bearing wear [7][9].

TABLE II - Comparison of Common Sensing Technologies

Sensing Modality

Vibration (Accel.)

Acoustic Emission

Motor Current (MCSA)

Temperature

Oil Debris

Fault Types Detected

Bearing, gear, shaft

Cracks, early spalls

Motor faults, eccentricity

Overheating, lubrication

Gear/bearing wear

Freq. Range

0-20 kHz

100 kHz-1 MHz

DC-1 kHz

Quasi-static

Quasi-static

Key Advantages

High sensitivity

Detects incipient faults

Non-intrusive

Low cost, easy install

Direct wear quantification

Limitations

Sensor placement critical

Signal attenuation

Load dependency

Slow response

Offline, periodic

III. MACHINERY DIAGNOSTIC APPROACHES

A. Signal Processing Techniques

Time-domain analysis employs statistical indicators such as root mean square (RMS), kurtosis, crest factor, and peak-to-peak amplitude to characterize signal energy and impulsiveness. Frequency-domain analysis via Fast Fourier Transform (FFT) enables identification of characteristic  defect  frequencies  and  their harmonics, providing  clear physical interpretability  for bearing  and gear  faults [5][9]. Time-frequency methods, including Short-Time Fourier Transform (STFT) and wavelet transform, address the non-stationary nature of machinery signals under variable operating conditions [11].

B. Classical Machine Learning for Fault Classification

Support vector machines (SVM) have been extensively applied to bearing and tool wear diagnostics, demonstrating robust performance in high-dimensional feature spaces with limited training data. Ensemble methods such as random forests and gradient boosting further improve diagnostic robustness by aggregating predictions from multiple base learners [15][16].

C. Deep Learning for Fault Diagnosis

Convolutional neural networks (CNN) have emerged as the dominant paradigm for automated feature extraction from raw vibration and acoustic  signals  [17][18].  Recurrent  neural networks  (RNN)  and  Long  Short-Term  Memory  (LSTM) networks  capture  temporal dependencies in sequential monitoring data, enabling joint fault detection and severity tracking [19][20].

TABLE III - ML Classifiers in CBM Diagnostic Tasks

Classifier

SVM

ANN / MLP

Fuzzy (TSK)

Random Forest

CNN (1D/2D)

LSTM / RNN

HybridCNN-LSTM

Feature Type

Hand-crafted

Hand-crafted

Hand-crafted

Hand-crafted

Raw / TF image

Sequential raw

Raw multimodal

Data Need

Low-Medium

Medium

Low

Medium

High

High

Very High

Noise Robustness

High

Medium

Very High

High

High

Medium

High

Typical Accuracy

92-98%

90-97%

88-95%

93-99%

96-99.5%

94-99%

97-99.8%

IV. PROGNOSTIC METHODS AND RUL ESTIMATION

A. Physics-Based Prognosis

Physics-based prognostic models leverage degradation mechanics and material science to model failure propagation. Paris-Erdogan fatigue  crack  growth  models, Hertzian  contact  stress  models  for  bearing  spall  propagation,  and  Archard  wear  models  provide

2

mechanistic  descriptions  of  fault  evolution  [7][23].  Particle  filter  frameworks  fuse  physics-based  degradation  equations  with probabilistic state estimation, yielding probabilistic RUL distributions [24].

B. Data-Driven Prognosis

Data-driven prognostic approaches learn degradation trajectories directly from historical run-to-failure data. Deep learning prognostic models, particularly LSTM networks and temporal convolutional networks (TCN), capture complex long-range temporal dependencies in multi-sensor degradation signals, achieving state-of-the-art performance on benchmark turbofan (C-MAPSS) and bearing datasets [19][20].

TABLE IV - Comparison of Prognostic Approaches

Method

Paris-Erdogan

Particle Filter

Wiener Process

LSTM / TCN

GaussianProcess

Physics-Informed NN

Type

Physics

Physics+Stoch.

Stochastic

Data-driven DL

Bayesian

Hybrid

UncertaintyOutput

No (deterministic)

Yes (probabilistic)

Yes

With MC Dropout

Yes (calibrated)

Yes

Data Req.

Material params

Physics+sensors

Degradation data

Large run-to-fail

Medium

Physics + data

Application

Crack/fatigue

Multi-system

Bearing wear

Turbofan, bearing

Online RUL

Novel fault modes

V. APPLICATION DOMAINS

A. Rotating Machinery and Bearings

Rolling element bearings represent the most extensively studied component in the PHM literature [5][6]. Benchmark datasets, including the CWRU bearing dataset and the FEMTO-ST bearing accelerated life test dataset, have enabled systematic comparison of diagnostic and prognostic algorithms [12][13].

B. Gear and Gearbox Diagnostics

Gear fault diagnostics exploits mesh frequency harmonics, modulation sidebands, and cepstral analysis to detect pitting, chipping, and tooth fracture. Kacprzynski et al. demonstrated the fusion of diagnostics and physics-of-failure models for remaining gear life prediction [7].

C. Cutting Tools and Manufacturing Processes

Tool condition monitoring (TCM) in machining processes is essential to maintain part quality and prevent workpiece damage. Cutting force signals [9], acoustic emission, and spindle current have been employed as health indicators for drill and milling cutter wear monitoring. TSK fuzzy models demonstrated adaptive real-time tool wear estimation [10].

VI. BENCHMARKING DATASETS AND PERFORMANCE METRICS

A. Public Datasets

Publicly available datasets have played an indispensable role in advancing and benchmarking CBM research. The NASA prognostics data repository [6] provides bearing run-to-failure data and turbofan engine degradation data (C-MAPSS), widely used for RUL estimation benchmarking. The CWRU bearing dataset remains the most frequently cited diagnostic benchmark [12].

TABLE V - Key Open-Access Datasets for CBM Research

Dataset

CWRU Bearing

FEMTO/PRONOSTIA

C-MAPSS

IMS (Cincinnati)

XJTU-SY

Gearbox (PHM2009)

Source

Case Western Res. Univ.

FEMTO-ST Inst.

NASA Ames

Univ. of Cincinnati

Xi'an Jiaotong Univ.

PHM Society

Machinery

Ball bearing

Ball bearing

Turbofan engine

Ball bearing

Ball bearing

Gearbox

Fault Types

Inner, Outer, Ball

Run-to-failure

Degradation

Run-to-failure

Run-to-failure

Tooth wear/break

Primary Use

Fault diagnosis

RUL prognosis

RUL regression

RUL prognosis

RUL + diagnosis

Gear diagnostics

VII. PAM ANALYTICAL DASHBOARD - DESIGN AND IMPLEMENTATION

To bridge the gap between academic prognostic frameworks and operational deployment, we developed PAM (Predictive Analysis & Maintenance), a data-centric web-based analytical dashboard that operationalizes the CBM methodologies surveyed in the preceding sections. PAM provides plant engineers and maintenance managers with a unified interface for real-time fleet health monitoring, failure prognosis, root cause analysis, and maintenance scheduling-without requiring specialist data science expertise.

3

A. Dashboard Architecture and Feature Modules

PAM is structured around six functional modules, each corresponding to a layer of the OSA-CBM architecture [4]: (1) Dashboard Overview  -  fleet-level  KPIs  including  health  score,  active  alert  count,  and  aggregate  sensor  status;  (2)  Manual  Monitoring  -per-module, per-bearing real-time sensor readouts with configurable threshold alarms; (3) Upload Data - CSV ingestion pipeline for offline sensor datasets with automatic preview and validation; (4) Analyze Data - failure prediction engine with confidence scores, RUL bar charts, root cause breakdown, and maintenance priority ranking; (5) Module Status - a filterable card grid showing per-module health scores, bearing-level status indicators, and risk classification; and (6) Maintenance Schedule - task management board for planning, assigning, and tracking work orders generated from prognostic outputs.

The dashboard continuously simulates sensor streams for 12 machine modules (M-01 through M-12) across three production lines, each equipped with four bearings. Temperature and vibration signals are sampled at 1.5-second intervals, and module health scores are computed using a composite degradation index derived from threshold exceedance weighted by bearing proximity to critical limits.

B. Dashboard Screenshots and Explanation

Fig. 7a presents the Main Overview Panel of the PAM dashboard. The top KPI stripe provides at-a-glance fleet health metrics: overall fleet health score (87.3%), count of active alerts (3), number of monitored modules (12), and average fleet temperature (71.4°C). The central Module Health Table lists all monitored modules with per-module temperature, vibration, computed health score rendered as a color-coded progress bar (green >75%, amber 50-75%, red <50%), and a risk classification tag (Low / Medium / High). Modules M-03 and M-08 are flagged as High-Risk with health scores of 31% and 28% respectively due to temperature and vibration threshold breaches. The right-hand Active Alerts panel surfaces critical and warning events in real time, with severity-coded color indicators, enabling operators to triage faults without navigating away from the overview. Three gauge dials at the bottom present fleet averages  for temperature, vibration, and overall health in an arc-gauge format familiar to industrial HMI conventions.

Fig. 7a - PAM Dashboard: Main Overview Panel - Fleet KPIs, Module Health Table, Real-Time Alerts, and Sensor Gauges.

Fig.  7b  presents  the  Prognostics  and  Analysis  Panel.  The  Failure  Predictions  card  lists  imminent  failures  ranked  by  model confidence, with estimated time-to-failure (ETA) and a horizontal confidence bar for each prediction. M-03 is predicted to experience bearing seizure within 48 hours at 92% confidence, while M-08 faces thermal degradation within 72 hours at 87% confidence. The Remaining Useful Life (RUL) Chart renders a vertical bar chart with color-coded bars for each module-red bars indicate modules with fewer than 100 hours remaining, amber for 100-400 hours, and green for those exceeding 400 hours-directly implementing the RUL regression methodology discussed in Section IV. The Root Cause Analysis breakdown decomposes the predicted failure probability distribution across six fault categories (bearing wear 35%, shaft misalignment 22%, lubrication failure 18%, thermal overload  12%, mass imbalance 8%, structural looseness 5%), supporting maintenance engineers in directing intervention efforts. The Maintenance Priority list auto-generates ranked action items (P1/P2/P3) from prognostic outputs, while the Sensor Trend Chart at the bottom tracks temperature and vibration trajectories over the preceding 24 hours with threshold markers, enabling visual identification of degradation onset and rate.

4

Fig. 7b - PAM Dashboard:Prognostics & Analysis Panel - RULChart, Failure Predictions, Root Cause Analysis, and Sensor Trends.

C. Alignment with CBM Frameworks

PAM directly instantiates the five-layer OSA-CBM architecture reviewed in Section II. The data acquisition and processing layers are realized through the CSV upload pipeline and real-time simulated sensor streams. Health state assessment is implemented via the composite health score and threshold-based alert engine visible in Fig. 7a. Prognostic reasoning is surfaced through the RUL bar chart, failure prediction engine, and confidence-scored prediction cards in Fig. 7b. The maintenance decision support layer is operationalized through auto-generated priority lists and the Maintenance Schedule module. This end-to-end mapping demonstrates the practical viability of translating academic CBM methodologies into deployable industrial tools.

VIII. OPEN CHALLENGES AND FUTURE RESEARCH DIRECTIONS

A. Data Scarcity and Class Imbalance

Run-to-failure experiments are costly and time-consuming, resulting in limited labeled fault data for training deep learning models. GAN-based data augmentation [21][22] and physics-informed synthetic data generation offer promising solutions, though ensuring physical fidelity remains an open challenge.

B. Domain Adaptation and Transfer Learning

Diagnostic models trained under one operating condition frequently exhibit severe performance degradation when deployed in differing target domains. Transfer learning and domain adaptation techniques have been applied to bridge domain gaps [17][18][28]. Few-shot and zero-shot learning frameworks are being explored to enable rapid adaptation to new machinery types with minimal labeled data.

C. Interpretability and Edge Deployment

Deep learning models, despite superior predictive performance, are often criticized as black boxes. Grad-CAM, SHAP values, and physics-informed  neural  networks  (PINNs)  provide  pathways  toward  interpretable  diagnostics  [20][25].  Simultaneously,  IIoT deployments demand lightweight models for real-time inference on resource-constrained edge devices via knowledge distillation and quantization [17][18].

D. Digital Twins

Digital twin technology represents a transformative frontier for machinery PHM, creating continuously updated virtual replicas that assimilate real-time sensor data and simulate degradation trajectories [25][30]. Their integration with enterprise maintenance systems and autonomous decision-making frameworks is an active and high-impact research direction.

TABLE VII -Research Challenges to Methodological Solutions

Challenge	Current Best Practice	Emerging Solution	Maturity

5

Data scarcity

Class imbalance

Domain shift

Black-box models

Edge deployment

Uncertain RUL

Transfer learning

SMOTE, cost-sensitive loss

Adversarial DA

SHAP, Grad-CAM

Pruning, quantization

Bayesian NNs

GAN / physics synth.

Conditional GAN

Zero-shot learning

Physics-Informed NNs

Neural arch. search

Conformal prediction

Medium

Medium-High

Medium

Emerging

High

Medium

IX. CONCLUSION

This paper has presented a comprehensive review of machinery diagnostics and prognostics for condition-based maintenance, covering the evolution from classical signal processing and statistical methods to advanced deep learning and physics-data hybrid frameworks. Alongside  the  survey,  the  PAM  analytical  dashboard  was  introduced  as  a  practical,  data-centric  realization  of  the  reviewed methodologies,  demonstrating  that  academic  CBM  frameworks  can  be  translated  into  operational  tools  accessible  to  industrial practitioners. The dashboard's modular architecture-spanning real-time fleet monitoring, RUL visualization, root cause analysis, and maintenance scheduling-reflects the five-layer OSA-CBM standard and provides a template for future deployable PHM systems.

Key insights include: (1) deep learning, particularly CNN and LSTM architectures, has established new performance benchmarks on public diagnostic and prognostic datasets; (2) hybrid physics-data models offer superior interpretability and generalization; (3) domain adaptation and transfer learning are critical enablers for practical industrial deployment; (4) uncertainty quantification and explainability remain essential for safety-critical maintenance decisions; and (5) data-centric analytical dashboards provide a critical bridge between algorithmic prognostics and operational maintenance practice.

Future research should prioritize lightweight, interpretable, domain-adaptive PHM models deployable on industrial edge hardware, construction of large-scale diverse public datasets, and integration of digital twin frameworks with enterprise asset management systems.

REFERENCES

[1] Jardine, A. K., Lin, D. & Banjevic, D. (2006). A Review of Machinery Diagnostics and Prognostics Implementing Condition Based Maintenance. Mechanical Systems and Signal Processing, 20(7), 1483-1510.

[2] [ Heng, A., et al. (2009). Intelligent Condition Based Prediction of Machinery Reliability. Mechanical Systems and Signal Processing, 23(5), 1600-1614.

[3] Vachtsevanos,G.,et al. (2006). Intelligent Fault Diagnosis and Prognosis for Engineering Systems. Hoboken, NJ: Wiley.

[4] Lebold, M. & Thurston, M. (2001). Open Standards for Condition Based Maintenance and Prognostics Systems. Proceedings of the MARCON 2001.

[5] Heng, A., et al. (2009). Rotating Machinery Prognostics: State of the Art, Challenges and Opportunities. Mechanical Systems and Signal Processing, 23(3), 724-739.

[6] NSF I/UCRC Center forIntelligent Maintenance Systems. Prognostic Data Repository: Bearing Data Set. NASAAmes Research Center.

[7] Kacprzynski, G., et al. (2004). Predicting Remaining Gear Life by Fusing Diagnostics and the Physics of Failure Models. JOM Journal, 56(3), 29-35.

[8] Zhou, J., et al. (2006). Genetic Algorithms for Feature Subset Selection in Equipment Fault Diagnostics. Journal of Engineering Asset Management, 10, 1104-1113.

[9] Ertunc, H.M. & Oysu, C. (2004). Drill WearMonitoring using Cutting Force Signals. Mechatronics, 14, 533-548.

[10]	Ren, Q., et al. (2011). TSK Fuzzy Modelling for Tool Wear Conditions in Turning Processes. Engineering Applications of Artificial Intelligence, 24(2), 260-265.

[11]	Peng, Y. & Dong, M. (2011). A Prognosis Method using Age-dependent Hidden Semi-Markov Model. Mechanical Systems and Signal Processing, 25, 237-252.M. Subrahmanyam and C. Sujatha, "Using neural networks for the diagnosis of localized defects in ball bearings," Tribol. Int., vol. 30, no. 10, pp.  739-752, 1997.

[12] R. B. Randall and J. Antoni, "Rolling element bearing diagnostics - A tutorial," Mech. Syst. Signal Process., vol. 25, no. 2, pp. 485-520, 2011.

[13] X. Li, "A brief review: Acoustic emission method for tool wear monitoring during turning," Int. J. Mach. Tools Manuf., vol. 42, no. 2, pp. 157-165, 2002.

[14] L. Breiman, "Random forests," Mach. Learn., vol. 45, no. 1, pp. 5-32, 2001.

[15] J. H. Friedman, "Greedy function approximation: Agradient boosting machine," Ann. Stat., vol. 29, no. 5, pp. 1189-1232, 2001.

[16] W. Zhang et al., "A new deep learning model for fault diagnosis with good anti-noise and domain adaptation ability," Sensors, vol. 17, no. 2, pp. 425-447, 2017.

[17] F. Jia et al., "Deep neural networks: A promising tool for fault characteristic mining,"Mech. Syst. Signal Process., vol. 72-73, pp. 303-315, 2016. [18] S. Zheng et al., "Long short-term memory network for remaining useful life estimation," in Proc. IEEE ICPHM, 2017, pp. 88-95.

[19] C. Zhang et al., "Multiobjective deep belief networks ensemble for remaining useful life estimation," IEEE Trans. Neural Netw. Learn. Syst., vol. 28, no. 10, pp. 2306-2318, 2017.

[20] X. Yi et al., "Generative adversarial network in medical imaging: A review," Med. Image Anal., vol. 58, p. 101552, 2019.

6

[21] C. Li et al., "Bearing fault diagnosis using fully-connected winner-take-all autoencoder," IEEE Access, vol. 6, pp. 6103-6115, 2018.

[22] P. D. McFadden, ``Examination of a technique for the early detection of failure in gears," Mech. Syst. Signal Process., vol. 1, no. 2, pp. 173-183, 1987.

7