DESIGN AND PERFORMANCE OPTIMIZATION OF A LABORATORY SCALE DISTILLER FOR HIGH-PURITY BIOETHANOL PRODUCTION FROM BIOMASS FEEDSTOCKS

Emmanuel F. Borre1

1Research Faculty, College of Agriculture, Nueva Ecija University of Science and Technology, Gabaldon, Philippines

Email: emmanuelfborre@gmail.com/eborre@ineust.ph.education

## ABSTRACT

This study presents the design, fabrication, and performance evaluation of a laboratory-scale packed-column distillation system to produce high-grade hydrous bioethanol using locally available materials. The system was engineered as a cost-effective and scalable solution for biomass based ethanol purification, with relevance to agricultural waste such as mango residues.

The distillation apparatus incorporates a configurable reflux column utilizing alternative packing materials—steel wool and Raschig rings—to enhance vapor-liquid contact and improve separation efficiency. Experimental evaluation was conducted under varying feedstock ethanol concentrations (40%, 60%, and 80%) using a Completely Randomized Design (CRD) and statistical analyses including ANOVA and regression modeling.

The results demonstrate that packing material significantly influences system performance. Raschig rings achieved superior outcomes, yielding ethanol purity up to 97.5%, distillation efficiency of approximately 74%, and ethanol recovery of 41.55%. Distillation rates ranged from 2.4 to 3.89 L/hr depending on feed concentration and packing configuration. Operational parameters showed rapid startup (5–10 minutes) and efficient thermal performance within a boiling range of 70–83°C.

Economic analysis indicates that the system is financially viable, with a low capital cost (₱6,500), production cost of approximately ₱35/L, and a payback period of 1.18 years. The modular design and use of accessible materials make the technology suitable for decentralized bioethanol production in rural or small-scale industrial settings.

The study highlights the potential of optimized packed-column distillation systems as sustainable and economically feasible solutions for bioethanol purification, with strong implications for renewable energy production and waste valorization.

Keywords

bioethanol production, mango waste valorization, agricultural biomass conversion, distillation, distillation efficiency, ethanol recovery, hydrous ethanol purification, small-scale distillation system, renewable biofuels, sustainable energy systems, low-cost distillation technology, biomass-to-fuel conversion

## INTRODUCTION

The continuous rise in global energy demand, coupled with the depletion of fossil fuel reserves and escalating environmental concerns, has intensified the search for sustainable and renewable energy alternatives. Fossil fuels remain the dominant global energy source; however, their increasing cost, finite availability, and contribution to greenhouse gas emissions make them increasingly unsustainable (Balat et al., 2008; Goldemberg, 2007). In the Philippines, this challenge is further exacerbated by the country’s heavy dependence on imported petroleum products, making it highly vulnerable to global price fluctuations. The fuel prices in the Philippines have surged dramatically, marking the 13th to 15th consecutive week of increases. Gasoline prices have risen by approximately ₱4.90 to ₱5.90 per liter, while diesel prices have increased by as much as ₱17.95 to ₱19.80 per liter, pushing pump prices to nearly ₱120 per liter for gasoline and exceeding ₱170 per liter for diesel in some areas. These increases are largely driven by geopolitical tensions in the Middle East, particularly the ongoing conflict involving Iran, which has disrupted global oil supply chains and increased importation costs. Consequently, key sectors such as transportation, agriculture, and small-scale industries are experiencing substantial economic strain, highlighting the urgency of developing alternative, locally sourced energy solutions.

Bioethanol has emerged as one of the most promising renewable fuel alternatives due to its biodegradability, renewability, and compatibility with existing fuel infrastructure. It is a liquid biofuel produced from various biomass feedstocks, including agricultural residues and organic wastes, through biochemical and thermochemical conversion processes (Demirbas, 2007). As an oxygenated fuel, bioethanol promotes more complete combustion, thereby reducing particulate emissions and improving air quality in internal combustion engines (Balat & Balat, 2009). Furthermore, bioethanol has been widely recognized for its potential to reduce net greenhouse gas emissions compared to conventional petroleum fuels (Goldemberg, 2007).

The use of ethanol as a fuel dates back to the early development of internal combustion engines in the late 19th century, particularly following the invention of the Otto engine in 1876. Today, ethanol–gasoline blends are widely utilized in modern engines without requiring significant modifications, demonstrating their practicality and scalability (Heywood, 2018). In addition, bioethanol possesses favorable physicochemical properties, including a higher octane number, wider flammability limits, higher flame speeds, and greater heat of vaporization than gasoline. These characteristics enable improved engine performance through higher compression ratios, enhanced combustion efficiency, and reduced engine knocking, ultimately contributing to better thermal efficiency (Turns, 2012; Heywood, 2018).

In the Philippine context, the abundance of agricultural residues presents a significant opportunity for sustainable bioethanol production. Mango (Mangifera indica) waste, in particular, is an underutilized biomass resource generated from fruit processing activities, including peels, pulp residues, and rejected fruits. These wastes are rich in fermentable sugars, making them suitable feedstocks for bioethanol production. The valorization of mango waste not only supports renewable energy generation but also addresses environmental concerns associated with organic waste disposal while creating additional income streams for agricultural communities.

Despite the promising potential of bioethanol, one of the primary challenges in its production lies in the efficient and cost-effective purification of ethanol from fermentation mixtures. Distillation remains the most widely employed separation technique; however, conventional distillation systems are often energy-intensive and costly, limiting their applicability in decentralized or small-scale operations (Balat et al., 2008). Therefore, the development of a low-cost, efficient distillation system using locally available materials is essential to enable broader adoption of bioethanol technologies, particularly in rural areas.

This study focuses on the design, fabrication, and performance evaluation of a laboratory-scale distiller for producing high-grade hydrous bioethanol from biomass feedstocks such as mango waste. By optimizing column packing materials and operating conditions, the research aims to improve ethanol recovery, distillation efficiency, and overall system performance while maintaining economic feasibility. Ultimately, this work contributes to the advancement of sustainable energy technologies that can mitigate the impacts of rising fuel prices and enhance energy security in the Philippines.

2. OBJECTIVES

The general objective of this study was to design and develop a scaled-down laboratory distillation system to produce high-grade hydrous bioethanol using locally available and cost-effective materials.

Specifically, the study aimed to:

- design and fabricate a laboratory-scale distillation apparatus incorporating a packed-column system suitable for high-purity bioethanol production;

- evaluate the performance of the developed distillation unit in terms of ethanol recovery, distillation efficiency, and operational parameters under varying feedstock conditions; and

- conduct a techno-economic assessment of the system, including cost analysis, to determine its feasibility for small-scale and decentralized bioethanol production.

3. MATERIALS AND METHODS

3.1 Materials and Instruments

The following materials and instruments were used during the conduct of the study:

- Bioethanol

- Water

- Charcoal

- Hydrometer

- Thermometer

- Glass bottle (1000mL)

- Timer

- Graduated cylinder

3.2 Methods

The following process was done during the conduct of the study:

- Design of the distiller

- Fabrication of the distiller

- Preliminary testing

- Performance testing and data gathering under two different packing materials under three different purity of alcohol

- Analysis of data using analysis of variance and regression analysis.

## RESULTS AND DISCUSSION

The operational performance of the developed bioethanol distillation unit demonstrated efficient and stable functionality under varying feedstock conditions. The system exhibited a rapid start-up time of 5 to 10 minutes and a total operating duration ranging from 90 to 120 minutes. Boiling of 3L feedstock with varying ethanol concentrations (40%, 60%, and 80%) occurred within 3 to 5 minutes after start-up, with boiling temperatures ranging from 70°C to 83°C. These values are consistent with the thermodynamic properties of ethanol–water mixtures, where ethanol lowers the boiling point of the solution, facilitating earlier vaporization (Heywood, 2018). Other operational parameters, including coolant flow rate (65.12–172 L/min), distillate volume (700 mL to 2 L), and residual stillage alcohol content (2–5%), indicate effective separation and condensation performance of the system.

Effect of Feedstock Concentration on Distillation Rate

The results showed that distillation rate increased with higher ethanol concentration in the feedstock. At 80% ethanol concentration, the highest distillation rate of 3.75 L/hr was observed, compared to 3.15 L/hr and 3.43 L/hr at 40% and 60%, respectively. This trend can be attributed to the lower boiling point and higher volatility of ethanol relative to water, which enhances vaporization at higher concentrations (Turns, 2012). Similar findings have been reported by Balat et al. (2008), who emphasized that ethanol concentration significantly influences evaporation rates and overall distillation performance.

Influence of Packing Materials on Performance

The type of packing material used in the distillation column significantly affected system performance. Raschig rings demonstrated superior performance compared to steel wool, achieving a higher mean distillation rate (3.7 L/hr) and improved ethanol recovery (41.55%). This can be explained by the enhanced vapor–liquid contact area and improved mass transfer efficiency provided by structured packing materials such as Raschig rings (Coulson & Richardson, 1999). Efficient packing promotes better separation by increasing the number of theoretical stages within the column.

Table 1. Distillation rate of Bioethanol Distiller

| Packing Material |  |  | Purity of Ethanol |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B1 | B2 | B3 | MEAN |
| A1 |  | 2.4 | 3.37 | 3.81 | 3.19a |
| A2 |  | 3.89 | 3.49 | 3.72 | 3.7b |
| MEAN |  | 3.15x | 3.43x | 3.76x | 3.445 |

The ethanol recovery was highest at 80% feedstock concentration, reaching 58.44%, confirming that higher initial ethanol content leads to improved recovery efficiency. The results further revealed that raschig rings slightly outperformed steel wool in terms of recovery, although both materials showed comparable trends.

In terms of distillate quality, the system achieved high ethanol purity levels, with mean values of 96.44% for steel wool and 97.14% for Raschig rings. These values are within the expected range for hydrous ethanol distillation and indicate effective separation. According to Demirbas (2007), achieving ethanol purity above 95% through conventional distillation reflects efficient column design and adequate reflux conditions. The result in Table 2 confirms that the ethanol recovery as influenced packing materials of the column and purity of ethanol is significant.

Table 2. Ethanol Recovery on Bioethanol Distiller

| Packing Material |  |  | Purity of Ethanol |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B1 | B2 | B3 | MEAN |
| A1 |  | 25k | 38.88j | 60.55i | 41.47a |
| A2 |  | 26.11k | 42.22j | 56.33i | 41.55b |
| MEAN |  | 51.11z | 40.55y | 58.44x | 41.51 |

Higher recovery was due to the volume and purity of ethanol inside the boiler, the higher the alcohol content of the feedstock, the higher ethanol recovery.

At the end of distillate collection, the alcohol content was recorded under each treatments, each treatments producing a mean of 96.44% for steel wool and 97.14% for raschig rings. Table 3 showed that using of different packing materials, the alcohol percentage of distillate is affected.

Table 3.Alcohol content of distillate on Bioethanol Distiller

| Packing Material |  |  | Purity of Ethanol |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B1 | B2 | B3 | MEAN |
| A1 |  | 96.83j | 95.5m | 97j | 96.44b |
| A2 |  | 96.1l | 97.5i | 97.83i | 97.14a |
| MEAN |  | 96.47y | 96.5x | 97.415y | 96.79 |

Additionally, the raschig rings has an distillation efficiency at about 73.37% higher than 65.45% under steel wool. The result showed in Table 4 confirmed that the distillation efficiency of two packing material is highly significant.

Table 4. Distillation Efficiency on Bioethanol Distiller

| Packing Material |  |  | Purity of Ethanol |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B1 | B2 | B3 | MEAN |
| A1 |  | 60.52k | 61.81j | 74.03j | 65.45b |
| A2 |  | 80.17i | 68.63j | 71.33j | 73.37a |
| MEAN |  | 70.34x | 65.22y | 72.68x | 69.41 |

Distillation efficiency was significantly higher when using Raschig rings (73.37%) compared to steel wool (65.45%). This improvement is attributed to better mass transfer characteristics and reduced channeling effects in structured packing materials. The findings align with established distillation theory, where column efficiency is highly dependent on packing geometry and surface area (Coulson & Richardson, 1999). The results suggest that optimizing packing materials is critical for enhancing system performance in small-scale distillation units.

The result in Table 5 showed that the fuel consumption rate ranged from 3.19 kg/hr (steel wool) to 3.7 kg/hr (Raschig rings), with no statistically significant difference between treatments. This indicates that packing material has minimal influence on energy consumption, which is primarily governed by furnace efficiency and heat transfer rate. Increased heat input accelerates vaporization, leading to higher distillation rates, as supported by thermodynamic principles of phase change (Turns, 2012).

Table 5. Fuel consumption rate on Bioethanol Distiller

| Packing Material |  |  | Purity of Ethanol |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B1 | B2 | B3 | MEAN |
| A1 |  | 2.4 | 3.37 | 3.81 | 3.19a |
| A2 |  | 3.89 | 3.49 | 3.72 | 3.7a |
| MEAN |  | 3.145x | 3.43x | 3.765x | 3.44 |

The effect of the fuel consumption rate is depends to the capacity of the furnace to produce heat. . The more heat the faster is the vaporization of feedstock. Fast vaporization provides higher amount of vapor which is then condensed at the condenser.

The bioethanol production was collected by every 250 ml on the distiller. The graphical representation showed in Figure 1. Is the bioethanol production by using steel wool as the packing material on the reflux column with respect to the distillation time

Figure 1. Regression curve of bioethanol production of Distiller using Steel Wool

The Figure 1 shows the relationship between the volume of the bioethanol produce and the time of distillation. The volume of the bioethanol is dependent on the feedstock inside the column, if the feedstock inside the boiler has a higher percentage of alcohol the volume of distillate is higher also, and if the feedstock inside the boiler has an lower alcohol percentage, therefore the volume of alcohol distillate was lower also.

The bioethanol production was collected by every 250 ml on the distiller. The graphical representation showed in Figure 2. is the bioethanol production by using steel wool as the packing material on the reflux column with respect to the distillation time.

Figure 2. Regression curve of bioethanol production of Bioethanol Distiller using Raschig Rings

Figure 2 shows the relationship of volume and time in the production of bioethanol using raschig rings as the packing material inside the column. The packing material inside the column did not affect the production of bioethanol. The production of bioethanol is dependent on the volume and alcohol content inside the boiler, if the feedstock in the boiler has a higher volume and higher alcohol content, the volume of distillate must be also, and if the feedstock inside the boiler has an lower alcohol percentage, therefore the volume of alcohol distillate was lower also.

The costs incurred in operating the CBED and RHG is presented in Table 1. After the computations the cost involved are investment cost of P6500, total cost of P51,477 and production cost of P35/ L. The net income that could be obtained is from P5,496 each year of producing high grade hydrous bioethanol of using the unit. Payback period is within 1.18yrs and return on investment of 84.55%.

Table 1. Cost analysis of CBED using RHG

| MODEL |  | CBED | TOTAL |
| --- | --- | --- | --- |
| IC |  | 6,500 | 6,500 |
|  | MC | 6,000 | 6,000 |
|  | LC | 500 | 500 |
| FC |  | 3,055 | 3,055 |
|  | D | 1,170 |  |
|  | IOI | 195 |  |
|  | I | 1,690 |  |
| VC |  | 48,617 | 48,617 |
|  | fC | 1,715 | 1,715 |
|  | R&M | 195 | 195 |  |
|  | LC | 20,000 | 20,000 |  |
|  | WC | 26,707 | 26,707 |  |
| TC |  | 51,477 |  |  |
| C |  | 1,648.80 |  |  |

## IC - Investment cost

MC - Material cost

LC - Labor cost

FC - Fixed cost (Php/yr)

D - Depreciation

IOI - Interest on investment

I - Insurance

VC - Variable cost (Php/yr)

fC - Fuel cost

EC - Electrical cost

R&M - Repair and maintenance

WC – Water Cost

C - Capacity (L/yr)

## 5. CONCLUSIONS AND RECOMMENDATIONS

  - Conclusions

## This study successfully demonstrated the design, fabrication, and performance evaluation of a laboratory-scale packed-column distillation system for the production of high-grade hydrous bioethanol using locally available materials. The developed distiller exhibited stable and efficient operation, confirming its applicability as a low-cost and scalable solution for biomass-based ethanol purification.

## The results established that the distillation rate is directly influenced by both the volume and ethanol concentration of the feedstock, with higher concentrations facilitating faster vaporization and improved distillate yield. Furthermore, the performance of the distillation system—particularly in terms of ethanol recovery and distillation efficiency—was strongly dependent on the physicochemical properties of the feedstock. The use of appropriate packing materials, such as Raschig rings, significantly enhanced vapor–liquid interaction, resulting in higher ethanol purity (≥95%), improved recovery, and greater overall efficiency.

## From an economic perspective, the system proved to be financially viable for small-scale deployment, requiring a relatively low capital investment (₱6,500) and generating a production cost of approximately ₱30–35 per liter. The projected payback period of 1.18 years and return on investment of 84.55% indicate strong potential for adoption in decentralized and rural bioethanol production systems.

## Importantly, the developed distillation technology demonstrates flexibility for integration with various biomass feedstocks, including mango waste and other agricultural residues. This adaptability highlights its potential as a platform technology for sustainable biofuel production and waste valorization in agricultural regions.5.2

## 5.2 Recommendations and Future Directions

Based on the findings of this study, the following recommendations and future research directions are proposed:

- Future work should focus on scaling up the distillation system to pilot- and community-level applications, specifically targeting mango waste as a primary feedstock. Given the abundance of mango processing residues in the Philippines, large-scale integration can significantly enhance bioethanol production while addressing agricultural waste management challenges.

- The system should be further optimized for co-processing of mango waste with other high-value agricultural residues, such as pineapple peels, banana waste, sugarcane bagasse, and onion (Allium cepa) waste, which is currently under ongoing investigation. Onion waste, generated in significant quantities from food processing and market discard, contains fermentable carbohydrates that can be utilized for bioethanol production. The co-fermentation and co-distillation of these biomass sources can enhance substrate diversity, improve fermentable sugar availability, and increase overall ethanol yield. Moreover, integrating multiple feedstocks ensures a more stable and year-round supply of raw materials, reducing seasonal dependency and improving the sustainability and scalability of the bioethanol production system.

Future studies should focus on optimizing pretreatment methods, fermentation efficiency, and distillation performance for mixed-feedstock systems, particularly evaluating the synergistic effects of mango and onion waste in terms of sugar conversion efficiency and ethanol recovery. To improve productivity and thermal efficiency, the use of continuous-type furnaces, biomass gasifiers, or electric heating systems is recommended. These approaches can provide a stable heat source, reduce fuel consumption variability, and enhance overall system performance.

- The use of Raschig rings is strongly recommended due to their superior performance in achieving high ethanol purity and efficiency. Further studies may explore advanced or hybrid packing materials to maximize mass transfer efficiency and reduce energy requirements.

- Comprehensive life cycle assessment (LCA) and techno-economic analysis should be conducted for large-scale implementation to evaluate environmental benefits, carbon reduction potential, and long-term economic sustainability.

References:

Abenes, F. B., & Florendo, P. D. C. (2008). Validation paradigm: The Philippine carabao as a model for cellulosic ethanol production. Paper presented at the National Conference of the Philippine Council for Industry and Energy Research and Development (PCIERD-DOST), Central Luzon State University, Science City of Muñoz, Nueva Ecija, Philippines.

Ackland, T. (2014). Home distillation of alcohol. Retrieved August 28, 2014, from http://www.homedistiller.org/designs.htm

Balat, M., Balat, H., & Öz, C. (2008). Progress in bioethanol processing. Progress in Energy and Combustion Science.

Charm, S. E. (1978). Fundamentals of food engineering (pp. 435–465). AVI Publishing.

Concepcion, J. L. (2009). Cellulosic ethanol production using corn stover (Unpublished undergraduate thesis). Central Luzon State University, Science City of Muñoz, Nueva Ecija, Philippines.

Corpuz, K. M. (2011). Design, fabrication and evaluation of prototype solar-powered still for bioethanol production (Unpublished master’s thesis). Central Luzon State University, Science City of Muñoz, Nueva Ecija, Philippines.

Cruz, I. F. (2014). Performance evaluation of crude bioethanol distiller using rice husk gasifier as heat source (Unpublished undergraduate thesis). Central Luzon State University, Science City of Muñoz, Nueva Ecija, Philippines.

Damian, G. B. (2011). Design, fabrication and evaluation of prototype anaerobic bioreactor for biomass hydrolysis (Unpublished master’s thesis). Central Luzon State University, Science City of Muñoz, Nueva Ecija, Philippines.

Elbehri, A., Segerstedt, A., & Liu, P. (2013). Biofuels and the sustainability challenge: A global assessment of sustainability issues, trends, and policies for biofuels and related feedstock. Food and Agriculture Organization (FAO).

Food and Agriculture Organization of the United Nations. (1995). Selection, testing and evaluation of agricultural machines and equipment (FAO Agricultural Services Bulletin No. 115). FAO.

Food and Agriculture Organization of the United Nations. (1997). Renewable biological systems for alternative sustainable energy production. FAO.

Gonzales, A. M. (2010). Production of bioethanol from composted rice straw using Aspergillus oryzae, Rhizopus stolonifer, and Trichoderma harzianum with the action of rumen fluid from cow (Bos taurus) (Unpublished undergraduate thesis). Central Luzon State University, Science City of Muñoz, Nueva Ecija, Philippines.

Hansen, A. C., Zhang, Q., & Lyne, P. W. L. (2005). Ethanol–diesel fuel blends: A renewable bioresource technology.

Katzen, R., Madson, P. W., & Moon, G. D., Jr. Ethanol distillation: The fundamentals. Katzen International Inc.

Maroulis, Z. B., & Saravacos, G. D. (2011). Food process engineering operations. CRC Press.

McLean, H. L., & Lave, L. B. (2003). Evaluating automobile fuel/propulsion system technologies. Progress in Energy and Combustion Science.

National Renewable Energy Laboratory. (2010). Biofuels: Fueling a stronger economy. Retrieved July 30, 2014, from http://www.nrel.gov/docs/legosti/old/5572r2.pdf

Robinson, C. S., & Gilliland, E. R. (1950). Elements of fractional distillation. McGraw-Hill.

Tham, M. T. (2006). Introduction to distillation. Retrieved December 30, 2025, from http://lorien.ncl.ac.uk/ming/distil/distill0.htm