DESIGN OF A SECURE AND SCALABLE INTERNET CAFÉ NETWORK SYSTEM

# ABSTRACT

This study presents the design of a secure and scalable Internet café network system aimed at improving connectivity, performance, and manageability in a multi-user environment. Internet cafés require reliable network infrastructures to support multiple client devices accessing shared internet services simultaneously. The study employed a descriptive design approach focusing on network requirements, hardware and software selection, topology design, IP addressing scheme, and security integration. A star topology was adopted to ensure centralized management, easy troubleshooting, and scalability for future expansion. The proposed network utilizes a structured IP addressing scheme within the 192.168.1.0/24 subnet and incorporates essential security measures such as firewall protection, antivirus systems, and user access control. The results include a complete network design with hardware and software specifications, cost estimation, and implementation considerations. The proposed system provides a reliable, secure, and scalable solution suitable for Internet café operations, improving both network efficiency and user experience.

Keywords: Internet café network, LAN, star topology, network security, IP addressing

# INTRODUCTION

# Background

Internet cafés continue to provide affordable access to computers and the internet for students, professionals, gamers, and other users. As the demand for online services and digital applications increases, the need for a reliable, secure, and efficient network infrastructure becomes more important. A well-designed network enables stable connectivity, efficient resource sharing, and effective management of multiple computer units within an Internet café environment.

# Problem Statement

# Many Internets cafés experience network-related issues such as slow internet connectivity, poor network management, security vulnerabilities, and difficulty in accommodating future expansion. Without a proper network design, these problems can negatively affect service quality and user satisfaction. Therefore, there is a need to develop a secure and scalable network design that can support efficient Internet café operations.

# Objectives

# General Objectives

# To design a secure and scalable network system for an Internet café.

# Specific Objectives

# To develop an efficient network topology for an Internet café.

# To create an appropriate IP addressing scheme for network devices.

# To identify the necessary hardware and software requirements.

# To incorporate basic network security measures.

# To provide a network design that supports future expansion and management.

## Significance

This study will benefit Internet café owners, network administrators, and students by providing a structured network design that promotes efficient connectivity, improved security, and easier network management. The proposed design may serve as a reference for establishing or upgrading Internet café network infrastructures.

## Scope and Limitations

This study focuses on the design of an Internet café network, including network topology, hardware and software requirements, IP addressing, and basic security considerations. The study is limited to the planning and design phase only and does not include actual implementation, deployment, performance testing, or user surveys.

## Related Literature

## Local Area Network (LAN) Design

A Local Area Network (LAN) is a fundamental networking structure used to connect multiple computers within a limited geographical area such as homes, schools, offices, and Internet cafés. LANs enable fast communication, efficient data sharing, and centralized control of network resources. In Internet café environments, LAN design is essential because it allows multiple client machines to simultaneously access shared internet connections and services. A properly designed LAN improves performance by reducing latency, preventing congestion, and ensuring stable connectivity across all connected devices (Forouzan, 2017).

## Network Topologies (Star Topology)

Network topology refers to the physical or logical arrangement of devices in a network. Star topology is widely used in Internet café setups because of its simplicity, reliability, and ease of management. In this configuration, all client computers are connected to a central device such as a switch or router. This setup allows easy detection and isolation of faults since issues in one workstation do not affect the entire network. Additionally, star topology supports scalability, making it suitable for environments where additional computer units may be added in the future without major redesign (Tanenbaum & Wetherall, 2011).

## Network Security in LAN Systems

Network security is a critical component in designing Internet café networks due to the public nature of user access. Without proper security mechanisms, networks may be exposed to unauthorized access, malware attacks, and data breaches. Common security measures include firewalls, antivirus software, content filtering, and user access control policies. Implementing these measures ensures confidentiality, integrity, and availability of network resources (Stallings, 2018).

## Internet Café Management Systems and Network Efficiency

Modern Internet cafés often integrate management systems that allow administrators to monitor user activity, control session time, manage billing, and enforce usage policies. These systems improve operational efficiency and enhance user experience by ensuring fair usage and stable performance across all workstations. Centralized management also reduces downtime and simplifies network administration tasks.

## Scalability and Performance Optimization in Network Design

Scalability is an important consideration in network design, particularly in Internet cafés where user demand may increase over time. A scalable network allows additional devices to be integrated without significant changes to the existing infrastructure. Techniques such as bandwidth management, load balancing, and traffic monitoring help optimize performance and ensure stable internet access even during peak usage periods (Oppenheimer, 2010).

# METHODOLOGY

This study utilized a descriptive design approach in developing a secure and scalable Internet café network system. The methodology focused on identifying network requirements, selecting appropriate hardware and software components, designing the network topology, creating an IP addressing scheme, and incorporating security measures to ensure efficient and reliable network operations.

## Network Requirements

The proposed Internet café network was designed to support multiple client computers connected to a shared internet connection. The network must provide stable connectivity, efficient resource sharing, centralized management, and sufficient bandwidth for web browsing, online gaming, and other internet-based activities. The design also considered future expansion to accommodate additional workstations without significant infrastructure modifications.

## Hardware Requirements

The hardware components required for the proposed network design include:

- Internet Service Provider (ISP) modem

- Router for internet distribution and network management

- Managed or unmanaged network switch

- Client computers/workstations

- Server computer (for centralized management)

- Ethernet cables (Cat5e or Cat6)

- Uninterruptible Power Supply (UPS)

- Network Interface Cards (NICs)

These components form the physical infrastructure necessary to establish communication among network devices and ensure reliable network operation.

## Software Requirements

The software requirements for the proposed network include:

- Operating System (Windows or Linux)

- Network management software

- Internet café management software

- Antivirus and anti-malware software

- Firewall software

- Device drivers and network utilities

These software applications help manage network resources, monitor user activities, and provide protection against security threats.

Network Topology Design

The proposed network utilizes a star topology, where all client computers are connected to a central network switch. The switch is connected to a router that provides internet access to all devices. Star topology was selected because it offers easy management, fault isolation, reliability, and scalability. If a single workstation experiences a failure, the rest of the network remains operational.

## IP Addressing Scheme

A private IP addressing scheme was developed to organize network devices efficiently. The proposed network uses the 192.168.1.0/24 network address range.

Table 1. Proposed IP Addressing Scheme for the Network

| Device | IP Address |
| --- | --- |
| Router (Gateway) | 192.168.1.1 |
| Server | 192.168.1.2 |
| Client PC 1 | 192.168.1.10 |
| Client PC 2 | 192.168.1.11 |
| Client PC 3 | 192.168.1.12 |
| Client PC 4 | 192.168.1.13 |
| Client PC 5 | 192.168.1.14 |
| Additional Clients | 192.168.1.15–192.168.1.254 |

The Dynamic Host Configuration Protocol (DHCP) may also be used to automate IP address assignment and simplify network administration.

## Security Design

To ensure network security, several protective measures were incorporated into the design. A firewall is deployed at the network gateway to monitor and filter incoming and outgoing traffic. Antivirus software is installed on all client computers to protect against malware and malicious software. User access restrictions are implemented to prevent unauthorized system modifications. Regular software updates and network monitoring are also recommended to maintain security, improve system performance, and reduce vulnerabilities.

The integration of these security measures helps protect network resources, maintain operational stability, and provide a safer computing environment for Internet café users.

RESULTS

## Network Topology Diagram

The proposed Internet café network uses a Star Topology, where all client computers are connected to a central switch. The switch is connected to a router that provides internet access through an ISP modem. This design ensures centralized control, easy troubleshooting, and efficient expansion.

Figure 1. Network Layout of the Proposed Internet Café System

Text Representation of Topology:

ISP Modem → Router → Switch → Client PCs / Server

## Hardware Requirements Table

## Table 2. Required Hardware for Network Implementation

| ## Component | ## Description | ## Quantity |
| --- | --- | --- |
| ## ISP Modem | ## Provides internet connection | ## 1 |
| ## Router | ## Manages network traffic | ## 1 |
| ## Network Switch (24-port) | ## Connects all devices | ## 1 |
| ## Client PCs | ## Workstations for users | ## 10 – 20 |
| ## Server PC | ## Management system host | ## 1 |
| ## Ethernet Cables (Cat5e or Cat6) | ## Wired connections | ## As needed |
| ## UPS | ## Power backup protection | ## 1 – 2 |
| ## NIC (Network Cards) | ## Network interface for PCs | ## Built-in |

## Software Requirements Table

## Table 3. Software requirements for the Proposed Internet Café Network

| ## Software | ## Purpose |
| --- | --- |
| ## Windows OS / Linux | ## Operating system for PCs |
| ## Internet Café Management System | ## Time tracking and billing |
| ## Antivirus Software | ## Protection from malware |
| ## Firewall System | ## Network security control |
| ## DHCP Service | ## Automatic IP assignment |
| ## Network Monitoring Tools | ## Performance tracking |

## IP Addressing Table

## Table 4. IP Address Allocation for Networking Devices

| ## Device | ## IP Address |
| --- | --- |
| ## Router (Gateway) | ## 192.168.1.1 |
| ## Server | ## 192.168.1.2 |
| ## PC 1 | ## 192.168.1.10 |
| ## PC 2 | ## 192.168.1.11 |
| ## PC 3 | ## 192.168.1.12 |
| ## PC 4 | ## 192.168.1.13 |
| ## PC 5 | ## 192.168.1.14 |
| ## PC 6–20 | ## 192.168.1.15 – 192.168.1.254 |

## Cost Estimation

## Table 5. Estimated Budget Requirements for Network Deployment

| ## Item | ## Estimated Cost (PHP) |
| --- | --- |
| ## Router | ## 2,500 – 5,000 |
| ## Switch (24-port) | ## 3,000 – 6,000 |
| ## ISP Installation | ## 1,500 – 3,000 |
| ## Client PCs (10 units) | ## 150,000 – 250,000 |
| ## Server PC | ## 20,000 – 40,000 |
| ## Cables & Accessories | ## 3,000 – 8,000 |
| ## UPS | ## 3,000 – 10,000 |

Estimated Total Cost: ₱180,000 – ₱322,000 (depending on number of units and specifications)

# DISCUSSION

Explanation of the Design

# The proposed Internet café network is designed using a star topology, where all devices are connected to a central switch. This setup simplifies network management and ensures that each workstation has a stable and direct connection to the network backbone. The router manages internet traffic while the switch distributes connectivity to all client machines.

# The IP addressing scheme is structured within the private Class C range (192.168.1.0/24), allowing efficient organization and easy management of devices. DHCP can also be implemented to automate IP distribution.

Advantages of the Design

The proposed network design offers several advantages:

- Centralized network management

- Easy troubleshooting and maintenance

- Scalable structure for future expansion

- Efficient resource sharing among users

- Stable and consistent internet connectivity

- Reduced network complexity

## Security Features

The network includes several security mechanisms to ensure safe operation:

- Firewall protection at the router level

- Antivirus software on all client machines

- User access restrictions to prevent system misuse

- Network monitoring tools for activity tracking

- Optional content filtering for public usage control

These features help protect the system from malware, unauthorized access, and network misuse.

## Scalability

The design supports scalability by allowing additional client computers to be added without modifying the entire network structure. The use of a switch-based star topology makes expansion simple and cost-effective. Additional IP addresses within the subnet allow up to 254 devices in the same network range, making it suitable for future growth of the Internet café.

CONCLUSION

The study successfully designed a secure and scalable Internet café network system using a star topology. The proposed design includes a structured IP addressing scheme, appropriate hardware and software requirements, and essential security measures such as firewall protection and antivirus integration. The use of a centralized router for DHCP improved network manageability and reduced configuration complexity. Overall, the design meets the requirements of a stable, efficient, and expandable Internet café network system.

RECOMMENDATION

It is recommended that future improvements include the implementation of advanced network monitoring tools for real-time traffic analysis and performance optimization. The Internet café may also consider upgrading to higher bandwidth internet connections to support increasing user demand. Additionally, implementing more advanced security features such as intrusion detection systems (IDS) and user authentication portals can further enhance network protection and control.

ACKNOWLEDGEMENT

The researchers humbly extend their sincere gratitude to Engr. Minerva C. Zoleta, their research adviser, for her invaluable guidance, unwavering support, and constructive supervision throughout this study. Her insightful feedback, technical expertise, and thoughtful suggestions greatly contributed to the improvement of the research design, methodology, and overall quality of this work. Her mentorship continuously motivated the researchers to pursue academic excellence and professional growth.

The researchers would also like to express their appreciation to their classmates, friends, and fellow students who provided encouragement, helpful suggestions, and moral support throughout the conduct of this study. Their cooperation and motivation played a significant role in helping the researchers overcome challenges and complete this work successfully.

Above all, the researchers offer their deepest gratitude to God Almighty for His guidance, wisdom, strength, and countless blessings throughout the research journey. His grace and protection made the completion of this work possible and meaningful.

# REFERENCES

- Forouzan, B. A. (2017). Data communications and networking (5th ed.). McGraw-Hill Education. https://openlibrary.org/works/OL35531W/Data_Communications_and_Networking

- Kurose, J. F., & Ross, K. W. (2021). Computer networking: A top-down approach (8th ed.). Pearson. https://www.pearson.com/en-us/subject-catalog/p/computer-networking-a-top-down-approach/P200000003282

- Oppenheimer, P. (2010). Top-down network design (3rd ed.). Cisco Press. https://www.ciscopress.com/store/top-down-network-design-9781587202838

- Stallings, W. (2018). Network security essentials: Applications and standards (6th ed.). Pearson. https://www.pearson.com/en-us/subject-catalog/p/network-security-essentials-applications-and-standards/P200000003279

- Tanenbaum, A. S., & Wetherall, D. J. (2011). Computer networks (5th ed.). Pearson. https://www.pearson.com/en-us/subject-catalog/p/computer-networks/P200000003285

# ABOUT THE AUTHORS

Jefferson Dacpano is a Bachelor of Science in Computer Engineering student at Eulogio Amang Rodriguez Institute of Science and Technology with a strong interest in computer hardware, electronics, and system maintenance. He has developed knowledge and practical skills in computer assembly, hardware troubleshooting, preventive maintenance, and system configuration through his academic training and hands-on activities. In addition, he has obtained a Computer Systems Servicing (CSS) NC II certification, which has strengthened his competencies in installing, maintaining, and repairing computer systems and networks. Dedicated to continuous learning and professional development, he seeks opportunities to apply his technical knowledge and problem-solving skills in real-world environments while contributing to efficient and reliable technology solutions.

Robin Ace Golez is a third-year Computer Engineering student at Eulogio “Amang” Rodriguez Institute of Science and Technology (EARIST). He has completed training in Arduino Programming under the Special Opportunity Program and holds TESDA National Certificates II (NC II) in Gas Metal Arc Welding (GMAW) and Computer System Servicing (CSS). His interests include automation and mechatronic systems, with hands-on experience in programming and microcontroller-based projects. His academic focus includes control systems, automation, and the integration of hardware and software for engineering applications.

Roilan Lagunero is a third-year Bachelor of Science in Computer Engineering student at Eulogio Amang Rodriguez Institute of Science and Technology and a Computer Systems Servicing (CSS) NC II holder. He has a strong interest in computer hardware, system maintenance, troubleshooting, and hands-on engineering applications. Through his academic training and technical certification, he has developed practical skills in computer assembly, network setup, and hardware diagnostics. Passionate about problem-solving and system design, he aims to further enhance his technical expertise and contribute effectively to engineering and technology-related projects in the future.

Joshua Philip Pascual is a 3rd year Computer Engineering college student in Eulogio "Amang" Rodriguez Institute of Science and Technology (EARIST). He has obtained TESDA National Certificate II (NC II) in Computer Systems Servicing (CSS). He is a student that has a strong programming foundation and is leaning towards learning more about mechatronics, robotics, and hardware engineering. He is particularly interested in developing efficient, reliable solutions that bridge embedded systems and intelligent automation. And he is eager to continue to expand his technical skills through hands-on projects and collaborative engineering projects.

Arjay Satorre is a third-year Bachelor of Science in Computer Engineering student at Eulogio Amang Rodriguez Institute of Science and Technology with a strong passion for technology and innovation. He is committed to developing practical and impactful engineering solutions that address real-world challenges and contribute to technological advancement. Aspiring to establish his own technology-based business in the future, he continuously enhances his technical knowledge and entrepreneurial skills through academic projects and hands-on learning experiences. He is also a holder of TESDA Computer Systems Servicing (CSS) COC I and COC II, demonstrating his competence in computer systems servicing, installation, maintenance, and troubleshooting.

Mark Anthony Zamora is a third-year Computer Engineering student at Eulogio “Amang” Rodriguez Institute of Science and Technology who is passionate about hands-on engineering, problem-solving, and system design. He enjoys working on practical projects that enhance his technical skills and deepen his understanding of engineering concepts. He has obtained a TESDA National Certificate II (NC II) in Computer Systems Servicing (CSS), which strengthened his knowledge in hardware, troubleshooting, and system maintenance.

Engr. Minerva C. Zoleta, a Professional Computer Engineer, is a dedicated Computer Engineering Professor at the Eulogio “Amang” Rodriguez Institute of Science and Technology in the Philippines, specializing in Embedded Systems, Operating Systems, and Computer Network and Security. With a strong background in academia and industry. She has been instrumental in shaping the next generation of Engineers through innovative teaching methods and hands-on research. Engr. Zoleta holds a Master’s degree in Electrical Engineering major in Computer Engineering at Technological University of the Philippines, Manila and is pursuing her doctorate degree in Engineering with specialization in Computer Engineering AT Technological Institute of the Philippines. She has presented published research on topics such as Embedded System, IoT applications, and wireless communication international conferences and journals. Passionate about technology-driven solutions, She has led various projects integrating smart systems into real-world applications, contributing to the advancement of local and international engineering communities.