A REVIEW ON INSTRUMENTATION AND VALIDATION METHOD FOR THE ANALYSIS OF DIFFERENT DRUGS

Author: Devendra Singh, School of Pharmacy, Vivek University Bijnor, 246701 (U.P), India.

Co-Author: Dr. Saurabh Sharma, School of Pharmacy, Vivek University Bijnor, 246701 (U.P), India.

Co-Author: Dr. Piyush Pankaj, Sardar Patel College of Pharmacy, Gorakhpur, 273013, (U.P), India.

Co-Author: Vipin Kumar, School of Pharmacy, Vivek University Bijnor, 246701 (U.P), India.

Abstract:

Finding applications for UV/visible spectroscopy and High Performance Liquid Chromatography was the goal of the current investigation. UV-visible spectroscopy is used in analysis to examine different kinds of solvents and compounds. Spectroscopy is usually preferred by small businesses since the equipment is inexpensive and requires little upkeep. The analysis method is based on calculating how much monochromatic light is absorbed by colourless molecules in the near-ultraviolet region of the spectrum (200-400 nm). Determining the "identity, intensity, consistency, and purity of such compounds" is a key step in pharmaceutical analysis. Analysis of intermediates and raw materials used in the production of drugs is also included. It is often recognised that the most important factor in the synthesis and optimisation of a novel molecule for the successful creation of formulations is the dissociation constant.

High performance liquid chromatography, fluid visible ultraviolet spectrophotometry, difference spectrophotometry, and simultaneous equation systems were all tested. High performance liquid chromatography is a useful analytical technique for assessing pharmaceutical products. High performance liquid chromatography techniques should be able to separate, identify, and measure various medicines and drug-related degradants that can create, identify, and measure any drugs and drug-related contaminants that may be introduced during synthesis. The process of identifying the performance qualities and limits of a technique and defining the elements that alter its characteristics and to what degree is known as validation.

Keywords: High performance liquid Chromatography, Spectrophotometer, Spectroscopy, Spectrum

Introduction:

Validation of analytical techniques is a crucial regulatory criterion for pharmaceutical research. High performance liquid chromatography is a common analytical technique used in the development and validation of test techniques for pharmacological compounds and constituents. A procedure's validation provides documented proof and a high level of assurance that an empirical method employed for a specific test is adequate for its intended usage. In recent years, regulatory bodies have been more conscious of the necessity of ensuring that data given to them in marketing authorisation applications is collected using approved analytical techniques. The most widely utilised validation features are precision accuracy (repeatability and intermediate accuracy), specificity, range, detection limit, quantitation limit, linearity, and stability of analytical solutions.

The type of procedure and its applications determine which parameters require validation and which approach is used in each particular situation. Before beginning the process of method validation, it is crucial that the analytical system itself be established, maintained, calibrated, and verified. These combination products might provide formidable hurdles for analytical chemists who are in charge of creating and validating analytical procedures. The identification and measurement of contaminants is a crucial step in the development of pharmaceutical processes for quality and safety.

[1, 2].

### Analytical chemistry:

Analytical chemistry is the examination of material samples to get a knowledge of their chemical composition and structure. Analytical chemistry is the science of quantitative measurement. Analyte measurement in a complicated sample turns into a problem-solving task. Chemical knowledge that can be used in sample analysis, sample handling techniques for a variety of issues (the tools-of-the-trade), appropriate data analysis, and accurate and trustworthy record keeping are all included in sample analysis.

#### Qualitative:

The identification of elements, ions, or compounds present in a sample is the focus of qualitative analysis.

- Quantitative:

Determining the amount of one or more elements is the focus of quantitative analysis.

## Analytical methods for pharmaceutical analysis:

## UV/VIS-spectrophotometric methods:

Spectroscopy in the ultraviolet-visible spectrum refers to the utilisation of visible light. Particularly, absorption or reflectance affects the apparent colour in the optical spectrum. In that region of the electromagnetic spectrum, molecules go through electronic changes. The method is supplementary to fluorescence spectroscopy, in which absorption steps determine transitions from the ground state to the excited state, whereas fluorescence deals with transitions from the excited state to the ground state [3].

- Electroanalytical methods:

High sensitivity and low detection limits are achieved by combining electrochemical techniques with straightforward, low-cost equipment. Coulometry, electrogravimetry, potentiometry, conductometry, polarography, amperometry, and voltammetry are the electrochemical techniques utilised in dose formulations for drug examination. However, voltammetric methods are by far the most used electrochemical techniques, and their application in dose forms of drug analysis has been well reviewed in the literature [4].

- Capillary electrophoresis:

Capillary electrophoresis is a potent isolation method that works with both big and tiny compounds. Among its numerous uses, pharmaceutical quantification merits mention. This technique has a high separation efficiency, which enables challenging separations, short analytical times, quick method expansion, little sample and solvent consumption, and simple, automated instrumentation [5].

- Turbulent-Flow chromatography (TFC):

One excellent option for cutting down on measurement times is turbulent-flow chromatography (TFC). Turbulent flow chromatography is a method that uses the differing diffusion coefficients of proteins and small molecules close to the stationary phase surface in a bulk turbulent flow regime to differentiate between small and big molecules [6]. This technology is particularly enticing since it is online and requires little manual manipulation for high throughput. TFC is mostly used as a sample preparation tool in bio-analysis since it makes it possible to separate the target analytes from the big molecules that comprise the majority of the biological matrix.

- Counter current chromatography in analytical chemistry:

Counter current chromatography is a kind of liquid chromatography that employs two immiscible liquid phases in the absence of a solid support. As a liquid chromatography method, counter current chromatography makes use of a number of terminologies that have previously been developed for chromatography. Countercurrent chromatography is acceptable since the two liquid phases do not flow countercurrent to one another. A stationary step of support-free liquid is used in countercurrent chromatography, a liquid chromatography method.

#### Advantages of a support-free liquid stationary phase:

- It is adaptable.

- It's a special opportunity.

- The extremely large volume of stationary phase may be used by Solutes.

- Applications:

- Countercurrent chromatography is becoming more and more important in the science of separation. By doing this, the permanent adsorption and contamination of the samples may be almost eliminated and all the components in the sample solution injected into the column can be recovered.

- A blunt sample may be injected straight into the column, making sample preparation easier. Countercurrent chromatography is an efficient method for separating organic and inorganic chemicals from a complicated mixture [7].

- Traditional analytical techniques:

Current analytical chemistry is governed by sophisticated apparatus; ancient methods, many of which are still in use today, are also the foundation of analytical chemistry and some of the concepts that underpin current instruments [8].

- Titration:

In titration, a reactant is added to a solution under analysis until an equivalent point is achieved. It is possible to calculate the amount of substance in solution under analysis.

#### Gravimetric analysis:

Determining the quantity of material in the sample is the goal of gravimetric analysis. Weighing the sample either before or after a transformation is how it is accomplished.

- Inorganic qualitative analysis:

In general, inorganic qualitative analysis refers to a methodical approach that verifies the existence of certain, often aqueous, ions or elements by carrying out a sequence of reactions that rule out ranges of possibilities before verifying suspected ions with a confirming test.

#### Miscellaneous techniques:

The drug quantification process uses the several sorts of diverse approaches listed below [9, 10].
i. Dielectric permittivity and loss factor in Dielectric Thermal Analysis (DEA). 
ii. Temperature difference: Differential Thermal Analysis (DTA). 
iii. Heat difference in Differential Scanning Calorimetry (DSC). 
iv. Dilatometry: volume (DIL). 
v. Mechanical stiffness and damping in dynamic mechanical analysis (DMA). 
vi. Gaseous breakdown products: evolved gas analysis (EGA). 
vii. Thermal conductivity and diffusivity in laser flash analysis (LFA). 
viii. Mass in thermogravimetric analysis (TGA). 
Ix. Dimension in thermomechanical analysis (TMA).

#### Hyphenated techniques:

"Hybrid" or "hyphenated" procedures are combinations of the aforementioned methods. New hybrid approaches are being developed, and some examples are currently in widespread usage [11, 12]. 
It is possible to separate and quantify organic samples by employing hyphenated procedures.
i. Gas Chromatography-Mass Spectrometry, or GC-MS. 
ii. Inductively Coupled Plasma-Mass Spectrometry, or ICP-MS. 
iii. Gas Chromatography-Infrared Spectroscopy, or GC-IR. 
iv. Mass Spectrometry-Mass Spectrometry, or MS-MS. 
liquid chromatography-mass spectrometry, or LC-MS. 
Instrumental procedures need a minimal quantity of material and are sensitive. With enough accuracy and dependability, complex mixtures can be analysed with or without their previous separation.

Table 1: Comparison of various analytical methods

| Technique | Specificity | Properties |
| --- | --- | --- |
| GC-MS | High | High resolution, Good for unknowns |
| HPLC-UV(And DAD) | Good Moderate better with DAD | Good for soy food and conjugates |
| Fluorescence | Good | Sensitive |
| ED (And array) | Better with detection array | Suitable for biological samples |
| MS | High | Ease of use and sensitive |
| UV (DAD) | Moderate better with DAD | High separation resolution Excellence mass sensitivity |
| Fluorescence | Moderate | Sensitive |
| ED(And array) | Moderate | Sensitive |
| MS | High | Sensitivity |
| UV and IR spectroscopy | High | High throughput |
| MALDI-MS | High | High throughput |

### Spectrophotometry:

### Spectrophotometry is the scientific field that studies how chemical samples interact with electromagnetic light. When a sample's molecules, atoms, or ions change from one energy state to another, the absorption or emission of electromagnetic radiation (EMR) must be calculated and analysed. This transition might occur from an excited to a ground state or from a ground state to an enthusiastic state. The total of a molecule's ground-level rotational, vibrational, and electronic energy is its energy. To put it another way, spectroscopy determines changes in rotational, vibrational, or electronic energy.

### It is the most effective tool for studying atomic and molecular structures and is used to analyse a variety of materials. Optical spectroscopy uses the region of the electromagnetic spectrum between 200 and 400 nm. Table 2 lists the areas of the electromagnetic spectrum.

Table 2: Regions of electromagnetic spectrum

| Region | Wavelength |
| --- | --- |
| Far (or vacuum) ultraviolet | 10-200 nm |
| Near ultraviolet | 200-400 nm |
| Visible | 400-800 nm |
| Near infrared | 0.75-2.5μm |
| Mid infrared | 2.5-25 μm |
| Far infrared | 25-300 μm |

#### Ultraviolet spectrophotometers:

#### UV-visible spectrophotometry is the most popular technique in pharmaceutical research. Calculating how much UV or visible light a solution material absorbs is necessary. The percentage or function of the ratio of the intensity of two light beams in the UV-visible field is measured by ultraviolet-visible spectrophotometers [13]. If reported data is available, organic substances can be categorised using a spectrophotometer in qualitative research. Quantitative spectrophotometric analysis is performed to ascertain the number of molecular species that absorb radiation. Spectrophotometry is a quick, fast, somewhat specific method that may be used with tiny quantities of substances.

The principle involved in Ultraviolet Spectrophotometry:

A compound solution is used to disperse radiation with wavelengths between 200 and 400 nm. The bond's electrons get stimulated inside the molecule, occupying a higher quantum state and using up some of the energy flowing through the solution. The absorbed radiation's wavelength (lower energy) increases with the degree of electron looseness the bond of the molecule. The power of photons travelling through a sample that contains the analyte is reduced as a result of absorption. This measurement of attenuation, which we refer to as absorbance, provides a negative signal [14].

Only when the photon's energy matches the energy differential between the two energy levels can absorption take place. A depiction of absorbance as a function of photon energy is known as an absorbance spectrum. When an analyte transitions from a higher-energy state to a lower-energy state, photon-emission takes place.

#### The principle involved in Visible Spectrophotometry (Colorimetry):

The study of visual photon absorption at wavelengths between 400 and 800 nm is known as colorimetry. Radiation in the wavelength range may be absorbed by any coloured material. We get an absorption curve (absorbance against wavelength) because coloured materials absorb light at different wavelengths in different ways. The wavelength on this absorption curve where the most light is absorbed is called λmax. This λmax is a qualitative characteristic that aids in substance identification and is unique or typical for any coloured material.

#### Instrumentation of UV Spectrophotometry:

#### Following are the instrumentation of single beam and double beam UV Spectrophotometry [15].

Figure 1: UV-Visible single beam Spectrophotometer

The components of UV-Visible double beam spectrophotometer are:

i. Source of light.

ii. Monochromators.

iii. Sample cells.

iv. Detector.

Figure 2: UV Visible double beam Spectrophotometer

Figure 3: Photo Multiplier Tube

The many light sources utilised in UV are: 
i. A light that emits hydrogen.
ii. A light with deuterium. 
iii. A light with xenon discharge. 
iv. Arc light made of mercury. 
Monochromators 
When it comes to transforming polychromatic light into monochromatic light, monochromators are superior and more effective than filters. 
The units of a monochromator are as follows: 
i. The entrance slit. 
ii. Collimator. 
iii. Prism/grating. 
iv. The exits are illuminated.

Grating monochromators, which are composed of quartz, are the most often used monochromator in UV. A band pass of 0.4–2 nm is provided using grating. 
There are two kinds of grating monochromators. 
i. Diffraction gratings.
ii. Gratings for gearbox.

Sample cells:

Since glass cells will absorb UV light, quartz is the sole material used in sample cells. These cells have a path length of 10 mm, or 1 cm.

Choice of solvent:

The following criteria should be met by a suitable solvent for UV spectroscopy. 
i. It shouldn't absorb radiation in the area being studied.
ii. To minimise contact with the solute molecules, it should be less polar. 
iii. A highly pure solvent should be utilised.

Detectors:

A detector is a transducer that transforms EMR into an electron flow, which is then transformed into a voltage or current flow in the readout circuit. As detectors, photoelectric or photomultiplier tubes are typically utilised. The following crucial conditions must be met by the detector. It needs to react to radiant radiation across a wide range of wavelengths. 
a. It ought to be sensitive to low radiation power levels.
b. It must to react to the radiation quickly and provide an electrical signal. It is easily magnified. 
c. For stability, it should have a comparatively low noise level. 
d. The power of the beam that strikes it immediately affects the signal that is generated.

The most sensitive, costly, and complex devices employ detectors. The detector works on the principle that photoelectrons are multiplied by secondary electron emission, as shown in Figure 3. A photocathode and up to ten dynodes of anodes are used to accomplish this. Every dynode is kept between 75 and 100 volts higher than the one before it. Due to secondary electron emission, the electron emission is increased by a factor of four or five at each stage.

- Quantitative analysis in UV spectrophotometry:

#### Assay of substances in a single component sample:

One of three main methods is then used to compute the assay of single component samples that contain additional absorbing compounds from the obtained absorbance. 
Sample concentration may be calculated using a variety of techniques, including:
a. Using the absorptivity value (E1%,1cm). 
b. The multiple standard technique or calibration curve method. 
c. Direct comparison or a single standard.

#### Using absorptivity (E1%, 1cm) values:

Its absorptivity is calculated using the standard absorptive value technique, which uses standard A (1%, 1cm) or E values. benefit when obtaining a sample of the reference material is costly or difficult [16].

Calibration curve method or multiple standard methods:

Using the calibration curve approach, a calibration graph is created by measuring the absorbances of many standard solutions of the reference material at concentrations that span the sample concentrations. Because the concentration is similar to the answer's absorbance, the concentration of the analyte in the sample solution may be determined from the graph.

#### Single standard or direct comparison method:

The absorbance of a sample solution and a standard solution of the reference material are measured as part of the single point standardisation process. The proportionate connection between absorbance and concentration is used to calculate the concentration of the chemicals in the sample. The characteristic that every wavelength is the foundation of all spectro-photometric methods for multi-component samples. The total absorbance of a solution is the sum of the absorbances of its constituent parts; measured absorbance is the difference between the total absorbance of the solution in the reference cell and the sample cell. One of the following methods is used to determine the concentration of the absorbing material in multi-component formulations based on the observed absorbance [17, 18].

### Principle of High Performance Liquid Chromatography:

The method uses a flow of liquid solvent to pass a liquid sample over a solid adsorbent that has been packed into a column. The analytes' flow is slowed down because each analyte in the sample interacts with the adsorbent somewhat differently. The analytes flow out of the column quickly if the connection is weak, and the elution period is lengthy if the interaction is strong. High performance liquid chromatography has been applied in research (e.g., separating the components of a complex biological sample, or of comparable synthetic chemicals from each other), manufacturing, medicine, and law (e.g., detecting vitamin D levels in blood serum, detecting performance enhancement drugs in urine) [19]. (For instance, when pharmaceutical and biological goods are being produced).Chromatography is frequently characterised as an adsorption-based mass transfer method. In order to separate the sample components, high performance liquid chromatography uses pumps to move a pressurised liquid and a sample combination along a sorbent-filled column. The sorbent, the column's active component, is typically a granular substance composed of solid particles (such as silica, polymers, etc.) that range in size from 2 to 50 µm. Due to varying degrees of contact with the sorbent particles, the components of the sample mixture are separated from one another. Pressurised liquids are known as a "mobile phase" and are usually a mixture of solvents (such as water, acetonitrile, and/or methanol) [20].

#### Advantages of High Performance Liquid Chromatography:

i. Speed (analysis may be completed in less than 20 minutes). 
ii. Increased sensitivity (other detectors might be used).
iii. Higher resolution (several different stationary phases). 
iv. Reusable columns, which are costly yet useful for numerous analyses. 
v. Perfect for low-volatility chemicals. 
vi. Simple sample management, recovery, and upkeep.

#### Modern High Performance Liquid Chromatography:

Many sectors, including pharmaceuticals, agrochemicals, dyes, petrochemicals, natural products, and others, now depend on highly advanced, dependable, and quick liquid chromatographic (LC) separation processes. Gravity-fed open tubular columns with particles as small as 100 microns were utilised in early liquid chromatography; separations frequently developed using the human eye as a detector.

Instrumentation of High Performance Liquid Chromatography:

High performance liquid chromatography uses two fundamental elution modes: isocratic and gradient liquid chromatography system operation. The first is referred to as isocratic elution. The mobile phase, which might be a mixture or a pure solvent, doesn't change during the run in this mode.

Figure 4: Isocratic high performance liquid chromatography system

The second kind is known as gradient elution, in which the composition of the mobile phase varies throughout the separation process, as the name suggests. Samples containing chemicals with a broad range of chromatographic polarity can benefit from this mode. As separation advances, the mobile phase's elution strength is raised to elute the sample components that are more firmly held. Two solvent bottles and two pumps are included in the most basic scenario, which is seen in Figure 5. The gradient controller controls each pump's speed to provide varying amounts of each solvent during the separation process.

The real mobile phase composition, which is gradually supplied to the column, is created in the mixer by combining two streams. Initially, there is more of the weaker solvent [Solvent A] in the mobile phase [21]. In accordance with a preset schedule, the percentage of the stronger solvent [Solvent B] is gradually raised. 
Because the mixer is located downstream of the pumps, a gradient is produced under high pressure. Other high-performance liquid chromatography systems are made to combine many solvent streams at low pressure in front of a single pump. Gradient proportioning valves change the mobile phase's strength over time by selecting among the four solvent bottles.

Figure 5: High Pressure Gradient High Performance Liquid Chromatography System

#### High Performance Liquid Chromatography Gradient Mixtures:

High performance liquid chromatography gradient mixers must offer extremely accurate solvent composition management in order to preserve a repeatable gradient profile. This can be complicated in high performance liquid chromatography by the tiny quantities of elution required by several systems. Creating a consistent gradient is far more challenging when combining tiny amounts than when mixing big volumes. Low dispersion flows in the mixer are necessary for low pressure systems, and the process must be quite precise. For multi-pump high pressure systems, this necessitates extremely accurate flow rate control, allowing for very small flow rate modifications [22].

- High Performance Liquid Chromatography Pumps:

Because contemporary high performance liquid chromatography uses tiny particles, modern liquid chromatography pumps must operate safely and effectively at pressures of 10,000 psi or at least 6,000 psi. Sapphire pistons, stainless steel cylinders, and return valves with sapphire balls and stainless steel sheets enable high performance liquid chromatography pumps to function at these pressures while being sensitively inert to the variety of solvents utilised. For analytical applications, high performance liquid chromatography pumps should have flow rates between 0 and 10 ml/min; however, preparatory high performance liquid chromatography may require flow rates more than 100 ml/min. It is exceedingly challenging to provide a highly constant flow rate at very low flow rates.

A flow variance of less than 10μl/min is needed for 1 ml/min if 1 percent is deemed acceptable. The majority of high performance liquid chromatography detectors are flow sensitive, and variations in flow rate can produce quantisation mistakes, hence the degree of consistency is necessary.

- High Performance Liquid Chromatography Sample Valves:

Since the sample valves are positioned between the pump and the column, High Performance Liquid Chromatography sample valves must also be able to withstand pressures of up to 10,000 psi. In analytical high performance liquid chromatography, the sample volume should be selectable between sub-microliters and a few microlitres; in preparative high performance liquid chromatography, the sample volume may even exceed 10 millilitres. To maintain device performance, the sample valve must be designed with extremely low dispersion characteristics. This applies not only to flow dispersion but also to the less obvious dispersion issues brought on by sample adsorption/desorption on valve surfaces and sample diffusion between valve moving parts into and out of the mating surfaces.

Naturally, the valves must provide a highly consistent sample size, but a constant size sample loop is typically used to achieve this.

- High Performance Liquid Chromatography Columns:

The particles in the box of High Performance Liquid Chromatography columns are extremely small, typically a few microns in diameter. Very small particles are required to provide the low dispersion that results in the large plate counts required by contemporary high performance liquid chromatography. Plate counts exceeding 25,000 per column are feasible with contemporary columns; however, these relatively high efficiencies are rarely observed with real samples because of the dispersion caused by injection valves, detectors, data acquisition systems, and the higher molecular weight of real samples in comparison to conventional test samples [23].

Although loading very tiny particles into the column is a challenging technical issue, careful attention must be paid to the column end fittings as well as the input and output connectors to minimise dispersion, even with proper packing. High performance liquid chromatography's main benefit is the far greater variety of solvents and packing materials that may be utilised due to the much lower quantities needed for each. Specifically, extremely expensive optically pure chemicals can be utilised as (disposable) HPLC solvents and as stationary phases for Chiral High Performance Liquid Chromatography.

- High Performance Liquid Chromatography Detectors:

Analytes are detected as they elute from the chromatographic column by the HPLC detector, which is situated at the end of the column. Evaporative light scattering, UV spectroscopy, fluorescence, mass spectrometry, and electrochemical detectors are frequently employed. The individual molecules that emerge (elute) from the column can be seen (detected) by the detector. In order for the chemist to do a quantitative analysis of the components of the sample, a detector is used to quantify the quantity of those molecules.

The liquid chromatogram, or the graph of the detector response, is produced by the detector's output to a computer or recorder. In addition to providing the necessary sensitivity, a detector delivers a particular response for the components divided by the column. It must be unaffected by variations in the composition of the mobile phase. keeping an eye on the mobile phase as it leaves the column.

Table 3: Normalvs. Reversed Phase Chromatography Comparison

| Parameters | Normal | Reverse |
| --- | --- | --- |
| Packing polarity | Higher | Lower |
| Solvent polarity | Lower | Higher |
| Elution order | Non-polar first, then polar | Polar first, then non-polar |
| Effect of increasing solvent polarity | Decrease retention time | Increases retention time |

## Analytical method validation:

An analytical technique has been developed to assess the quality of pharmaceutical items. This is crucial to guarantee that the developed analytical procedure's performance characteristics meet the needs of the planned analytical application. The process that ensures this through laboratory research is described as method validation. The process of proving that analytical techniques are adequate for their intended application and that they enable the identification, intensity, and consistency of drug substances and drug products for quantification is known as system validation. Any pharmaceutical plant's primary goal is to continuously produce goods of the necessary quality and attributes at the lowest feasible cost.

The pharmaceutical business has been conducting validation studies for a long time. Because the sector has placed more focus on quality assurance programs in recent years, there is a growing interest in validation, which is essential to an effective production operation. Validation is a notion that has developed in the United States with the intention of facilitating the identification, intensity, and consistency of drug substances and drug products for measurement. Any pharmaceutical plant's primary goal is to continuously produce goods of the necessary quality and attributes at the lowest feasible cost.

The pharmaceutical business has been conducting validation studies for a long time. Because the sector has placed more focus on quality assurance programs in recent years, there is a growing interest in validation, which is essential to an effective production operation. The idea of validation first appeared in the United States in 1978. Over time, the concept of validation has grown to encompass a wide range of activities, from computerised systems for clinical trials, labelling, or process control to analytical techniques used for the quality control of drug substances and drug products. Validation is based on regulatory requirements, but it is best understood as a crucial and essential component of cGMP.

Simply put, validation is the process of evaluating validity or demonstrating efficacy. The validation is a collaborative endeavour including individuals from different plant disciplines [24, 25, 26].

## Importance of validation:

- It's quality control.

- It has a time limit.

- Process optimisation is what it is.

- A decrease in quality costs.

- Nominal errors and bottlenecks.

- Better, fewer batch failures.

- Effectiveness and productivity.

- The output went increased.

- Steer clear of capital investments.

#### Planning for validation:

Every validation task has to be organised. Clearly specified and documented invalidation master plans or similar papers are essential components of a validation program. 
i. The validation master plan need to be a succinct, unambiguous, and compact summary document. 
ii. Information on at least the following should be included in the validation master plan: 
a. Policy for validation. 
b. The validation activities' organisational structure. 
c. An overview of the facilities, systems, tools, and procedures that need to be verified. 
iii. Documentation format: The report and protocol format. 
a. Scheduling and planning. 
b. Control of change. 
b. Citing an already-existing document. 
d. Separate validation master plans can be required for large projects.

- Strategy for the validation of methods:

The process of developing and validating methods is iterative. The impact of operational parameters on the method's performance may be assessed during the validation stage, which was not carried out during the method's development and optimisation phases. The most important reason for validation is that a method's validity can only be shown by laboratory trials. Examining past results is insufficient; experimental investigations that are meant to validate the specific strategy must be carried out, and such investigations must be pre-planned and specified in the necessary paperwork.

The report should provide a clear explanation of the method's intended application and operating principles, as well as the validation parameters that will be examined and a rationale for the method's selection [27]. A description of the analytical procedure and predetermined admittance requirements must also be included.

- Types of validation:

#### Prospective validation:

This is described as documented evidence that the system performs as intended based on a pre-planned process. Usually, it is validated before a new product or a product with a modified production method is delivered. carried done in a minimum of three consecutive batches of output sizes. Before the process is put into commercial usage, a validation methodology is carried out in prospective validation. During the product development stage, the manufacturing process can be separated into distinct stages.

Each step should be assessed based on experience or theoretical considerations in order to identify the crucial characteristics that may impact the final product's quality. Determine how important these factors are, and then organise a number of trials. The authorised protocol should be followed when preparing and documenting the studies.

- Concurrent Validation:

It is accomplished in two situations: first, for equipment that is already in use; second, appropriate installation verification is carried out in addition to special operating tests. For a contemporary, regularly produced product, the dates are gathered from at least three successful trials. This is comparable to both prospective and retrospective validation, with the exception that the running firm will sell the product to the public at its selling price throughout the certification runs. In-process inspection and product testing at crucial production stages are examples of validation.

#### Retrospective Validation:

This is described as the current documentation demonstrating that a system performs as intended while reviewing and analysing previous data. It is accomplished by analysing factory testing data from the past to demonstrate that the procedure has always been in order. It is a type of process validation for an already-sold product. Retrospective validation is only appropriate for well-established systems; it would not be acceptable if the product's composition, operating methods, or equipment had recently changed.

It is crucial to use past data to validate such methods. The steps include preparing a specific technique and publishing the data analysis results. It is an homage and leads to a conclusion. Records of batch processing and packaging, process control charts, log books of maintenance, records of staff changes, studies of process capacity, completed product data, including trend cards and storage stability findings, should all be used as data sources for this validation. The batches chosen for retrospective validation should be sufficiently numerous to show process consistency and should be representative of all batches produced during the review period, including any batches that did not satisfy the standards.

- Re-validation:

It offers proof that process characteristics and product quality are not adversely impacted by implemented process and/or process environment enhancements. The documentation requirements would be the same as those for the initial validation of the method. Services, systems, equipment, and procedures, including cleaning, should be routinely examined to ensure that they continue to be correct. A check with evidence that services, structures, equipment, and procedures satisfy the requirements satisfies the need for revalidation if no significant modifications have been made to the validated state. In some circumstances, revalidation is required.

Properties like density, viscosity, particle size distribution, and moisture are a few of the modifications that need to be validated. 
i. The active raw material manufacturer's source is altered.
ii. The packing material is altered. 
iii. The facility or plant is altered. 
iv. It modifies the procedure (e.g., batch size, drying temperatures, and mixing time). 
v. It modifies the apparatus (such as adding an automated detecting system).

Re-validation is often not necessary for equipment changes that entail "like for like" replacements, with the exception that the new equipment has to be certified. 
vi. The decision to forego revalidation studies has to be thoroughly explained and recorded [28].

Conclusion:

HPLC and UV-visible spectroscopy techniques for concurrent drug estimation. How validation techniques affect each drug study's analytical procedure. This page contains a large number of validation parameters for the appropriate validation and estimation of analytical procedures. If we are using any instrument for analysis, all of the instrumentation information is crucial. For a better outcome, the validation must be adequate at the conclusion of any computation.

References:

1. Knox J. H., Done J. N. High-Performance Liquid Chromatography. International Journal of Pharmaceutical Compounding. 2004; 8(3):223-225.

2. Simpson C.F. Practical High-Performance Liquid Chromatography. International Journal of Pharmaceutical Compounding. 2004;8(3): 116-118.

3. Rudy B., Magali A., Herida S., Recent applications of analytical techniques for quantitative pharmaceutical analysis,A review. WSEAS transaction on biology and biomedicine.2010; 7(4): 316-338.

4. Alain B. Lain B., Tatyana M., Boris S., Oleg S., Counter current chromatography in analytical chemistry. International Union on Pure and Applied Chemistry. 2009; 81(2): 355-387.

5. Istvan B. Modern analytical techniques in the pharmaceutical and bioanalysis. Budapest, 2011; 1-60.

6. Malviya R., Bansal V., Omprakash P., Sharma P., High performance liquid chromatography: A short review. Journal of Global Pharma Tehnology, 2010; 2(5): 22-26.

7. Guido F. P., Samuel M. Pro., J. Brent F. Countercurrent Separation of Natural Products. Journal of Natural Product. 2008; 71(8): 1489-1508.

8. Sathish K. K,Sujana. K; Development and validation Of UV Spectroscopic Method for determination of Atazanavir Sulphate in Bulk and Formulation; International Journal of Pharmacy and Pharmaceutical Sciences; 2012, 4(3): 109-112

9. Khanage S.G., Deshmukh V.K., Mohite P.B., Dhamak V.M., Appala Raju; Development of Derivative Spectrophotometric Estimation of Atazanavir sulfate in Bulk Drug and Pharmaceutical Dosage Forms; International Journal of Pharm & Health Science, Vol. 1 (3), 2010, 149-154

10. Kitson F. G., Larsen B. S., McEwen C. N. Gas Chromatography and Mass Spectroscopy-A Practical Guide. Academic Press, London.6th Edn , 1996; 522-536.

11. Scott P.W. Gas Chromatography. Library Science, UK. 2003; 245-250.

12. Kennedy J. H. Analytical Chemistry Principles. Harcourt Brace Jovanovic, London.6th Edn, 1984; 540-549.

13. Lena O., AntonyJ.S. Hand book of Pharmaceutical Analysis. Marel DekkerInc. New York, Basel. 2002; 462.

14. Christian G. D. Analytical Chemistry. John Wiley and Sons, New York, 6th Edn. 2003; 471.

15. Chatwal G.R., ShamK. A. Instrumental methods of chemical Analysis, Himalaya Publishing house, 5th Edn. 2002; 107-116.

16. Becket A. H., Stenlake J. B. Practical pharmaceutical chemistry, CBS Publication, NewDelhi. 4thEdn., 2001;284-288.

17. Skoog D. A., Holler F. J., Crouch S. R. Principle of Instrumental Analysis, Thomson Publications, USA, 6th Edn., 2007; 1-3.

18. David H. Modern analytical chemistry, McGraw Hill Higher Education, 3rd Edn ., 2000; 38-84.

19. Dey S., Sureshbabu N.,Murthy P.N.,Panda S.K., Development And Validation of A Stability Indicating Rp-Hplc Method For Estimation of Atazanavir Sulphate in Bulk. Journal of Pharmaceutical Analysis. 2012; 7 (3): 149-154.

20. Ahuja S., Rasmussen H. Overview of HPLC Method Development for Pharmaceuticals, Elsevier Academic Press. 2007; 45-83.

21. Lloyd R.S., Joseph L., Kirklan J., Rihard W.A., Practical HPLC method development, Elsevier Analytica Chemia Acta, Wiley, New York.1991; 277-288.

22. Maccrehan W. A., Differential pulse detection in liquid chromatography and its application to the measurement of organometal cations Analytical Chemistry, 1981, 5(1); 74-77.

23. Kissinger P.T., Szulc M.E., Fischer D.H., KrullI.S., Marcel D. In Electrochemical Detection in Liquid Chromatography and Capillary Electrophoresis. Chromatography Science Series. New York. 1997; 137.

24. Kissinger P. T., Heineman W. R., Laboratory Techniques in Electroanalytical Chemistry, Marcel Dekker, NewYork, 2ndEdn., 1984; 51-125.

25. Snyder L R., Joseph J., Kirikl and J. Practical HPLC method development. John Wiley and Sons, 2nd Edition. 1988; 740-746.

26. Jeevana Jyothi B., Uv Spectrophotometric Method For Estimation of New Drug,

Daclatasvir Dihydrochloride. International Research Journal of Pharmacy, 2016, 7(9):

1-3.

27. Shabir A.M., Moloy M., Bhasin P.S., HPLC Method Development and Validation: A Review. International Research Journal of Pharmacy, 2015, 4(4): 39-46

28. Indian Pharmcopoeia 2014, Volume I., Government of India Ministry Of Health & Family Welfare., 7th Edn, January 2014: 770