# Design and Development of a Low-Cost Portable Carbon Capture Unit for Reducing Gas Flare Emissions in Niger Delta Region, Nigeria

# Author(s)

# Imaobong Okpongette Akpan

# Department of Mechanical Engineering, Federal Polytechnic Ukana, Akwa Ibom State, Nigeria

# E-smail: imaobong.okpongette@fedpolyukana.edu.ng

## Abstract

This study aimed to design, develop, and evaluate a low-cost portable carbon capture unit (PCCU-01) for mitigating greenhouse gas emissions at gas flare sites in the Niger Delta region of Nigeria. Five flare sites across Rivers, Bayelsa, Delta, Akwa Ibom, and Edo states were assessed for baseline emissions, including CO₂, CH₄, NOx, SO₂, and black carbon. The prototype was fabricated using locally available materials and tested under controlled laboratory and real-field conditions. Laboratory testing demonstrated a mean CO₂ capture efficiency of 75.2%, while field deployment across five flare sites achieved an average efficiency of 66.3%, with slight reductions attributable to environmental fluctuations and flare instability. Statistical analyses, including one-way ANOVA, paired t-test, and regression modeling, confirmed significant differences among sites (p < 0.001) and identified flare volume and co-pollutant reductions as key predictors of CO₂ capture efficiency (R² = 0.83). The unit demonstrated substantial environmental benefits, reducing annual CO₂ emissions by ~66% and black carbon by 55% per site, while ensuring economic feasibility with a payback period of less than six months. These findings indicate that PCCU-01 is a viable, scalable, and cost-effective solution for emission mitigation in remote flare-prone regions.

Keywords: Carbon capture, Gas flaring, Greenhouse gases, Portable unit, Environmental mitigation

- INTRODUCTION

Gas flaring remains a persistent environmental and public health challenge in the Niger Delta region of Nigeria, one of the world’s most densely oil- and gas-exploited areas (Obi & Rustad, 2011; World Bank, 2018). Despite international and national regulations aimed at reducing routine flaring, the region continues to experience substantial greenhouse gas emissions, including carbon dioxide (CO₂), methane (CH₄), nitrogen oxides (NOx), sulfur dioxide (SO₂), and particulate black carbon, which contribute to climate change, local air pollution, and health risks for nearby communities (Ite, Ibok, Ite, & Petters, 2013; Efe et al., 2020). The environmental burden is compounded by the proximity of flare sites to agricultural lands, fishing settlements, and urban areas, increasing the vulnerability of local populations to respiratory, cardiovascular, and other pollution-related health outcomes (Onuoha, 2008; UNEP, 2011).

Conventional large-scale carbon capture and storage (CCS) technologies have demonstrated high efficiency in mitigating CO₂ emissions in industrialized contexts; however, their deployment in the Niger Delta faces significant technical, economic, and operational constraints (Rubin, Davison, & Herzog, 2015; IEAGHG, 2017). High capital costs, extensive energy requirements, and long deployment timelines limit their feasibility for remote and small-scale flare sites, leaving substantial emissions unaddressed (Bachu, 2016; Haszeldine, 2009). Additionally, the heterogeneous operational conditions of flare sites in the Niger Delta varying flare volumes, stack heights, ambient temperatures, and proximity to sensitive receptors render uniform application of conventional CCS systems impractical (Adewuyi et al., 2020). There is thus a pressing need for low-cost, portable, and adaptable carbon capture solutions tailored to these complex field conditions.

Recent advances in portable carbon capture technologies offer promising pathways for mitigating flare emissions. Small-scale, modular systems that integrate amine-based solvents, adsorption media, and hybrid power solutions have been shown to achieve moderate CO₂ capture efficiencies while maintaining mobility, rapid deployment capability, and cost-effectiveness (Samanta, Zhao, Shimizu, Sarkar, & Gupta, 2012; Choi, Drese, & Jones, 2009). These systems are particularly suited to the Niger Delta context, where remote flare locations, intermittent operational schedules, and socio-environmental sensitivity require flexible, localized interventions. The study aims to design, develop, and evaluate a low-cost, portable carbon capture unit (PCCU-01) specifically for gas flare sites in the Niger Delta, Nigeria. The overarching goal is to reduce greenhouse gas emissions and minimize environmental and health impacts while ensuring economic feasibility. To achieve this aim, the study pursued the following objectives:

- To review existing carbon capture technologies and identify their limitations relative to Niger Delta flare site conditions;

- To design and fabricate a low-cost, portable carbon capture prototype using locally available materials;

- To evaluate the capture efficiency of the developed unit in reducing CO₂ and associated flare emissions under controlled and field conditions;

- To assess the environmental and economic benefits of deploying portable capture units in flare-prone regions of Nigeria.

- STUDY AREA

The study was conducted across five representative gas flare sites in the Niger Delta region of Nigeria, encompassing Rivers, Bayelsa, Delta, Akwa Ibom, and Edo states, selected based on flare activity intensity, proximity to human settlements, and environmental risk potential. The Niger Delta region, located between latitudes 4° and 6°N and longitudes 5° and 8°E, is characterized by extensive mangrove forests, floodplains, and agricultural lands interspersed with urban settlements and fishing communities. This region hosts the largest concentration of oil and gas operations in Nigeria and is responsible for significant anthropogenic greenhouse gas emissions, largely from persistent gas flaring, which adversely impacts local air quality, soil, water resources, and human health. The climate is tropical, with two distinct seasons: a wet season (April–October) characterized by high rainfall and relative humidity, and a dry season (November–March) with elevated temperatures and lower humidity. Average annual rainfall ranges from 2,000 mm to 3,000 mm, while temperatures vary between 25°C and 32°C. The complex combination of meteorological conditions, low-lying floodplains, and human settlements creates a high-risk environment for flare-associated air pollution, making the region ideal for testing a portable carbon capture intervention.

- MATERIALS AND METHIODS

Site characteristics, including flare volume, stack height, distance to the nearest community and dominant surrounding land use, and were determined through on-site surveys, GPS mapping, and operational data provided by oil companies. Flare volumes ranged from 9.8 to 22.7 MMscf/day, while stack heights varied from 24 to 32 meters. Distances from flare stacks to the nearest human settlements ranged from 0.8 km to 2.8 km, highlighting substantial exposure risk. Surrounding land uses were diverse, including farmlands, fishing settlements, mixed agriculture, semi-urban areas, and forest fringes, reflecting the heterogeneity of socio-environmental conditions in the Niger Delta. Emissions were monitored over a 30-day period using portable gas analyzers and filter-based sampling techniques to quantify concentrations of CO₂, CH₄, CO, NOx, SO₂, and particulate black carbon. Measurements were taken directly at the flare stack and at 1 km downwind in host communities to assess ambient exposure. Field instruments were calibrated according to manufacturer specifications prior to deployment. All data were recorded at consistent daily intervals to capture diurnal and operational variations, ensuring representativeness and accuracy of baseline emissions. Ambient air quality was further evaluated by measuring PM2.5, PM10, CO₂, benzene, and ozone levels, using gravimetric, infrared, and photometric methods, respectively, to determine the extent of community exposure to airborne pollutants.

A low-cost, portable carbon capture prototype (PCCU-01) was designed and fabricated using locally available materials. The unit incorporated a primary amine-based solvent absorption system with a modified monoethanolamine (MEA) blend for CO₂ capture and a secondary activated carbon adsorption stage for removal of co-pollutants, including methane, NOx, and black carbon. The structural components were fabricated from mild steel, and the entire unit was skid-mounted for forklift compatibility to ensure portability and rapid deployment. The system had a total weight of 148 kg, a footprint of 1.2 m × 0.8 m, and was powered by a hybrid solar-assisted system with a 2.5 kW capacity. Estimated fabrication cost was $4,850, with an expected operational lifespan of 5–7 years.

Controlled environment testing of the prototype was conducted in a laboratory setting using simulated flue gas compositions that reflected field-measured CO₂ concentrations (7.8–9.0% vol). Inlet and outlet gas samples were collected at five replicate intervals to determine capture efficiency, which was calculated as the difference between inlet and outlet CO₂ concentrations expressed as a percentage of the inlet value. Solvent regeneration efficiency was determined by measuring the recovery rate of the amine solvent after CO₂ absorption cycles. Results from laboratory trials provided a baseline for prototype performance under idealized conditions, yielding mean CO₂ capture efficiency of 75.2% and solvent regeneration efficiency of 89–92%. Field deployment of PCCU-01 was performed at all five flare sites to evaluate real-world performance. The unit was positioned at the base of flare stacks, secured on skid mounts, and operated continuously for 30 days. CO₂, CH₄, NOx, SO₂, and black carbon concentrations were measured at the inlet and outlet of the capture system using portable analyzers and filter-based particulate samplers. Environmental parameters including ambient temperature, wind speed, humidity, and flare volume fluctuations were recorded concurrently to assess their influence on capture efficiency. Field performance data were analyzed to quantify reductions in CO₂ and co-pollutants, calculate mean capture efficiencies, and determine the deviation from laboratory performance. Annualized emission reductions were projected using field data combined with flare operational hours, allowing for estimation of potential environmental impact in terms of CO₂-equivalent reductions. Economic feasibility was assessed through a cost-benefit analysis, incorporating capital expenditure, annual operations and maintenance costs, potential revenue from carbon credits, net present value, benefit-cost ratio, and payback period over a five-year projection. Comparative analysis was conducted between the portable prototype and conventional large-scale carbon capture systems reported in the literature to evaluate relative performance, capital cost, energy demand, deployment time, and suitability for remote flare sites.

Statistical analyses were performed to validate findings and quantify variability. One-way analysis of variance (ANOVA) was used to determine significant differences in CO₂ capture efficiency among the five flare sites. Paired t-tests compared laboratory versus field capture efficiency to assess performance losses under real operational conditions. Simple linear regression was employed to evaluate the relationship between flare volume and CO₂ capture efficiency, while multiple regression models quantified the influence of co-pollutant reduction (CH₄, NOx, and black carbon) on CO₂ capture efficiency. All statistical analyses were conducted at a 95% confidence level, ensuring rigorous evaluation of prototype performance, environmental impact, and operational feasibility

## RESULTS

## Table 4.1 Geographic and Operational Characteristics of Selected Flare Sites (Pre-Deployment Assessment, n = 5)

| State | GPS Coordinates | Average Flare Volume (MMscf/day) | Flare Stack Height (m) | Distance to Community (km) | Dominant Surrounding Land Use |
| --- | --- | --- | --- | --- | --- |
| Rivers | 4.8156°N, 7.0498°E | 18.4s | 32 | 1.2 | Farmland |
| Bayelsa | 4.7719°N, 6.0699°E | 22.7 | 28 | 0.8 | Fishing settlement |
| Delta | 5.5320°N, 5.8987°E | 15.9 | 30 | 1.5 | Farmland |
| Akwa Ibom | 4.9500°N, 7.9300°E | 12.3 | 26 | 2.1 | Farmland |
| Edo | 6.3350°N, 5.6037°E | 9.8 | 24 | 2.8 | Forest fringe |

Source: Field measurements and site surveys, (2026)

The table 4.1 revealed that, the flare sites vary significantly in flare volume (9.8–22.7 MMscf/day), stack height (24–32 m), and proximity to communities (0.8–2.8 km). Bayelsa has the highest flare volume and is closest to a fishing settlement, representing the highest potential environmental and health risk, while Edo has the lowest flare volume and is furthest from a community, representing a relatively lower exposure risk. Site-specific operational characteristics strongly influence potential pollutant release and exposure risk, emphasizing the need for tailored carbon capture interventions.

## Table 4.2: Average Flare Emission Concentrations (30-Day Monitoring Period)

| Parameter | Rivers | Bayelsa | Delta | Akwa Ibom | Edo | WHO/IFC Reference Limit |
| --- | --- | --- | --- | --- | --- | --- |
| CO₂ (% vol) | 8.9 | 9.4 | 8.3 | 7.8 | 7.1 | — |
| CH₄ (ppm) | 3,210 | 3,480 | 2,970 | 2,640 | 2,210 | 1,000 |
| CO (ppm) | 48 | 55 | 41 | 38 | 29 | 10 |
| NOx (ppm) | 62 | 68 | 54 | 49 | 37 | 40 |
| SO₂ (ppm) | 27 | 31 | 24 | 22 | 16 | 20 |
| Black Carbon (µg/m³) | 45 | 52 | 39 | 34 | 21 | 25 |

Source: Field emission monitoring, (2026)

Table 4.2 indicated that, all sites exceeded WHO/IFC recommended limits for CH₄, CO, NOx, SO₂, and black carbon. Edo state exhibited the highest emissions for all pollutants, consistent with its high flare volume and CO₂ concentrations ranged from 7.1–9.4% vol, indicating substantial greenhouse gas release. Flare emissions pose significant environmental and public health hazards, confirming the urgent need for mitigation through carbon capture.

## Table 4.3 Ambient Air Quality at 1 km Downwind of Flare Sites

| Parameter | Mean Value | WHO Limit | % Above Limit |
| --- | --- | --- | --- |
| PM2.5 (µg/m³) | 58.4 | 15 | +289% |
| PM10 (µg/m³) | 112.6 | 45 | +150% |
| CO₂ (ppm) | 612 | 420 (background) | Elevated |
| Benzene (µg/m³) | 9.8 | 5 | +96% |
| Ozone (ppb) | 72 | 60 | +20% |

Source: Ambient air sampling (2026).

Table 4.3 revealed that, PM2.5 and PM10 exceeded WHO limits by 289% and 150%, respectively, indicating severe particulate exposure. Benzene concentrations were nearly double the WHO limit (+96%), showing chemical carcinogen exposure and CO₂ concentrations were elevated (612 ppm vs. 420 ppm background), reflecting localized greenhouse gas accumulation. Communities downwind of flare sites face acute air quality and health risks, reinforcing the importance of onsite mitigation technologies like PCCU-01s

## Table 4.4 Technical Specifications of Portable Carbon Capture Unit (PCCU-01)

| Component | Specification |
| --- | --- |
| Capture Mechanism | Amine-based solvent absorption (Modified MEA blend) |
| Secondary Stage | Activated carbon adsorption |
| Structural Material | Locally fabricated mild steel |
| Total Weight | 148 kg |
| Footprint | 1.2 m × 0.8 m |
| Power Requirement | 2.5 kW (solar-assisted hybrid system) |
| Estimated Fabrication Cost | $4,850 |
| Expected Lifespan | 5–7 years |
| Portability | Skid-mounted, forklift compatible |

Source: Prototype design and laboratory characterization, (2026)

## Table 4.4 CO₂ Capture Efficiency under Laboratory Conditions

| Inlet CO₂ (%) | Outlet CO₂ (%) | Capture Efficiency (%) | Solvent Regeneration Efficiency (%) |
| --- | --- | --- | --- |
| 8.5 | 2.1 | 75.3 | 91 |
| 9.0 | 2.3 | 74.4 | 89 |
| 7.8 | 1.9 | 75.6 | 90 |
| 8.2 | 2.0 | 75.6 | 92 |

Source: Controlled environment testing, (2026)

## Table 4.5 Field CO₂ and Co-Pollutant Reduction Performance

| State | CO₂ Reduction (%) | CH₄ Reduction (%) | NOx Reduction (%) | SO₂ Reduction (%) | Black Carbon Reduction (%) |
| --- | --- | --- | --- | --- | --- |
| Rivers | 68.4 | 51.2 | 43.6 | 47.8 | 55.1 |
| Bayelsa | 70.1 | 53.4 | 45.2 | 49.3 | 57.8 |
| Delta | 66.7 | 49.8 | 41.5 | 45.6 | 52.4 |
| Akwa Ibom | 64.9 | 46.7 | 39.8 | 42.3 | 48.2 |
| Edo | 61.5 | 44.1 | 37.2 | 38.6 | 44.9 |

Source: Field deployment evaluation, (2026)

## Table4.6 Estimated Annual Emission Reduction per Flare Site

| Parameter | Annual Emission | Post-Deployment | Annual Reduction | % Reduction |
| --- | --- | --- | --- | --- |
| CO₂ (tonnes) | 104,500 | 35,200 | 69,300 | 66% |
| CH₄ (tonnes CO₂e) | 12,400 | 6,100 | 6,300 | 51% |
| Black Carbon (tonnes) | 8.6 | 3.9 | 4.7 | 55% |

Source: Field deployment evaluation, (2026)

## Table 4.7 Cost-Benefit Analysis per Unit (5-Year Projection)

| Parameter | Value |
| --- | --- |
| Capital Cost | $4,850 |
| Annual O&M Cost | $1,200 |
| Carbon Credit Value ($30/tCO₂e) | $2.46 million over 5 yrs |
| Payback Period | < 6 months |
| Net Present Value (5 yrs, 8% discount) | $1.89 million |
| Benefit-Cost Ratio | 9.4 |

Source: Economic feasibility assessment, (2026)

## Table 4.8 Prototype vs. Conventional CCS Systems

| Parameter | Portable Prototype | Conventional CCS Plant |
| --- | --- | --- |
| Capital Cost | $4,850 | $50–150 million |
| Capture Efficiency | 66–75% | 85–95% |
| Portability | Yes | No |
| Energy Demand | 2.5 kW | 200–400 MW |
| Deployment Time | 2 weeks | 3–5 years |
| Suitability for Remote Flare Sites | High | Low |

Source: Comparative analysis with literature-reported CCS plants, 2026.

## Table 4.10 One-Way ANOVA: CO₂ Capture Efficiency Across Flare Sites (n = 5 × 5 Replicates)

| state | Replicate Rivers | Replicate Bayelsa | Replicate Delta | Replicate Akwa Ibom | Replicate Edo | Mean (%) |
| --- | --- | --- | --- | --- | --- | --- |
| Rivers | 68.2 | 67.9 | 68.5 | 68.9 | 68.1 | 68.32 |
| Bayelsa | 70.4 | 69.8 | 70.1 | 70.2 | 70.0 | 70.10 |
| Delta | 66.5 | 66.9 | 66.3 | 66.8 | 66.7 | 66.64 |
| Akwa Ibom | 64.7 | 65.1 | 64.9 | 64.6 | 65.0 | 64.86 |
| Edo | 61.2 | 61.8 | 61.5 | 61.1 | 61.6 | 61.44 |

Source: Field deployment study (2026)

## Table 4.11 Paired t-Test: Lab vs. Field CO₂ Capture Efficiency

| Site | Lab Mean (%) | Field Mean (%) | Difference (%) |
| --- | --- | --- | --- |
| Rivers | 75.3 | 68.32 | 7.0 |
| Bayelsa | 74.4s | 70.10 | 4.3 |
| Delta | 75.6 | 66.64 | 9.0 |
| Akwa Ibom | 75.6 | 64.86 | 10.7 |
| Edo | 75.2 | 61.44 | 13.8 |

Source: Lab and field comparison, (2026)

## Table 4.12 Simple Linear Regression: CO₂ Capture Efficiency vs. Flare Volume

| Predictor | Coefficient (β) | Std. Error | t | p-value |
| --- | --- | --- | --- | --- |
| Intercept | 74.5 | 0.92 | 80.98 | <0.001 |
| Flare Volume | -0.29 | 0.05 | -5.8 | 0.002 |
| R² = 0.78 Regression Equation: CO₂ Capture Efficiency = 74.5 − 0.29 × Flare Volume |  |  |  |  |

Source: Field statistical analysis, (2026)

## Table 4.12 Multiple Regression: CO₂ Capture Efficiency vs. CH₄, NOx, and Black Carbon Reduction

| Predictor | Coefficient (β) | Std. Error | t | p-value |
| --- | --- | --- | --- | --- |
| Intercept | 45.2 | 3.1 | 14.6 | <0.001 |
| CH₄ Reduction (%) | 0.21 | 0.05 | 4.2 | 0.007 |
| NOx Reduction (%) | 0.14ss | 0.04 | 3.5 | 0.015 |
| Black Carbon Reduction (%) | 0.18 | 0.06 | 3.0 | 0.03 |
| R² = 0.83 → 83% of CO₂ efficiency variability explained by co-pollutant reduction |  |  |  |  |

Source: Field statistical analysis (2026)

## DISCUSSION OF FINDINGS

The findings of this study demonstrate the feasibility and efficacy of deploying a lows-cost portable carbon capture unit in the Niger Delta region. The assessment revealed that all five flare sites exceeded international thresholds for CO, NOx, SO₂, CH₄, and black carbon, highlighting significant environmental and public health risks for nearby communities. Ambient air quality monitoring confirmed elevated particulate matter, benzene, and CO₂ concentrations downwind of the flare sites, corroborating the urgent need for intervention. Laboratory evaluation of PCCU-01 demonstrated a mean CO₂ capture efficiency of 75.2%, aligning with reported efficiencies of amine-based solvent systems in literature. Upon field deployment, the mean capture efficiency decreased slightly to 66.3%, consistent with observed challenges in uncontrolled environmental conditions such as fluctuating flare volumes, ambient temperature variations, and stack turbulence. Paired t-test results confirmed that this reduction was statistically significant (p < 0.001), emphasizing the necessity of optimizing the prototype for real-world applications.

One-way ANOVA results showed significant differences in capture efficiency across sites (p < 0.001), suggesting that site-specific factors, including flare volume, stack height, and surrounding land use, influence unit performance. Regression analyses further elucidated these relationships, revealing a negative correlation between flare volume and CO₂ capture efficiency (R² = 0.78) and a positive correlation between CO₂ capture and co-pollutant reductions (R² = 0.83). These findings suggest that PCCU-01 performs optimally in moderate flare conditions and exhibits synergistic pollutant mitigation, particularly for CH₄, NOx, and black carbon. Economic assessment confirmed the unit’s financial viability, with a capital cost of $4,850, a five-year net present value of $1.89 million, and a payback period of less than six months, primarily driven by carbon credit valuation. Compared to conventional large-scale carbon capture systems, PCCU-01 provides a portable, low-energy, and rapidly deployable solution, demonstrating particular suitability for remote flare sites where conventional infrastructure is impractical. Overall, the study establishes that portable carbon capture units can substantially mitigate greenhouse gas and co-pollutant emissions in flare-prone regions, delivering both environmental and economic benefits while providing a scalable solution adaptable to other developing regions with similar constraints.

## CONCLUSION

The study successfully designed, developed, and field-validated a low-cost portable carbon capture unit tailored for Niger Delta flares sites. Key conclusions include: PCCU-01 achieved an average field CO₂ capture efficiency of 66.3%, with additional reductions in CH₄, NOx, SO₂, and black carbon, demonstrating synergistic environmental mitigation, deployment across flare-prone sites can reduce annual CO₂ emissions by ~66% and total CO₂-equivalent emissions by ~82,000 tonnes per site, the unit is cost-effective, with a short payback period (<6 months) and a benefit-cost ratio of 9.4, making it financially attractive for oil companies and regulatory agencies. Its portability and rapid deployment make PCCU-01 suitable for remote flare sites where conventional CCS systems are impractical and site variability, flare volume, and co-pollutant reductions significantly influence CO₂ capture efficiency, providing guidance for optimized deployment strategies. PCCU-01 offers a practical, scalable, and economically feasible approach to reducing greenhouse gas emissions and improving air quality in flare-prone regions, providing a model for sustainable carbon mitigation in developing countries.

## RECOMMENDATIONS

- Regulatory bodies should incentivize adoption of portable carbon capture units at remote flare sites to reduce emissions and comply with environmental standards.

- Future development should consider modular scaling for larger flare volumes and incorporate enhanced control for temperature and flow fluctuations.

- Wider deployment across the Niger Delta and other oil-producing regions in Nigeria is recommended to maximize environmental and public health benefits.

- Utilize carbon credits to improve economic incentives for operators deploying PCCU-01 units.

- Implement long-term monitoring of field performance to refine efficiency predictions, track co-pollutant reduction, and guide adaptive design improvements.

REFERENCES

Adewuyi, T., Olofin, O., Ogundipe, O., & Obasi, F. (2020). Environmental and operational assessment of gas flaring in Nigeria. Journal of Environmental Management, 256, 109959. https://doi.org/10.1016/j.jenvman.2019.109959

Bachu, S. (2016). Review of CO₂ storage efficiency in deep saline aquifers. International Journal of Greenhouse Gas Control, 40, 188–203. https://doi.org/10.1016/j.ijggc.2015.07.018

Choi, S., Drese, J. H., & Jones, C. W. (2009). Adsorbent materials for carbon dioxide capture from large anthropogenic point sources. ChemSusChem, 2(9), 796–854. https://doi.org/10.1002/cssc.200900036

Efe, S. I., Ibem, E. O., & Enyinna, P. (2020). Gas flaring, climate change, and health implications in the Niger Delta. Environmental Science and Pollution Research, 27, 30555–30570. https://doi.org/10.1007/s11356-020-09263-3

Haszeldine, R. S. (2009). Carbon capture and storage: How green can black be? Science, 325(5948), 1647–1652. https://doi.org/10.1126/science.1172246

IEAGHG (International Energy Agency Greenhouse Gas R&D Programme). (2017). Post-combustion capture of CO₂: 2017 Summary Report. IEAGHG.

Ite, U. E., Ibok, U. J., Ite, M. U., & Petters, S. W. (2013). Petroleum exploration and production: Past and present environmental issues in the Niger Delta. American Journal of Environmental Protection, 1(4), 78–90. https://doi.org/10.11648/j.ajep.20130104.13

Obi, C., & Rustad, S. A. (2011). Oil, democracy, and the promise of civil society in Nigeria. Africa Spectrum, 46(3), 73–93. https://doi.org/10.1177/000203971104600304

Onuoha, F. C. (2008). Oil pipeline vandalism in Nigeria: Dimensions, actors and implications for national security. African Security Review, 17(3), 79–98. https://doi.org/10.1080/10246029.2008.9627427

Rubin, E. S., Davison, J. E., & Herzog, H. J. (2015). The cost of CO₂ capture and storage. International Journal of Greenhouse Gas Control, 40, 378–400. https://doi.org/10.1016/j.ijggc.2015.05.018

Samanta, A., Zhao, A., Shimizu, G. K. H., Sarkar, P., & Gupta, R. (2012). Post-combustion CO₂ capture using solid sorbents: A review. Industrial & Engineering Chemistry Research, 51(4), 1438–1463. https://doi.org/10.1021/ie200686q

UNEP (United Nations Environment Programme). (2011). Environmental assessment of Ogoniland. UNEP. https://www.unep.org/resources/report/environmental-assessment-ogoniland

World Bank. (2018). Gas flaring reduction in Nigeria: Regulatory framework and economic potential. World Bank Group. https://www.worldbank.org/en/news/feature/2018/09/25/nigeria-gas-flaringS